3GPP TSG-RAN WG4 #112bis	R4-2415839

Hefei, China, 14 - 18 October 2024

Source: 	Huawei, HiSilicon

Title:  	Discussions on RRM requirements for Rel-19 ATG

Agenda Item:	6.10.4

Document for:	Discussion

Introduction

In the last meeting, we started the discussion of the Rel-19 ATG CA enhancement. The points and views of companies are detailed in the WF [1]. This paper presents our observations and views on RRM impacts on ATG CA.

RRM core requirement analysis

Compared with the requirements defined in Rel-18 ATG for single-Cell, we provide our analysis on the potential RRM impacts to support CA in case by case manner in the following part.

Co-location signalling

Based on the discussions regarding the co-location of ATG DL Cells, as clarified in the WID, it is established that ATG PCell and SCell will be co-located. Since there will be only one deployment method for ATG Cells—co-located deployment—both the ATG UE and the NW can have predefined configurations specifically for co-located deployment.

Proposal 1: There is no necessity for NW to introduce additional signalling to provide co-location information to the UE.

Fast CA/DC requirement

EMR measurement report introduced in previous release is for fast CA/DC setup upon entering connected mode. NW can utilize the measurement result from IDLE mode, and RAN4 discuss the validity of the results to avoid outdated results. However, for Rel-19 ATG, it is important to note that the co-location of the ATG PCell and SCell facilitates a seamless transition for the ATG UE from idle to active mode with CA. The NW can directly configure both the PCell and SCell to establish CA, a process that is already sufficiently fast and efficient. Therefore, there is no necessity to implement additional fast CA-related requirements.

Proposal 2: There is no necessity to introduce idle/inactive mode CA measurement requirements and NR CA EMR in ATG CA.

MTTD

The definition of MTTD is captured from spec 38.133[2] as follows:

The UE shall be capable of handling at least a relative transmission timing difference between slot timing of all pairs of TAGs in FR1 and/or FR2-1 as shown in Table 7.5.4-1, provided that the UE is: - configured with the pTAG and the sTAG for inter-band NR carrier aggregation in SA or NR-DC mode, or - configured with more than one sTAG for inter-band NR carrier aggregation in EN-DC or NE-DC mode.

Due to the co-location of the ATG PCell and SCell, their physical positions can be considered identical which means the TA will be the same when the UE transmits uplink signals to these two Cells. Consequently, as both the PCell and SCell operate within the same TAG, there is no need to define the MTTD.

Observation 1: There are no multi-TAGs in Rel-19 ATG CA.

Proposal 3: There is no need to define MTTD for Rel-19 ATG CA.

MRTD

The reuse of legacy intra-band MRTD requirements represents a prudent choice for ATG CA. For inter-band CA, UE has separate RF chains. We see no implementation issue to maintain the same MRTD value for inter-band CA, which is 33 us.

Proposal 4: Reused the legacy MRTD requirement for ATG inter-band CA: 33us.

Interruption

For UE supporting CA, it is straightforward that there could be an interruption in one Cell caused by certain procedures from the other Cells. Based on the current interruption requirements in chapter 8.2 of 38.133, it is evident that there is no distinction made between different scenarios or UE types. Following the same logic, we think current interruption requirements can also apply to ATG-supporting CA without the need for any additional specifications. If there are ATG new procedures which are not covered in current interruption requirements, RAN4 could discuss whether to introduce new interruption requirements.

Proposal 5: Existing interruption requirements can also apply to ATG supporting CA. RAN4 could discuss whether to introduce new interruption requirements if there are ATG new procedures which are not covered in current interruption requirements.

SCell Activation and Deactivation Delay

In previous releases, different types of SCell activation enhancement were specified. For instance, direct SCell activation/PUCCH SCell activation/fast SCell activation/multiple SCell activation. Based on WID and RAN4 RF discussion so far, there are no multiple SCells scenarios. Considering the characteristic of ATG scenario, we think at least following SCell activation requirements shall be defined:

- SCell activation

- Direct SCell activation at SCell addition

- Direct SCell activation at Handover

Proposal 6: RAN4 to define SCell activation for single-SCell for at least:

- SCell activation

- Direct SCell activation at SCell addition

- Direct SCell activation at Handover

Measurement GAP

In the last meeting, companies provided three proposals from WF [1] captured as follows:

- Proposal 1: RAN4 to reuse NR carrier aggregation measurement gap as the baseline for ATG DL CA - Proposal 2: The measurement gap applicability rule and scheduling applicability for SA single carrier shall be also applied for NR CA configuration. - Proposal 3: When define the SCC interruption time, only consider aligned frame boundaries scenario, the interruption requirement in single carrier case can be reused for CA configuration.

Reusing the legacy NR CA measurement gap for ATG CA is a straightforward approach, and the scenario of aligned frame boundaries is among the most effective solutions for achieving ATG CA.

Proposal 7: For interruption time of MGL, only consider aligned frame boundaries scenario, the interruption requirement in single carrier case can be reused for CA configuration.

Deactivated SCell measurement

In Rel-18 ATG, RAN4 defined L3 intra-frequency and inter-frequency measurement. For Rel-19 ATG supporting CA, RAN4 shall also consider the case for intra-frequency measurement on deactivated SCell.

Proposal 8: RAN4 to define intra-frequency measurement requirements for deactivated SCell.

Conclusion

Proposal 1: There is no necessity for NW to introduce additional signalling to provide co-location information to the UE.

Proposal 2: There is no necessity to introduce idle/inactive mode CA measurement requirements and NR CA EMR in ATG CA.

Observation 1: There are no multi-TAGs in Rel-19 ATG CA.

Proposal 3: There is no need to define MTTD for Rel-19 ATG CA.

Proposal 4: Reused the legacy MRTD requirement for ATG inter-band CA: 33us.

Proposal 5: Existing interruption requirements can also apply to ATG supporting CA. RAN4 could discuss whether to introduce new interruption requirements if there are ATG new procedures which are not covered in current interruption requirements.

Proposal 6: RAN4 to define SCell activation for single-SCell for at least:

- SCell activation

- Direct SCell activation at SCell addition

- Direct SCell activation at Handover

Proposal 7: For interruption time of MGL, only consider aligned frame boundaries scenario, the interruption requirement in single carrier case can be reused for CA configuration.

Proposal 8: RAN4 to define intra-frequency measurement requirements for deactivated SCell.

References

[1] R4-2413959 WF on RRM requirements for NR_ATG_enh

[2] 3GPP TS 38.133 V18.6.0