3GPP TSG-RAN WG4 Meeting #112bis	  R4-2415844

Hefei, China, October 14th – 18th, 2024

Agenda Item:	6.18.1

Source:	Huawei, HiSilicon

Title:	Discussion on general aspects in AIML mobility

Document for:	Discussion and Decision

- Introduction

According to the revised SID on AIML for mobility in [1], RAN4 will start the study on AI mobility from this meeting until August 2025. In this paper, we discuss the general aspects in AIML mobility.

- General aspects of AI mobility in RAN4

Revised SID on AIML for mobility [1]: Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] General aspects in RAN2 AI mobility #126 [2]: - Conclusion 1: 1st study goal of evaluation is to reduce measurement overhead - Conclusion 2: 2nd study goal of evaluation is to enhance handover performance

For mobility enhancement, AI is introduced to predict the RRM measurement and event prediction. The identified use cases are AI based cell-level/beam-level measurement prediction, AI based HO failure/RLF prediction and AI based measurement event prediction.

As shown above, two goals are defined for verifying the gain introduced by AI in RAN2. One is to reduce the measurement overhead, another is to enhance handover performance.

Revised SID on AIML for mobility [1]: - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

For the identified use cases, the testability, interoperability, and impacts on RRM requirements and performance will be studied in RAN4.

In legacy, the requirement of measurement accuracy is defined in RAN4. For AI based cell-level/beam-level measurement prediction, RAN4 can study the requirement of prediction accuracy. For AI based AI based HO failure/RLF prediction and AI based measurement event prediction, if the eventual prediction is derived from predicted measurement, RAN4 can also study the requirement of prediction accuracy.

In the cases where the measurement can be reduced by AI prediction, there are two options for defining the testing goal.

- Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy margin requirement.

- Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

  - FFS: The value of MRRT.

  - FFS: Other parameter and the values, e.g., the prediction window.

Proposal 1: RAN4 can discuss the following two options:

- Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement.

- Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

  - FFS: The value of MRRT.

  - FFS: Other parameter and the values, e.g., the prediction window.

With option 1, RAN4 can ensure that the accuracy of the predicted measurement will not worse than that of legacy measurement. Therefore, no handover performance degradation will be introduced.

With option 2, whether the predicted measurement will have an impact on handover performance may need to be evaluated. If there is no handover performance degradation or the degradation is smaller enough to accept, then the benefit of AI can still be verified in terms of reducing the effort of measurement.

It is observed that in some cases, for example, in the inter frequency prediction, the measurement reduction may help reduce the measurement gap. If the measurement gap can be reduced, more resources for data transmission can be obtained. In these cases, additional benefit of AI is seen in terms of improving the use experience of services like XR in mobility.

Observation 1: AI based prediction may help reduce the power consuming due to the measurement reduction.

Observation 2: AI based prediction may help reduce the delay of data transmission and improve the throughput due to the reduction of measurement gap.

Proposal 2: When defining measurement prediction requirements, the gain derived from power consumption reduction and measurement gap reduction shall also be taken into account.

- Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: RAN4 can discuss the following two options:

- Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement.

- Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

  - FFS: The value of MRRT.

  - FFS: Other parameter and the values, e.g., the prediction window.

Observation 1: AI based prediction may help reduce the power consuming due to the measurement reduction.

Observation 2: AI based prediction may help reduce the delay of data transmission and improve the throughput due to the reduction of measurement gap.

Proposal 2: When defining measurement prediction requirements, the gain derived from power consumption reduction and measurement gap reduction shall also be taken into account.

References

- R4-2410570, “Revised SID on AIML for mobility in NR”, 3GPP RAN#105, September 9-12, 2024.

- R2-2405941, “Summary of [POST125bis][021][AIML mobility] Simulation assumptions and methodology (OPPO)”, 3GPP RAN2#126, May 20th -24th, 2024.