3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415847

Hefei, China, 14 – 18 October, 2024

Title: 	Discussion on core requirements for SL positioning and CPP

Source: 	Huawei, HiSilicon

Agenda item:	4.2.1.2

Document for:	Discussion

Introduction

Maintenance of RRM core requirements for SL positioning and CPP are discussed in RAN4#112, and the outcomes are captured in [1]. Further discussions are needed for the following issues.

- SL positioning

- CPP

In this paper, we will provide our views on remaining issues in core requirements for SL positioning and CPP.

Discussion

SL positioning

### Issue 1-1-1: End point of SL-PRS based RSTD measurement period requirements Conclusion: - No common understanding on RAN2 procedure for reporting behaviour. Companies check offline. - No consensus at the moment to revisit previous agreement

The following was agreed in RAN4#110-bis.

#### Issue 1-1-1: End point of SL-PRS based RSTD measurement period requirements Agreements: - The SL RSTD measurement period ends after the UE has measured SL PRS resources from at least two different Tx UEs including target and reference UEs. - FFS whether any updates to the TS 38.133 are needed.

In our understanding, however, the TSL RSTD,Total in current requirement is for per TX UE or per resource, and it is same as other SL PRS measurement e.g. SL Rx-Tx that is performed over only one TX UE. Note that this is different from Uu, where TRSTD,Total is the measurement period for all configured resources.

The SL RSTD measurement shall be performed during the measurement period TSL RSTD,Total is defined as: TSL RSTD,Total=s=1STSL RSTD,effect,s , where S is the number of samples per measured link, defined below: S = 1 for SL-PRS bandwidth>48 PRBs, S = 4 for SL-PRS bandwidth≤48 PRBs, For each SL-PRS sample s of the target measured link, which is received within a slot where the UE receives SCI and the associated SL-PRS within its capabilities [Components 2 and 3 of FG 41-1-1], TSL RSTD,effect,s is defined as:

Therefore, we suggest to update the SL RSTD requirements to reflect that measurement period ends after the UE has measured SL PRS resources from at least two different Tx UEs.

In last meeting, there were different views on whether UE can use multiple measurement reports to report different SL RSTD, each report with a different reference TX UE. After checking, we understand this is not supported by the current 38.355, so we can assume a single measurement report, and the total measurement period is based on the individual measurement period for the TX UE which is measured latest.

A TP is provided in our companion draftCR.

Proposal 1: Update the SL RSTD requirements to reflect that measurement period ends after the UE has measured SL PRS resources from at least two different Tx UEs.

CPP

### Issue 2-1-1: Measurement period requirements for DL RSCP/DL RSCPD with aperiodic time window Agreements: - Define the requirement for one-shot window. - The requirement is defined by taking the time window length + processing time.

RAN4 has agreed to define the requirement for one-shot window, and the requirements are to be defined by taking the time window length + processing time. We will discuss the exact requirements next.

In RAN4#111, some companies suggested that UE is assumed to suspend all PRS measurements until it has completed the measurement of PRS resources in the one-shot time window. In our view, this proposal is reasonable as it avoids the impact between periodic and one-shot measurements, and effectively prioritizes the one-shot measurement which is critical for CPP. Otherwise, UE may need to interrupt ongoing periodic measurement to perform the one-shot measurement, or UE may miss the one-shot time window due to ongoing periodic measurement, both of which are not desirable.

Following the proposal, the requirements for CPP measurement should be the sum of the time from the start of measurement period to the time window and the processing time. After UE finishes processing resources in the one-shot time window for CPP measurement, it should start RSTD or Rx-Tx measurement as in legacy way. Note that above only applies when UE supports CPP measurement in the time window.

Proposal 2: When one-shot time window is configured,

- the measurement period for CPP is Tuncertainty+Ti,

- the measurement period for RSTD/Rx-Tx is Tuncertainty+maxTeffective,i+Tlegacy,

- Tuncertainty is the time from the start of the measurement to the start of the one-shot time window,

- Ti is the processing time for PFL i,

- Tlegacy is the legacy requirements for RSTD and Rx-Tx.

Conclusions

In this paper we provided provide our views on remaining issues in core requirements for SL positioning and CPP.

Proposal 1: Update the SL RSTD requirements to reflect that measurement period ends after the UE has measured SL PRS resources from at least two different Tx UEs.

Proposal 2: When one-shot time window is configured,

- the measurement period for CPP is Tuncertainty+Ti,

- the measurement period for RSTD/Rx-Tx is Tuncertainty+maxTeffective,i+Tlegacy,

- Tuncertainty is the time from the start of the measurement to the start of the one-shot time window,

- Ti is the processing time for PFL i,

- Tlegacy is the legacy requirements for RSTD and Rx-Tx.

Reference

- R4-2414033, WF on SL positioning and carrier phase positioning, CATT