3GPP TSG-RAN4 Meeting #112-bis	R4-2415853

# Hefei, China, 14 – 18 October, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on PRS RMC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are some issues with SL PRS RMC: - RAN4 simulation are based on (comb size, symbol size) = (4,2) and (4,4). In particular, (4,2) is for partial staggering which is specific for SL PRS. However, (4,2) is not supported in SL PRS RMC. - It is not clear which of the 15kHz and 30kHz SCS in SL PRS RMC apply for each test configuration of each test case. - The slot offset of the two SL PRS resources should be based on the capability of the UE under test in terms of active slot and active resource, and a fixed slot offset 4 does not serve the test purpose. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update SL PRS RMC: - Change (comb size, symbol size) (4,2) to (4,2). - Clarify that the SCS of SL PRS is same as that for the corresponding NR SL configuration in the test case. - Change slot offset of the second SL PRS resourcesto be based on the capability of the UE under test in terms of active slot and active resource. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SL PRS RMC does not serve the test purpose. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.21A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The draftCR is based on revised R4-2413983 shared on RAN4 reflector before RAN4#112-bis. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### A.3.21A.2	NR SL-PRS configurations

### A.3.21A.2.1	NR SL-PRS configurations for FR1

Table A.3.21A.2.1-1: SL PRS.1 FR1: SL-PRS configuration

| SL PRS Parameters | Values |  |  |  |
| --- | --- | --- | --- | --- |
| Reference channel | SL PRS.1.1 FR1 | SL PRS.1.2 FR1 | SL PRS.1.3 FR1 | SL PRS.1.4 FR1 |
| SCS | 15 kHz, 30 kHz Note 2 |  |  |  |
| SL-PRS comb size | 4 |  | 4 |  |
| Number of SL-PRS symbols | 2 |  | 4 |  |
| SL-PRS comb offset Note 1 | 1 |  | 1 |  |
| SL-PRS resource slot offset (slot) Note 1 | 0 | Note 3 | 0 | Note 3 |
| RB numbers containing SL PRS within channel Bandwidth Note 1 | 24 |  | 48 |  |
| SL-PRS starting PRB | 4 |  | 4 |  |
| NOTE 1: Unless otherwise specified in the test case. NOTE 2: Same as the SCS for the corresponding NR SL configuration in the test case. NOTE 3: Unless otherwise specified in the test case, if the UE under test indicates other values than ‘n1’ for both maxNumOfActiveSL-PRS-ResourcesInOneSlot and maxNumOfSlotsWithActiveSL-PRS-Resources, 1; Otherwise, 160 for SCS of 15kHz and 320 for SCS of 30kHz. |  |  |  |  |

<End of Change 1>