3GPP TSG-RAN WG4 Meeting#112-bis	R4-2415860

Hefei, China, 14 – 18 October, 2024

Title: 	Discussion on performance requirements for CPP

Source: 	Huawei, HiSilicon

Agenda item:	4.2.2.6

Document for:	Discussion

Introduction

Maintenance of RRM performance requirements for CPP are discussed in RAN4#112, and the outcomes are captured in [1]. Based on [1], further discussions are needed for the accuracy requirements.

In this paper, we will provide our views on remaining issues in performance requirements for CPP.

Discussion

### Issue 2-2-2: Additional margins due to frequency drift and RF calibration Agreements: - RAN4 to clarify that the RSCPD accuracy requirement and the relative RSCP accuracy requirement apply given that the carrier phase measurements are performed on PRS resources within a slot/a set of symbols, which can be controlled at least by the window configuration. - Companies can bring proposals on other scenarios in WI performance maintenance phase.

Due to existence of carrier frequency offset, the error in measured RSCPD or relative RSCP will increase along with the time separation between the two resources for calculating RSCPD or relative RSCP. Assuming 4GHz and 0.1 ppm frequency error, the phase error will be 72 deg if the two resources are separated by one slot of 30kHz SCS (0.5ms). This error is much larger than the BB estimation error which is less than 20 deg based on companies’ results and will make the CP measurement useless. To mitigate the impact of frequency error, RAN4 agreed to define accuracy requirements for CPP under the condition where CPP measurements are performed on PRS resources within a slot/a set of symbols.

We do not support to consider other scenarios besides the agreed one. To define a meaningful requirement for other scenarios, UE needs to estimate the carrier frequency offset when UE receives from TRP l as well as the carrier frequency offset when UE receives from another TRP 2. However, we understand UE does not have an accurate estimate of frequency error, as long as it meets the 0.1ppm requirements defined in 38.101.

Proposal 1: RAN4 to only define accuracy for the scenarios where CPP measurements are performed on PRS resources within a slot/a set of symbols (i.e. the agreed scenario).

On top of the applicability condition, some additional margin still needs to be added for the expected RSTD uncertainty. This is because even the two resources are in the same set of symbols after accounting for the expected RSTD, there can still be RTD at UE side due to expected RSTD uncertainty, and this RTD will cause error in RSCPD or relative RSCP if there exists frequency carrier frequency offset.

It is noted that in our submitted simulation results, we did not model the impact of residual frequency error, i.e. the results are based on zero frequency error. We do not see update of the simulation assumption needed or additional simulation needed to derive the additional margin. Instead, the additional margin can be derived based on the expected RSTD uncertainty and the assumed carrier frequency offset.

Based on the maximum RSTD uncertainty of 32us in FR1 and 8us in FR2, and assuming PFL center frequency is 4GHz in FR1 and 40GHz in FR2, the additional margin on the phase accuracy would be 9.2 deg for FR1 and 23 deg for FR2.

Proposal 2: For defining CP measurement accuracy requirements, add additional margin on top of simulation results to account for the impact of carrier frequency offset and RSTD uncertainty.

- 9.2 deg for FR1

- 23 deg for FR2

Conclusions

In this paper we provided our views on remaining issues in performance requirements for CPP.

Proposal 1: RAN4 to only define accuracy for the scenarios where CPP measurements are performed on PRS resources within a slot/a set of symbols (i.e. the agreed scenario).

Proposal 2: For defining CP measurement accuracy requirements, add additional margin on top of simulation results to account for the impact of carrier frequency offset and RSTD uncertainty.

- 9.2 deg for FR1

- 23 deg for FR2

Reference

- R4-2414033, WF on SL positioning and carrier phase positioning, CATT