3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415862

Hefei, China, 14 – 18 October, 2024

Title: 	Discussion on RRM requirements for less than 5MHz Ph2

Source: 	Huawei, HiSilicon

Agenda item:	6.4.3

Document for:	Discussion

Introduction

RRM requirements less than 5MHz in Rel-19 are discussed in RAN4#112, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the following issues.

- SCell related requirements

- PSCell related requirements

- Other requirements

In this paper, we will provide our views on RRM requirements for less than 5MHz in Rel-19.

Discussion

SCell related requirements

Issue 2-1-1: SCell activation and deactivation delay requirements: - Agreement - Specify new SCell activation delay requirements to support less than 5MHz bandwidth in the target SCell in FR1. - Agreement - Start from single SCell activation delay requirements.

RAN4 agreed to define new requirements for SCell with less than 5MHz. In our understanding, the new requirements should account for the additional time needed for SSB index reading during activation.

- For unknow SCell, UE needs to read SSB index during activation, so the delay extension is needed.

- For known SCell, UE may or may not have read SSB index of the SCell because the known condition does not require beam level reporting in FR1.

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown.

In our view, there are two options for known SCell, and we are open to further discuss.

- Option 1: always extend the delay considering the worst case

- Option 2: extend the delay when UE is not configured with beam level reporting, otherwise do not extend the delay

For the exact delay extension, since the side condition is same as HO (i.e. -2dB), the extension in R18 HO requirements (i.e. 2 samples) can be re-used.

Proposal 1: For SCell with less than 5MHz BW,

- SCell activation delay for unknown SCell is extended by [2] SMTC period,

- FFS whether SCell activation delay for known SCell needs to be extended or not.

Issue 2-1-3: Whether intra-band CA is in the scope: - Proposals - Option 1: Consider intra-band CA (both contiguous and non-contiguous) within the scope of the RRM requirements in Rel-19. - Option 2: Wait until RF room has concrete conclusion on intra-band CA combinations.

Based on discussion last meeting, intra-band CA may have impact on the RRM requirements, e.g. the SCell activation delay for the case where the SCell being activated is intra-band contiguous to an active cell. We suggest to wait for RF session conclusion before deciding whether such requirements are considered or not.

Proposal 2: Wait for RF session conclusion before deciding whether to consider intra-band CA in RRM requirements for less than 5MHz in Rel-19.

Issue 2-1-4: IDLE mode CA measurement reporting: - Proposals - Option 1: For 3MHz channel bandwidth, N2=[3+3] if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR1.

This issue is related to EMR (Issue 2-3-3) and not limited to CA. We will discuss it in section 2.3.

PSCell related requirements

Issue 2-2-2: PSCell change, conditional PSCell change delay requirements: - Proposals - Option 1: RAN4 to discuss the impacts of less than 5MHz, in particular PBCH puncturing, to PSCell change delay requirements.

In last meeting there is common understanding that PSCell change requirements would need to be extended due to PBCH puncturing. The only issue is that some of the PSCell change requirements are referring to PSCell addition requirements, thus do not need to be changed.

In our view, this is something related to spec drafting, and does not impact technical agreement. Also, some of the PSCell change requirements are defined without reference.

Proposal 3: Specify new PSCell change delay requirements for PSCell configured with less than 5MHz channel bandwidth.

Issue 2-2-4: SCG activation delay requirements: Agreement: - FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the SCG activation requirement requirement.

We support to define SCG activation requirements for PSCell with 12RB PBCH. On one hand, fast SCG recovery is always desirable for CA/DC; on the other hand, we do not see much efforts for UE to support SCG activation for PSCell with 12RB PBCH, if UE supports both SCG deactivation and less than 5MHz BW for PSCell, and the delay for SCG activation is extended for PBCH puncturing.

On the exact extension, since SCG activation is similar to PSCell addition or PSCell change, the same extension as in R18 HO requirements (i.e. 2 samples) can be re-used.

Proposal 4: RAN4 to define new SCG activation requirements for PSCell with 12RB PBCH, and same delay extension as for PSCell addition or PSCell change is re-used.

Other requirements

Issue 2-3-1: NW indication on PBCH BW in MO configuration and/or HO command: - Proposals - Option 1: RAN4 to discuss whether NW indication on PBCH BW in MO configuration is needed considering a cell with less than 5MHz can be SCell. - Option 2: If SCell can use 12PRB SSB bandwidth in R19, it’s necessary to provide assistance information to UE regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command.

One issue that RAN4 may need to discuss in the PBCH BW indication in MO. The issue was discussed in R18, and it was concluded that UE supporting less than 5MHz can determine the PBCH BW for an SSB configured in MO for measurement by checking whether the SSB frequency is on the new sync raster or not. The assumption in R18 was that PBCH puncturing is only applicable to SSB on the new sync raster because the cell with less than 5MHz can only be PCell when serving a UE.

In R19, the cell with less than 5MHz can be SCell, and it is not clear if the SSB of such a cell can be off the new sync rater. If it is possible, then UE may need additional indication from NW to determine if an SSB frequency in an MO is with punctured PBCH or not. Otherwise, such an indication is not needed and UE can follow R18 approach.

In last meeting, some companies commented that RF session has decided that the SSB of SCell with less than 5MHz should be also on the new sync raster. After checking, we understand the conclusion is only for SCell with 12RB Tx BW configuration in band n100. For other cases e.g. SCell with 15RB in band n100 or other bands, we may still need to wait for RF conclusion.

Proposal 5: RAN4 to discuss whether NW indication on PBCH BW in MO configuration is needed considering a cell with less than 5MHz can be SCell.

Issue 2-3-3: EMR requirements: Further discuss: - Option 2: RAN4 to study impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on EMR measurement with SSB index reading. - Option 3: not define IDLE mode EMR requirement

We support to define EMR requirements for target cell with 12RB PBCH. On one hand, fast setup is always desirable for CA/DC; on the other hand, we do not see much efforts for UE to support EMR measurement on a target cell with 12RB PBCH, if UE supports both EMR and less than 5MHz BW for SCell, and the delay for SSB index reading in EMR is extended.

On the exact delay extension, since the side condition is same as inter-frequency measurement (i.e. -4dB), the extension in R18 inter-frequency requirements (i.e. 3 samples) can be re-used.

Proposal 6: RAN4 to define EMR requirements for target cell with 12RB PBCH. Delay requirements for SSB index reading in EMR is extended by [3] samples.

Conclusions

In this paper we provided provide our views on RRM requirements for less than 5MHz in Rel-19.

Proposal 1: For SCell with less than 5MHz BW,

- SCell activation delay for unknown SCell is extended by [2] SMTC period,

- FFS whether SCell activation delay for known SCell needs to be extended or not.

Proposal 2: Wait for RF session conclusion before deciding whether to consider intra-band CA in RRM requirements for less than 5MHz in Rel-19.

Proposal 3: Specify new PSCell change delay requirements for PSCell configured with less than 5MHz channel bandwidth.

Proposal 4: RAN4 to define new SCG activation requirements for PSCell with 12RB PBCH, and same delay extension as for PSCell addition or PSCell change is re-used.

Proposal 5: RAN4 to discuss whether NW indication on PBCH BW in MO configuration is needed considering a cell with less than 5MHz can be SCell.

Proposal 6: RAN4 to define EMR requirements for target cell with 12RB PBCH. Delay requirements for SSB index reading in EMR is extended by [3] samples.

Reference

- R4-2413883, WF for [112][214] NR_FR1_lessthan_5MHz_BW_Ph2, Intel