3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415868

Hefei, China, 14 – 18 October, 2024

Title: 	Discussion on other RRM requirements for Rel-19 NTN

Source: 	Huawei, HiSilicon

Agenda item:	6.26.4.2

Document for:	Discussion

Introduction

RRM requirements Rel-19 NTN are discussed in RAN4#112, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the RRM requirements for following objectives.

- Downlink coverage enhancements

- Uplink Capacity/Cell Throughput Enhancement

- Support of regenerative payload

In this paper, we will provide our views on other RRM requirements for Rel-19 NTN.

Discussion

Downlink coverage enhancements

Issue 2-1-1: RRM impact of SSB periodicity enhancement <Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, Apple, Xiaomi, Samsung, HW, QC): - RAN4 to wait for more progress from RAN1/2 on SSB periodicity extension and other possible system level enhancements to discuss the RRM impacts of DL coverage enhancement. - Proposal 1a (Samsung): - If SSB periodicity is extended, almost all RRM requirements should be impacted including idle mode/inactive mode and connected mode such as: cell (re)-selection, L3 measurement/L1 measurement and so on. Need further RAN1 conclusion on whether SSB periodicity is changed. - Proposal 1b (Ericsson): RAN4 to check if RRM requirements could be impacted by scaled measurement time delay due to extension of SSB/SMTC periodicity. - RAN4 to check if RRM requirements could be impacted by SSB periodicity extension to a value larger than 20ms during cell search, including the remaining physical channels and signals (e.g., SIB, paging, etc.). #### Issue 2-2-1: RRM impact of DTX/DRX <Way Forward> : Further discuss the following proposal - Proposal 1 (Apple): - RAN4 to wait more conclusions from RAN1 to decide whether and how NTN RRM can be impacted by DTX/DRX from NES. - Proposal 2 (Samsung): - All the measurements related to DL-RS should be impacts in “off” state. RAN4 to discuss and define new RRM requirements to enhance DL coverage. Need further progress from other WGs. - Proposal 3 (Ericsson): - RAN4 to check if RRM requirements could be impacted by scaled measurement time delay due to extension of DRX periodicity. #### Issue 2-3-1: RRM impact of dynamic and flexible power sharing <Way Forward> : Further discuss the following proposal - Proposal 1 (LG): - After RAN1 concludes the solution for dynamic and flexible power sharing among satellite beams or different satellite beam patterns/size, RAN4 should check whether the outcome from RAN1 is affect RRM requirements. - Proposal 2 (Xiaomi): - RAN4 to study the RRM impact of satellite beams activation/deactivation scheme after RAN1/2 reaching more concrete conclusions. #### Issue 2-4-1: Beam switching related issues <Way Forward> : Further discuss the following proposal - Proposal 1 (Ericsson): RAN4 to check if beam steering latency other than 0 may impact RRM requirements. - Proposal 2 (Ericsson): RAN4 to check if timing offset/error (between beam switching timing and DL/UL framework) may impact RRM requirements. - Proposal 3 (Ericsson): If applicable, the requirements for quasi-earth_fixed cell and earth_ moving cell both shall be studied with respect to the beam hopping scheme.

In RAN1#118 the following is agreed related to DL coverage enhancement.

Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

In RAN2#127 the following is agreed related to DL coverage enhancement.

- From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects) - From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms. - RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity - If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

It can be seen that extension of SSB periodicity for initial access will be considered by RAN1, and RAN1 will further discuss the default value(s) among the candidate values ranging from 40ms to 640ms. RAN2 also discussed potential impacts and enhancements.

From RAN4 perspective, we understand the existing RRM requirements can be re-used as baseline with SSB periodicity extension, for the following reasons.

- There is no requirement on cell selection (initial access) in RAN4.

- Existing RRM requirements are based on configured SSB or SMTC period which can be up to 160ms. If SSB periodicity is extended beyond 160ms, RAN2 will extend the signaling range, and we also do not see direct impact from it, as we already have DRX requirements where measurement interval will follow DRX cycle which can be much larger than 160ms.

On the other hand, if SSB periodicity is extended beyond 160ms there may be other RRM impacts. For example, the timing accuracy requirements (Te) are defined based on the assumption that SSB is available at least every 160ms, and RAN4 needs to discuss whether the current requirements apply with 320ms.

We are also open to discuss potential impacts of beam hopping which is used together with SSB periodicity extension to improve DL coverage, e.g. DTX aspects or SMTC impacts as discussed in RAN2. Since this is pending on conclusion from other WGs, RAN4 can further wait before discussing the exact RRM impact.

Proposal 1: Existing RRM requirements are re-used as baseline for extended SSB periodicity

- FFS other RRM impacts if extended SSB periodicity is larger than 160ms, e.g. on UL timing accuracy (Te).

- Updates can be considered based on conclusion from other WGs, e.g. on DTX aspects or SMTC impacts due to beam-hopping / larger SSB periodicity.

Uplink Capacity/Cell Throughput Enhancement

Issue 3-1-1: RRM impact for uplink Capacity/Cell Throughput Enhancement < Way Forward > - No need for RAN4 to discuss RRM requirement for uplink Capacity/Cell Throughput Enhancement.

We understand the objective is to support Uplink Capacity/Cell Throughput Enhancement by using of OCC in the transmissions from multiple UEs on the same time frequency resource. We do not see it impacts any existing RRM requirement or requires new RRM requirement.

Proposal 2: RAN4 to confirm no RRM impact for uplink Capacity/Cell Throughput Enhancement.

Support of regenerative payload

In RAN2#127 the following is agreed related to regenerative payload.

- RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario. - Send an LS to RAN1 and RAN4 asking whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for ta-Common or whether we should e.g. introduce negative values for ta-Common. Also indicate that in any case legacy UEs would have to rely on existing signalling and then on a minimum value equal to 0. - RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed. - We do not optimize UE power saving for SDT procedure in regenerative payload architecture

An LS [2] is sent to RAN4 asking about the need of negative value in common TA.

In RAN2#127 meeting, the setting of common TA and Kmac for regenerative payload with full gNB on board was discussed, and a question related to the value setting was raised. Related to this, RAN2 kindly asks RAN1 and RAN4: Question: Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common. Additionally, RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0.

In our view, negative value in common TA is needed for regenerative payload at least for PRACH.

- In TN, the PRACH will arrive at gNB later than the symbol boundary w.r.t. gNB UL timing due to propagation delay between UE and gNB. As a result, gNB does not need to try to detect PRACH earlier than gNB UL timing.

- In NTN, UE will compensate the propagation delay between UE and gNB when transmitting PRACH, and due to the error in the compensation, e.g. GNSS error, PRACH may arrive at gNB earlier than the symbol boundary w.r.t. gNB UL timing.

- This is not an issue for transparent payload because gNB can use a smaller common TA than what is actually needed, but for regenerative payload the actually needed common TA is zero, so the issue cannot be resolved by gNB implementation.

The other way to resolve the issue is that gNB tries to detect PRACH earlier than gNB UL timing. However, this would require large change in gNB implementation, and the performance in multi-path channel will be degraded since part of CP is used for detection. Besides, with negative timing offset not corrected, there will be performance degradation for the subsequent PUCCH/PUSCH, e.g. Msg3 and Msg5.

In our view, introducing negative values in common TA is the most reasonable way to resolve this issue. It will simplify the gNB implementation and avoid possible performance degradation in PRACH detection and PUCCH/PUSCH demodulation due to negative timing offset. The impact at UE side is marginal, i.e. UE just follows the configured value in common TA, which is same as existing implementation.

Proposal 3: RAN4 to confirm to RAN2 that negative values in common TA are needed.

Issue 5-2-1: Timing requirements for regenerative payload <Way Forward> - The common understanding is that NTA,adjcommon is configured by network. - Further discuss the impact on timing requirements in TS 38.133 for regenerative mode of NTN.

Regardless of negative values for common TA is introduced or not, we do not see any impact on timing requirements due to regenerative payload. In the existing timing requirements, common TA is used to define the reference timing, and the requirements are generic enough, i.e. no matter the common TA is fixed as zero or configured as some value, there is no issue to apply the existing requirements.

Proposal 4: RAN4 to confirm no impact on timing requirements due to regenerative payload.

Issue 5-3-1: RRM requirements for RRC_INACTIVE state mobility with regenerative payload <Way Forward> - Whether to update the specific RRM requirements for inactive state with regenerative payload should wait for RAN2/RAN3 progress. Issue 5-3-2: RRM requirements for RRC Connection Mobility Control with regenerative payload <Way Forward> : Further discuss the following proposal - Whether to update the specific RRM requirements for RRC re-establishment with regenerative payload should wait for RAN2/RAN3 progress. - Whether to update the specific RRM requirements for satellite switch and RACH-less handover with regenerative payload should wait for RAN2 progress. - FFS: RAN4 to clarify requirement of satellite switching without PCI change is not applied for regenerative mode of NTN.

The two issues 5-3-1 and 5-3-2 arise from RAN2#127 conclusion.

On mobility in RRC_INACTIVE and RRC re-establishment, there is no further conclusion from RAN2#128. On satellite switch with re-sync, there is no conclusion from RAN#105 whether it will be supported or not. We do not see the need for RAN4 to discuss requirements for them in this meeting.

On RACH-less HO, RAN2#128 agreed that it is supported without any enhancement. Existing requirements for RACH-less HO can thus be re-used. This is aligned with agreement from last meeting that the existing requirements for NR NTN can be a baseline, so we do not see the need for further discussion in RAN4.

#### Issue 5-4-1: RRM requirements for network verified UE positioning <Way Forward> - Proposal 1 (CATT): - RAN4 should define the RRM requirements for network verified UE positioning for regenerative architecture. #### Issue 5-4-2: Multiple SMTCs mechanism <Way Forward> - Proposal 1 (ZTE): - RAN4 shall clarify whether the legacy multiple SMTCs mechanism can be used for regenerative payload or not.

RAN4 has agreed last meeting that the existing requirements for NR NTN can be a baseline for regenerative payload. In our view, this means the existing requirements for NW verified location and related to multiple SMTC are also applicable for regenerative payload. We do not see the need for further discussion in RAN4.

Conclusions

In this paper we provided provide our views on other RRM requirements for Rel-19 NTN.

Proposal 1: Existing RRM requirements are re-used as baseline for extended SSB periodicity

- FFS other RRM impacts if extended SSB periodicity is larger than 160ms, e.g. on UL timing accuracy (Te).

- Updates can be considered based on conclusion from other WGs, e.g. on DTX aspects or SMTC impacts due to beam-hopping / larger SSB periodicity.

Proposal 2: RAN4 to confirm no RRM impact for uplink Capacity/Cell Throughput Enhancement.

Proposal 3: RAN4 to confirm to RAN2 that negative values in common TA are needed.

Proposal 4: RAN4 to confirm no impact on timing requirements due to regenerative payload.

Reference

- R4-2414037, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT

- R2-2407623, LS on common TA in a regenerative payload scenario, RAN2