3GPP TSG-RAN WG4 Meeting #112bis	R4-2415872

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	China Telecom

Title:	Discussion on PC2 RedCap for TDD bands

Agenda Item:	6.7.1

Document for:	Approval

- Introduction

In RAN4#112, the WF on RedCap PC2 requirements was approved [1] with the following agreements captured:

Issue 1-1-1: PC2 RedCap UE for TDD bands

Agreement: Add one sentence in the clause of 6.2I.1 to state that PC2 also applies for RedCap/eRedCap UE in a band agnostic way for TDD bands

This contribution discusses how to introduce PC2 requirements for RedCap on TDD bands as highlighted in the WF.

- Discussion

How to introduce PC2 requirements for RedCap on TDD bands in the band agnostic way

In last RAN4 meeting, companies proposed their changes to the specification for introducing RedCap PC2 requirements. In contribution [2], it is proposed to add one sub-clause for 6.1I to generally describe the RedCap UE requirements. In contribution [3-8], companies propose their changes/views in 6.2I.1 to specify maximum output power for PC2 RedCap. Although, the words are slightly different, the intended changes of meaning are quite similar. Thus, it was recommended in topic summary to consider the following changes as starting point for introducing RedCap PC2 requirements for TDD bands. In this contribution, we repropose to make the following changes to enable PC2 RedCap feature in 38.101-1.

### 6.2I Transmitter power for RedCap For Redcap and eRedCap UE the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I. ### 6.2I.1 Maximum output power for RedCap For Redcap and eRedCap UE, the requirements for power class 3 specified in clause 6.2.1 apply. For RedCap and eRedCap UE, the requirements for power class 2 specified in clause 6.2.1 [for TDD bands] apply.

Table 1 Changes to 38.101-1

- Conclusions

In this contribution, we discuss enabling PC2 RedCap for TDD bands and have the following proposals:

Proposal 1:. Make the changes as shown in table 1 to enable PC2 RedCap feature for TDD bands in 38.101-1.

The corresponding draft CR is also prepared and submitted in [9].

- References

R4-2414286, WF on RedCap PC2 requirements, China Telecom

R4-2411115, On UE RF requirements for PC2 RedCap, CATT

R4-2411171, On PC2 RedCap UE in FR1, Apple

R4-2411242, Enabling PC2 RedCap UEs in TDD and FDD bands, Skyworks Solutions Inc.

R4-2411744, PC2 requirements for TDD RedCap and eRedCap, CMCC

R4-2412467, draft CR on power class 2 RedCap for TDD bands, China Telecom, ZTE

R4-2412957, Discussion on the RF impacts for Rel-19 PC2 TDD RedCap UE	Huawei, HiSilicon

R4-2412984, PC2 RedCap RF overview	Ericsson

R4-2415873, Draft CR to 38.101-1: power class 2 RedCap for TDD bands, China Telecom, ZTE