3GPP TSG-RAN WG4 Meeting #112bis	R4-2415876

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	China Telecom

Title:	Discussion on the note for DL interruption clarification for 3 or 4 bands switching

Agenda Item:	5.10.2

Document for:	Approval

- Introduction

In RAN4#112, the WF on downlink interruption for UL Tx switching was approved [1] with the following Wayforward captured:

Issue 2-2-1: Definition format for indicating no DL interruption requirement

Wayforward: Considering the following format as starting point for indicating no DL interruption requirement for TX switching among 3 or 4 bands

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) |
| --- | --- | --- |
| CA_n1-n3-n783 | n1, n3, n78 | No for CA_n1-n78, CA_n3-n78 |
| CA_n3-n5-n78 | n3, n5, n78 | No for CA_n3-n78, CA_n5-n78 |
| CA_n1-n3-n5-n78 | n1, n3, n5, n78 | No for CA_n1-n78, CA_n3-n78, CA_n5-n78 |

This contribution discusses how to introduce the note for DL interruption clarification for 3 or 4 bands including the above CA combinations captured in the WF

- Discussion

how to introduce the note for DL interruption clarification for dynamic switching across 3 or 4 bands

In last RAN4 meeting, companies proposed their views on if DL interruption is mandated or not for the combination of CA_n1-n3-n5-n78 and the corresponding fallbacks[2][3]. In contribution [2], it shows the format of mandating no DL interruption for rel-18 combinations is same with rel-17/16 combos at uplink tx switching.

If on any two of the bands the UE is configured with switchingOptionConfigForBandPair set to 'dualUL' for CA, then more than two uplink bands are configured, which means the switched BandPair is different from CA combinations before and after switching. For example the two band pairs supporting dualUL are denoted as band pairs of (band X and band Z) and (band Y and band Z), while one transmitter is switched between band X and band Y. Now RAN4 existing requirement format such as “no for CA_nX-nY” of no DL interruption clarification for 2 uplink bands CA can not apply to 3 or 4 bands, which may need to be specified by another note based on band pair rather than band combination.

Thus, it is proposed to make the following changes to introduce the note for DL interruption clarification for 3 or 4 bands in 38.101-1.

Table 1 Changes to 38.101-1

| NR CA Band | NR Band (Table 5.2-1) | DL interruption allowed (Note 4) (Note 5) |
| --- | --- | --- |
| CA_n1-n3-n783 | n1, n3, n78 | No for CA_n1-n78, CA_n3-n78 |
| CA_n3-n5-n78 | n3, n5, n78 | No for CA_n3-n78, CA_n5-n78 |
| CA_n1-n3-n5-n78 | n1, n3, n5, n78 | No for CA_n1-n78, CA_n3-n78, CA_n5-n78 |
| NOTE 4: Applicable when dynamic Tx switching is conducted across 2 UL bands. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. NOTE 5: Applicable when dynamic Tx switching is conducted across 3 or 4 UL bands. The requirement of “no for CA_nX-nY” refer to band pair(s) for dynamic switching. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13]. |  |  |

- Conclusions

In this contribution, we discuss how to introduce the note for DL interruption clarification for dynamic switching across 3 or 4 bands have the following proposals:

Proposal 1: It is proposed to make the changes as shown in table 1 to introduce the note 5 for DL interruption clarification for dynamic switching across 3 or 4 bands in 38.101-1.

- Note 5: Applicable when dynamic Tx switching is conducted across 3 or 4 UL bands. The requirement of “no for CA_nX-nY” refer to band pair(s) for dynamic switching. The DL interruption requirement is specified in clause 8.2.2.2.10 of 38.133 [13].

- References

R4-2414435, WF on downlink interruption for UL Tx switching, China Telecom

R4-2411150, On DL interruption for Tx switching across 3 and 4 bands, Apple

R4-2413318, DL interruptions for mid band combinations, Qualcomm Incorporated