3GPP TSG-RAN WG4 Meeting # 112-bis	R4- 2415881

# Hefei, Anhui, China, 14th – 18th Octorber, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for (e)RedCap in NTN

Agenda item:	6.26.4.1

Document for:	Discussion

Introduction

In this contritubtion, we provide our views on RRM requirements for RedCap NTN based on the approved WF [1].

Discussion

Applicability clarification

The SCS considered for (e)Redcap UE with FR1-NTN

The one of proposals for SCS for RedCap NTN is as below.

<Way Forward> - For RRM requirements, the SCS considered for (e)Redcap UE with FR1-NTN support 15kHz, 30kHz and 60kHz.

In existing FR1 NTN, there is no the timing error requirements for 60kHz, so 15kHz and 30kHz SCS for RedCap NTN UE sould be also considered.

- Proposal 1: Consider 15kHz and 30kHz SCS for RedCap NTN to define RRM requirements

General consideration on RRM requirements

What RRM requirements are defined for (e)RedCap UE with FR1-NTN

<Agreement> - FR2 related requirements shall not be defined for (e)RedCap UE with FR1-NTN bands. - The requirements defined for both NTN and (e)RedCap UE requirements should be defined for (e)RedCap UE with FR1-NTN bands, including the following: - Cell Re-selection for RRC_IDLE state mobility - Cell Re-selection for RRC_INACTIVE state mobility - NR Handover - NR FR1 - NR FR1 RACH-based Handover - RRC Connection Mobility Control - SA: RRC Re-establishment - Random access - SA: RRC Connection Release with Redirection - Timing - UE transmit timing - UE timer accuracy - Timing advance - Signalling characteristics - Radio Link Monitoring - Link Recovery Procedures - Active BWP switch delay - Active TCI state switching delay - UE-specific CBW change - Measurement Procedure - General measurement requirement - NR intra-frequency measurements - NR inter-frequency measurements - L1-RSRP measurements for Reporting - For the requirements that have only been defined for NTN, RAN4 to define the following FR1-NTN requirements for (e)Redcap: - NR SAN Satellite switching with re-synchronization - Pathloss reference signal switching delay - Network verified UE location - For the requirements that have only been defined for (e)Redcap and not been defined for FR1-NTN, RAN4 NOT to define the following requirements for NTN (e)RedCap in Rel-19: - NR measurements for positioning in RRC_INACTIVE state - NR Handover to other RATs - Note: For NTN (e)RedCap, it refers to, for example, NTN to E-UTRA handover - RRC connection release with redirection to E-UTRAN - deriveSSB-IndexFromCell tolerance - Uplink spatial relation switch delay - Inter-RAT measurements in idle/inactive/connected mode - PRS-RSRPP measurements - NR measurements with autonomous gaps <Way Forward> - FFS for the following requirements: - Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state - NR Rel-17/18 Conditional Handover, and RACH-less Handover - Note: further check whether they are supported for RedCap NTN in RAN1/2 spec. - FFS: Send LS to RAN1/2 to ask the status. RAN4 does not trigger the RAN1/2 discussion to support these features. - Configured Grant based Small Data Transmissions (CG-SDT) - Random access based Small Data Transmissions (RA-SDT)

RAN4 agreed what RRM requirements would be defined for RedCap NTN, and some of RRM requirements are FFS. In Rel-17/18 NR NTN, time and location based conditional handover has been defined, and this CHO should be considered if a UE supports NTN feature. So, CHO requirements for RedCap NTN should be defiend. Also, the UE supporting NTN feature is required to perform timing/frequency pre-compensation, so RACH-less handover defined in NR NTN could be supported for RedCap NTN. In our understanding, based on RAN1/2 specification, there are no restriction in case RedCap NTN supports CHO and RACH-less handover.

- Proposal 2: RAN4 to define requirements for conditional HO and RACH-less HO for RedCap NTN

For SDT, in the last RAN2 meeting, there was the agreement related to SDT procedure for regenerative payload below.

<RAN2#127 Agreements> - The objective for Support of regenerative payload - We do not optimize UE power saving for SDT procedure in regenerative payload architecture

With this agreement, SDT procedure is already supported in NR NTN from the RAN2 perspective even RAN4 does not have related requirements. Since the SDT operation could be supported for both NTN and (e)RedCap UE, the requirements for SDT should be defined for RedCap NTN.

- Proposal 3: RAN4 to define requriements for SDT for RedCap NTN

Reduction in the number of UE Rx branches

How to relax the requirements for 1Rx (e)RedCap UEs with FR1-NTN

In RAN4 specification, extended measurement /evaluation period and handover delay for RRM requirements for 1Rx RedCap UE have already considered, so RRM requirements for 1Rx RedCap NTN can be defined based on existing 1Rx RedCap RRM requirements.

- Proposal 4: RRM requirements for 1Rx RedCap NTN can be defined based on existing 1Rx RedCap RRM requirements

eDRX enhancement

How to consider the impact of eDRX enhancement for (e)Redcap UE with FR1-NTN

In NR NTN, RAN4 has considered {0.32, 0.64, 1.28, 2.56}sec DRX cycle for RRM requirements, and for 2.56s DRX cycle, UE is not required to meet the requirmeents for earth-moving LEO deployment. It implies that DRX cycle larger than 2.56s at least for earth-moving LEO based NTN scenario is meaningless. eDRX cycle defined in RRM requiqrements is larger than 2.56s, so eDRX configuration for earth-moving LEO deployment should be precluded. For GSO deployment, eDRX cycle for RedCap NTN could be considered, but RAN4 needs to further discuss whether eDRX cycle is applicable for quasi-earth fixed cell LEO deployment.

- Proposal 5: For eDRX configuration for RedCap NTN,

  - For GSO deploymenet, RRM requirements based on eDRX can be defined

  - For quasi-earth fixed cell LEO deployment, RAN4 needs to further discuss to define RRM requirements

  - For earth-moving LEO deployment, eDRX configuration should be precluded

Conclusion

In this contribution, we provide views on (e)RedCap NTN, and we propose

- Proposal 1: Consider 15kHz and 30kHz SCS for RedCap NTN to define RRM requirements

- Proposal 2: RAN4 to define requirements for conditional HO and RACH-less HO for RedCap NTN

- Proposal 3: RAN4 to define requriements for SDT for RedCap NTN

- Proposal 4: RRM requirements for 1Rx RedCap NTN can be defined based on existing 1Rx RedCap RRM requirements

- Proposal 5: For eDRX configuration for RedCap NTN,

  - For GSO deploymenet, RRM requirements based on eDRX can be defined

  - For quasi-earth fixed cell LEO deployment, RAN4 needs to further discuss to define RRM requirements

  - For earth-moving LEO deployment, eDRX configuration should be precluded

Reference

[1] R4-2414037, “The WF on RRM requirements for Rel-19 NR NTN phase3,” CATT