3GPP TSG-RAN4 Meeting #112bis	R4-2415885

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	LG Electronics

Title:	Duty cycle solutions of HPUE for inter-band UL CA and EN-DC

Agenda item:	6.1.1.1.1

Document for:	Discussion

Introduction

In this paper, we provide our views on duty cycle solution of HPUE for inter-band UL CA and EN-DC.

Discussion

In last RAN4 2 meetings, the following WFs were approved [1, 2]. This paper discusses the duty cycle solution requirement of HPUE for inter-band UL CA and EN-DC.

[1] 2.1 Configuration Adhoc agreement: - For PC1.5 inter-band UL CA/EN-DC, the work for the case PC2 FDD band as one of the constituent band would be triggered after example combo(s) is added in the WID 2.2 Spec organization Adhoc agreement: Adopt same way as in Rel-18 2.3 3UL CC with 3Tx Adhoc agreement: Don not consider 3ULCC with 3Tx CA_nXA-nY(2A)/ DC_XA_nY(2A) scenario 3.1 Methodology - Use higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability - No new power class is introduced 3.2 Scenarios to be considered in Rel-19 - For both 2Tx and 3Tx, higherPowerLimit-R17/higherPowerLimtMRDC-R17 is enabled for any specified inter-band band combination 4.2 Duty cycle solution for SAR compliance (WF) - FFS [2] 1.1 Duty cycle solution for SAR compliance - Duty cycle solution is considered for both intra-band CA and inter-band CA/EN-DC 3.1 Requirements between handheld UE and FWA - <WF>: - For PC2 and PC1.5 2-band inter-band UL CA with 2Tx or 3Tx, - only define one set of UE RF requirements for both handheld UE and FWA - For PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx, - only define one set of UE RF requirements for both handheld UE and FWA. 3.2 Whether to collect configurations to derive general requirements? - <WF>: - General requirements can be specified including considerations of different duplex mode configurations: - For PC2, - FDD+FDD, FDD+TDD, TDD+TDD with 2Tx and 3Tx - For PC1.5, - [FDD+FDD], FDD+TDD, TDD+TDD with 2Tx and 3Tx 3.3 3UL CC with 3Tx - <WF>: RAN4 to further check whether there is demand for 3 ULCC with 3Tx scenario (such as CA_nXA-nY(2A) and CA_ nXA-nYB), and whether it is desirable to be introduced in Rel-19. 3.4 Release independence - <WF>: Discuss the release independence for 3Tx band combination for handheld UE and FWA at later stage of this WI - For FWA, only discuss the configurations that were not introduced in Rel-18 - 4.4 MSD and SAR compliance Way forward: - If higherPowerLimit-R17/higherPowerLimtMRDC-R17 capability is adopted for increasing UE transmission power, then taking same methodology as for higherPowerLimit-R17/ higherPowerLimtMRDC-R17 for SAR compliance and MSD. More specific, - For a given band combination, no additional MSD requirements are specified in addition to the MSD requirements of its legacy power class(es) - No new duty-cycle solution/mechanism for SAR compliance is considered - P-MPR can be used for SAR mitigation

PC1.5 UE for 2- band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA

For PC1.5 UE supporting NR inter-band UL CA with 2Tx,

PC2+PC2 for 2-band NR bands is new power combination.

- Case 1 : PC2 NR FDD + PC2 NR TDD

- Case 2 : PC2 NR TDD + PC2 NR TDD

Based on WF [1], case 1 can be treated after example combo(s) is added in the WID. however, case 2 can be treated in RAN4. For case2, the existing PC2 duty cycle solution should be modified.

For example,

If a UE supports power class 1.5 for the band combination and the supported power class enables the higher maximum output power than that of the default power class: - if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and - if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or - if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 23dBm or lower; - shall apply all requirements for the default power class and set the configured transmitted power as specified in in clause 6.2A.4.1.3; - if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5maxUplinkDutyCycle-interBandCA-PC2 but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or - if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is between 23dBm and 26dBm; - shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2A.4.1.3; - else; - shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent). The average percentage of uplink symbols is defined as 50% ( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331. For NR Band x or NR Band y, – if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%; – else if the band is configured with power class 3; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

Proposal 1: Modify the existing PC2 duty cycle solution if PC2 TDD + PC2 TDD with 2Tx is introduced.

For PC1.5 UE supporting NR inter-band UL CA with 3Tx,

In Rel-18, the following combination of PC for NR bands is supported.

- Case 1 : PC3 NR FDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

Whether to support the following combinations in Rel-19 need to be decided. Based on WF [1], case 2, and case 3 can be treated after example combo(s) is added in the WID.

- Case 2 : PC2 NR FDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 3 : PC2 NR FDD + PC2 NR TDD (UL-MIMO or Tx Diversity)

- Case 4 : PC3 NR TDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 5 : PC2 NR TDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 6 : PC2 NR TDD + PC2 NR TDD (UL-MIMO or Tx Diversity)

Among the cases, the total maximum output power can be higher than 29dBm in case 1, case 4 and case 5. The difference from 29dBm is 0.97dB for case 1 and case 4 and 1.76dB for case 5.

- Case 1 & Case 4 : 23dBm + 29dBm = 29.97 dBm

- Case 5 : 26dBm + 29dBm  = 30.76 dBm

If higherPowerLimit is indicated, PPower class, CA is replaced with 29.97dBm for case 1 and case 4 and 30.76dBm for case 5.

And, the existing duty-cycle solution can be reused for case1, case 4 and case 5.

Proposal 2: For PC1.5 UE supporting NR inter-band UL CA with 3Tx,  the existing duty-cycle solution can be applied if PC3 NR TDD + PC1.5 NR TDD (2Tx), and PC2 NR TDD + PC1.5 NR TDD (2Tx) are introduced.

PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA

For PC2 UE supporting two band EN-DC with 2Tx,

In Rel-18, the following combination of PC for E-UTRA band and PC for NR band is supported.

- Case 1 : PC3 E-UTRA FDD + PC3 NR TDD

- Case 2 : PC3 E-UTRA FDD + PC2 NR TDD

- Case 3 : PC3 E-UTRA TDD + PC3 NR TDD

- Case 4 : PC3 E-UTRA TDD + PC2 NR TDD

Whether to support the following combinations in Rel-19 need to be decided. Based on WF [1], case 6 can be treated after example combo(s) is added in the WID.

- Case 5 : PC3 E-UTRA FDD + PC3 NR FDD

- Case 6 : PC3 E-UTRA FDD + PC2 NR FDD

- Case 7 : PC2 E-UTRA TDD + PC3 NR TDD

For Case 5 Case 6, the existing E-UTRA FDD + NR TDD duty cycle solution can be reused with a new capability, for example, maxUplinkDutyCycle-interBandENDC-FDD-PC2 which comprises of maxUplinkDutyCycle-FDD-EN-DC1 and  maxUplinkDutyCycle-FDD-EN-DC2,  instead of maxUplinkDutyCycle-FDD-TDD-EN-DC1 and  maxUplinkDutyCycle-FDD-TDD-EN-DC2 which are applicable only for inter-band FDD+TDD EN-DC.

For Case 7, the existing E-UTRA TDD + NR TDD duty cycle solution needs to be modified, because it is based on PC3 E-UTRA TDD. PC2 E-UTRA TDD based duty cycle solution is needed.

For example, Table 2.1 shows the percentage of maximum E-UTRA/NR uplink transmission for Case 7.

Table 2.1: Percentage of maximum E-UTRA/NR uplink transmission in PC2 EN-DC (Case 7)

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D:threshold) |  |
| --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (23dBm) |  |  |
|  | Y=0~50 | Z=0~100, D=100-2*Y | Z <= D | Z>D |
| 0 | 70 | N/A | PC2 | PC3 |
| 1 | 50 | N/A |  |  |
| 2 | 30 | 40 |  |  |
| 3 | 40 | 20 |  |  |
| 4 | 30 | 40 |  |  |
| 5 | 20 | 60 |  |  |
| 6 | 60 | N/A |  |  |

- X : E-UTRA TDD Uplink-downlink Configuration

- Y : maximum percentage of E-UTRA uplink transmission during any evaluation period of time

- Z : actual percentage of NR uplink transmission during any evaluation period of time

- D : threshold of NR uplink transmission during any evaluation period of time

Here, E-UTRA TDD uplink-downlink configuration ‘0’, ‘1’ and ‘6’ are not applicable for EN-DC uplink transmission with maximum output power for SAR compliance. A minimum percentage of 20% for maximum NR uplink transmission during evaluation period can be assumed as threshold when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent.

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 3 and threshold of NR uplink transmission = 20,

- Z > 20,        PC3 EN-DC is applied

- Z ≤ 20,      PC2 EN-DC is applied

Table 2.2 shows the percentage of maximum E-UTRA/NR uplink transmission for Case 3. Threshold of 30% was specified with the minimum percentage of maximum NR uplink transmission for PC3 E-UTRA TDD+PC3 NR TDD

when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent.

Table 2.2: Percentage of maximum E-UTRA/NR uplink transmission in PC2 EN-DC (Case 3)

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D:threshold) |  |
| --- | --- | --- | --- | --- |
|  | E-UTRA (23dBm) | NR (23dBm) |  |  |
|  | Y=0~100 | Z=0~100, D=100-Y | Z <= D | Z>D |
| 0 | 70 | 30 | PC2 | PC3 |
| 1 | 50 | 50 |  |  |
| 2 | 30 | 70 |  |  |
| 3 | 40 | 60 |  |  |
| 4 | 30 | 70 |  |  |
| 5 | 20 | 80 |  |  |
| 6 | 60 | 40 |  |  |

Proposal 3: For PC2 UE supporting two band EN-DC with 2Tx,  consider  a new capability if PC3 E-UTRA FDD + PC3 NR FDD is introduced. For example,

- maxUplinkDutyCycle-interBandENDC-FDD-PC2

  - comprises of maxUplinkDutyCycle-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-EN-DC2

Proposal 4: For PC2 UE supporting two band EN-DC with 2Tx, consider a minimum percentage of 20% as threshold for maximum NR uplink transmission if PC2 E-UTRA TDD + PC3 NR TDD is introduced.

For PC1.5 UE supporting two band EN-DC with 2Tx,

In Rel-18, it is not supported.

Whether to support the following combination in Rel-19 needs to be decided.

- Case 1 : PC2 E-UTRA TDD + PC2 NR TDD

For Case 1, the existing E-UTRA TDD + NR TDD duty cycle solution needs to be modified, because it is based on PC3 E-UTRA TDD. PC2 E-UTRA TDD based duty cycle solution is needed.

For example, Table 2.3 shows the percentage of maximum E-UTRA/NR uplink transmission for Case 1.

Table 2.3: Percentage of maximum E-UTRA/NR uplink transmission in PC1.5 EN-DC (26dBm(E-UTRA TDD)+26dBm(NR TDD))

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D(D1,D2) :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (26dBm) (Z=Z1 or Z2) |  |  |  |  |
|  | Y=0~50 | Z1=0~50, D1=50-Y | Z2=0~50, D2=25-Y = D1-25 | Z > D1 | D2<=Z<=D1 or D1>=Z | Z < D2 |
| 0 | 70 | N/A | N/A | N/A | N/A | N/A |
| 1 | 50 | N/A | N/A | N/A | N/A | N/A |
| 2 | 30 | 20 | N/A | PC3 | PC2 | N/A |
| 3 | 40 | 10 | N/A | PC3 | PC2 | N/A |
| 4 | 30 | 20 | N/A | PC3 | PC2 | N/A |
| 5 | 20 | 30 | 5 | PC3 | PC2 | PC1.5 |
| 6 | 60 | N/A | N/A | N/A | N/A | N/A |

Here, E-UTRA TDD uplink-downlink configuration ‘0’, ‘1’ and ‘6’ are not applicable for EN-DC uplink transmission with maximum output power for SAR compliance.

- X : E-UTRA TDD Uplink-downlink Configuration

- Y : maximum percentage of E-UTRA uplink transmission during any evaluation period of time

- Z : actual percentage of NR uplink transmission during any evaluation period of time

- D : threshold of NR uplink transmission during any evaluation period of time

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 5 and threshold of NR uplink transmission = 30 (D1=30, D2=5),

- Z > 30,        PC3 EN-DC is applied

- 5 < Z ≤ 30, PC2 EN-DC is applied

- Z ≤ 5,         PC1.5 EN-DC is applied

Proposal 5: For PC1.5 UE supporting two band EN-DC with 2Tx,  define new duty cycle solution considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission if PC2 E-UTRA TDD + PC2 NR TDD is introduced. For example,

- E-UTRA TDD Uplink-downlink Configuration (X) = 5

- minimum percentage of 30% and 5% as 2 thresholds for maximum NR uplink transmission

For PC2 UE supporting two band EN-DC with 3Tx,

In Rel-18, the following combination of PC for E-UTRA band and PC for NR band is supported.

- Case 1 : PC3 E-UTRA FDD + PC3 NR TDD (UL-MIMO or Tx diversity)

- Case 2 : PC3 E-UTRA FDD + PC2 NR TDD (UL-MIMO or Tx diversity)

- Case 3 : PC3 E-UTRA TDD + PC3 NR TDD (UL-MIMO or Tx diversity)

- Case 4 : PC3 E-UTRA TDD + PC2 NR TDD (UL-MIMO or Tx diversity)

Whether to support the following combinations in Rel-19 need to be decided. Based on WF [1], case 6 can be treated after example combo(s) is added in the WID.

- Case 5 : PC3 E-UTRA FDD + PC3 NR FDD (UL-MIMO or Tx diversity)

- Case 6 : PC3 E-UTRA FDD + PC2 NR FDD (UL-MIMO or Tx diversity)

- Case 7 : PC2 E-UTRA TDD + PC3 NR TDD (UL-MIMO or Tx diversity)

For Case 5, the existing E-UTRA FDD + NR TDD duty cycle solution can be reused with a new capability, for example, maxUplinkDutyCycle-interBandENDC-FDD-PC2 which comprises of maxUplinkDutyCycle-FDD-EN-DC1 and  maxUplinkDutyCycle-FDD-EN-DC2,  instead of maxUplinkDutyCycle-FDD-TDD-EN-DC1 and  maxUplinkDutyCycle-FDD-TDD-EN-DC2 which are applicable only for inter-band FDD+TDD EN-DC.

For Case 7 , the existing E-UTRA TDD + NR TDD duty cycle solution needs to be modified, because it is based on PC3 E-UTRA TDD. PC2 E-UTRA TDD based duty cycle solution is needed. Table 2.1 can be considered.

Proposal 6: For PC2 UE supporting two band EN-DC with 3Tx,  consider  a new capability if PC3 E-UTRA FDD + PC3 NR FDD is introduced. For example,

- maxUplinkDutyCycle-interBandENDC-FDD-PC2

  - comprises of maxUplinkDutyCycle-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-EN-DC2

Proposal 7: For PC2 UE supporting two band EN-DC with 3Tx, consider a minimum percentage of 20% as threshold for maximum NR uplink transmission if PC2 E-UTRA TDD + PC3 NR TDD is introduced.

For PC1.5 UE supporting two band EN-DC with 3Tx,

In Rel-18, it is not supported.

Whether to support the following combinations in Rel-19 needs to be decided.

- Case 1 : PC3 E-UTRA FDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 2 : PC3 E-UTRA TDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 3 : PC2 E-UTRA TDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 4 : PC2 E-UTRA TDD + PC2 NR TDD (UL-MIMO or Tx diversity)

For Case 1, the following formula can be assumed between the LTE UL duty cycle, NR UL duty cycle and the overall duty threshold.

-   DutyLTE x( PLTE/ P29) + DutyNR x(PNR/ P29)  ≤ Duty threshold

-   0.25xDutyLTE + DutyNR  ≤ Duty threshold

DutyLTE is from 0 to 100%.

DutyNR is from 0 to 25%.

PLTE, PNR, P29 represent the maximum linear power (mW) of LTE, NR, and EN-DC PC1.5 respectively; DutyLTE, DutyNR represent the maximum uplink percentage of LTE, NR respectively. Duty threshold represents the maximum UL duty cycle that can maintain the PC1.5 power class for FDD+TDD EN-DC.

The LTE reference configuration can be DutyLTE=70%. The default value of NR UL duty capability can be 7.5% i.e. "DutyLTE=70% and PLTE=23dBm", corresponding to default DutyNR=7.5%, PNR=29dBm.

For better flexibility, two LTE configurations are supported with LTE reference configuration of DutyLTE=70% and 40%.

The LTE reference configuration can be DutyLTE=40%. The default value of NR UL duty capability can be 15% i.e. "DutyLTE=40% and PLTE=23dBm", corresponding to default DutyNR=15%, PNR=29dBm.

It means that Duty threshold is scaled with 0.25.

For Case 2, the corresponding duty cycle solution needs to be specified.

Table 2.4 shows this example for the percentage of maximum E-UTRA/NR UL transmission. The possible E-UTRA TDD uplink-downlink configurations and the percentage need to be considered when specifying duty cycle solution.

Table 2.4: Percentage of maximum E-UTRA/NR uplink transmission in PC1.5 EN-DC (23dBm(E-UTRA TDD)+2x26dBm(NR TDD))

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D(D1,D2) :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (23dBm) | NR (29dBm) (Z=Z1 or Z2) |  |  |  |  |
|  | Y=0~100 | Z1=0~25, D1=(100-Y)/4 =25-Y/4 | Z2=0~25 D2=(50-Y)/4 =D1-12.5 | Z > D1 | D2<=Z<=D1 or D1>=Z | Z < D2 |
| 0 | 70 | 7.5 | N/A | N/A | N/A | N/A |
| 1 | 50 | 12.5 | N/A | N/A | N/A | N/A |
| 2 | 30 | 17.5 | 5 | PC3 | PC2 | N/A |
| 3 | 40 | 15 | 2.5 | PC3 | PC2 | N/A |
| 4 | 30 | 17.5 | 5 | PC3 | PC2 | N/A |
| 5 | 20 | 20 | 7.5 | PC3 | PC2 | PC1.5 |
| 6 | 60 | 10 | N/A | N/A | N/A | N/A |

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 5 and threshold of NR uplink transmission = 20 (D1=20, D2=7.5),

- Z > 20,            PC3 EN-DC is applied

- 7.5 < Z ≤ 20, PC2 EN-DC is applied

- Z ≤ 7.5,         PC1.5 EN-DC is applied

For Case 3, the corresponding duty cycle solution needs to be specified.

Table 2.5 shows this example for the percentage of maximum E-UTRA/NR UL transmission. The possible E-UTRA TDD uplink-downlink configurations and the percentage need to be considered when specifying SAR solution.

Table 2.5: Percentage of maximum E-UTRA/NR uplink transmission in PC1.5 EN-DC (26dBm(E-UTRA TDD)+2x26dBm(NR TDD))

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D(D1,D2) :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (29dBm) (Z=Z1 or Z2) |  |  |  |  |
|  | Y=0~50 | Z1=0~25, D1=(50-Y)/2 | Z2=0~25 D2=(25-Y)/2 =Z1-12.5 | Z > D1 | D2<=Z<=D1 or D1>=Z | Z < D2 |
| 0 | 70 | N/A | N/A | N/A | N/A | N/A |
| 1 | 50 | N/A | N/A | N/A | N/A | N/A |
| 2 | 30 | 10 | N/A | PC3 | PC2 | N/A |
| 3 | 40 | 5 | N/A | PC3 | PC2 | N/A |
| 4 | 30 | 10 | N/A | PC3 | PC2 | N/A |
| 5 | 20 | 15 | 2.5 | PC3 | PC2 | PC1.5 |
| 6 | 60 | N/A | N/A | N/A | N/A | N/A |

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 5 and threshold of NR uplink transmission = 15 (D1=15, D2=2.5),

- Z > 15,           PC3 EN-DC is applied

- 2.5 < Z ≤ 15, PC2 EN-DC is applied

- Z ≤ 2.5,         PC1.5 EN-DC is applied

For Case 4, the corresponding duty cycle solution needs to be specified.

Table 2.6 shows this example for the percentage of maximum E-UTRA/NR UL transmission. The possible E-UTRA TDD uplink-downlink configurations and the percentage need to be considered when specifying duty cycle solution.

Table 2.6: Percentage of maximum E-UTRA/NR uplink transmission in PC1.5 EN-DC (26dBm(E-UTRA TDD)+2x23dBm(NR TDD))

| E-UTRA TDD Uplink-downlink Configuration (X) | Percentage of maximum E-UTRA/NR uplink transmission during evaluation period (%) |  |  | Supported EN-DC power class (Z:actual NR % of uplink transmission, D(D1,D2) :threshold) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA (26dBm) | NR (26dBm) (Z=Z1 or Z2) |  |  |  |  |
|  | Y=0~50 | Z1=0~50, D1=50-Y | Z2=0~50 D2=25-Y = D1-25 | Z > D1 | D2<=Z<=D1 or D1>=Z | Z < D2 |
| 0 | 70 | N/A | N/A | N/A | N/A | N/A |
| 1 | 50 | N/A | N/A | N/A | N/A | N/A |
| 2 | 30 | 20 | N/A | PC3 | PC2 | N/A |
| 3 | 40 | 10 | N/A | PC3 | PC2 | N/A |
| 4 | 30 | 20 | N/A | PC3 | PC2 | N/A |
| 5 | 20 | 30 | 5 | PC3 | PC2 | PC1.5 |
| 6 | 60 | N/A | N/A | N/A | N/A | N/A |

For example,

When E-UTRA TDD Uplink-downlink Configuration (X) = 5 and threshold of NR uplink transmission = 30 (D1=30, D2=5),

- Z > 30,        PC3 EN-DC is applied

- 5 < Z ≤ 30, PC2 EN-DC is applied

- Z ≤ 5,         PC1.5 EN-DC is applied

Proposal 8: For PC1.5 UE supporting two band EN-DC with 3Tx, define new duty cycle solutios considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission if the following band combinations are introduce.

- PC3 E-UTRA FDD/TDD + PC1.5 NR TDD

- PC2 E-UTRA TDD + PC2/PC1.5 NR TDD

For example of PC2 E-UTRA TDD + PC2 NR TDD,

- E-UTRA TDD Uplink-downlink Configuration (X) = 5

- minimum percentage of 30% and 5% as 2 thresholds for maximum NR uplink transmission

Conclusion

In this contribution, we proposed our view on the duty cycle solution of HPUE for inter-band CA and EN-DC. The proposals are as follows.

For PC1.5 UE supporting NR inter-band UL CA with 2Tx,

Proposal 1: Modify the existing PC2 duty cycle solution if PC2 TDD + PC2 TDD with 2Tx is introduced.

For example,

If a UE supports power class 1.5 for the band combination and the supported power class enables the higher maximum output power than that of the default power class: - if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and - if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or - if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 23dBm or lower; - shall apply all requirements for the default power class and set the configured transmitted power as specified in in clause 6.2A.4.1.3; - if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5maxUplinkDutyCycle-interBandCA-PC2 but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or - if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is between 23dBm and 26dBm; - shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2A.4.1.3; - else; - shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent). The average percentage of uplink symbols is defined as 50% ( DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.331. For NR Band x or NR Band y, – if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%; – else if the band is configured with power class 3; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

For PC1.5 UE supporting NR inter-band UL CA with 3Tx,

Proposal 2: For PC1.5 UE supporting NR inter-band UL CA with 3Tx,  the existing duty-cycle solution can be applied if PC3 NR TDD + PC1.5 NR TDD (2Tx), and PC2 NR TDD + PC1.5 NR TDD (2Tx) are introduced.

For PC2 UE supporting two band EN-DC with 2Tx,

Proposal 3: For PC2 UE supporting two band EN-DC with 2Tx,  consider  a new capability if PC3 E-UTRA FDD + PC3 NR FDD is introduced. For example,

- maxUplinkDutyCycle-interBandENDC-FDD-PC2

  - comprises of maxUplinkDutyCycle-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-EN-DC2

Proposal 4: For PC2 UE supporting two band EN-DC with 2Tx, consider a minimum percentage of 20% as threshold for maximum NR uplink transmission if PC2 E-UTRA TDD + PC3 NR TDD is introduced.

For PC1.5 UE supporting two band EN-DC with 2Tx,

Proposal 5: For PC1.5 UE supporting two band EN-DC with 2Tx,  define new duty cycle solution considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission if PC2 E-UTRA TDD + PC2 NR TDD is introduced. For example,

- E-UTRA TDD Uplink-downlink Configuration (X) = 5

- minimum percentage of 30% and 5% as 2 thresholds for maximum NR uplink transmission

For PC2 UE supporting two band EN-DC with 3Tx,

Proposal 6: For PC2 UE supporting two band EN-DC with 3Tx,  consider  a new capability if PC3 E-UTRA FDD + PC3 NR FDD is introduced. For example,

- maxUplinkDutyCycle-interBandENDC-FDD-PC2

  - comprises of maxUplinkDutyCycle-FDD-EN-DC1 and maxUplinkDutyCycle-FDD-EN-DC2

Proposal 7: For PC2 UE supporting two band EN-DC with 3Tx, consider a minimum percentage of 20% as threshold for maximum NR uplink transmission if PC2 E-UTRA TDD + PC3 NR TDD is introduced.

For PC1.5 UE supporting two band EN-DC with 3Tx,

Proposal 8: For PC1.5 UE supporting two band EN-DC with 3Tx, define new duty cycle solutios considering the possible E-UTRA TDD uplink-downlink configurations and the percentage of maximum E-UTRA/NR uplink transmission if the following band combinations are introduce.

- PC3 E-UTRA FDD/TDD + PC1.5 NR TDD

- PC2 E-UTRA TDD + PC2/PC1.5 NR TDD

For example of PC2 E-UTRA TDD + PC2 NR TDD,

- E-UTRA TDD Uplink-downlink Configuration (X) = 5

- minimum percentage of 30% and 5% as 2 thresholds for maximum NR uplink transmission

- References

[1] R4-2414277, “WF on HPUE for CA in TN”, Samsung

[2] R4-2410750, “WF on higher power UE”, Samsung

[3] RP-240828, “New WID : UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, ATT