3GPP TSG-RAN4 Meeting #112bis	R4-2415886

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	LG Electronics

Title:	HPUE for inter-band UL CA and EN-DC

Agenda item:	6.1.1.1.3

Document for:	Discussion

Introduction

In this paper, we provide our views on HPUE for inter-band UL CA and EN-DC.

Discussion

In last RAN4 2 meetings, the following WFs were approved [1, 2]. This paper discusses HPUE for inter-band UL CA and EN-DC.

[1] 2.1 Configuration Adhoc agreement: - For PC1.5 inter-band UL CA/EN-DC, the work for the case PC2 FDD band as one of the constituent band would be triggered after example combo(s) is added in the WID 2.2 Spec organization Adhoc agreement: Adopt same way as in Rel-18 2.3 3UL CC with 3Tx Adhoc agreement: Don not consider 3ULCC with 3Tx CA_nXA-nY(2A)/ DC_XA_nY(2A) scenario 3.1 Methodology - Use higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability - No new power class is introduced 3.2 Scenarios to be considered in Rel-19 - For both 2Tx and 3Tx, higherPowerLimit-R17/higherPowerLimtMRDC-R17 is enabled for any specified inter-band band combination 4.2 Duty cycle solution for SAR compliance (WF) - FFS [2] 1.1 Duty cycle solution for SAR compliance - Duty cycle solution is considered for both intra-band CA and inter-band CA/EN-DC 3.1 Requirements between handheld UE and FWA - <WF>: - For PC2 and PC1.5 2-band inter-band UL CA with 2Tx or 3Tx, - only define one set of UE RF requirements for both handheld UE and FWA - For PC2 and PC1.5 2-band inter-band EN-DC with 2Tx or 3Tx, - only define one set of UE RF requirements for both handheld UE and FWA. 3.2 Whether to collect configurations to derive general requirements? - <WF>: - General requirements can be specified including considerations of different duplex mode configurations: - For PC2, - FDD+FDD, FDD+TDD, TDD+TDD with 2Tx and 3Tx - For PC1.5, - [FDD+FDD], FDD+TDD, TDD+TDD with 2Tx and 3Tx 3.3 3UL CC with 3Tx - <WF>: RAN4 to further check whether there is demand for 3 ULCC with 3Tx scenario (such as CA_nXA-nY(2A) and CA_ nXA-nYB), and whether it is desirable to be introduced in Rel-19. 3.4 Release independence - <WF>: Discuss the release independence for 3Tx band combination for handheld UE and FWA at later stage of this WI - For FWA, only discuss the configurations that were not introduced in Rel-18 - 4.4 MSD and SAR compliance Way forward: - If higherPowerLimit-R17/higherPowerLimtMRDC-R17 capability is adopted for increasing UE transmission power, then taking same methodology as for higherPowerLimit-R17/ higherPowerLimtMRDC-R17 for SAR compliance and MSD. More specific, - For a given band combination, no additional MSD requirements are specified in addition to the MSD requirements of its legacy power class(es) - No new duty-cycle solution/mechanism for SAR compliance is considered - P-MPR can be used for SAR mitigation

PC1.5 UE for 2- band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA

For PC1.5 UE supporting NR inter-band UL CA with 2Tx,

PC2+PC2 for 2-band NR bands is new power combination.

- Case 1 : PC2 NR FDD + PC2 NR TDD

- Case 2 : PC2 NR TDD + PC2 NR TDD

Based on WF [1], case 1 can be treated after example combo(s) is added in the WID. however, case 2 can be treated in RAN4.

Proposal 1: For PC1.5 UE supporting NR inter-band UL CA with 2Tx, consider  following band combination(s) in Rel-19

- PC2 NR TDD + PC2 NR TDD

For PC1.5 UE supporting NR inter-band UL CA with 3Tx,

In Rel-18, the following combination of PC for NR bands is supported.

- Case 1 : PC3 NR FDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

Whether to support the following combinations in Rel-19 need to be decided. Based on WF [1], case 2, and case 3 can be treated after example combo(s) is added in the WID.

- Case 2 : PC2 NR FDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 3 : PC2 NR FDD + PC2 NR TDD (UL-MIMO or Tx Diversity)

- Case 4 : PC3 NR TDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 5 : PC2 NR TDD + PC1.5 NR TDD (UL-MIMO or Tx Diversity)

- Case 6 : PC2 NR TDD + PC2 NR TDD (UL-MIMO or Tx Diversity)

Proposal 2: For PC1.5 UE supporting NR inter-band UL CA with 3Tx,  consider  following band combination(s) in Rel-19.

- PC3/PC2 NR TDD + PC1.5 NR TDD

- PC2 NR TDD + PC2 NR TDD

PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA

For PC2 UE supporting two band EN-DC with 2Tx,

In Rel-18, the following combination of PC for E-UTRA band and PC for NR band is supported.

- Case 1 : PC3 E-UTRA FDD + PC3 NR TDD

- Case 2 : PC3 E-UTRA FDD + PC2 NR TDD

- Case 3 : PC3 E-UTRA TDD + PC3 NR TDD

- Case 4 : PC3 E-UTRA TDD + PC2 NR TDD

Whether to support the following combinations in Rel-19 need to be decided. Based on WF [1], case 6 can be treated after example combo(s) is added in the WID.

- Case 5 : PC3 E-UTRA FDD + PC3 NR FDD

- Case 6 : PC3 E-UTRA FDD + PC2 NR FDD

- Case 7 : PC2 E-UTRA TDD + PC3 NR TDD

Proposal 3: For PC2 UE supporting two band EN-DC with 2Tx, consider following band combination(s) in Rel-19.

- PC3 E-UTRA FDD + PC3 NR FDD

- PC2 E-UTRA TDD + PC3 NR TDD.

For PC1.5 UE supporting two band EN-DC with 2Tx,

In Rel-18, it is not supported.

Whether to support the following combination in Rel-19 needs to be decided.

- Case 1 : PC2 E-UTRA TDD + PC2 NR TDD

Proposal 4: For PC1.5 UE supporting two band EN-DC with 2Tx,  consider following band combination(s) in Rel-19.

- PC2 E-UTRA TDD + PC2 NR TDD

For PC2 UE supporting two band EN-DC with 3Tx,

In Rel-18, the following combination of PC for E-UTRA band and PC for NR band is supported.

- Case 1 : PC3 E-UTRA FDD + PC3 NR TDD (UL-MIMO or Tx diversity)

- Case 2 : PC3 E-UTRA FDD + PC2 NR TDD (UL-MIMO or Tx diversity)

- Case 3 : PC3 E-UTRA TDD + PC3 NR TDD (UL-MIMO or Tx diversity)

- Case 4 : PC3 E-UTRA TDD + PC2 NR TDD (UL-MIMO or Tx diversity)

Whether to support the following combinations in Rel-19 need to be decided. Based on WF [1], case 6 can be treated after example combo(s) is added in the WID.

- Case 5 : PC3 E-UTRA FDD + PC3 NR FDD (UL-MIMO or Tx diversity)

- Case 6 : PC3 E-UTRA FDD + PC2 NR FDD (UL-MIMO or Tx diversity)

- Case 7 : PC2 E-UTRA TDD + PC3 NR TDD (UL-MIMO or Tx diversity)

Proposal 5: For PC2 UE supporting two band EN-DC with 3Tx,  consider following band combination(s) in Rel-19.

- PC3 E-UTRA FDD + PC3 NR FDD

- PC2 E-UTRA TDD + PC3 NR TDD

For PC1.5 UE supporting two band EN-DC with 3Tx,

In Rel-18, it is not supported.

Whether to support the following combinations in Rel-19 needs to be decided.

- Case 1 : PC3 E-UTRA FDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 2 : PC3 E-UTRA TDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 3 : PC2 E-UTRA TDD + PC1.5 NR TDD (UL-MIMO or Tx diversity)

- Case 4 : PC2 E-UTRA TDD + PC2 NR TDD (UL-MIMO or Tx diversity)

Proposal 6: For PC1.5 UE supporting two band EN-DC with 3Tx, consider following band combination(s) in Rel-19.

- PC3 E-UTRA FDD/TDD + PC1.5 NR TDD

- PC2 E-UTRA TDD + PC2/PC1.5 NR TDD

Conclusion

In this contribution, we proposed our view on HPUE band combinations for inter-band CA and EN-DC. The proposals are as follows.

For PC1.5 UE supporting NR inter-band UL CA with 2Tx,

Proposal 1: For PC1.5 UE supporting NR inter-band UL CA with 2Tx, consider  following band combination(s) in Rel-19

- PC2 NR TDD + PC2 NR TDD

For PC1.5 UE supporting NR inter-band UL CA with 3Tx,

Proposal 2: For PC1.5 UE supporting NR inter-band UL CA with 3Tx,  consider  following band combination(s) in Rel-19.

- PC3/PC2 NR TDD + PC1.5 NR TDD

- PC2 NR TDD + PC2 NR TDD

For PC2 UE supporting two band EN-DC with 2Tx,

Proposal 3: For PC2 UE supporting two band EN-DC with 2Tx, consider following band combination(s) in Rel-19.

- PC3 E-UTRA FDD + PC3 NR FDD

- PC2 E-UTRA TDD + PC3 NR TDD.

For PC1.5 UE supporting two band EN-DC with 2Tx,

Proposal 4: For PC1.5 UE supporting two band EN-DC with 2Tx,  consider following band combination(s) in Rel-19.

- PC2 E-UTRA TDD + PC2 NR TDD

For PC2 UE supporting two band EN-DC with 3Tx,

Proposal 5: For PC2 UE supporting two band EN-DC with 3Tx,  consider following band combination(s) in Rel-19.

- PC3 E-UTRA FDD + PC3 NR FDD

- PC2 E-UTRA TDD + PC3 NR TDD

For PC1.5 UE supporting two band EN-DC with 3Tx,

Proposal 6: For PC1.5 UE supporting two band EN-DC with 3Tx, consider following band combination(s) in Rel-19.

- PC3 E-UTRA FDD/TDD + PC1.5 NR TDD

- PC2 E-UTRA TDD + PC2/PC1.5 NR TDD

- References

[1] R4-2414277, “WF on HPUE for CA in TN”, Samsung

[2] R4-2410750, “WF on higher power UE”, Samsung

[3] RP-240828, “New WID : UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, ATT