3GPP TSG-RAN4 Meeting #112-bis	R4-2415887

# Hefei, China, 14th – 18th October, 2024

Source: 	Qualcomm Incorporated

Title: 	FR2-1 L3 measurement delay by optimizing RX beam sweeping factor

Agenda item:	 6.15.2.1

Document for:	Discussion

# Introduction

In this paper, we are presenting view for R19 RRM enhancement phase-5 WI.

# Discussion

## Prior Agreements

A subset of agreements and open issues from last meeting [1] are listed below.

Applicability requirement: Updated Agreement in online session: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion. UE Power class: Agreement in online session: Baseline: RAN4 to consider UE supporting FR2-1 power class 3 as first priority. Note: whether other power classes could apply the outcome of the WI discussion can be FFS after concluding on PC3. These extra FFS parts will NOT delay the WI completion. Other clarification on WID: Agreement in online session: Remove {FFS: “For UE supporting multiple-Rx simultaneous reception for L3 delay enhancement” means UE supporting “simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window. But it does not mean “UE can process multiple SSBs from different directions of the target frequency in parallel.”}. Issue 1-2-1: FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature Agreement in online session: - The conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF is that multi-Rx simultaneous reception of UE is in active mode. And it does not assume that the condition of in active mode is same as that for Rel-18 multi-Rx simultaneous reception. Issue 1-2-2: FFS: UE’s mobility status, e.g., whether HST is precluded or not Agreement in online session: - RAN4 to consider UE in non-HST case as first priority. - Note: whether or how HST case could use the outcome of the WI discussion can be FFS after concluding on non-HST case. These extra FFS parts will NOT delay the WI completion. Issue 1-2-3: FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier [Way forward]: FFS: - Option A/1a: Two searchers are occupied by this single carrier to the requirements of enhanced BSF for single carrier. Discuss CA case later. (HW, CATT, Apple, OPPO, LGE, Xiaomi) - Option B: Consider one searcher for single carrier. (MTK, ZTE, OPPO, QC, Nokia, Intel, vivo, Ericsson, Apple, Xiaomi, CATT) Issue 1-2-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC [Way forward]: FFS: - Option 1 (CATT, vivo): No need to add additional processing time due to multiple SSBs within one SMTC. - Option 2 (Apple): allow additional processing time for UE supporting multiple-Rx simultaneous reception for L3 delay enhancement if there is only one searcher available for the processing - Option 3 (Ericsson): Extra SSB post-processing time in several ms, as side effect of BSF enhancement, may be acceptable, but it depends on how much we can gain with BSF enhancement - Option 4 (Intel): - Consider UE baseband processing capabilities when specifying the L3 delay reduction for simultaneous receptions on multiple FR2 SSB-s. - Different (or whether or not) delay reduction applies when the ratio of number of SSB within a burst and time duration of the measurement periodicity varies. - Option 5 (Nokia): - If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as - a. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms - b. Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms - c. where TSSB_processing = 2 ms Issue 1-2-5: FFS: Power consumption issue (including conditions to trigger UE using FBS for L3 measurement) [Way forward]: discussion can be mainly focus on 3 directions: (1)“additional triggering for this R19 L3 measurement with FBS” (use option 8 for discussion) or (2)“L3 measurement with FBS can be activated/deactivation following on R18 mechanism” (use option 1 for discussion) or (3)“up to network indication” (use option 3 for discussion). Discuss option 1/3/8 together, and then add details from other options if needed. Issue 1-2-8: FFS: cell-centre UE or cell-edge UE [Way forward]: this issue can be discussed in issue 1-2-5. No more duplicated discussion in issue 1-2-8. Issue 1-3: Scenarios to use L3 measurement delay reduction by optimizing Rx BSF Which scenarios are considered to use L3 measurement delay reduction by optimizing Rx BSF: - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 5: Handover - Scenario 6: PSCell addition - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection - Scenario 8: SCell activation - Scenario 9: SCG activation - Scenario 10: CGI identification - Scenario 11: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB. The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded. Agreement: - Scenario 1/2/3/4/5/7 can be considered as first priority. After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the baseline scenarios 1/2/3/4/5/7, RAN4 can discuss whether and how the solutions(s) can be extended to the other scenarios. The discussion on other scenarios will NOT delay the WI completion.

## Impact of Channel Condition on Triggering Fast Beam Sweeping

It is important to determine if mobility or cell edge conditions can influence fast beam sweeping. If UE is in cell center and if it has low mobility, it is less likely to require measurement of neighbor cell. UE does not need to employ multi-panel and fast beam sweeping in this scenario. On the other hand, UE is more likely to be handed over to neighbor cell if UE is at cell edge or if UE is moving quickly. Hence, mobility or cell edge conditions should determine the triggering of fast beam sweeping.

Observation 1: If UE is in cell center and if it has low mobility, it does not need to measure neighboring cells frequently. UE does not need to employ multi-panel and fast beam sweeping in this scenario.

Observation 2: UE is more likely to be handed over to neighbor cell if UE is at cell edge or if UE is moving quickly.

Figure 1: Transition between fast beam sweeping and legacy beam sweeping

Figure 1 shows transition between fast and baseline beam sweeping based on channel conditions. Case B and case M refer to baseline beam sweeping and fast beam sweeping respectively. 40ms is the SMTC period. 8 and 4 denote the scaling factor for evaluation periods during baseline beam sweeping and fast beam sweeping respectively.

3GPP has already defined the criteria to determine cell edge and high-speed conditions for relaxed idle/inactive mode measurement and for RLM/BFD evaluation. Appendix A shows the criteria for low mobility and good serving cell quality. These criteria can be reused to determine the cell edge and high-speed conditions to apply fast beam sweeping, too.

Figure 2: Reporting of current channel condition for beam sweeping status and its impact on mobility procedures

In procedures like handover, network needs to know UE’s expected time to complete the procedure. This allows network to reserve contention free random access preambles during handover. If UE’s beam sweep factor depends on channel conditions, it may change from time to time and network needs to be aware of UE’s current channel conditions. Hence, UE needs to inform network when not cell-edge and not high-speed conditions are fulfilled at UE.

Observation 3: In some mobility procedures (e.g., handover), network needs to estimate UE’s expected time to complete the procedure. It is better for network to know when not cell-edge and not high-speed conditions are fulfilled at UE if these conditions influence triggering of fast beam sweeping.

3GPP has already introduced not cell-edge and not high-speed conditions for idle/inactive mode measurement and RLM/BFD relaxation. Appendix A shows the detailed procedures that UE can use to determine if it has fulfilled cell edge or high-speed conditions for RLM/BFD relaxation. Those same procedures can be used to determine “not cell edge (and not high-speed)” conditions and beam sweeping, too.

Observation 4: 3GPP has already introduced not cell-edge and not high-speed conditions for idle/inactive mode measurement and RLM/BFD relaxation.

Existing RLM/BFD relaxation mechanisms have introduced signaling and procedures to report RLM/BFD relaxation status. These signaling and procedures are shown in appendix B. These reports for conveying RLM/BFD relaxation status can be reused or served as a baseline for conveying beam sweeping status.

UE should not immediately report to the network after “not cell-edge and not high-speed” conditions are fulfilled. UE’s condition may change, and immediate reporting may lead to a ping-pong effect. UE should wait for a certain time to report to the network after “not cell-edge and not high-speed” conditions are fulfilled. TTT-like time window or N310 like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge (and not high-speed)” is observed by the UE, can be configured to avoid frequent status transitions and reports.

Observation 5: 3GPP has introduced reporting configurations to convey RLM/BFD relaxation status.

Observation 6: After “not cell-edge and not high-speed” condition is fulfilled for the first time, UE should wait for a certain time to report its status to the network.

UE should not relax its beam sweep factor to eight after it has received explicit signaling (e.g., FR2 CHO, FR HO, GBBR, etc.) from the network. Changing beam sweep factor after receiving these explicit signaling will confuse network regarding the expected timeline of the mobility procedure.

Observation 7: Relaxing beam sweep factor after receiving explicit signaling like HO, FR2 CHO, etc. until the completion of the procedure will confuse network regarding the procedure’s expected timeline.

Figure 2 shows how UE’s change of beam sweep factor can influence a mobility procedure. UE’s steps in this figure are shown below.

- UE initially reports its capability of fast beam sweep to network

- Later, UE receives measurement object configuration from network.

- After a certain time, UE observes that the conditions of “not cell-edge” and “not high-speed” are fulfilled.

- UE waits for a certain time, governed via a TTT-like window or N310-like timer, and reports its relaxation of beam sweep status to network.

- UE also starts using legacy beam sweeping factor from that instance.

- Later, UE receives an explicit command from network to perform a mobility procedure (e.g., handover).

- UE completes handover using legacy beam sweeping factor.

- After an application delay that corresponds to HO timeline, UE resorts to using fast beam sweeping factor.

This whole procedure can be described through the following proposal:

Proposal 1: RAN4 to adopt the following framework for the fast UE Rx beam sweeping based L3 measurement and mobility requirements:

  - NW provides the following criteria for fast beam sweeping application, and the signaling details are FFS

    - Cell edge condition: Threshold value of absolute L3 SSB-RSRP of SpCell

    - High speed condition: Threshold value of L3 SSB-RSRP variation on SpCell over a time period T

    - When the condition of not cell-edge (and not high-speed, if configured) is met, the UE is allowed to fallback Rx beam sweeping factor to the existing N value

    - Note: the existing criteria defined for the relaxed idle/inactive mode measurement and/or RLM/BFD evaluation can be reused or served as a baseline

  - Report configuration for the status of fast beam sweeping factor application, and the signaling details are FFS

    - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge (and not high-speed)” is observed by the UE, can be configured to avoid frequent status transitions and reports

    - Note: the existing report defined for RLM/BFD relaxation status can be reused or served as a baseline

  - Besides, other explicit signaling (e.g. FR2 CHO, FR HO, GBBR, etc.) may disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed.

  - FFS on the application delay of UE Rx beam sweeping factor switch

## Number of Searchers and Processing Time

UE does not need multiple searchers to simultaneously receive signals from different cells with different beams. One searcher is sufficient to simultaneously receive signals with different beams.

Observation 8: One searcher is sufficient to simultaneously receive signals with different beams.

3GPP assumes that a UE has two searchers while defining the carrier specific scaling factor in CA and DC scenarios. CA and DC are not included in the scope of this work item since this work item only focuses on single carrier. Hence, theoretically, one can assume two searchers to simultaneously process signals from multiple directions. However, this would complicate the CSSF table later if optimization of RX beam sweeping factor gets extended to CA/DC scenarios in the future. Besides, assumption of one searcher does not significantly increase the timeline of the procedures where the requirements are applicable. Hence, use of one searcher is reasonable to avoid complexity in potential future work items.

Observation 9: Although this work item focuses on single carrier, the assumption of two searchers to simultaneously process SSBs from multiple directions in this work item can potentially complicate the CSSF table if optimization of RX beam sweeping factor gets extended to CA/DC scenarios in the future.

Proposal 2: UE uses one searcher per carrier while applying enhanced beam sweeping factor.

RAN4 agreed to investigate six scenarios (scenario 1/2/3/4/5/7 of [1]) for this work item. Among these six scenarios, only the legacy handover requirement includes SSB processing time. Hence, SSB processing time (2 ms) can be extended by Rx beam sweeping factor N in the handover delay equation when UE reduces handover delay by simultaneously receiving signals from N directions using N different beams. The delay requirements in the other scenarios are quite long and they don’t include SSB processing time. Hence, those delay requirements don’t need to be extended to handle simultaneous processing of multiple SSBs.

Observation 10: Among the RAN4 agreed six scenarios, only the legacy handover requirement includes SSB processing time. The delay requirements in the other scenarios are quite long and they don’t include SSB processing time.

Proposal 3: In the handover delay equation, SSB processing time (2 ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions.

- With above proposal, FR2 – FR2 HO timeline takes the following form (changes from legacy HO timeline is shown in blue):

  - Tinterrupt = Tsearch + TIU +  Tprocessing + T∆ + M * Tmargin ms

  - Where,

    - M is the number of RX beams that UE employs to simultaneously receive signals from multiple directions.

    - Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

    - Tsearch is the time required to search the target cell when the handover command is received by the UE. If the target cell is a known cell, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = ceil(N/M)* Trs  ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = ceil(N/M)*3* Trs  ms. N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

Proposal 4: SSB processing time does not need to be introduced to handle optimized Rx BSF in following scenarios.

  - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

  - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

  - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

# Conclusions

Observation 1: If UE is in cell center and if it has low mobility, it does not need to measure neighboring cells frequently. UE does not need to employ multi-panel and fast beam sweeping in this scenario.

Observation 2: UE is more likely to be handed over to neighbor cell if UE is at cell edge or if UE is moving quickly.

Observation 3: In some mobility procedures (e.g., handover), network needs to estimate UE’s expected time to complete the procedure. It is better for network to know when not cell-edge and not high-speed conditions are fulfilled at UE if these conditions influence triggering of fast beam sweeping.

Observation 4: 3GPP has already introduced not cell-edge and not high-speed conditions for idle/inactive ode measurement and RLM/BFD relaxation.

Observation 5: 3GPP has introduced reporting configurations to convey RLM/BFD relaxation status.

Observation 6: After “not cell-edge and not high-speed” condition is fulfilled for the first time, UE should wait for a certain time to report its status to the network.

Observation 7: Relaxing beam sweep factor after receiving explicit signaling like HO, FR2 CHO, etc. until the completion of the procedure will confuse network regarding the procedure’s expected timeline.

Observation 8: One searcher is sufficient to simultaneously receive signals with different beams.

Observation 9: Although this work item focuses on single carrier, the assumption of two searchers to simultaneously process SSBs from multiple directions in this work item can potentially complicate the CSSF table if optimization of RX beam sweeping factor gets extended to CA/DC scenarios in the future.

Observation 10: Among the RAN4 agreed six scenarios, only the legacy handover requirement includes SSB processing time. The delay requirements in the other scenarios are quite long and they don’t include SSB processing time.

Proposal 1: RAN4 to adopt the following framework for the fast UE Rx beam sweeping based L3 measurement and mobility requirements:

  - NW provides the following criteria for fast beam sweeping application, and the signaling details are FFS

    - Cell edge condition: Threshold value of absolute L3 SSB-RSRP of SpCell

    - High speed condition: Threshold value of L3 SSB-RSRP variation on SpCell over a time period T

    - When the condition of not cell-edge (and not high-speed, if configured) is met, the UE is allowed to fallback Rx beam sweeping factor to the existing N value

    - Note: the existing criteria defined for the relaxed idle/inactive mode measurement and/or RLM/BFD evaluation can be reused or served as a baseline

  - Report configuration for the status of fast beam sweeping factor application, and the signaling details are FFS

    - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge (and not high-speed)” is observed by the UE, can be configured to avoid frequent status transitions and reports

    - Note: the existing report defined for RLM/BFD relaxation status can be reused or served as a baseline

  - Besides, other explicit signaling (e.g. FR2 CHO, FR HO, GBBR, etc.) may disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed.

  - FFS on the application delay of UE Rx beam sweeping factor switch

Proposal 2: UE uses one searcher per carrier while applying enhanced beam sweeping factor.

Proposal 3: In the handover delay equation, SSB processing time (2 ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions.

- With above proposal, FR2 – FR2 HO timeline takes the following form (changes from legacy HO timeline is shown in blue):

  - Tinterrupt = Tsearch + TIU +  Tprocessing + T∆ + M * Tmargin ms

  - Where,

    - M is the number of RX beams that UE employs to simultaneously receive signals from multiple directions.

    - Tmargin is time for SSB post-processing. Tmargin can be up to 2ms.

    - Tsearch is the time required to search the target cell when the handover command is received by the UE. If the target cell is a known cell, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = ceil(N/M)* Trs  ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = ceil(N/M)*3* Trs  ms. N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times.

Proposal 4: SSB processing time does not need to be introduced to handle optimized Rx BSF in following scenarios.

  - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

  - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

  - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

# Appendix A: Criteria for RLM/BFD Relaxation

The RLM/BFD relaxed measurement criteria for low mobility and good serving cell quality are defined as follows: [2]

# Appendix B: Reporting of BFD/RLM Relaxation State via UAI

UE’s reporting regarding BFD and RLM relaxation status can be summarized through the following:

- bfd-RelaxationReportingConfig > OtherConfig

  - UE uses the config to report the relaxation state of BFD measurements.

- rlm-RelaxationReportingConfig > OtherConfig

  - UE uses the config to report the relaxation state of RLM measurements.

The requirements for providing RLM/BFD relaxation status are defined as follows in RAN2 and RAN4 specs [2] [3].

NR UE employs the following timers to report RLM/BFD relaxation status [2].

# References

[1] R4-2414026, Way Forward for NR_RRM_Ph5, Apple, RAN4 #112

[2] 38.331 V 17.7.0

[3] 38.133 V 17.11.0