3GPP TSG-RAN4 Meeting #112-bis	R4-2415892

# Hefei China, 14th – 18th October, 2024

Source: 	Qualcomm Incorporated

Title: 	RRM Core Requirements for Positioning Accuracy

Agenda item:	6.17.5.1

Document for:	Discussion

# Introduction

In this paper, we focus on RRM core requirements for positioning accuracy

# Prior Agreements

Following agreement happened during the last meeting [1].

Issue 3-2: Requirements for case 2a Agreement: RAN4 should not define any positioning accuracy requirements because positioning is LMF based Issue 3-3: Reported metrics for case 2a Agreement: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS Issue 3-5: Reported metrics for case 2b Agreement: postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported. Issue 3-6: Reported metrics for case 3a/3b Agreement: postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

The AI/ML WID shows the following priority for different cases of positioning [2]:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

# Discussion

## Case 2a and LOS/NLOS Metric

A paper submitted to RAN plenary [3] quoted the recent guidance from RAN2 chair regarding 2nd priority use cases:

#### 8.1.2.3 LCM for Positioning use case Contributions should focus on UE-sided model, but can discuss NW-sided model and should focus on 1st priority positioning use cases

Based on this RAN2 guidance, RAN plenary included the following note in AI/ML WID [2]: “Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.”

Since neither RAN1 nor RAN2 will discuss proposals related to 2nd priority work items (including LOS/NLOS metric of case 2a) in Q4 2024, RAN4 should postpone defining requirement for LOS/NLOS until RAN1/RAN2 make further progress regarding case 2a and this metric.

Observation 1: RAN2 chair has guided companies to focus on 1st priority positioning use cases. RAN plenary has suggested RAN1 not to discuss any proposal targeting 2nd priority issues in Q4 of 2024.

Proposal 1: RAN4 postpones defining requirements for LOS/NLOS metric until RAN1 and RAN2 make further progress regarding case 2a, including this metric.

## Case 1

UE based positioning with UE side model is a 1st priority item of AI/ML positioning. RAN4 accuracy tests were not defined for legacy (non AI-ML) UE based positioning algorithm with UE side model.

However, legacy spec has previously defined the signalling for reporting UE’s location via DL-TDoA IE in non-AI-ML scenario [4]. It is expected that the same DL-TDoA will be used to report UE’s position with UE-sided AI/ML model, too.

-- ASN1START NR-DL-TDOA-LocationInformation-r16 ::= SEQUENCE { measurementReferenceTime-r16 CHOICE { systemFrameNumber-r16 NR-TimeStamp-r16, utc-time-r16 UTCTime, ... } OPTIONAL, ..., [[ locationCoordinates-r17 LocationCoordinates OPTIONAL, -- Cond batch1 locationSource-r17 LocationSource-r13 OPTIONAL -- Cond batch2 ]] } -- ASN1STOP

Since 3GPP has classified UE-based positioning a 1st priority work item and legacy non-AI-ML has introduced signaling IE for reporting UE’s location, RAN4 should study to define requirements for Case 1.

Observation 2: UE based positioning with UE side model is a 1st priority item of AI/ML positioning.

Observation 3: Legacy spec has previously defined the signalling for reporting UE’s location via DL-TDoA IE in non-AI-ML scenario.

Proposal 2: RAN4 studies defining requirements for Case 1, i.e., UE-based positioning with UE-side model.

# Conclusions

Observation 1: RAN2 chair has guided companies to focus on 1st priority positioning use cases. RAN plenary has suggested RAN1 not to discuss any proposal targeting 2nd priority issues in Q4 of 2024.

Observation 2: UE based positioning with UE side model is a 1st priority item of AI/ML positioning.

Observation 3: Legacy spec has previously defined the signalling for reporting UE’s location via DL-TDoA IE in non-AI-ML scenario.

Proposal 1: RAN4 postpones defining requirements for LOS/NLOS metric until RAN1 and RAN2 make further progress regarding case 2a, including LOS/NLOS metric.

Proposal 2: RAN4 studies defining requirements for Case 1, i.e., UE-based positioning with UE-side model.

# References

[1] R4-2414323, “WF on AI/ML”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112.

[2] RP-2423999, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN 105, Sep 2024.

[3] RP-242030, “Checkpoint for AI/ML PHY”, Ericsson.

[4] 37.355, “LTE Positioning Protocol”, V 18.3.0