3GPP TSG-RAN4 Meeting #112-bis	R4-2415893

# Hefei China, 14th – 18th October, 2024

Source: 	Qualcomm Incorporated

Title: 	RRM Core Requirements for AI/ML Beam Management

Agenda item:	6.17.4.1

Document for:	Discussion

# Introduction

This paper focuses on RRM core requirements for AI/ML based beam management.

Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. impact on different prediction accuracy metrics should be taken into account studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy. Issue 2-8:Beam prediction and TCI state related issues - Proposals - Option 1: Tx beam prediction does not imply that UE knows the best UE Rx beam to receive the predicted beam - TCI state for the predicted beam is unknown unless it has been explicitly measured within 1280ms - Option 2: UE knows the Rx beam paired with the predicted Tx beam - FFS whether the TCI state associated with a predicted Tx beam is known or unknown - Option 3: UE knows the Rx beam paired with the predicted Tx beam - TCI state should be considered known , FFS whether side conditions for known TCI state should be modified (relaxed) for this case - Option 4: others

3GPP studied different performance metrics for AI/ML based beam management during study item discussion. These metrics are captured in [4]:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE - NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

RAN1 made the following agreements in [5][6]:

- RAN1#117: Working Assumption - Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study). - RAN1#118 agreement: For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list are associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

# Discussion

Based on previous agreements and the contents of work item document, RAN4 should define AI-ML related RRM core requirements in, at least, following three scenarios:

- Measurement period for prediction

- Accuracy of predicted metrics

- TCI state status of predicted TX beam

Proposal 1: RAN4 should define AI-ML beam management related RRM core requirements in, at least, following three areas:

- Measurement period for prediction

- Accuracy of predicted metrics

- TCI state status of predicted TX beam

## 2.1 Measurement Period for Prediction

Legacy RAN4 RRM spec allows UE to measure reference signals for a certain duration to meet corresponding accuracy requirements (e.g., L1-RSRP accuracy, SS-RSRP accuracy, etc.). Similarly, spec needs to allow UE to measure setB beams for a certain duration to predict beams and associated metrics of setA.

Figure 1 shows this concept of evaluation period for predicting AI-ML based beams and associated metrics. Spec needs to provide, at least, K samples of SSB during setB measurement to ensure accuracy of prediction. Some UEs can implement a more complex model and may require a smaller number of SSB samples to meet prediction accuracy metric. However, that should be transparent to the perspective of RAN4 test.

Figure 1: Evaluation Period for Predicting AI-ML Based Beams and Associated Metrics

Legacy RAN4 RRM spec introduces a sharing factor that extends evaluation period when relevant reference signals overlap with SMTC and measurement gap in time domain. Similarly, spec should allow UE to extend evaluation period for beam prediction if the reference signals of setB overlap with SMTC and measurement gaps. Evaluation period for beam prediction should also depend on the configuration of DRX.

Observation 1: Legacy RAN4 RRM spec allows UE to measure reference signals for a certain duration to meet corresponding accuracy requirements. The required duration depends on 1) SMTC and measurement gap based sharing factor and 2) DRX cycle.

RAN1 has introduced the concept of association ID that conveys the properties of a DL Tx beam set. UE may require different evaluation period for prediction for different properties of a DL Tx beam set. For example, UE may require less samples to measure setB beams if the number of beams are similar in setB and setA. On the other hand, UE may require more samples to measure setB beams if the number of setA beams is much larger than that of setB.

Observation 2: RAN1 has introduced the concept of association ID that conveys the properties of a DL Tx beam set. UE may require different evaluation period for prediction for different properties of a DL Tx beam set.

Similarly, UE may require different evaluation period for different prediction metrics (top beam ID, L1-RSRP accuracy, etc.). Required evaluation period may vary for different levels of accuracy.

Proposal 2: Spec allows a minimum duration of measurements on the SSBs/CSI-RS before UE reports a CSI report containing predicted metrics (i.e., channel characteristics).

- Note: Measurement period should consider measurement gaps, SMTCs, DRX operation, etc.

- FFS: Minimum duration may depend on other factors e.g., associated ID, level of accuracy, etc.

## 2.2 Selection and Accuracy of Predicted Metrics

3GPP focused on the following prediction metrics in [3]

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered. For metrics for beam management requirements/tests, the following test metrics/KPIs are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options The overhead/latency reduction should be considered for the requirements as the side condition.

Option 2 is the simplest option for testing. This has practical use cases, too. Network can use UE’s predicted top beam to change UE’s TCI state. Option 2’s drawback lies in the fact that option 2 (top-1 (%) version of option 2) does not distinguish between following two scenarios:

- UE’s top-1 predicted beam is adjacent to, but not same as, the top-1 strongest beam

- UE’s top-1 predicted beam is far from the top-1 strongest beam.

This drawback of option 2 is addressed in option 3. UE can pass option 3 if the maximum RSRP among UE’s top-K predicted beams lies within X dB of the RSRP of the strongest beam. Hence, option 3 allows UE to pass a test if UE’s top-1 predicted beam is adjacent to the top-1 strongest beam.

Observation 3: Option 2’s usefulness lies in the fact that network can use UE’s predicted top beam to change UE’s TCI state.

Observation 4: Option 2’s drawback lies in the fact that its pass/fail criteria does not depend on whether the top predicted beams are adjacent to the strongest beams or not. Option 3 does not suffer from this drawback and retains the same usefulness as option 2.

RAN1 also discussed these different prediction metrics during the last meeting and made the following agreement in [6]:

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

Hence, RAN4 can either wait for RAN1 to down-select prediction metrics. Or RAN4 can decide on its own. In the latter case, RAN4 should select, at least, option 2 and option 3 of 38.843 as predicted metrics.

Proposal 3: Regarding down-selection of prediction metrics in spatial beam prediction, RAN4 takes one of the following two approaches:

- Approach 1: Wait for RAN1 to down-select the prediction metrics.

- Approach 2: Down-select the prediction metrics for RAN4 tests in RAN4.

  - In approach 2, RAN4 should select option 2 and option 3 of 38.843.

## 2.3 TCI State Status of Predicted Tx Beam

During the last meeting, RAN4 discussed whether the TCI state of predicted Tx beam is known or unknown. RAN4 also discussed whether UE knows the RX beam paired with the predicted TX beam.

3GPP is considering L1-RSRP accuracy as a potential prediction metric for AI-ML based beam management. For both temporal and spatial beam prediction, UE must know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam. If UE does not know its RX beam corresponding to the gNB Tx beam, UE’s predicted L1-RSRP RSRP will be drastically different from the actual L1-RSRP.

Observation 5: 3GPP is considering L1-RSRP accuracy as a potential prediction metric for AI-ML based beam management. UE has to know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam.

3GPP is also considering top-1 (%) beam or top-K (%) beam as a prediction metric. UE has to predict the top beams of setA based on what UE would see at its baseband. To compare between gNB beams, UE needs to have some idea of the Rx gain associated with them. Hence, UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Observation 6: To compare between gNB beams of setA, UE needs to have some idea of the Rx gain associated with them. UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Some companies mentioned that the AI-ML model considered in RAN1 uses (gNB Tx beam, measured RSRP) as inputs and the model does not explicitly indicate UE’s Rx beam. RAN1 assumed that UE knows its corresponding RX beam for the output of the AI-ML model, including predicted gNB Tx beam. That’s why, RAN1 did not explicitly include UE’s RX beam in the AI-ML model.

Observation 7: Some companies mentioned that the AI-ML model considered in RAN1 uses (gNB Tx beam, measured RSRP) as inputs and the model does not explicitly indicate UE’s Rx beam. RAN1 assumed that UE knows its corresponding RX beam for the output of the AI-ML model, including predicted gNB Tx beam.

RAN4 needs to further discuss whether the TCI state corresponding to predicted TX beam is known or unknown.

RAN4 spec differentiates between known and unknown condition for TCI state in the following way [7]:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

As above text shows, the SNR of the SSB associated with the TCI state needs to be >= -3 dB for TCI state to be known. These types of side conditions need to revisited to determine if the TCI state corresponding to the strongest beam is known or unknown.

Proposal 4: UE knows the RX beam paired with the predicted Tx beam.

  - FFS: Whether TCI state should be considered known.

  - FFS: Whether side conditions for known TCI state should be modified (relaxed) for this case.

# Conclusions

Observation 1: Legacy RAN4 RRM spec allows UE to measure reference signals for a certain duration to meet corresponding accuracy requirements. The required duration depends on SMTC and measurement gap-based sharing factor, and DRX cycle.

Observation 2: RAN1 has introduced the concept of association ID that conveys the properties of a DL Tx beam set. UE may require different evaluation period for prediction for different properties of a DL Tx beam set.

Observation 3: Option 2’s usefulness lies in the fact that network can use UE’s predicted top beam to change UE’s TCI state.

Observation 4: Option 2’s drawback lies in the fact that its pass/fail criteria does not depend on whether the top predicted beams are adjacent to the strongest beams or not. Option 3 does not suffer from this drawback.

Observation 5: 3GPP is considering L1-RSRP accuracy as a potential prediction metric for AI-ML based beam management. UE has to know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam.

Observation 6: To compare between gNB beams of setA, UE needs to have some idea of the Rx gain associated with them. UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Observation 7: Some companies mentioned that the AI-ML model considered in RAN1 uses (gNB Tx beam, measured RSRP) as inputs and the model does not explicitly indicate UE’s Rx beam. RAN1 assumed that UE knows its corresponding RX beam for the output of the AI-ML model, including predicted gNB Tx beam.

Proposal 1: RAN4 should define AI-ML beam management related RRM core requirements in, at least, following three areas:

- Measurement period for prediction

- Accuracy of predicted metrics

- TCI state status of predicted TX beam

Proposal 2: Spec allows a minimum duration of measurements on the SSBs/CSI-RS before UE reports a CSI report containing predicted metrics (i.e., channel characteristics).

- Note: Measurement period should consider measurement gaps, SMTCs, DRX operation, etc.

- FFS: Minimum duration may depend on other factors e.g., associated ID, level of accuracy, etc.

Proposal 3: Regarding down-selection of prediction metrics, RAN4 takes one of the following two approaches:

- Approach 1: Wait for RAN1 to down-select the prediction metrics.

- Approach 2: Down-select the prediction metrics for RAN4 tests in RAN4.

  - In approach 2, RAN4 should select option 2 and option 3 of 38.843.

Proposal 4: UE knows the RX beam paired with the predicted Tx beam.

  - FFS: Whether TCI state should be considered known.

  - FFS: Whether side conditions for known TCI state should be modified (relaxed) for this case.

# References

- R4-2414446, ‘AI/ML ad-hoc meeting minutes’, Moderator (Qualcomm)

- R4-2414323, ‘WF on AI/ML’, Qualcomm

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP TSG RAN #105.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #117.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #118.

- 38.133, Requirements for support of radio resource management, V 17.11.0.