3GPP TSG-RAN4 Meeting #112bis	R4-2415894

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	LG Electronics

Title:	Tx requirements for SL intra-band contiguous CA

Agenda item:	6.3.3.2

Document for:	Discussion

Introduction

In this paper, we provide our views on UE RF requirements for intra-band contiguous SL CA for the Band n47 based on the approved WF[1].

Discussion

Based on the approved WFs[1][2], we would like to discuss the UE RF requirements for SL intra-band contiguous CA in n47.

General - For PC2 SL intra-band C CA, reuse the same channel bandwidth of PC3 SL intra-band C CA MOP - For PC2 intra-band contiguous SL CA, the following two aspects should be specified in Rel-19 - Minimum requirements for Power Class 2 should be applicable for SL intra-band C CA combination configuration, and update the maximum output power for sidelink CA - Define SL intra-band C CA PC2 MOP as total transmitted power (Per UE) for PC2 SL CA UE MPR - MPR/A-MPR requirements for SL intra-band C CA with PC2 should be specified in Rel-19 referring to the methodology of that with PC3 in Rel-18 including, PSSCH/PSCCH, PSFCH and S-SSB - For PSSCH/PSCCH: specify MPR for PC2 contiguous CA with contiguous/non-contiguous RB allocation, including inner and outer. - For PSFCH: specify MPR for PC2 contiguous CA based on different range of R, R is the ratio of the gap bandwidth between the two PSFCH transmitted on the two intra-band carrier by the total bandwidth of the two carrier - For S-SSB: specify MPR for PC2 contiguous CA by single S-SSB and two S-SSB, separately Simulation assumption for MPR - WF: Keep 25dBc carrier leakage as agreed WF(R4-2406611) for PC2 SL intra-band C CA Architecture for MPR/A-MPR evaluation - The PC2 UE RF architecture of NR intra-band contiguous CA for MPR can be found as below for information (1 PA, dual Tx(Tx diversity), and dualPA) - High priority for MPR evaluations - 1 PA : 1x26dBm + 1LO - Dual Tx : 2x23dBm + 1LO - Second priority for MPR evaluations - dualPA : 2x23dBm + 2LO ACLR - Reuse 31dB ACLR for intra-band contiguous PC2 CA ΔPPowerClass,SL CA - Not to introduce ΔPPowerClass,SL CA due to no SAR related issues for SL CA on the ITS band Regulation (FCC) - Study and specify the related A-MPR requirements with NS_52 after final FCC announcement for the additional emission limits in US Regulation (EU) - No A-MPR requirement based on EU regulation is needed for SL intra-band C CA Other Tx requirements - WF: Reuse the requirements of PC3 SL intra-band C CA - Minimum output / transmit OFF power /transmit ON/OFF time mask - Power control - Transmit signal quality - SEM/ - General spurious emissions/UE-to-UE coexistence SE - Transmit intermodulation Rx requirements - The Rx requirements including REFSENS, ACS, maximum input power and blockings for PC3 SL intra-band C CA in current Spec shall apply to the PC2 SL intra-band C CA directly

PC2 MPR (PSSCH)

UE RF architecture

Architecture #1-1, #1-2, and #2-1 in Table 2-1 are considered for MPR evaluation.

Table 2-1. UE Architecture for PC2 SL contiguous CA

| Arch | description | Remarks |
| --- | --- | --- |
| #1-1 | 1x26dBm PA+ 1LO | Single–Tx |
| #1-2 | 2x23dBm PA+ 1LO | dual-Tx or txDiversity |
| #2-2 | 2x23dBm PA+ 2LO | dualPA-Architecture |

Simulation assumption

The simulation assumption in Table 2-2 is considered.

Table 2-2. Simulation assumption

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz aggregated CBW: 10+10, 10+30, 20+20, 20+30, 20+40, 30+30 (MHz) |
| Maximum output power for aggregated CBW | 26dBm |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| ACLR | 31dBc |
| Carrier leakage | 25dBc |
| IQ image | 34dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

Evaluation scenario

The PSSCH MPR evaluation scenarios in Table 2-3 and Table 2-4 are considered for contiguous RB allocation and non-contiguous RB allocation.

Table 2-3: SL contiguous CA MPR evaluation scenarios for PSSCH with contiguous RB allocation

| Aggregated CBW | Scenario | CC1 | CC2 | Inner/Outer RB allocation | SCS |
| --- | --- | --- | --- | --- | --- |
| 10MHz + 10MHz | 1 | 10RB42 | 10RB0 | Inner | 15 |
|  | 2 | 10RB42 | 12RB0 | Inner | 15 |
|  | 3 | 10RB42 | 15RB0 | Inner | 15 |
|  | 4 | 10RB42 | 25RB0 | Inner | 15 |
|  | 5 | 10RB42 | 30RB0 | Inner | 15 |
|  | 6 | 10RB42 | 36RB0 | Outer | 15 |
|  | 7 | 10RB42 | 50RB0 | Outer | 15 |
|  | 8 | 12RB40 | 36RB0 | Outer | 15 |
|  | 9 | 25RB27 | 36RB0 | Outer | 15 |
|  | 10 | 40RB12 | 40RB0 | Outer | 15 |
|  | 11 | 50RB2 | 50RB0 | Outer | 15 |
| 20MHz + 30MHz | 12 | 10RB41 | 10RB0 | Inner | 30 |
|  | 13 | 10RB41 | 12RB0 | Inner | 30 |
|  | 14 | 10RB41 | 25RB0 | Inner | 30 |
|  | 15 | 10RB41 | 30RB0 | Inner | 30 |
|  | 16 | 10RB41 | 48RB0 | Inner | 30 |
|  | 17 | 10RB41 | 50RB0 | Outer | 30 |
|  | 18 | 25RB26 | 36RB0 | Outer | 30 |
|  | 19 | 36RB15 | 36RB0 | Outer | 30 |
|  | 20 | 40RB11 | 40RB0 | Outer | 30 |
|  | 21 | 50RB1 | 75RB0 | Outer | 30 |
| 20MHz + 40MHz | 22 | 10RB41 | 10RB0 | Inner | 30 |
|  | 23 | 10RB41 | 25RB0 | Inner | 30 |
|  | 24 | 10RB41 | 36RB0 | Inner | 30 |
|  | 25 | 10RB41 | 48RB0 | Inner | 30 |
|  | 26 | 10RB41 | 60RB0 | Inner | 30 |
|  | 27 | 10RB41 | 70RB0 | Outer | 30 |
|  | 28 | 25RB26 | 36RB0 | Outer | 30 |
|  | 29 | 36RB15 | 70RB0 | Outer | 30 |
|  | 30 | 40RB11 | 90RB0 | Outer | 30 |
|  | 31 | 50RB1 | 105RB0 | Outer | 30 |
| 30MHz + 40MHz | 32 | 10RB68 | 10RB0 | Inner | 30 |
|  | 33 | 10RB68 | 25RB0 | Inner | 30 |
|  | 34 | 10RB68 | 36RB0 | Inner | 30 |
|  | 35 | 10RB68 | 48RB0 | Inner | 30 |
|  | 36 | 10RB68 | 60RB0 | Inner | 30 |
|  | 37 | 10RB68 | 70RB0 | Outer | 30 |
|  | 38 | 10RB68 | 105RB0 | Outer | 30 |
|  | 39 | 25RB53 | 70RB0 | Outer | 30 |
|  | 40 | 36RB42 | 90RB0 | Outer | 30 |
|  | 41 | 40RB38 | 90RB0 | Outer | 30 |
|  | 42 | 75RB3 | 105RB0 | Outer | 30 |
| 20MHz + 20MHz | 43 | 10RB41 | 10RB0 | Inner | 30 |
|  | 44 | 10RB41 | 12RB0 | Inner | 30 |
|  | 45 | 10RB41 | 15RB0 | Inner | 30 |
|  | 45 | 10RB41 | 25RB0 | Inner | 30 |
|  | 46 | 10RB41 | 30RB0 | Inner | 30 |
|  | 47 | 10RB41 | 36RB0 | Outer | 30 |
|  | 48 | 10RB41 | 50RB0 | Outer | 30 |
|  | 49 | 12RB39 | 36RB0 | Outer | 30 |
|  | 50 | 25RB26 | 36RB0 | Outer | 30 |
|  | 51 | 40RB11 | 40RB0 | Outer | 30 |
|  | 52 | 50RB1 | 50RB0 | Outer | 30 |
| 10MHz + 30MHz | 53 | 10RB41 | 10RB0 | Inner | 30 |
|  | 54 | 10RB41 | 12RB0 | Inner | 30 |
|  | 55 | 10RB41 | 15RB0 | Inner | 30 |
|  | 56 | 10RB41 | 20RB0 | Inner | 30 |
|  | 57 | 10RB41 | 25RB0 | Outer | 30 |
|  | 58 | 10RB41 | 50RB0 | Outer | 30 |
|  | 59 | 10RB41 | 75RB0 | Outer | 30 |
|  | 60 | 12RB12 | 36RB0 | Outer | 30 |
|  | 61 | 15RB9 | 60RB0 | Outer | 30 |
|  | 62 | 20RB4 | 75RB0 | Outer |  |
|  | 63 | 24RB0 | 75RB0 | Outer | 30 |

Table 2-4: SL contiguous CA MPR evaluation scenarios for PSSCH with non-contiguous RB allocation

| Aggregated CBW | Scenario | CC1 | CC2 | Inner/Outer1/Outer2 RB allocation | SCS |
| --- | --- | --- | --- | --- | --- |
| 10MHz + 10MHz | 1 | 10RB0 | 10RB42 | Outer2 | 15 |
|  | 2 | 10RB18 | 10RB22 | Outer2 | 15 |
|  | 3 | 10RB19 | 10RB22 | Outer2 | 15 |
|  | 4 | 10RB27 | 10RB14 | Outer2 | 15 |
|  | 5 | 10RB33 | 10RB7 | Outer1 | 15 |
|  | 6 | 10RB34 | 10RB7 | Outer1 | 15 |
|  | 7 | 10RB42 | 10RB1 | Inner | 15 |
|  | 8 | 25RB0 | 25RB27 | Outer2 | 15 |
|  | 9 | 25RB19 | 25RB8 | Outer2 | 15 |
|  | 10 | 25RB20 | 25RB8 | Outer2 | 15 |
|  | 11 | 25RB27 | 25RB1 | Outer1 | 15 |
|  | 12 | 30RB0 | 30RB22 | Outer2 | 15 |
|  | 13 | 30RB19 | 30RB3 | Outer2 | 15 |
|  | 14 | 30RB19 | 30RB2 | Outer2 | 15 |
|  | 15 | 30RB22 | 30RB1 | Outer1 | 15 |
|  | 16 | 50RB1 | 50RB0 | Outer2 | 15 |
| 20MHz + 30MHz | 17 | 10RB0 | 10RB68 | Outer2 | 30 |
|  | 18 | 10RB15 | 10RB32 | Outer2 | 30 |
|  | 19 | 10RB15 | 10RB31 | Outer2 | 30 |
|  | 20 | 10RB32 | 10RB10 | Outer1 | 30 |
|  | 21 | 10RB34 | 10RB9 | Outer1 | 30 |
|  | 22 | 10RB34 | 10RB8 | Outer1 | 30 |
|  | 23 | 10RB41 | 10RB1 | Inner | 30 |
|  | 24 | 25RB0 | 25RB53 | Outer2 | 30 |
|  | 25 | 25RB20 | 25RB25 | Outer2 | 30 |
|  | 26 | 25RB21 | 25RB25 | Outer2 | 30 |
|  | 27 | 25RB26 | 25RB1 | Outer1 | 30 |
|  | 28 | 30RB0 | 30RB48 | Outer2 | 30 |
|  | 29 | 30RB15 | 30RB12 | Outer2 | 30 |
|  | 30 | 30RB15 | 30RB11 | Outer2 | 30 |
|  | 31 | 30RB21 | 30RB1 | Outer1 | 30 |
|  | 32 | 50RB0 | 50RB28 | Outer2 | 30 |
|  | 33 | 50RB0 | 50RB0 | Outer2 | 30 |
|  | 34 | 50RB0 | 78RB0 | Outer2 | 30 |
| 20MHz + 40MHz | 35 | 10RB0 | 10RB96 | Outer2 | 30 |
|  | 36 | 10RB4 | 10RB30 | Outer2 | 30 |
|  | 37 | 10RB5 | 10RB30 | Outer2 | 30 |
|  | 38 | 10RB37 | 10RB15 | Outer1 | 30 |
|  | 39 | 10RB38 | 10RB15 | Inner | 30 |
|  | 40 | 10RB41 | 10RB1 | Inner | 30 |
|  | 41 | 25RB0 | 25RB81 | Outer2 | 30 |
|  | 42 | 25RB20 | 25RB38 | Outer2 | 30 |
|  | 43 | 25RB20 | 25RB37 | Outer2 | 30 |
|  | 44 | 25RB26 | 25RB1 | Outer1 | 30 |
|  | 45 | 30RB0 | 30RB76 | Outer2 | 30 |
|  | 46 | 30RB15 | 30RB26 | Outer2 | 30 |
|  | 47 | 30RB15 | 30RB25 | Outer2 | 30 |
|  | 48 | 30RB21 | 30RB1 | Outer1 | 30 |
|  | 49 | 50RB0 | 50RB56 | Outer2 | 30 |
|  | 50 | 50RB0 | 50RB0 | Outer2 | 30 |
|  | 51 | 50RB0 | 105RB0 | Outer2 | 30 |
| 30MHz + 40MHz | 52 | 10RB0 | 10RB96 | Outer2 | 30 |
|  | 53 | 10RB13 | 10RB30 | Outer2 | 30 |
|  | 54 | 10RB14 | 10RB30 | Outer1 | 30 |
|  | 55 | 10RB50 | 10RB15 | Outer1 | 30 |
|  | 56 | 10RB51 | 10RB15 | Inner | 30 |
|  | 57 | 10RB68 | 10RB1 | Inner | 30 |
|  | 58 | 25RB0 | 25RB81 | Outer2 | 30 |
|  | 59 | 25RB28 | 25RB37 | Outer2 | 30 |
|  | 60 | 25RB28 | 25RB36 | Outer1 | 30 |
|  | 61 | 25RB51 | 25RB1 | Outer1 | 30 |
|  | 62 | 25RB52 | 25RB1 | Inner | 30 |
|  | 63 | 30RB0 | 30RB76 | Outer2 | 30 |
|  | 64 | 30RB24 | 30RB26 | Outer2 | 30 |
|  | 65 | 30RB24 | 30RB25 | Outer1 | 30 |
|  | 66 | 30RB48 | 30RB1 | Outer1 | 30 |
|  | 67 | 75RB0 | 50RB56 | Outer2 | 30 |
|  | 68 | 75RB0 | 50RB0 | Outer2 | 30 |
|  | 69 | 75RB0 | 105RB0 | Outer2 | 30 |
| 20MHz + 20MHz | 70 | 10RB0 | 10RB41 | Outer2 | 30 |
|  | 71 | 10RB17 | 10RB22 | Outer2 | 30 |
|  | 72 | 10RB18 | 10RB22 | Outer1 | 30 |
|  | 73 | 10RB27 | 10RB14 | Outer1 | 30 |
|  | 74 | 10RB33 | 10RB7 | Outer1 | 30 |
|  | 75 | 10RB34 | 10RB7 | Inner | 30 |
|  | 76 | 10RB41 | 10RB1 | Inner | 30 |
|  | 77 | 25RB0 | 25RB26 | Outer2 | 30 |
|  | 78 | 25RB18 | 25RB8 | Outer2 | 30 |
|  | 79 | 25RB19 | 25RB8 | Outer1 | 30 |
|  | 80 | 25RB26 | 25RB1 | Outer1 | 30 |
|  | 81 | 30RB0 | 30RB21 | Outer2 | 30 |
|  | 82 | 30RB18 | 30RB3 | Outer1 | 30 |
|  | 83 | 30RB18 | 30RB2 | Outer1 | 30 |
|  | 84 | 30RB21 | 30RB1 | Outer2 | 30 |
|  | 85 | 50RB0 | 50RB0 | Outer2 | 30 |
| 10MHz + 30MHz | 86 | 10RB0 | 10RB68 | Outer2 | 30 |
|  | 87 | 10RB6 | 10RB32 | Outer2 | 30 |
|  | 88 | 10RB6 | 10RB31 | Outer1 | 30 |
|  | 89 | 10RB14 | 10RB10 | Outer1 | 30 |
|  | 90 | 10RB14 | 10RB1 | Outer1 | 30 |
|  | 91 | 10RB0 | 12RB53 | Outer2 | 30 |
|  | 92 | 10RB3 | 12RB25 | Outer2 | 30 |
|  | 93 | 25RB4 | 12RB25 | Outer1 | 30 |
|  | 94 | 25RB12 | 12RB1 | Outer1 | 30 |
|  | 95 | 25RB0 | 30RB48 | Outer2 | 30 |
|  | 96 | 25RB3 | 30RB7 | Outer2 | 30 |
|  | 97 | 30RB3 | 30RB6 | Outer1 | 30 |
|  | 98 | 30RB12 | 30RB1 | Outer1 | 30 |
|  | 99 | 30RB0 | 50RB28 | Outer2 | 30 |
|  | 100 | 30RB0 | 50RB1 | Outer2 | 30 |
|  | 101 | 50RB0 | 75RB1 | Outer2 | 30 |

Simulation results for contiguous RB allocations

Table 2-5, Table 2-6, and Table 2-7 show the MPR simulation results for the SL contiguous CA scenarios of contiguous RB allocations with architecture #1-1, #1-2, and #2-1 in Table 2-1 respectively.

Table 2-5: PSSCH/PSCCH MPR simulation results for contiguous RB allocations with 1x26dBm+1LO

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 0.0 | 0.0 | 0.0 | 0.0 | 0.6 | 1.7 | 3.1 | 2.1 | 2.1 | 2.4 | 3.1 |  |  |  |  |  |  |  |
|  | '16QAM' | 0.9 | 0.6 | 0.6 | 0.6 | 0.6 | 2.1 | 3.1 | 2.1 | 2.1 | 2.4 | 3.1 |  |  |  |  |  |  |  |
|  | '64QAM' | 2.4 | 2.1 | 2.4 | 2.4 | 2.1 | 2.1 | 3.1 | 2.1 | 2.1 | 2.4 | 3.1 |  |  |  |  |  |  |  |
|  | '256QAM' | 4.6 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 |  |  |  |  |  |  |  |
| '20MHz+30MHz' | Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 0.0 | 0.0 | 0.0 | 0.0 | 0.6 | 0.6 | 0.8 | 2.1 | 2.4 | 3.1 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 0.3 | 0.3 | 0.6 | 0.6 | 0.6 | 0.8 | 0.8 | 2.1 | 2.4 | 3.1 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 1.7 | 2.1 | 2.1 | 2.1 | 2.1 | 2.1 | 2.1 | 2.1 | 2.4 | 3.1 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 |  |  |  |  |  |  |  |  |
| '20MHz+40MHz' | Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.6 | 1.1 | 2.4 | 2.7 | 3.1 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 0.0 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 1.1 | 2.4 | 2.7 | 3.1 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 1.7 | 2.1 | 2.1 | 2.1 | 2.1 | 2.4 | 2.1 | 2.4 | 2.7 | 3.1 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 |  |  |  |  |  |  |  |  |
| '30MHz+40MHz' | Scenario # | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 | #41 | #43 |  |  |  |  |  |  |  |
|  | 'QPSK' | 0.0 | 0.0 | 0.0 | 0.0 | 0.8 | 1.7 | 3.1 | 1.7 | 2.1 | 2.4 | 3.1 |  |  |  |  |  |  |  |
|  | '16QAM' | 0.3 | 0.3 | 0.3 | 0.6 | 0.8 | 1.7 | 3.1 | 1.7 | 2.1 | 2.7 | 3.1 |  |  |  |  |  |  |  |
|  | '64QAM' | 1.7 | 2.1 | 1.7 | 2.1 | 2.1 | 2.1 | 3.1 | 2.1 | 2.1 | 2.4 | 3.1 |  |  |  |  |  |  |  |
|  | '256QAM' | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 |  |  |  |  |  |  |  |
| '20MHz+20MHz' | Scenario # | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 |
|  | 'QPSK' | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.6 | 3.1 | 1.1 | 2.7 | 2.7 | 3.1 |
|  | '16QAM' | 0.0 | 0.6 | 0.6 | 0.6 | 0.6 | 0.6 | 3.1 | 1.1 | 2.7 | 2.7 | 3.1 |
|  | '64QAM' | 1.7 | 2.1 | 2.1 | 2.1 | 2.1 | 2.4 | 3.1 | 2.1 | 2.7 | 2.7 | 3.1 |
|  | '256QAM' | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 |
| '10MHz+30MHz' | Scenario # | #55 | #56 | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 | #65 |
|  | 'QPSK' | 0.3 | 0.8 | 1.1 | 1.4 | 1.4 | 1.7 | 3.1 | 1.7 | 2.4 | 3.1 | 3.1 |
|  | '16QAM' | 0.3 | 0.8 | 1.1 | 1.4 | 1.4 | 1.7 | 3.1 | 1.7 | 2.4 | 3.1 | 3.1 |
|  | '64QAM' | 1.7 | 2.1 | 2.1 | 2.1 | 2.1 | 2.1 | 3.1 | 2.1 | 2.4 | 3.1 | 3.1 |
|  | '256QAM' | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 |

Table 2-6: PSSCH/PSCCH MPR simulation results for contiguous RB allocations with 2x23dBm+1LO

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 0.6 | 0.6 | 0.6 | 0.6 | 1.8 | 3.0 | 4.7 | 3.3 | 3.6 | 4.3 | 4.7 |  |  |  |  |  |  |  |
|  | '16QAM' | 1.8 | 1.8 | 2.1 | 1.8 | 1.8 | 3.3 | 4.7 | 3.3 | 3.6 | 4.3 | 4.7 |  |  |  |  |  |  |  |
|  | '64QAM' | 3.6 | 3.6 | 3.6 | 3.6 | 3.3 | 3.3 | 4.7 | 3.3 | 3.3 | 4.3 | 4.7 |  |  |  |  |  |  |  |
|  | '256QAM' | 5.8 | 5.8 | 5.8 | 5.8 | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 |  |  |  |  |  |  |  |
| '20MHz+30MHz' | Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 0.2 | 0.2 | 0.4 | 0.4 | 1.8 | 2.1 | 2.4 | 4.0 | 4.3 | 4.7 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 1.5 | 1.5 | 1.8 | 1.8 | 1.8 | 2.1 | 2.4 | 4.0 | 4.3 | 4.7 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.0 | 3.3 | 3.3 | 3.3 | 3.6 | 3.3 | 3.3 | 4.0 | 4.3 | 5.0 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 |  |  |  |  |  |  |  |  |
| '20MHz+40MHz' | Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 0.0 | 0.4 | 0.4 | 0.6 | 1.0 | 2.1 | 2.7 | 4.3 | 4.7 | 5.0 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 1.5 | 1.8 | 1.8 | 1.8 | 2.1 | 2.1 | 2.7 | 4.3 | 4.7 | 5.0 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.0 | 3.3 | 3.3 | 3.3 | 3.6 | 3.6 | 3.3 | 4.3 | 4.7 | 5.0 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.4 | 5.4 | 5.4 | 5.4 | 5.8 | 5.8 | 5.4 | 5.4 | 5.8 | 5.8 |  |  |  |  |  |  |  |  |
| '30MHz+40MHz' | Scenario # | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 | #41 | #43 |  |  |  |  |  |  |  |
|  | 'QPSK' | 0.2 | 0.2 | 0.2 | 0.8 | 2.1 | 3.0 | 4.7 | 3.3 | 3.6 | 4.3 | 5.0 |  |  |  |  |  |  |  |
|  | '16QAM' | 1.5 | 1.5 | 1.5 | 1.8 | 2.1 | 3.0 | 4.7 | 3.3 | 3.6 | 4.3 | 5.0 |  |  |  |  |  |  |  |
|  | '64QAM' | 3.0 | 3.3 | 3.3 | 3.3 | 3.3 | 3.3 | 4.7 | 3.3 | 3.6 | 4.3 | 5.0 |  |  |  |  |  |  |  |
|  | '256QAM' | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 | 5.8 | 5.4 |  |  |  |  |  |  |  |
| '20MHz+20MHz' | Scenario # | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 |
|  | 'QPSK' | 0.0 | 0.4 | 0.4 | 0.6 | 0.8 | 2.1 | 5.0 | 2.7 | 4.3 | 4.3 | 5.0 |
|  | '16QAM' | 1.3 | 1.8 | 1.8 | 1.8 | 2.1 | 2.1 | 5.0 | 2.7 | 4.3 | 4.3 | 5.0 |
|  | '64QAM' | 3.0 | 3.3 | 3.3 | 3.6 | 3.6 | 3.6 | 5.0 | 3.3 | 4.3 | 4.3 | 5.0 |
|  | '256QAM' | 5.4 | 5.4 | 5.4 | 5.4 | 5.8 | 5.8 | 5.8 | 5.4 | 5.8 | 5.8 | 5.8 |
| '10MHz+30MHz' | Scenario # | #55 | #56 | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 | #65 |
|  | 'QPSK' | 1.5 | 1.8 | 2.4 | 2.7 | 2.7 | 3.3 | 4.7 | 3.3 | 4.3 | 4.7 | 5.0 |
|  | '16QAM' | 1.5 | 2.1 | 2.4 | 2.7 | 2.7 | 3.3 | 4.7 | 3.3 | 4.3 | 4.7 | 5.0 |
|  | '64QAM' | 3.0 | 3.3 | 3.3 | 3.3 | 3.3 | 3.6 | 4.7 | 3.3 | 4.3 | 4.7 | 5.0 |
|  | '256QAM' | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 | 5.4 | 5.8 | 5.4 | 5.8 | 5.4 | 5.4 |

Table 2-7: PSSCH/PSCCH MPR simulation results for contiguous RB allocations with 2x23dBm+2LO

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 0.1 | 0.1 | 0.3 | 0.1 | 0.1 | 0.1 | 2.2 | 0.3 | 0.6 | 1.9 | 3.0 |  |  |  |  |  |  |  |
|  | '16QAM' | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 | 1.2 | 2.2 | 1.2 | 1.2 | 1.9 | 3.0 |  |  |  |  |  |  |  |
|  | '64QAM' | 2.2 | 2.2 | 2.6 | 2.2 | 2.2 | 2.2 | 2.2 | 2.6 | 2.2 | 2.2 | 3.0 |  |  |  |  |  |  |  |
|  | '256QAM' | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.6 | 4.6 | 4.6 | 4.2 | 4.2 |  |  |  |  |  |  |  |
| '20MHz+30MHz' | Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 1.2 | 1.9 | 3.0 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 0.9 | 0.9 | 0.9 | 1.2 | 1.2 | 1.2 | 0.9 | 1.2 | 1.8 | 3.0 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.2 | 1.8 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 3.0 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 3.8 | 3.8 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 |  |  |  |  |  |  |  |  |
| '20MHz+40MHz' | Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 | 0.6 | 0.1 | 1.8 | 2.6 | 3.4 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 0.9 | 0.9 | 0.9 | 1.2 | 1.2 | 1.2 | 0.9 | 1.8 | 2.6 | 3.4 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.6 | 3.4 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 3.8 | 3.8 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 |  |  |  |  |  |  |  |  |
| '30MHz+40MHz' | Scenario # | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 | #41 | #43 |  |  |  |  |  |  |  |
|  | 'QPSK' | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 | 0.3 | 2.2 | 0.3 | 0.9 | 2.2 | 3.0 |  |  |  |  |  |  |  |
|  | '16QAM' | 0.9 | 0.9 | 0.9 | 0.9 | 1.2 | 1.2 | 2.2 | 1.2 | 0.9 | 2.2 | 3.0 |  |  |  |  |  |  |  |
|  | '64QAM' | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 3.0 |  |  |  |  |  |  |  |
|  | '256QAM' | 3.8 | 3.8 | 3.8 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 |  |  |  |  |  |  |  |
| '20MHz+20MHz' | Scenario # | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 | #54 |
|  | 'QPSK' | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 2.6 | 0.3 | 2.2 | 2.2 | 3.0 |
|  | '16QAM' | 0.9 | 0.9 | 1.2 | 1.2 | 1.2 | 1.2 | 2.6 | 1.2 | 2.2 | 2.2 | 3.0 |
|  | '64QAM' | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.6 | 2.2 | 2.2 | 2.2 | 3.0 |
|  | '256QAM' | 3.8 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 |
| '10MHz+30MHz' | Scenario # | #55 | #56 | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 | #65 |
|  | 'QPSK' | 0.0 | 0.0 | 0.1 | 0.1 | 0.3 | 1.2 | 3.0 | 1.2 | 2.2 | 3.0 | 3.4 |
|  | '16QAM' | 0.9 | 0.9 | 0.9 | 0.9 | 0.9 | 1.2 | 3.0 | 1.2 | 2.2 | 3.4 | 3.4 |
|  | '64QAM' | 2.2 | 1.8 | 2.2 | 2.2 | 2.2 | 2.2 | 3.0 | 2.2 | 2.2 | 3.4 | 3.4 |
|  | '256QAM' | 3.8 | 3.8 | 3.8 | 3.8 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 |

Table 2-8, Table 2-9, and Table 2-10 show the maximum value of simulation results for SL Contiguous CA of Contiguous RB allocations with architecture #1-1, #1-2, and #2-1 respectively considering Inner RB allocation and Outer RB allocation as NR uplink Contiguous CA.

Table 2-8: PSSCH/PSCCH MPR simulation results for contiguous RB allocations with 1x26dBm+1LO

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | 1.4 | 3.1 |
|  | 16QAM | 1.4 | 3.1 |
|  | 64QAM | 2.4 | 3.1 |
|  | 256QAM | 4.6 | 4.2 |

Table 2-9: PSSCH/PSCCH MPR simulation results for contiguous RB allocations with 2x23dBm+1LO

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | 2.7 | 5.0 |
|  | 16QAM | 2.7 | 5.0 |
|  | 64QAM | 3.6 | 5.0 |
|  | 256QAM | 5.8 | 5.8 |

Table 2-10: PSSCH/PSCCH MPR simulation results for contiguous RB allocations with 2x23dBm+2LO

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | 0.3 | 3.4 |
|  | 16QAM | 1.2 | 3.4 |
|  | 64QAM | 2.6 | 3.4 |
|  | 256QAM | 4.2 | 4.6 |

The MPR can be proposed as Table 2-11, Table 2-12, and Table 2-13 based on the simulation results when considering implementation margin.

Table 2-11: PSSCH/PSCCH MPR for contiguous RB allocations with 1x26dBm+1LO

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 2.5 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 |

Table 2-12: PSSCH/PSCCH MPR for contiguous RB allocations with 2x23dBm+1LO

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 6.0 |
|  | 16QAM | ≤ 3.5 | ≤ 6.0 |
|  | 64QAM | ≤ 4.5 | ≤ 6.0 |
|  | 256QAM | ≤ 7.5 | ≤ 7.5 |

Table 2-13: PSSCH/PSCCH MPR for contiguous RB allocations with 2x23dBm+2LO

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 2.0 | ≤ 5.0 |
|  | 16QAM | ≤ 2.5 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 |

Proposal 1: Consider PC2 PSSCH MPR in Table 2-11 for SL contiguous CA of contiguous RB allocation with 1x26dBm PA + 1LO.

Proposal 2: Consider PC2 PSSCH MPR in Table 2-12 for SL contiguous CA of contiguous RB allocation with 2x23dBm PA + 1LO.

Proposal 3: Consider PC2 PSSCH MPR in Table 2-13 for SL contiguous CA of contiguous RB allocation with 2x23dBm PA + 2LO.

Simulation results for non-contiguous RB allocations

Table 2-14, Table 2-15, and Table 2-16 show the MPR simulation results for non-contiguous RB allocations with architecture #1-1, #1-2, and #2-1 in Table 2-1 respectively.

Table 2-14: PSSCH/PSCCH MPR simulation results for Non-contiguous RB allocations with 1x26dBm+1LO

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 7.1 | 3.5 | 3.5 | 3.1 | 0.0 | 0.0 | 0.2 | 5.2 | 2.7 | 2.7 | 0.5 | 4.8 | 2.4 | 2.4 | 2.0 | 3.5 |  |  |
|  | '16QAM' | 7.1 | 3.5 | 3.5 | 3.1 | 1.0 | 1.1 | 1.4 | 5.2 | 2.7 | 2.7 | 1.0 | 4.8 | 2.4 | 2.4 | 2.0 | 3.5 |  |  |
|  | '64QAM' | 7.1 | 3.5 | 3.5 | 3.1 | 2.3 | 2.4 | 2.4 | 5.2 | 2.7 | 2.7 | 2.4 | 4.8 | 2.4 | 2.4 | 2.3 | 3.5 |  |  |
|  | '256QAM' | 7.1 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 4.3 | 5.2 | 3.9 | 3.9 | 4.3 | 4.8 | 3.9 | 3.9 | 4.3 | 4.3 |  |  |
| '20MHz+30MHz' | Scenario # | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 |
|  | 'QPSK' | 5.7 | 3.5 | 3.5 | 0.8 | 0.0 | 0.0 | 0.0 | 3.5 | 3.1 | 3.1 | 0.8 | 3.5 | 3.1 | 3.1 | 2.0 | 3.5 | 3.5 | 3.5 |
|  | '16QAM' | 5.7 | 3.5 | 3.5 | 0.8 | 0.7 | 0.8 | 1.0 | 3.5 | 3.1 | 3.1 | 1.0 | 3.5 | 3.1 | 3.1 | 2.0 | 3.5 | 3.5 | 3.5 |
|  | '64QAM' | 5.7 | 3.5 | 3.5 | 2.0 | 2.0 | 2.0 | 2.0 | 3.5 | 3.1 | 3.1 | 2.3 | 3.5 | 3.1 | 3.1 | 2.3 | 3.5 | 3.5 | 3.5 |
|  | '256QAM' | 5.7 | 3.5 | 3.9 | 3.5 | 3.9 | 3.5 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 4.3 | 3.9 | 4.3 | 4.3 |
| '20MHz+40MHz' | Scenario # | #35 | #36 | #37 | #38 | #39 | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 |  |
|  | 'QPSK' | 5.7 | 3.5 | 3.5 | 0.0 | 0.0 | 0.0 | 3.9 | 3.5 | 3.5 | 1.3 | 3.9 | 3.5 | 3.5 | 2.3 | 3.9 | 3.9 | 3.5 |  |
|  | '16QAM' | 5.7 | 3.5 | 3.5 | 0.8 | 0.7 | 0.7 | 3.5 | 3.5 | 3.5 | 1.3 | 3.5 | 3.5 | 3.5 | 2.3 | 3.9 | 3.9 | 3.5 |  |
|  | '64QAM' | 5.7 | 3.5 | 3.5 | 2.0 | 2.0 | 2.0 | 3.9 | 3.5 | 3.5 | 2.3 | 3.5 | 3.5 | 3.5 | 2.3 | 3.9 | 3.9 | 3.5 |  |
|  | '256QAM' | 5.7 | 3.9 | 3.5 | 3.5 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 4.3 | 3.9 | 4.3 | 4.3 |  |
| '30MHz+40MHz' | Scenario # | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 |
|  | 'QPSK' | 5.7 | 3.5 | 3.5 | 0.0 | 0.0 | 0.0 | 3.5 | 3.5 | 3.5 | 0.0 | 0.0 | 3.5 | 3.5 | 3.5 | 0.2 | 3.9 | 3.5 | 3.5 |
|  | '16QAM' | 5.7 | 3.5 | 3.5 | 0.8 | 0.7 | 0.7 | 3.5 | 3.5 | 3.5 | 1.0 | 1.0 | 3.5 | 3.5 | 3.5 | 1.0 | 3.9 | 3.5 | 3.5 |
|  | '64QAM' | 5.7 | 3.5 | 3.5 | 2.0 | 2.0 | 2.0 | 3.5 | 3.5 | 3.5 | 2.0 | 2.0 | 3.5 | 3.5 | 3.5 | 2.3 | 3.9 | 3.9 | 3.5 |
|  | '256QAM' | 5.7 | 3.9 | 3.5 | 3.9 | 3.5 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 4.3 | 3.9 | 4.3 | 4.3 |
| '20MHz+20MHz' | Scenario # | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 | #78 | #79 | #80 | #81 | #82 | #84 | #84 | #85 |
|  | 'QPSK' | 5.7 | 3.5 | 3.5 | 3.1 | 0.0 | 0.0 | 0.0 | 3.5 | 3.1 | 3.1 | 1.0 | 3.9 | 2.7 | 2.7 | 2.0 | 3.5 |
|  | '16QAM' | 5.7 | 3.5 | 3.5 | 3.1 | 0.7 | 0.8 | 0.7 | 3.5 | 3.1 | 3.1 | 1.0 | 3.9 | 2.7 | 2.7 | 2.0 | 3.5 |
|  | '64QAM' | 5.7 | 3.5 | 3.5 | 3.1 | 2.0 | 2.0 | 2.0 | 3.9 | 3.1 | 3.1 | 2.3 | 3.9 | 2.7 | 2.7 | 2.4 | 3.5 |
|  | '256QAM' | 5.7 | 3.5 | 3.5 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 4.3 | 4.3 |
| '10MHz+30MHz' | Scenario # | #86 | #87 | #88 | #89 | #90 | #91 | #92 | #93 | #94 | #95 | #96 | #97 | #98 | #99 | #100 | #101 |
|  | 'QPSK' | 5.7 | 3.5 | 3.5 | 2.7 | 0.8 | 5.2 | 3.5 | 3.5 | 1.3 | 3.9 | 3.1 | 3.5 | 2.3 | 3.9 | 3.5 | 3.5 |
|  | '16QAM' | 5.7 | 3.5 | 3.5 | 2.7 | 0.8 | 5.2 | 3.5 | 3.5 | 1.3 | 3.9 | 3.5 | 3.5 | 2.0 | 3.9 | 3.5 | 3.5 |
|  | '64QAM' | 5.7 | 3.5 | 3.5 | 2.7 | 2.0 | 5.2 | 3.5 | 3.5 | 2.0 | 3.9 | 3.5 | 3.5 | 2.4 | 3.9 | 3.5 | 3.5 |
|  | '256QAM' | 5.7 | 3.9 | 3.5 | 3.9 | 3.9 | 5.2 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 3.9 | 4.3 | 4.3 |

Table 2-15: PSSCH/PSCCH MPR simulation results for Non-contiguous RB allocations with 2x23dBm+1LO

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 8.2 | 5.0 | 5.0 | 4.6 | 1.1 | 1.1 | 1.3 | 6.3 | 4.2 | 4.2 | 2.1 | 5.9 | 3.8 | 3.8 | 3.4 | 5.0 |  |  |
|  | '16QAM' | 8.2 | 5.0 | 5.0 | 4.6 | 2.1 | 2.1 | 2.1 | 6.3 | 4.2 | 4.2 | 2.1 | 5.9 | 3.8 | 3.8 | 3.4 | 5.0 |  |  |
|  | '64QAM' | 8.2 | 5.0 | 5.0 | 4.6 | 3.4 | 3.1 | 3.4 | 6.3 | 4.6 | 4.2 | 3.4 | 5.9 | 3.8 | 3.8 | 3.4 | 5.0 |  |  |
|  | '256QAM' | 8.2 | 5.0 | 5.0 | 5.0 | 5.0 | 5.0 | 5.0 | 6.3 | 5.0 | 5.0 | 5.0 | 5.9 | 5.0 | 5.0 | 5.0 | 5.0 |  |  |
| '20MHz+30MHz' | Scenario # | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 |
|  | 'QPSK' | 6.8 | 5.0 | 5.0 | 1.8 | 0.9 | 0.6 | 0.9 | 5.0 | 4.6 | 4.6 | 2.4 | 5.0 | 4.6 | 4.6 | 3.4 | 5.0 | 5.0 | 5.0 |
|  | '16QAM' | 6.8 | 5.0 | 5.0 | 1.8 | 1.8 | 1.8 | 1.8 | 5.0 | 4.6 | 4.6 | 2.4 | 5.4 | 4.6 | 4.6 | 3.4 | 5.0 | 5.0 | 5.0 |
|  | '64QAM' | 6.8 | 5.0 | 5.0 | 3.1 | 3.1 | 3.1 | 3.1 | 5.5 | 4.6 | 4.6 | 3.4 | 5.0 | 4.6 | 4.6 | 3.4 | 5.0 | 5.0 | 5.0 |
|  | '256QAM' | 6.8 | 5.0 | 5.0 | 4.6 | 4.6 | 4.6 | 5.0 | 5.0 | 4.6 | 4.6 | 5.0 | 5.0 | 5.0 | 5.0 | 5.0 | 5.4 | 5.0 | 5.0 |
| '20MHz+40MHz' | Scenario # | #35 | #36 | #37 | #38 | #39 | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 |  |
|  | 'QPSK' | 6.8 | 5.4 | 5.0 | 0.6 | 0.8 | 0.8 | 5.4 | 5.0 | 5.0 | 2.7 | 5.4 | 5.0 | 5.0 | 3.8 | 5.4 | 5.4 | 5.0 |  |
|  | '16QAM' | 6.8 | 5.0 | 5.0 | 1.8 | 1.8 | 1.8 | 5.4 | 5.0 | 5.0 | 2.7 | 5.4 | 5.0 | 5.0 | 3.8 | 5.4 | 5.4 | 5.0 |  |
|  | '64QAM' | 6.8 | 5.4 | 5.0 | 3.1 | 3.1 | 3.1 | 5.4 | 5.0 | 5.0 | 3.1 | 5.4 | 5.0 | 5.0 | 3.8 | 5.4 | 5.4 | 5.0 |  |
|  | '256QAM' | 6.8 | 5.4 | 5.0 | 4.6 | 4.6 | 4.6 | 5.4 | 5.0 | 5.0 | 5.0 | 5.4 | 5.0 | 5.0 | 5.0 | 5.4 | 5.4 | 5.4 |  |
| '30MHz+40MHz' | Scenario # | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 |
|  | 'QPSK' | 6.8 | 5.0 | 5.0 | 0.9 | 0.8 | 0.8 | 5.4 | 5.0 | 5.0 | 1.1 | 1.1 | 5.0 | 5.0 | 5.0 | 1.1 | 5.4 | 5.4 | 5.0 |
|  | '16QAM' | 6.8 | 5.0 | 5.0 | 1.8 | 1.8 | 1.8 | 5.4 | 5.0 | 5.0 | 2.1 | 2.1 | 5.0 | 5.0 | 5.0 | 2.1 | 5.4 | 5.0 | 5.0 |
|  | '64QAM' | 6.8 | 5.0 | 5.0 | 3.1 | 3.1 | 3.1 | 5.4 | 5.0 | 5.0 | 3.1 | 3.1 | 5.4 | 5.0 | 5.0 | 3.4 | 5.4 | 5.4 | 5.0 |
|  | '256QAM' | 6.8 | 5.0 | 5.0 | 4.6 | 4.6 | 5.0 | 5.4 | 5.0 | 5.0 | 5.0 | 5.0 | 5.4 | 5.0 | 5.0 | 5.0 | 5.4 | 5.4 | 5.4 |
| '20MHz+20MHz' | Scenario # | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 | #78 | #79 | #80 | #81 | #82 | #84 | #84 | #85 |
|  | 'QPSK' | 6.8 | 5.0 | 5.0 | 1.8 | 0.9 | 0.6 | 0.9 | 5.0 | 4.6 | 4.6 | 2.4 | 5.0 | 4.6 | 4.6 | 3.4 | 5.0 |
|  | '16QAM' | 6.8 | 5.0 | 5.0 | 1.8 | 1.8 | 1.8 | 1.8 | 5.0 | 4.6 | 4.6 | 2.4 | 5.4 | 4.6 | 4.6 | 3.4 | 5.0 |
|  | '64QAM' | 6.8 | 5.0 | 5.0 | 3.1 | 3.1 | 3.1 | 3.1 | 5.5 | 4.6 | 4.6 | 3.4 | 5.0 | 4.6 | 4.6 | 3.4 | 5.0 |
|  | '256QAM' | 6.8 | 5.0 | 5.0 | 4.6 | 4.6 | 4.6 | 5.0 | 5.0 | 4.6 | 4.6 | 5.0 | 5.0 | 5.0 | 5.0 | 5.0 | 5.4 |
| '10MHz+30MHz' | Scenario # | #86 | #87 | #88 | #89 | #90 | #91 | #92 | #93 | #94 | #95 | #96 | #97 | #98 | #99 | #100 | #101 |
|  | 'QPSK' | 6.8 | 5.0 | 5.0 | 4.2 | 1.8 | 6.3 | 5.4 | 5.4 | 2.7 | 5.4 | 5.0 | 5.0 | 3.8 | 5.4 | 5.4 | 5.0 |
|  | '16QAM' | 6.8 | 5.0 | 5.0 | 4.2 | 1.8 | 6.3 | 5.4 | 5.0 | 2.7 | 5.5 | 5.0 | 5.0 | 3.8 | 5.4 | 5.0 | 5.0 |
|  | '64QAM' | 6.8 | 5.0 | 5.0 | 4.2 | 3.1 | 6.3 | 5.4 | 5.4 | 3.1 | 5.9 | 5.0 | 5.0 | 3.8 | 5.4 | 5.0 | 5.0 |
|  | '256QAM' | 6.8 | 5.0 | 5.0 | 4.6 | 5.0 | 6.3 | 5.4 | 5.0 | 5.0 | 5.4 | 5.0 | 5.0 | 5.0 | 5.4 | 5.0 | 5.4 |

Table 2-16: PSSCH/PSCCH MPR simulation results for Non-contiguous RB allocations with 2x23dBm+2LO

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 3.0 | 2.6 | 2.6 | 1.5 | 0.1 | 0.1 | 0.1 | 3.4 | 1.5 | 1.5 | 0.1 | 3.4 | 0.9 | 0.9 | 0.3 | 3.0 |  |  |
|  | '16QAM' | 3.0 | 2.6 | 2.6 | 1.5 | 1.2 | 1.2 | 1.2 | 3.4 | 1.5 | 1.5 | 1.2 | 3.4 | 1.2 | 1.2 | 1.2 | 3.0 |  |  |
|  | '64QAM' | 3.0 | 2.6 | 2.6 | 2.2 | 2.2 | 2.2 | 2.2 | 3.4 | 2.2 | 2.2 | 2.2 | 3.4 | 2.2 | 2.2 | 2.2 | 3.0 |  |  |
|  | '256QAM' | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 | 4.2 |  |  |
| '20MHz+30MHz' | Scenario # | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 |
|  | 'QPSK' | 2.6 | 2.2 | 2.2 | 0.0 | 0.0 | 0.0 | 0.0 | 3.0 | 1.8 | 1.8 | 0.0 | 3.0 | 1.8 | 1.9 | 0.1 | 3.4 | 3.0 | 3.0 |
|  | '16QAM' | 2.6 | 2.2 | 2.2 | 0.9 | 0.9 | 0.9 | 0.9 | 3.0 | 1.9 | 1.9 | 0.9 | 3.0 | 1.8 | 1.8 | 1.2 | 3.4 | 3.0 | 3.0 |
|  | '64QAM' | 2.6 | 2.2 | 2.2 | 2.2 | 2.2 | 1.8 | 2.2 | 3.0 | 2.2 | 2.2 | 2.2 | 3.0 | 2.2 | 2.2 | 2.2 | 3.4 | 3.0 | 3.0 |
|  | '256QAM' | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 4.2 | 3.8 | 3.8 | 3.8 | 4.2 | 3.8 | 4.2 | 4.2 |
| '20MHz+40MHz' | Scenario # | #35 | #36 | #37 | #38 | #39 | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 |  |
|  | 'QPSK' | 3.0 | 2.2 | 2.2 | 0.0 | 0.0 | 0.0 | 3.4 | 2.2 | 2.2 | 0.1 | 3.4 | 2.2 | 2.2 | 0.3 | 3.4 | 3.0 | 3.4 |  |
|  | '16QAM' | 3.0 | 2.2 | 2.2 | 0.9 | 0.9 | 0.9 | 3.4 | 2.2 | 2.2 | 0.9 | 3.4 | 2.2 | 2.2 | 0.9 | 3.4 | 3.0 | 3.4 |  |
|  | '64QAM' | 3.0 | 2.2 | 2.2 | 2.2 | 1.8 | 2.2 | 3.4 | 2.2 | 2.2 | 2.2 | 3.4 | 2.2 | 2.2 | 2.2 | 3.4 | 3.0 | 3.4 |  |
|  | '256QAM' | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 4.2 | 3.8 | 4.2 | 3.8 | 4.2 | 4.2 |  |
| '30MHz+40MHz' | Scenario # | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 |
|  | 'QPSK' | 3.0 | 2.2 | 2.2 | 0.0 | 0.0 | 0.0 | 3.0 | 2.2 | 2.2 | 0.0 | 0.0 | 3.0 | 2.2 | 1.8 | 0.1 | 3.4 | 2.6 | 3.4 |
|  | '16QAM' | 3.0 | 2.2 | 2.2 | 0.9 | 0.9 | 0.9 | 3.0 | 2.2 | 2.2 | 0.9 | 0.9 | 3.4 | 2.2 | 2.2 | 0.9 | 3.4 | 2.6 | 3.4 |
|  | '64QAM' | 3.0 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 3.0 | 2.2 | 2.2 | 2.2 | 2.2 | 3.4 | 2.2 | 2.2 | 2.2 | 3.4 | 2.6 | 3.4 |
|  | '256QAM' | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 4.2 | 3.8 | 3.8 | 3.8 | 3.8 | 4.2 | 3.8 | 4.2 | 4.2 |
| '20MHz+20MHz' | Scenario # | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 | #78 | #79 | #80 | #81 | #82 | #84 | #84 | #85 |
|  | 'QPSK' | 3.0 | 2.2 | 2.2 | 1.8 | 0.0 | 0.0 | 0.0 | 3.0 | 1.5 | 1.5 | 0.1 | 3.4 | 1.2 | 1.2 | 0.6 | 3.0 |
|  | '16QAM' | 3.0 | 2.2 | 2.2 | 1.9 | 0.9 | 0.9 | 0.9 | 3.0 | 1.9 | 1.5 | 0.9 | 3.4 | 1.2 | 1.2 | 0.9 | 3.0 |
|  | '64QAM' | 3.0 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 2.2 | 3.0 | 2.2 | 2.2 | 2.2 | 3.4 | 2.2 | 2.2 | 2.2 | 3.0 |
|  | '256QAM' | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 4.2 | 3.8 | 4.2 | 4.2 | 4.2 | 4.2 |
| '10MHz+30MHz' | Scenario # | #86 | #87 | #88 | #89 | #90 | #91 | #92 | #93 | #94 | #95 | #96 | #97 | #98 | #99 | #100 | #101 |
|  | 'QPSK' | 3.0 | 2.2 | 2.2 | 0.9 | 0.1 | 3.0 | 2.2 | 2.2 | 0.3 | 3.4 | 2.2 | 2.2 | 0.9 | 3.4 | 2.6 | 3.4 |
|  | '16QAM' | 3.0 | 2.2 | 2.2 | 0.9 | 0.9 | 3.0 | 2.2 | 2.2 | 0.9 | 3.4 | 2.2 | 2.2 | 1.2 | 3.4 | 2.6 | 3.4 |
|  | '64QAM' | 3.0 | 2.2 | 2.2 | 1.8 | 2.2 | 3.0 | 2.2 | 2.2 | 2.2 | 3.4 | 2.2 | 2.2 | 2.2 | 3.4 | 2.6 | 3.4 |
|  | '256QAM' | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 3.8 | 4.2 | 3.8 | 4.2 | 4.2 |

Table 2-17, Table 2-18, and Table 2-19 show the maximum value of simulation results for Non-contiguous RB allocations with architecture #1-1, #1-2,  and #2-1 respectively considering Inner/Outer1/Outer2 RB allocation as NR uplink Contiguous CA.

Table 2-17: PSSCH/PSCCH MPR simulation results for Non-contiguous RB allocations with 1x26dBm+1LO

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | 0.2 | 3.5 | 7.1 |
|  | 16QAM | 1.4 | 3.5 | 7.1 |
|  | 64QAM | 2.4 | 3.5 | 7.1 |
|  | 256QAM | 4.3 | 4.3 | 7.1 |

Table 2-18: PSSCH/PSCCH MPR simulation results for Non-contiguous RB allocations with 2x23dBm+1LO

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | 1.3 | 5.4 | 8.2 |
|  | 16QAM | 2.1 | 5.0 | 8.2 |
|  | 64QAM | 3.4 | 5.4 | 8.2 |
|  | 256QAM | 5.0 | 5.0 | 8.2 |

Table 2-19: PSSCH/PSCCH MPR simulation results for Non-contiguous RB allocations with 2x23dBm+2LO

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | 0.1 | 2.6 | 3.4 |
|  | 16QAM | 1.2 | 2.6 | 3.4 |
|  | 64QAM | 2.2 | 2.6 | 3.4 |
|  | 256QAM | 4.2 | 4.2 | 4.2 |

The MPR can be proposed as Table 2-20, Table 2-21, and Table 2-22 based on the simulation results when considering implementation margin.

Table 2-20: PSSCH/PSCCH MPR for Non-contiguous RB allocations with 1x26dBm+1LO

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 7.5 | ≤ 11.0 |
|  | 16QAM | ≤ 4.0 | ≤ 7.5 | ≤ 11.0 |
|  | 64QAM | ≤ 5.5 | ≤ 7.5 | ≤ 11.0 |
|  | 256QAM | ≤ 8.0 | ≤ 8.0 | ≤ 11.0 |

Table 2-21: PSSCH/PSCCH MPR for Non-contiguous RB allocations with 2x23dBm+1LO

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 8.5 | ≤ 12.0 |
|  | 16QAM | ≤ 4.5 | ≤ 8.5 | ≤ 12.0 |
|  | 64QAM | ≤ 7.0 | ≤ 8.5 | ≤ 12.0 |
|  | 256QAM | ≤ 9.0 | ≤ 9.0 | ≤ 12.0 |

Table 2-22: PSSCH/PSCCH MPR for Non-contiguous RB allocations with 2x23dBm+2LO

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 6.5 | ≤ 7.5 |
|  | 16QAM | ≤ 4.0 | ≤ 6.5 | ≤ 7.5 |
|  | 64QAM | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 |
|  | 256QAM | ≤ 8.0 | ≤ 8.0 | ≤ 8.0 |

Proposal 4: Consider PC2 PSSCH MPR in Table 2-20 for SL contiguous CA of non-contiguous RB allocation with 1x26dBm PA + 1LO.

Proposal 5: Consider PC2 PSSCH MPR in Table 2-21 for SL contiguous CA of non-contiguous RB allocation with 2x23dBm PA + 1LO.

Proposal 6: Consider PC2 PSSCH MPR in Table 2-22 for SL contiguous CA of non-contiguous RB allocation with 2x23dBm PA + 2LO.

PC2 MPR (PSFCH)

UE RF architecture

Architecture #1-1, #1-2, and #2-1 in Table 2-1 are considered for MPR evaluation.

Simulation assumption

The simulation assumption in Table 2-23 is considered.

Table 2-23. Simulation assumption

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz aggregated CBW: 10+10, 10+30, 20+20, 20+30, 20+40, 30+30 (MHz) |
| Maximum output power for aggregated CBW | 26dBm |
| Modulation for PSFCH | QPSK |
| PSFCH | ZC sequence |
| Structure of Slot | Baseline is to follow RAN1 agreements |
| Waveform | CP-OFDM |
| ACLR | 31dBc |
| Carrier leakage | 25dBc |
| IQ image | 34dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

Evaluation scenario

The PSFCH MPR evaluation scenarios in Table 2-34 are considered for SL contiguous CA.

Table 2-24: SL contiguous CA MPR evaluation scenarios for PSFCH

| Aggregated CBW | Scenario | CC1 | CC2 | R | SCS |
| --- | --- | --- | --- | --- | --- |
| 10MHz + 10MHz | 1 | 1RB0 | - |  | 15 |
|  | 2 | 1RB51 | - |  | 15 |
|  | 3 | 1RB0 | 1RB0 | 0.5255 | 15 |
|  | 4 | 1RB0 | 1RB10 | 0.6185 | 15 |
|  | 5 | 1RB0 | 1RB20 | 0.7116 | 15 |
|  | 6 | 1RB0 | 1RB30 | 0.8046 | 15 |
|  | 7 | 1RB0 | 1RB40 | 0.8976 | 15 |
|  | 8 | 1RB0 | 1RB51 | 1.0 | 15 |
|  | 9 | 1RB10 | 1RB0 | 0.4324 | 15 |
|  | 10 | 1RB20 | 1RB0 | 0.3394 | 15 |
|  | 11 | 1RB30 | 1RB0 | 0.2463 | 15 |
|  | 12 | 1RB40 | 1RB0 | 0.1533 |  |
|  | 13 | 1RB51 | 1RB0 | 0.0509 | 15 |
| 20MHz + 30MHz | 14 | 1RB0 | - |  | 30 |
|  | 15 | 1RB50 | - |  | 30 |
|  | 16 | 1RB0 | 1RB0 | 0.4248 | 30 |
|  | 17 | 1RB0 | 1RB10 | 0.4995 | 30 |
|  | 18 | 1RB0 | 1RB20 | 0.5742 | 30 |
|  | 19 | 1RB0 | 1RB30 | 0.6489 | 30 |
|  | 20 | 1RB0 | 1RB40 | 0.7236 | 30 |
|  | 21 | 1RB0 | 1RB50 | 0.7983 | 30 |
|  | 22 | 1RB0 | 1RB60 | 0.8730 | 30 |
|  | 23 | 1RB0 | 1RB70 | 0.9477 | 30 |
|  | 24 | 1RB0 | 1RB77 | 1.0 | 30 |
|  | 25 | 1RB10 | 1RB0 | 0.3501 | 30 |
|  | 26 | 1RB20 | 1RB0 | 0.2754 | 30 |
|  | 27 | 1RB30 | 1RB0 | 0.2007 | 30 |
|  | 28 | 1RB40 | 1RB0 | 0.1260 | 30 |
|  | 29 | 1RB50 | 1RB0 | 0.0513 | 30 |
| 20MHz + 40MHz | 30 | 1RB0 | - |  | 30 |
|  | 31 | 1RB50 | - |  | 30 |
|  | 32 | 1RB0 | 1RB0 | 0.3509 | 30 |
|  | 33 | 1RB0 | 1RB10 | 0.4127 | 30 |
|  | 34 | 1RB0 | 1RB20 | 0.4745 | 30 |
|  | 35 | 1RB0 | 1RB30 | 0.5363 | 30 |
|  | 36 | 1RB0 | 1RB40 | 0.5981 | 30 |
|  | 37 | 1RB0 | 1RB50 | 0.6600 | 30 |
|  | 38 | 1RB0 | 1RB60 | 0.7218 | 30 |
|  | 39 | 1RB0 | 1RB70 | 0.7836 | 30 |
|  | 40 | 1RB0 | 1RB80 | 0.8454 | 30 |
|  | 41 | 1RB0 | 1RB90 | 0.9073 | 30 |
|  | 42 | 1RB0 | 1RB100 | 0.9691 | 30 |
|  | 43 | 1RB0 | 1RB105 | 1.0 | 30 |
|  | 44 | 1RB10 | 1RB0 | 0.2890 | 30 |
|  | 45 | 1RB20 | 1RB0 | 0.2272 | 30 |
|  | 46 | 1RB30 | 1RB0 | 0.1654 | 30 |
|  | 47 | 1RB40 | 1RB0 | 0.1036 | 30 |
|  | 48 | 1RB50 | 1RB0 | 0.0417 | 30 |
| 30MHz + 40MHz | 49 | 1RB0 | - |  | 30 |
|  | 50 | 1RB77 | - |  | 30 |
|  | 51 | 1RB0 | 1RB0 | 0.4449 | 30 |
|  | 52 | 1RB0 | 1RB10 | 0.4977 | 30 |
|  | 53 | 1RB0 | 1RB20 | 0.5506 | 30 |
|  | 54 | 1RB0 | 1RB30 | 0.6035 | 30 |
|  | 55 | 1RB0 | 1RB40 | 0.6563 | 30 |
|  | 56 | 1RB0 | 1RB50 | 0.7092 | 30 |
|  | 57 | 1RB0 | 1RB60 | 0.7621 | 30 |
|  | 58 | 1RB0 | 1RB70 | 0.8150 | 30 |
|  | 59 | 1RB0 | 1RB80 | 0.8678 | 30 |
|  | 60 | 1RB0 | 1RB90 | 0.9207 | 30 |
|  | 61 | 1RB0 | 1RB100 | 0.9736 | 30 |
|  | 62 | 1RB0 | 1RB105 | 1.0000 | 30 |
|  | 63 | 1RB10 | 1RB0 | 0.3920 | 30 |
|  | 64 | 1RB20 | 1RB0 | 0.3391 | 30 |
|  | 65 | 1RB30 | 1RB0 | 0.2862 | 30 |
|  | 66 | 1RB40 | 1RB0 | 0.2334 | 30 |
|  | 67 | 1RB50 | 1RB0 | 0.1805 | 30 |
|  | 68 | 1RB60 | 1RB0 | 0.1276 | 30 |
|  | 69 | 1RB70 | 1RB0 | 0.0748 | 30 |
|  | 70 | 1RB77 | 1RB0 | 0.0377 | 30 |
| 20MHz + 20MHz | 71 | 1RB0 | - |  | 30 |
|  | 72 | 1RB51 | - |  | 30 |
|  | 73 | 1RB0 | 1RB0 | 0.5304 | 30 |
|  | 74 | 1RB0 | 1RB10 | 0.6243 | 30 |
|  | 75 | 1RB0 | 1RB20 | 0.7182 | 30 |
|  | 76 | 1RB0 | 1RB30 | 0.8122 | 30 |
|  | 77 | 1RB0 | 1RB40 | 0.9061 | 30 |
|  | 78 | 1RB0 | 1RB50 | 1.0 | 30 |
|  | 79 | 1RB10 | 1RB0 | 0.4365 | 30 |
|  | 80 | 1RB20 | 1RB0 | 0.3426 | 30 |
|  | 81 | 1RB30 | 1RB0 | 0.2486 | 30 |
|  | 82 | 1RB40 | 1RB0 | 0.1547 | 30 |
|  | 83 | 1RB50 | 1RB0 | 0.0608 | 30 |
| 10MHz + 30MHz | 84 | 1RB0 | - |  | 30 |
|  | 85 | 1RB23 | - |  | 30 |
|  | 86 | 1RB0 | 1RB0 | 0.2768 | 30 |
|  | 87 | 1RB0 | 1RB10 | 0.3707 | 30 |
|  | 88 | 1RB0 | 1RB20 | 0.4646 | 30 |
|  | 89 | 1RB0 | 1RB30 | 0.5586 | 30 |
|  | 90 | 1RB0 | 1RB40 | 0.6525 | 30 |
|  | 91 | 1RB0 | 1RB50 | 0.7464 | 30 |
|  | 92 | 1RB0 | 1RB60 | 0.8403 | 30 |
|  | 93 | 1RB0 | 1RB70 | 0.9343 | 30 |
|  | 94 | 1RB0 | 1RB77 | 1.0 | 30 |
|  | 95 | 1RB10 | 1RB0 | 0.1829 | 30 |
|  | 96 | 1RB20 | 1RB0 | 0.0890 | 30 |
|  | 97 | 1RB23 | 1RB0 | 0.0608 | 30 |

Here, R is the ratio of the gap bandwidth between the two PSFCH transmitted on the two intra-band carrier by the total bandwidth of the two carrier.

Simulation results for PSFCH

Table 2-25, Table 2-26, and Table 2-27 show the MPR simulation results for the SL contiguous CA scenarios with architecture #1-1, #1-2, and #2-1 in Table 2-1 respectively.

Table 2-25: PSFCH MPR simulation results for SL contiguous CA with 1x26dBm+1LO

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R | - | - | 0.53 | 0.62 | 0.71 | 0.80 | 0.90 | 1.00 | 0.43 | 0.34 | 0.25 | 0.15 | 0.05 |  |  |  |  |  |
|  |  | 2.68 | 0.00 | 5.92 | 7.38 | 10.68 | 10.68 | 9.75 | 9.77 | 1.65 | 1.26 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| '20MHz+30MHz' | Scenario # | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 |  |  |
|  | R | - | - | 0.42 | 0.50 | 0.57 | 0.65 | 0.72 | 0.80 | 0.87 | 0.95 | 1.00 | 0.35 | 0.28 | 0.20 | 0.13 | 0.05 |  |  |
|  |  | 3.06 | 0.00 | 5.87 | 5.38 | 7.33 | 10.16 | 10.62 | 10.16 | 9.70 | 9.23 | 10.17 | 1.65 | 0.90 | 0.00 | 0.00 | 0.00 |  |  |
| '20MHz+40MHz' | Scenario # | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |  |  |  |  |  |  |
|  | R | - | - | 0.35 | 0.41 | 0.47 | 0.54 | 0.60 | 0.66 | 0.72 | 0.78 |  |  |  |  |  |  |  |  |
|  |  | 4.58 | 0.00 | 1.32 | 5.88 | 5.39 | 6.86 | 10.18 | 10.18 | 9.71 | 9.24 |  |  |  |  |  |  |  |  |
|  | Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 |  |  |  |  |  |  |  |  |  |
|  | R | 0.85 | 0.91 | 0.97 | 1.00 | 0.29 | 0.23 | 0.17 | 0.10 | 0.04 |  |  |  |  |  |  |  |  |  |
|  |  | 10.18 | 10.18 | 10.18 | 10.19 | 1.65 | 1.26 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |  |  |  |  |
| '30MHz+40MHz' | Scenario # | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |  |  |  |  |  |
|  | R | - | - | 0.44 | 0.50 | 0.55 | 0.60 | 0.66 | 0.71 | 0.76 | 0.82 | 0.87 |  |  |  |  |  |
|  |  | 4.66 | 0.00 | 4.94 | 5.92 | 5.43 | 10.68 | 10.21 | 9.28 | 10.22 | 10.22 | 10.67 |  |  |  |  |  |
|  | Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 |  |  |  |  |  |
|  | R | 0.92 | 0.97 | 1.00 | 0.39 | 0.34 | 0.29 | 0.23 | 0.18 | 0.13 | 0.07 | 0.04 |  |  |  |  |  |
|  |  | 10.21 | 9.75 | 10.70 | 1.65 | 1.65 | 1.26 | 1.26 | 0.00 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| '20MHz+20MHz' | Scenario # | #71 | #72 | #73 | #74 | #75 | #76 | #77 | #78 | #79 | #80 | #81 | #82 | #83 |  |  |  |  |  |
|  | R | - | - | 0.53 | 0.62 | 0.72 | 0.81 | 0.91 | 1.00 | 0.44 | 0.34 | 0.25 | 0.15 | 0.06 |  |  |  |  |  |
|  |  | 4.16 | 0.00 | 5.92 | 10.22 | 10.22 | 10.22 | 10.22 | 9.77 | 2.07 | 1.26 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| '10MHz+30MHz' | Scenario # | #84 | #85 | #86 | #87 | #88 | #89 | #90 | #91 | #92 | #93 | #94 | #95 | #96 | #97 |  |  |  |  |
|  | R | - | - | 0.28 | 0.37 | 0.46 | 0.56 | 0.65 | 0.75 | 0.84 | 0.93 | 1.00 | 0.18 | 0.09 | 0.06 |  |  |  |  |
|  |  | 4.16 | 0.00 | 1.75 | 1.75 | 4.95 | 7.38 | 9.75 | 10.22 | 10.22 | 9.75 | 9.77 | 1.26 | 0.00 | 0.00 |  |  |  |  |

Table 2-26: PSFCH MPR simulation results for SL contiguous CA with 2x23dBm+1LO

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R | - | - | 0.53 | 0.62 | 0.71 | 0.80 | 0.90 | 1.00 | 0.43 | 0.34 | 0.25 | 0.15 | 0.05 |  |  |  |  |  |
|  |  | 3.80 | 0.00 | 7.05 | 8.50 | 11.79 | 11.80 | 10.87 | 10.89 | 3.61 | 2.36 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| '20MHz+30MHz' | Scenario # | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 |  |  |
|  | R | - | - | 0.42 | 0.50 | 0.57 | 0.65 | 0.72 | 0.80 | 0.87 | 0.95 | 1.00 | 0.35 | 0.28 | 0.20 | 0.13 | 0.05 |  |  |
|  |  | 4.19 | 0.00 | 6.99 | 6.50 | 8.45 | 11.28 | 11.74 | 11.28 | 10.82 | 10.35 | 11.29 | 2.76 | 2.36 | 0.00 | 0.00 | 0.00 |  |  |
| '20MHz+40MHz' | Scenario # | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |  |  |  |  |  |  |
|  | R | - | - | 0.35 | 0.41 | 0.47 | 0.54 | 0.60 | 0.66 | 0.72 | 0.78 |  |  |  |  |  |  |  |  |
|  |  | 5.71 | 0.00 | 2.42 | 7.01 | 6.52 | 7.98 | 11.29 | 11.29 | 10.83 | 10.36 |  |  |  |  |  |  |  |  |
|  | Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 |  |  |  |  |  |  |  |  |  |
|  | R | 0.85 | 0.91 | 0.97 | 1.00 | 0.29 | 0.23 | 0.17 | 0.10 | 0.04 |  |  |  |  |  |  |  |  |  |
|  |  | 11.29 | 11.29 | 11.30 | 11.31 | 2.76 | 2.36 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |  |  |  |  |
| '30MHz+40MHz' | Scenario # | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |  |  |  |  |  |
|  | R | - | - | 0.44 | 0.50 | 0.55 | 0.60 | 0.66 | 0.71 | 0.76 | 0.82 | 0.87 |  |  |  |  |  |
|  |  | 6.28 | 0.00 | 6.07 | 7.05 | 7.05 | 11.79 | 11.33 | 10.40 | 11.33 | 11.33 | 11.79 |  |  |  |  |  |
|  | Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 |  |  |  |  |  |
|  | R | 0.92 | 0.97 | 1.00 | 0.39 | 0.34 | 0.29 | 0.23 | 0.18 | 0.13 | 0.07 | 0.04 |  |  |  |  |  |
|  |  | 11.33 | 10.87 | 11.81 | 2.76 | 2.76 | 2.36 | 2.36 | 0.00 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| '20MHz+20MHz' | Scenario # | #71 | #72 | #73 | #74 | #75 | #76 | #77 | #78 | #79 | #80 | #81 | #82 | #83 |  |  |  |  |  |
|  | R | - | - | 0.53 | 0.62 | 0.72 | 0.81 | 0.91 | 1.00 | 0.44 | 0.34 | 0.25 | 0.15 | 0.06 |  |  |  |  |  |
|  |  | 5.29 | 0.00 | 7.05 | 11.33 | 10.87 | 11.33 | 11.33 | 10.89 | 3.17 | 2.36 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| '10MHz+30MHz' | Scenario # | #84 | #85 | #86 | #87 | #88 | #89 | #90 | #91 | #92 | #93 | #94 | #95 | #96 | #97 |  |  |  |  |
|  | R | - | - | 0.28 | 0.37 | 0.46 | 0.56 | 0.65 | 0.75 | 0.84 | 0.93 | 1.00 | 0.18 | 0.09 | 0.06 |  |  |  |  |
|  |  | 5.29 | 0.00 | 2.86 | 2.86 | 6.07 | 8.51 | 10.87 | 11.33 | 11.33 | 10.87 | 10.89 | 2.36 | 0.00 | 0.00 |  |  |  |  |

Table 2-27: PSFCH MPR simulation results for SL contiguous CA with 2x23dBm+2LO

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 | #13 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R | - | - | 0.53 | 0.62 | 0.71 | 0.80 | 0.90 | 1.00 | 0.43 | 0.34 | 0.25 | 0.15 | 0.05 |  |  |  |  |  |
|  |  | 1.31 | 1.27 | 0.81 | 1.66 | 3.14 | 3.61 | 2.62 | 2.65 | 1.15 | 0.77 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| '20MHz+30MHz' | Scenario # | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 |  |  |
|  | R | - | - | 0.42 | 0.50 | 0.57 | 0.65 | 0.72 | 0.80 | 0.87 | 0.95 | 1.00 | 0.35 | 0.28 | 0.20 | 0.13 | 0.05 |  |  |
|  |  | 1.77 | 1.75 | 1.27 | 0.84 | 1.32 | 2.70 | 3.11 | 2.73 | 2.66 | 2.70 | 2.68 | 1.15 | 0.36 | 0.00 | 0.00 | 0.00 |  |  |
| '20MHz+40MHz' | Scenario # | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |  |  |  |  |  |  |
|  | R | - | - | 0.35 | 0.41 | 0.47 | 0.54 | 0.60 | 0.66 | 0.72 | 0.78 |  |  |  |  |  |  |  |  |
|  |  | 1.78 | 1.75 | 1.73 | 1.69 | 1.78 | 1.73 | 3.12 | 2.73 | 2.69 | 2.63 |  |  |  |  |  |  |  |  |
|  | Scenario # | #40 | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 |  |  |  |  |  |  |  |  |  |
|  | R | 0.85 | 0.91 | 0.97 | 1.00 | 0.29 | 0.23 | 0.17 | 0.10 | 0.04 |  |  |  |  |  |  |  |  |  |
|  |  | 2.72 | 3.15 | 2.71 | 2.71 | 1.17 | 0.78 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |  |  |  |  |
| '30MHz+40MHz' | Scenario # | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 | #57 | #58 | #59 |  |  |  |  |  |
|  | R | - | - | 0.44 | 0.50 | 0.55 | 0.60 | 0.66 | 0.71 | 0.76 | 0.82 | 0.87 |  |  |  |  |  |
|  |  | 1.75 | 1.69 | 1.25 | 1.21 | 1.30 | 3.11 | 2.60 | 2.21 | 2.65 | 3.07 | 2.68 |  |  |  |  |  |
|  | Scenario # | #60 | #61 | #62 | #63 | #64 | #65 | #66 | #67 | #68 | #69 | #70 |  |  |  |  |  |
|  | R | 0.92 | 0.97 | 1.00 | 0.39 | 0.34 | 0.29 | 0.23 | 0.18 | 0.13 | 0.07 | 0.04 |  |  |  |  |  |
|  |  | 2.63 | 2.67 | 3.15 | 1.17 | 0.79 | 0.78 | 0.73 | 0.00 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| '20MHz+20MHz' | Scenario # | #71 | #72 | #73 | #74 | #75 | #76 | #77 | #78 | #79 | #80 | #81 | #82 | #83 |  |  |  |  |  |
|  | R | - | - | 0.53 | 0.62 | 0.72 | 0.81 | 0.91 | 1.00 | 0.44 | 0.34 | 0.25 | 0.15 | 0.06 |  |  |  |  |  |
|  |  | 1.80 | 1.74 | 1.30 | 3.13 | 2.69 | 3.17 | 3.13 | 2.71 | 1.18 | 0.39 | 0.00 | 0.00 | 0.00 |  |  |  |  |  |
| '10MHz+30MHz' | Scenario # | #84 | #85 | #86 | #87 | #88 | #89 | #90 | #91 | #92 | #93 | #94 | #95 | #96 | #97 |  |  |  |  |
|  | R | - | - | 0.28 | 0.37 | 0.46 | 0.56 | 0.65 | 0.75 | 0.84 | 0.93 | 1.00 | 0.18 | 0.09 | 0.06 |  |  |  |  |
|  |  | 1.73 | 1.32 | 0.80 | 1.23 | 0.84 | 1.27 | 2.59 | 2.69 | 2.62 | 2.66 | 2.64 | 0.78 | 0.00 | 0.00 |  |  |  |  |

Table 2-28 shows the maximum value of simulation results for SL Contiguous CA with architecture #1-1, #1-2, and #2-1 respectively

Table 2-28: PSFCH MPR simulation results for SL contiguous CA

|  | MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  | R ≤ 0. 3 | 0.3 < R ≤ 0. 5 | 0.5 < R ≤ 1.0 |
| 1x26dBm+1LO | 1.8 | 6.9 | 10.7 |
| 2x23dBm+1LO | 2.9 | 8.0 | 11.8 |
| 2x23dBm+2LO | 0.8 | 1.8 | 3.6 |

The MPR can be proposed as Table 2-29 based on the simulation results when considering implementation margin.

Table 2-29: PSFCH MPR for SL contiguous CA

|  | MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  | R ≤ 0. 3 | 0.3 < R ≤ 0. 5 | 0.5 < R ≤ 1.0 |
| 1x26dBm+1LO | ≤ 5.0 | ≤ 10.0 | ≤ 14.0 |
| 2x23dBm+1LO | ≤ 6.0 | ≤ 11.0 | ≤ 15.0 |
| 2x23dBm+2LO | ≤ 4.0 | ≤ 5.0 | ≤ 7.0 |

Proposal 7: Consider PC2 PSFCH MPR in Table 2-19 for SL contiguous CA with 1x26dBm PA + 1LO, 2x23dBm PA + 1LO, and 2x23dBm PA + 2LO.

PC2 MPR (S-SSB)

UE RF architecture

Architecture #1-1, #1-2, and #2-1 in Table 2-1 are considered for MPR evaluation.

Simulation assumption

The simulation assumption in Table 2-30 is considered.

Table 2-30. Simulation assumption

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz aggregated CBW: 10+10, 10+30, 20+20, 20+30, 20+40, 30+30 (MHz) |
| Maximum output power for aggregated CBW | 26dBm |
| Modulation for PSBCH | QPSK |
| S-PSS | M-sequence |
| S-SSS | Golden-sequence |
| S-SSB structure |  |
| RB allocation | RBstart: All the possible cases LCRB: 11 RB |
| Structure of Slot | Baseline is to follow RAN1 agreements |
| Waveform | CP-OFDM |
| ACLR | 31dBc |
| Carrier leakage | 25dBc |
| IQ image | 34dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

Evaluation scenario

The S-SSB MPR evaluation scenarios in Table 2-31 are considered for SL contiguous CA.

Table 2-31: SL contiguous CA MPR evaluation scenarios for S-SSB

| Aggregated CBW | Scenario | CC1 | CC2 | Inner/Outer1/Outer2 RB allocation | SCS |
| --- | --- | --- | --- | --- | --- |
| 10MHz + 10MHz | 1 | 11RB0 | 11RB41 | Outer2 | 15 |
|  | 2 | 11RB18 | 11RB22 | Outer2 | 15 |
|  | 3 | 11RB20 | 11RB22 | Outer1 | 15 |
|  | 4 | 11RB33 | 11RB17 | Outer1 | 15 |
|  | 5 | 11RB35 | 11RB7 | Inner | 15 |
|  | 6 | 11RB41 | 11RB0 | Inner | 15 |
| 20MHz + 30MHz | 7 | 11RB0 | 11RB67 | Outer2 | 30 |
|  | 8 | 11RB15 | 11RB32 | Outer2 | 30 |
|  | 9 | 11RB15 | 11RB30 | Outer1 | 30 |
|  | 10 | 11RB33 | 11RB7 | Outer1 | 30 |
|  | 11 | 11RB34 | 11RB7 | Inner | 30 |
|  | 12 | 11RB40 | 11RB0 | Inner | 30 |
| 20MHz + 40MHz | 13 | 11RB0 | 11RB95 | Outer2 | 30 |
|  | 14 | 11RB4 | 11RB30 | Outer2 | 30 |
|  | 15 | 11RB15 | 11RB30 | Outer1 | 30 |
|  | 16 | 11RB33 | 11RB7 | Outer1 | 30 |
|  | 17 | 11RB34 | 11RB7 | Inner | 30 |
|  | 18 | 11RB40 | 11RB0 | Inner | 30 |
| 30MHz + 40MHz | 19 | 11RB0 | 11RB95 | Outer2 | 30 |
|  | 20 | 11RB4 | 11RB30 | Outer2 | 30 |
|  | 21 | 11RB15 | 11RB30 | Outer1 | 30 |
|  | 22 | 11RB47 | 11RB7 | Outer1 | 30 |
|  | 23 | 11RB48 | 11RB7 | Inner | 30 |
|  | 24 | 11RB67 | 11RB0 | Inner | 30 |
| 20MHz + 20MHz | 25 | 11RB0 | 11RB40 | Outer2 | 30 |
|  | 26 | 11RB4 | 11RB30 | Outer2 | 30 |
|  | 27 | 11RB28 | 11RB7 | Outer1 | 30 |
|  | 28 | 11RB33 | 11RB7 | Outer1 | 30 |
|  | 29 | 11RB34 | 11RB0 | Inner | 30 |
|  | 30 | 11RB40 | 11RB0 | Inner | 30 |
| 10MHz + 30MHz | 31 | 11RB0 | 11RB67 | Outer2 | 30 |
|  | 32 | 11RB7 | 11RB32 | Outer2 | 30 |
|  | 33 | 11RB7 | 11RB31 | Outer1 | 30 |
|  | 34 | 11RB13 | 11RB1 | Outer1 | 30 |
|  | 35 | 11RB13 | 11RB0 | Inner | 30 |

Simulation results for S-SSB

Table 2-32, Table 2-33, and Table 2-34 show the MPR simulation results for the SL contiguous CA scenarios with architecture #1-1, #1-2, and #2-1 in Table 2-1 respectively.

Table 2-32: S-SSB MPR simulation results for SL Contiguous CA with 1x26dBm + 1LO

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 9.7 | 5.4 | 5.5 | 0.8 | 0.8 | 0.3 |
| '20MHz+30MHz' | Scenario # | #7 | #8 | #9 | #10 | #11 | #12 |
|  |  | 6.9 | 4.6 | 5.0 | 0.7 | 0.8 | 0.3 |
| '20MHz+40MHz' | Scenario # | #13 | #14 | #15 | #16 | #17 | #18 |
|  |  | 6.9 | 5.1 | 4.5 | 0.7 | 0.8 | 0.2 |
| '30MHz+40MHz' | Scenario # | #19 | #20 | #21 | #22 | #23 | #24 |
|  |  | 6.9 | 6.0 | 4.6 | 0.8 | 0.7 | 0.2 |
| '20MHz+20MHz' | Scenario # | #25 | #26 | #27 | #28 | #29 | #30 |
|  |  | 6.9 | 6.9 | 4.6 | 0.7 | 0.8 | 0.3 |
| '10MHz+30MHz' | Scenario # | #31 | #32 | #33 | #34 | #35 |  |
|  |  | 6.9 | 4.6 | 4.6 | 1.1 | 1.0 |  |

Table 2-33: S-SSB MPR simulation results for SL Contiguous CA with 2x23dBm + 1LO

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10.1 | 5.8 | 5.8 | 1.1 | 1.1 | 0.2 |
| '20MHz+30MHz' | Scenario # | #7 | #8 | #9 | #10 | #11 | #12 |
|  |  | 7.7 | 5.3 | 5.3 | 1.1 | 1.1 | 0.1 |
| '20MHz+40MHz' | Scenario # | #13 | #14 | #15 | #16 | #17 | #18 |
|  |  | 7.2 | 4.9 | 5.3 | 1.0 | 1.1 | 0.2 |
| '30MHz+40MHz' | Scenario # | #19 | #20 | #21 | #22 | #23 | #24 |
|  |  | 7.6 | 6.2 | 5.4 | 1.4 | 1.1 | 0.2 |
| '20MHz+20MHz' | Scenario # | #25 | #26 | #27 | #28 | #29 | #30 |
|  |  | 7.2 | 7.2 | 4.9 | 1.1 | 1.1 | 0.2 |
| '10MHz+30MHz' | Scenario # | #31 | #32 | #33 | #34 | #35 |  |
|  |  | 7.7 | 4.9 | 5.3 | 1.6 | 1.3 |  |

Table 2-34: S-SSB MPR simulation results for SL Contiguous CA with 2x23dBm + 2LO

| '10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2.7 | 2.7 | 2.8 | 0.0 | 0.0 | 0.0 |
| '20MHz+30MHz' | Scenario # | #7 | #8 | #9 | #10 | #11 | #12 |
|  |  | 2.7 | 1.9 | 2.4 | 0.0 | 0.0 | 0.0 |
| '20MHz+40MHz' | Scenario # | #13 | #14 | #15 | #16 | #17 | #18 |
|  |  | 2.8 | 1.9 | 1.9 | 0.0 | 0.0 | 0.0 |
| '30MHz+40MHz' | Scenario # | #19 | #20 | #21 | #22 | #23 | #24 |
|  |  | 2.3 | 1.9 | 1.8 | 0.0 | 0.0 | 0.0 |
| '20MHz+20MHz' | Scenario # | #25 | #26 | #27 | #28 | #29 | #30 |
|  |  | 2.0 | 2.4 | 2.0 | 0.0 | 0.0 | 0.0 |
| '10MHz+30MHz' | Scenario # | #31 | #32 | #33 | #34 | #35 |  |
|  |  | 2.4 | 1.9 | 1.9 | 0.0 | 0.0 |  |

Table 2-35 shows the maximum value of simulation results for architecture #1-1, #1-2, and #2-1 respectively considering Inner, Outer1, and Outer 2 RB allocation ranges.

Table 2-35 : S-SSB MPR simulation results for SL Contiguous CA

|  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  | Inner | Outer1 | Outer2 |
| 1x26dBm + 1LO | 1.0 | 5.5 | 9.7 |
| 2x23dBm + 1LO | 1.3 | 5.8 | 10.1 |
| 2x23dBm + 2LO | 0.0 | 2.8 | 2.8 |

The MPR can be proposed as Table 2-36 based on the simulation results when considering implementation margin.

Table 2-36: S-SSB MPR for SL contiguous CA

|  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  | Inner | Outer1 | Outer2 |
| 1x26dBm + 1LO | ≤ 4.0 | ≤ 9.5 | ≤ 14.0 |
| 2x23dBm + 1LO | ≤ 4.5 | ≤ 10.0 | ≤ 14.5 |
| 2x23dBm + 2LO | ≤ 3.0 | ≤ 7.0 | ≤ 7.0 |

Proposal 8: Consider PC2 S-SSB MPR in Table 2-36 for SL contiguous CA with 1x26dBm PA + 1LO, 2x23dBm PA + 1LO, and 2x23dBm PA + 2LO.

Conclusion

In this contribution, we provided PC2 PSSCH, PSFCH, and S-SSB MPR simulation results for SL intra-band contiguous CA. Based on those, the followings are proposed.

- MPR for SLCA PSSCH/PSCCH

Proposal 1: Consider PC2 PSSCH MPR in Table 2-11 for SL contiguous CA of contiguous RB allocation with 1x26dBm PA + 1LO.

Table 2-11: PSSCH/PSCCH MPR for contiguous RB allocations with 1x26dBm+1LO

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 2.5 | ≤ 5.0 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 |

Proposal 2: Consider PC2 PSSCH MPR in Table 2-12 for SL contiguous CA of contiguous RB allocation with 2x23dBm PA + 1LO.

Table 2-12: PSSCH/PSCCH MPR for contiguous RB allocations with 2x23dBm+1LO

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 6.0 |
|  | 16QAM | ≤ 3.5 | ≤ 6.0 |
|  | 64QAM | ≤ 4.5 | ≤ 6.0 |
|  | 256QAM | ≤ 7.5 | ≤ 7.5 |

Proposal 3: Consider PC2 PSSCH MPR in Table 2-13 for SL contiguous CA of contiguous RB allocation with 2x23dBm PA + 2LO.

Table 2-13: PSSCH/PSCCH MPR for contiguous RB allocations with 2x23dBm+2LO

| Modulation |  | MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 2.0 | ≤ 5.0 |
|  | 16QAM | ≤ 2.5 | ≤ 5.0 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 |

Proposal 4: Consider PC2 PSSCH MPR in Table 2-20 for SL contiguous CA of non-contiguous RB allocation with 1x26dBm PA + 1LO.

Table 2-20: PSSCH/PSCCH MPR for Non-contiguous RB allocations with 1x26dBm+1LO

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 7.5 | ≤ 11.0 |
|  | 16QAM | ≤ 4.0 | ≤ 7.5 | ≤ 11.0 |
|  | 64QAM | ≤ 5.5 | ≤ 7.5 | ≤ 11.0 |
|  | 256QAM | ≤ 8.0 | ≤ 8.0 | ≤ 11.0 |

Proposal 5: Consider PC2 PSSCH MPR in Table 2-21 for SL contiguous CA of non-contiguous RB allocation with 2x23dBm PA + 1LO.

Table 2-21: PSSCH/PSCCH MPR for Non-contiguous RB allocations with 2x23dBm+1LO

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 8.5 | ≤ 12.0 |
|  | 16QAM | ≤ 4.5 | ≤ 8.5 | ≤ 12.0 |
|  | 64QAM | ≤ 7.0 | ≤ 8.5 | ≤ 12.0 |
|  | 256QAM | ≤ 9.0 | ≤ 9.0 | ≤ 12.0 |

Proposal 6: Consider PC2 PSSCH MPR in Table 2-22 for SL contiguous CA of non-contiguous RB allocation with 2x23dBm PA + 2LO.

Table 2-22: PSSCH/PSCCH MPR for Non-contiguous RB allocations with 2x23dBm+2LO

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 6.5 | ≤ 7.5 |
|  | 16QAM | ≤ 4.0 | ≤ 6.5 | ≤ 7.5 |
|  | 64QAM | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 |
|  | 256QAM | ≤ 8.0 | ≤ 8.0 | ≤ 8.0 |

- MPR for SLCA PSFCH

Proposal 7: Consider PC2 PSFCH MPR in Table 2-19 for SL contiguous CA with 1x26dBm PA + 1LO, 2x23dBm PA + 1LO, and 2x23dBm PA + 2LO.

Table 2-29: PSFCH MPR for SL contiguous CA

|  | MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  | R ≤ 0. 3 | 0.3 < R ≤ 0. 5 | 0.5 < R ≤ 1.0 |
| 1x26dBm+1LO | ≤ 5.0 | ≤ 10.0 | ≤ 14.0 |
| 2x23dBm+1LO | ≤ 6.0 | ≤ 11.0 | ≤ 15.0 |
| 2x23dBm+2LO | ≤ 4.0 | ≤ 5.0 | ≤ 7.0 |

- MPR for SLCA S-SSB

Proposal 8: Consider PC2 S-SSB MPR in Table 2-36 for SL contiguous CA with 1x26dBm PA + 1LO, 2x23dBm PA + 1LO, and 2x23dBm PA + 2LO.

Table 2-36: S-SSB MPR for SL contiguous CA

|  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- |
|  | Inner | Outer1 | Outer2 |
| 1x26dBm + 1LO | ≤ 4.0 | ≤ 9.5 | ≤ 14.0 |
| 2x23dBm + 1LO | ≤ 4.5 | ≤ 10.0 | ≤ 14.5 |
| 2x23dBm + 2LO | ≤ 3.0 | ≤ 7.0 | ≤ 7.0 |

- References

[1] R4-2410587, “WF on NR_SL_ intraB_CA_ITS_part2”, Huawei, LGE, Meta Ireland

[2] R4-2406611, “WF on NR_SL_ intraB_CA_ITS”, OPPO, Meta Ireland

[3] RP-240840, “New WID on NR Sidelink: Intra-band Carrier Aggregation in ITS band”, Moderator(Apple)