3GPP TSG-RAN4 Meeting #112bis	R4-2415897

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	LG Electronics

Title:	ATG UE RF requirements for UL-MIMO

Agenda item:	6.10.2.3

Document for:	Discussion

Introduction

In this paper, we provide our views on ATG UE RF requirements for UL-MIMO based on the approved WF[1].

Discussion

Based on the approved WF[1], we would like to discuss ULFPTx mode and single antenna port related requirement for ATG UL-MIMO.

4-1-1: ULFPTx mode for UL-MIMO - FFS on whether to apply ULFPTx mode for ATG UE 4-1-2: whether preclude the single antenna port related requirement - FFS on whether to preclude the single antenna port related requirement. 4-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18 - If the legacy requirement for UL MIMO is defined at each antenna connector, - For ATG UE with antenna array, the requirement should be updated [at each TAB connector]. - If the legacy requirement for UL MIMO is defined per layer, FFS - Option 1: for ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer. - Option 2: for ATG UE with capability antennaArrayType-r18, the requirement should be updated per layer without explicitly emphasize the TAB connector. 4-2-2: whether to apply coherent UL MIMO requirement for ATG UE - FFS on whether to apply coherent UL MIMO requirement for ATG UE 4-3: receiver requirements - Reuse ATG single carrier requirements for following Rx requirements (ZTE, Ericsson) - REFSENSE - Max input level - ACS - Blocking - Spurious emission - Intermodulation - FFS whether to test the Rx requirements under the UL MIMO scenario since Rx requirements is the same for R18 ATG UE and R19 ATG UE supporting UL-MIMO.

ULFPTx mode for UL-MIMO

In NR, ULFPTx mode is specified for PC2 and PC1.5 as follows.

- Single layer for 2 Tx port & 4 Tx port

  - PC2/PC1.5

- 2 layers for 4 Tx port

  - PC1.5

For ATG single layer UL MIMO operation and two antenna-ports, in case that R-MOP of 26dBm is declared for UL-MIMO, ULFPTx mode 1/2/Full in Table 2-1 can be supported depending on UE PA implementation.

.

Table 2.1: Example of ATG ULFPTx (R-MOP = 26dBm)

| ATG ULFPTx mode | PA1(dBm) | PA2(dBm) |
| --- | --- | --- |
| Mode2 | 26 | 23~25 |
| Mode-Full | 26 | 26 |
| Mode1 | 23~25 | 23~25 |

Proposal 1: Consider ULFPTx mode for ATG UL-MIMO.

Whether to preclude the single antenna port related requirement

The following requirements same as the existing NR UL-MIMO need to be considered.

For single layer UL MIMO operation and two antenna-ports,

- the rated maximum output power shall be met to a UE that supports ULFPTx feature and is configured in its declared full power mode.

For single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity,

- the requirements in clause 6.2J apply for at least one antenna connector for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.2J apply for at least one TAB connector for the ATG UE indicating the capability antennaArrayType-r18.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity,

- the rated maximum output power applies for sum of two antenna connectors for the ATG UE not indicating the capability antennaArrayType-r18.

- the rated maximum output power applies for sum of two TAB connectors for the ATG UE indicating the capability antennaArrayType-r18.

For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity and the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16,

- the requirements in clause 6.2J apply for at least one antenna connector for the ATG UE not indicating the capability antennaArrayType-r18.

- the requirements in clause 6.2J apply for at least one TAB connector for the ATG UE indicating the capability antennaArrayType-r18.

Proposal 2: Consider the single antenna port related requirement.

- For single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity

- For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity,

- For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity and the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

Conclusion

In this contribution, we provided our view on ULFPTx mode and single antenna port related requirement for ATG UL-MIMO.

Proposal 1: Consider ULFPTx mode for ATG UL-MIMO.

Proposal 2: Consider the single antenna port related requirement.

- For single layer UL MIMO operation and single antenna-port(W=1) without indicating txDiversity

- For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity,

- For single layer UL MIMO operation and single antenna-port(W=1) with indicating txDiversity and the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

- References

[1] R4-2414322, “WF on Rel-19 ATG UE RF requirements”, CMCC

[2] RP-240839, “New WID on Enhancements for Air-to-ground network for NR”, Moderator(Apple)