# 3GPP TSG-RAN WG4 Meeting # 112-bis	R4-2415899

# Hefei, China, Oct.14th – 18th, 2024

# Title: 	Further discussion on PC2 n28 40MHz

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	5.20.2

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, a new WID on Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28 was approved in [1], where the objectives are listed as below:

The objectives of this WI are to introduce requirements for UE Power Class 2 and 40MHz Channel Bandwidth for NR band n28 as optional features, including the following aspects: - Introduce requirements for PC2 and those associated with following bullets - Both 1Tx and 2Tx PC2 are supported - UL-MIMO is supported. - Specify A-MPR requirements for PC2 - Introduce 40MHz Channel bandwidth located in UL: 703 - 743MHz and DL: 758 - 798MHz for band n28 supporting PC3 and PC2. - Allow flexible channel allocations within the frequency range (UL: 703 - 743MHz and DL: 758 - 798MHz) for BW<40MHz. - Release independence of UE 40MHz Channel bandwidth to Rel-15 is anticipated. - No changes to the existing requirements (38.101-1 v18.5.0) (including A-MPR values and RB allocation regions) for power class 3 with existing channel bandwidths up to and including 30MHz to avoid impacts on existing power class 3 networks. In the event of new regulatory requirements, this assumption can be revisited.

In last meeting, some agreements are captured in the WF[2], in which there are several open issues.

In this contribution, we give some further discussions on the RF requirements of 40MHz for n28 PC2.

# 2	Discussion

1. Single set or two sets of RF requirement

Issue 2-1-1: UE architecture assumption Agreement: - No need to study the feasibility of UE architecture, companies are encouraged to provide analysis on RF requirements based on their own implementation. - RAN4 RF requirements should accommodate different UE architecture assumption. - FFS on whether single set of requirements for different UE architectures should be defined.

In terms of the description of the WID, evolve duplexer implementation is desired from industry, i.e. single band duplexer implementation, would be used for studied. This is different with the existing specs in which dual duplexer architecture for UE is assumed.

Due to different filter solutions of dual-duplexer and full-band duplexer implementations, two sets of requirements means there is a need to introduce UE capabily to inform NW about the UE filter implementations. For PC2 FDD band, some RF requirements such as RSD have already been defined as 1Tx and 2Tx with reporting the Tx diversity capabily. If further split the RF requirements into different filter solutions, then it seems it would become duplicated for the RF requirements definition. Without specifying new capability, single set of RF requirements can be applied to either filter implementation.

Proposal: Without specifying new capability, single set of RF requirements can be applied to either filter implementation.

2. ΔMPR&A-MPR

Similar as PC3 40MHz, the ΔMPR requirements for PC2 n28 40MHz. In current spec, ΔMPR are defined for PC2 n40 and n71, and it use the same ΔMPR requirements as PC3. To our understanding, ∆MPR is used for different implementations and accommodating the more pronounced PA memory effect. Given that band n71 falls within a comparable frequency band to band n28, it is anticipated that the similar approach would be reused. Therefore we propose to use the same ∆MPR as PC3.

Proposal 2: To use the same ∆MPR as PC3 for PC2 n28 40MHz.

For A-MPR, there are two network signalling (i.e. NS_17 and NS_18) for band n28 in current spec. During the R18, the PC2 A-MPR for 5/10/15/20/25/30MHz have been agreed and captured in WF [5], and the PC2 A-MPR are applicable for both dual-duplexer and full-band duplexer implementations. Thus only 40MHz is left for PC2 A-MPR evaluation.

For NS_17, the evaluation results from companies in R18 shows A-MPR requirements are needed for PC2.

# 3 Conclusion

In this contribution, we give some initial discussion on the RF requirements of 40MHz for n28 PC3. The conclusions are summarized in the following:

Proposal 1: Without specifying new capability, single set of RF requirements can be applied to either filter implementation.

Proposal 2: To use the same ∆MPR as PC3 for PC2 n28 40MHz.

# 4	Reference

[1] RP-241660, New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC, CBN

[2] R4-2414274, WF on introduction of PC2 and 40MHz CBW in NR band n28, CMCC, CBN, China Broadnet, Approved

[3] R4-2411879, Discussion on PC2 n28 40MHz, ZTE Corporation, Sanechips.