# 3GPP TSG-RAN WG4 Meeting # 112-bis	R4-2415900

# Hefei, China, Oct.14th – 18th, 2024

# Title: 	Further discussion on 6Rx Receiver requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.1.1.3.1

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements are included in the approved WF[2], in which the REFSEN requirements were approved after long debate, as shown below:

# Topic 1: REFSENS (delta RIB,6R) ## Sub-topic 1-1: General considerations for specifying ΔRIB,6R value Issue 1-1-1: Whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case Agreement: Include n104 in the high band (n77, n78, n79) category. Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA Agreement: Different value for handheld UE and FWA. Issue 1-1-3: Verification of 6Rx receiver requirements Way Forward: Companies encouraged to provide their views on the ZTE proposal in Proposal 5 in R4-2411883 and any further positions on verification of 6Rx receiver requirements at future meetings given that ΔRIB,6R values are now defined. ## Sub-topic 1-2: ΔRIB,6R values for handheld UE and FWA Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA Agreement: Based on the outcome of Issue 1-1-2, RAN4 to specify ΔRIB,6R values for handheld UE and FWA as follows. - Adopt the average value of ΔRIB,6R for handheld UE from company proposals (updated to include company proposals from RAN4#111) and specify it for handheld UE devices. Table: Averaged values for handheld UE based on proposals Operating Bands; ΔRIB,6R (dB). MediaTek; Band n41; -3.4. Band n77, n78, n79, n104; -3.1. Xiaomi; Band n41; -3.2. Band n77, n78, n79, n104; -3.0. LGE; Band n41; -3.5. Band n77, n78, n79, n104; -3.0. Spreadtrum; Band n41; -3.3. Band n77, n78, n79, n104; -3.0. Meta; Band n41; -3.5. Band n77, n78, n79, n104; -3.1. vivo #1; Band n41; -3.0. Band n77, n78, n79, n104; -3.0. vivo #2; Band n41; -3.2. Band n77, n78, n79, n104; -3.0. ZTE; Band n41; -3.5. Band n77, n78, n79, n104; -3.0. Ericsson; Band n41; -3.6. Band n77, n78, n79, n104; -3.2. Google; Band n41; -3.4. Band n77, n78, n79; -3.0. Huawei; Band n41; -3.3. Band n77, n78, n79, n104; -3.0. Qualcomm; Band n41; -3.4. Band n77, n78, n79, n104; -3.0. Apple (R4-2407071); Band n41; -3.3. Band n77, n78, n79, n104; -3.0. OPPO (R4-2408759); Band n41; -3.6. Band n77, n78, n79, n104; -3.2. Average; Band n41; -3.4. Band n77, n78, n79, n104; -3.0. - Adopt the average value of ΔRIB,6R for FWA UE from company proposals indicating different FWA UE values from handheld UE values and specify it for FWA UE devices. Table: Averaged values for FWA UE based on proposals Operating Bands; ΔRIB,6R (dB). LGE; Band n41; -3.7. Band n77, n78, n79, n104; -3.4. Spreadtrum; Band n41; -3.6. Band n77, n78, n79, n104; -3.2. vivo; Band n41; -3.4. Band n77, n78, n79, n104; -3.2. Google; Band n41; -3.7. Band n77, n78, n79; -3.3. Nokia; Band n41; -4.0. Band n77, n78, n79, n104; -3.4. Average; Band n41; -3.7. Band n77, n78, n79, n104; -3.3.

As seen as above, for the receiver requirements, the only open issue is how to verify the 6Rx requirements, which was initial proposed in our contribution [2].

In this contribution, we give some further discussions on the verification UE REFSEN requirements related to 6Rx.

# 2	Discussion

The verification of 6Rx requirements aim to describe whether or not the other 2Rx/4Rx UE Rx requirements should be applied for a UE equipped 6Rx, or how to verify the 6Rx requirements when a UE equips 8Rx. The Rx requirements verification are included in diversity characteristics section in TS38.101-1.

In [2], We summarize the diversity characteristics for 2Rx, 4Rx and 8Rx single carrier Rx requirements in the spec. Similarity, the verification of 6Rx requirements can be found in Table 1.

Table 1: diversity characteristics for 2Rx/4Rx/6Rx/8Rx

|  | Single carrier REFSEN requirement | Rx requirements other than single carrier REFSENS |
| --- | --- | --- |
| UE is equipped with 2Rx | 2Rx | 2Rx |
| UE is equipped with 4Rx | 2Rx, 4Rx | 4Rx, skip 2Rx |
| UE is equipped with 6Rx | 2Rx, 4Rx, 6Rx | 6Rx, skip both 2Rx and 4Rx |
| UE is equipped with 8Rx | 2Rx, 4Rx, 6Rx, 8Rx | 8Rx, skip both 2Rx, 4Rx and 6Rx |

In table 1, when 6Rx is introduced, in order to reduce the test burden, the diversity characteristics aims for the requirements test verification shall be discussed. We think the similar method as 4Rx/8Rx could be reused, which are:

- For single carrier REFSEN requirements, additional requirement for four Rx antenna ports and six Rx antenna ports shall be verified in operating bands where the UE is equipped with six Rx antenna ports.

- For Rx requirements other than single carrier REFSEN, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands where the UE is equipped with six Rx antenna ports.

In addition, when a UE equipped with eight Rx antenna ports, it seems the 6Rx REFSEN requirements shall be verified additionally, and there is no need to verify 6Rx for the Rx requirements other than REFSEN.

We have the following proposal 1 and proposal 2 for diversity characteristics:

Proposal 1. When the UE is equipped with six Rx antenna ports:

- For single carrier REFSEN requirements, additional requirement for four Rx antenna ports and six Rx antenna ports shall be verified in operating bands;

- For Rx requirements other than single carrier REFSEN, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands.

Proposal 2. When the UE is equipped with eight Rx antenna ports, except 2/4Rx antenna ports:

- For single carrier REFSEN requirements, additional requirement for six Rx antenna ports shall be verified in operating bands;

- For Rx requirements other than single carrier REFSEN, the UE shall skip six Rx antenna ports requirements in operating bands.

- With the above two proposals, the modifications for the specification TS38.101-1 are proposed in the companion CR[3] for information.

# 3 Conclusion

In this contribution, we give some further discussions on the verification of Rx requirements for 6Rx. The conclusions are:

Proposal 1. When the UE is equipped with six Rx antenna ports:

- For single carrier REFSEN requirements, additional requirement for four Rx antenna ports and six Rx antenna ports shall be verified in operating bands;

- For Rx requirements other than single carrier REFSEN, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands.

Proposal 2. When the UE is equipped with eight Rx antenna ports, except 2/4Rx antenna ports:

- For single carrier REFSEN requirements, additional requirement for six Rx antenna ports shall be verified in operating bands;

- For Rx requirements other than single carrier REFSEN, the UE shall skip six Rx antenna ports requirements in operating bands.

# 4	Reference

[1]  R4-2414285, WF on requirements for 6Rx, AT&T

[2]  R4-2411883, Further discussion on 6Rx RESXEN requirements, ZTE, Sanechips

[3]  R4-24xxxxx, draft CR on 38.101-1 introduction of 6Rx (Rx part), ZTE, Sanechips