# 3GPP TSG-RAN WG4 Meeting # 112-bis	R4-2415905

# Hefei, China, Oct.14th – 18th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Further discussion on R19 PC1.5 Intra-band UL CA

# Agenda item:	6.1.1.1.2

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements/open issues are included in the WF[2]. In this contribution, we give some further discussions on PC1.5 intra-band contiguous and non-contiguous UL CA.

# 2	Discussion

For the MPR evaluation, we think the methodology proposed in [3] is a promising method which can be avoid large simulation/measurement work. Even with simulation, we think the assumption would be the same with the existing one, i.e. equal PSD assumption.

### MPR evaluation methodology and assumption Online agreement: - MPR and CANS_04 A-MPR studies for PC1.5 contiguous intra-band ULCA focusses on TxD architecture and may account for PSD imbalance with up to 6dB. - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB - PC1.5 contiguous intra-band ULCA based on dualPA architecture with two LOs is not specified in R19 - MPR studies for PC1.5 non-contiguous intra-band ULCA focusses on dualPA architecture with two LOs and may account for PSD imbalance with up to 6dB. - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB - PC1.5 non-contiguous intra-band ULCA based on TxD architecture is not specified in R19

In current specification, the PSD information for MSD are not explicated in the texts of MPR definition, which can be interpreted that the requirements are applied for either equal PSD or imbalance PSD although the equal PSD is assumed for MPR evaluation.

- Proposal 1: To reuse the existing same methodology, i.e. equal PSD assumption to defined the requirements.

# 3 Conclusion

In this contribution, we give further discussions on PC1.5 Intra-band contiguous and non-contiguous UL CA. The proposals and conclusions are:

- Proposal 1: To reuse the existing same methodology, i.e. equal PSD assumption to defined the requirements.

# 4	Reference

[1] R4-2410750, WF on high power UE, Samsung

[2] R4-2414277, WF on HPUE for CA in TN, Samsung

[3] R4-2411168, MPR for PC1.5 intra-band contiguous UL CA , Apple.