# 3GPP TSG-RAN WG4 Meeting # 112-bis                              R4-2415907

# Hefei, China, Oct.14th – 18th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Further discussion on R19 Increasing UE transmission power limit

# Agenda item:	6.1.1.1.4

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements are included in the WF[1]. In this contribution, we give some further discussions on R19 increasing UE transmission power limit.

# 2	Discussion

The methodology was agreed as below:

## 3.1 The methodology Adhoc agreement: - Use higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability - No new power class is introduced

It can be seen that the above methodology are reused from the existing R17/R18 methodologies. Meanwhile, considering there are no new MSD values defined for a UE reporting the high power IEs, i.e. higherPowerLimit-R17 and higherPowerLimtMRDC-R17 capability. Therefore, the impacts for the specification are limited for R19 supporting the feature of increasing UE transmission power limit.

Observation 1. With the existing R17/R18 methodologies, the impacts for the specification are limited for R19 supporting the feature of increasing UE transmission power limit.

For the scenarios, it was agreed:

## 3.2 The scenarios to be considered in Rel-19 Adhoc agreement with offline modification: - For both 2Tx and 3Tx, higherPowerLimit-R17/higherPowerLimtMRDC-R17 is enabled for any specified inter-band band combination

It can be seen that the above agreement is a generic requirements, since it is somehow aligned with the approach for the NOTE in the specification where ‘A general way for the eligible band combination to enable increase higher power limit feature is used in the NOTE description in TS38.101-1/3.’

But still, as we mentioned in the contribution [3], three UL cluster cases are not supported: CA_nX(2A)-nYA, CA_nXA-nY(2A) , and combinations with four UL CCs are not supported for either 2Tx or 3Tx band combination. Combining the above, the ‘any specified inter-band band combination’ does not include the following two scenarios:

1: 3UL CC: CA_nX(2A)-nYA, CA_nXA-nY(2A)

2: Four UL CC (or >3UL CC)

The reason is the above two scenarios band combination will not be introduced/specified in the specifications.

Observation 2. Either UL inter-band CA with intra-band non-contiguous CA or four UL CA(or >3UL CC) are invalid inter-band band combination scenarios.

The intention for R19 increasing UE transmission power limit is to enable this feature for any possible HPUE band combination, this is reflected in the wording ‘...enabled for any specified inter-band band combination’ in the above agreements.

Our understanding for the ‘any specified inter-band band combination’ is the inter-band band combination specified in the spec via the corresponding basket WIDs, i.e.:

PC2 for 2Tx/3Tx UL CA_nX-nY with the following power configuration on each NR carrier:

- PC3+PC3

- PC2+PC3

- PC2+PC2

For the above configurations, only PC2+PC3(different power class for each constitute band) can be enable high power limit capability. For other power configurations like PC2+PC5 in future, the high power limit could also be enable.

- PC1.5 for UL CA_nX-nY with the following power configuration on each NR carrier:

- PC3+PC1.5

- PC2+PC1.5

- PC2+PC2

For the above configurations, only PC3+PC1.5 and PC2+PC1.5(different power class for each constitute band) can be enable high power limit capability. For other power configurations like PC5+PC1.5 in future, the high power limit could also be enable. However, for the power configurations like PC2+PC3, it cannot be enabled to the high power for PC1.5 band combination, since the sum of powers of CCs cannot exceed the 29dBm.

Therefore, the modification for the specifications should reflect the above.

In below texts, we give some modifications for the TS38.101-1 to reflect the current agreements to enable R19  increasing UE transmission power limit feature:

For 2Tx:

Modification on the NOTE in Table 6.2A.1.3-1 in section 6.2A.1.3	UE maximum output power for Inter-band CA

NOTE 7: The UE that supports a PC2 uplink CA configuration with a composite of supporting PC3 within an NR band and PC2 within a second NR band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. NOTE 8: The UE that supports a PC3 uplink CA configuration with a composite of supporting PC3 within an NR TDD or FDD band and PC5 within a second NR band listed in Table 6.2F.1-1 may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2A.4.1.3. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise.

Modification on the description for in sub-clause 6.2A.4.1.3:

For the case when the UE indicates higherPowerLimit-r17, PPowerClass,CA is replaced by 10 log10 (pPowerClass,A + pPowerClass,CA,B). Where - pPowerClass,A is the linear value of the maximum UE power for serving cell c on the operating band A specified in Table 6.2.1-1 according to ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise without taking into account the tolerance; - pPowerClass,CA,B is the linear value of the maximum UE power for serving cell(s) on the operating band B including intra-band carrier aggregation specified in Table 6.2F.1A.2-1 or Table 6.2A.1.1-1 according to ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass, otherwise without taking into account the tolerance.

For 3Tx with UL MIMO:

NOTE in Table 6.2H.3.1-1 in section 6.2H.3.1 UE maximum output power for inter-band UL CA with UL MIMO

NOTE 1: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. NOTE 4: The UE supports PC2 with UL MIMO in either one of the TDD bands and PC2 in the other TDD band. NOTE 5: Power class 3 is default power class unless otherwise stated. NOTE 6: FWA form factor is targeted unless otherwise stated. NOTE 7: The UE that supports a PC2 uplink CA configuration with a composite of supporting PC3 within an NR band and PC2 within a second NR band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2H.3.4. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. NOTE 8: The UE that supports a PC1.5 uplink CA configuration with a composite of supporting PC3 or PC2 within an NR band and PC1.5 within a second TDD NR band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2H.3.4. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise.

Modification on the description for in sub-clause 6.2H.3.4:

#### 6.2H.3.4 Configured transmitted power for inter-band UL CA with UL MIMO For inter-band UL CA with UL MIMO in one of the two frequency bands, the requirements in clause 6.2A.4.1.3 apply except that: - PPowerClass,CA is the maximum UE power specified in Table 6.2H.3.1-1 without taking into account the tolerance, If the UE indicates higherPowerLimit-r17 for an eligible CA configuration as specified in Table 6.2H.3.1-1 and ΔPPowerClass, CA = 0, PPowerClass,CA is replaced by 10 log10 ∑ pPowerClass,c.; - MPRc and A-MPRc are specified in clause 6.2D.2 and clause 6.2D.3 respectively for the component carrier configured with UL MIMO. - ΔPPowerClass,CA: – For a power class 2 UE, it is 3dB when the requirements of default power class are applied as specified in sub-clause 6.2.H.3.1, otherwise ΔPPowerClass, CA = 0 dB; – For a power class 1.5 UE, it is 6dB when the requirements of default power class are applied as specified in sub-clause 6.2.H.3.1; and it is 3dB when the requirements of power class 2 are applied as specified in sub-clause 6.2.H.3.1; otherwise ΔPPowerClass, CA = 0 dB;

- Similar changes are foreseen for 3Tx with TxD in sub-clauses 6.2L.3.1 and 6.2L.3.4.

For 3Tx with TxD:

NOTE in Table 6.2L.3.1-1 in section 6.2L.3.1	UE maximum output power for inter-band UL CA with Tx Diversity

NOTE 1: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: The UE supports PC3 in FDD band and PC2 with Tx Diversity in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with Tx Diversity in TDD band. NOTE 4: Power class 3 is the default power class unless otherwise stated. - NOTE 5: FWA form factor is targeted unless otherwise stated. NOTE 6: The UE that supports a PC2 uplink CA configuration with a composite of supporting PC3 within an NR band and PC2 within a second NR band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2L.3.4. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise. NOTE 7: The UE that supports a PC1.5 uplink CA configuration with a composite of supporting PC3 or PC2 within an NR band and PC1.5 within a second NR band may signal a higherPowerLimit-r17 capability whereby the maximum output power indicated in the table may be exceeded in accordance with sub-clause 6.2L.3.4. The power classes referenced are according to the reported ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass otherwise.

Modification on the description for in sub-clause 6.2H.3.4:

#### 6.2L.3.4 Configured transmitted power for inter-band UL CA with Tx Diversity For inter-band UL CA with Tx Diversity in one of the two frequency bands, the requirements in clause 6.2A.4.1.3 apply except that: - PPowerClass,CA is the maximum UE power specified in Table 6.2L.3.1-1 without taking into account the tolerance, If the UE indicates higherPowerLimit-r17 for an eligible CA configuration as specified in Table 6.2L.3.1-1 and ΔPPowerClass, CA = 0, PPowerClass,CA is replaced by 10 log10 ∑ pPowerClass,c.; - MPRc and A-MPRc are specified in clause 6.2G.2 and clause 6.2G.3 respectively for the component carrier configured with Tx Diversity. - ΔPPowerClass,CA: – For a power class 2 UE, it is 3dB when the requirements of default power class are applied as specified in sub-clause 6.2.J.3.1, otherwise ΔPPowerClass, CA = 0 dB; – For a power class 1.5 UE, it is 6dB when the requirements of default power class are applied as specified in sub-clause 6.2.J.3.1; and it is 3dB when the requirements of power class 2 are applied as specified in sub-clause 6.2.J.3.1; otherwise ΔPPowerClass, CA = 0 dB;

# 3 Conclusion

In this contribution, we give some discussions on the R19 increasing UE transmission power limit. the proposals and conclusions are:

Observation 1. With the existing R17/R18 methodologies, the impacts for the specification are limited for R19 supporting the feature of increasing UE transmission power limit.

Observation 2. Either UL inter-band CA with intra-band non-contiguous CA or four UL CA are invalid inter-band band combination scenarios.

Proposal: To take the modifications to TS38.101-1 into account.

# 4	Reference

[1] R4-2414277, WF on HPUE for CA in TN, Samsung

[2] R4-2408353, on R19 Increasing UE transmission power limit, ZTE Corporation, Sanechips

[3] R4-2411880, Further discussion on R19 Increasing UE transmission power limit, ZTE Corporation, Sanechips