# 3GPP TSG-RAN WG4 Meeting # 112-bis	R4-2415908

# Hefei, China, Oct.14th – 18th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	On EESS mmWave UE spurious emission

# Agenda item:	5.21.2

# Document for:	Approval

# 1 Introduction

In last RAN #105 meeting, a new work item for mmWave UE spurious emission was approved in [1], the objectives  are captured below:

The objective of this WI includes the following aspects: - For UE, specify additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258, where - Introduce new corresponding Network Signalling(s) and associated A-MPR(s) if needed by taking into the consideration of the relationship with the existing NS_200, NS_202, NS_203, CA_NS_200, CA_NS_202 and CA_NS_203 - All the power classes are considered in the above work - For BS, conduct necessary modifications on protection of Earth Exploration Satellite Service requirements if any

The work aims to study and define new NS_XX and the corresponding A-MPR requirements for the more stringent UE spurious emission requirement of -35dBW/200MHz or -5dBm/200MHz in EESS(Earth Exploration Satellite Service) frequency range of 23.6GHz-24GHz which shall be satisfied and required in China and EU at an earlier stage.

In this contribution, we give some discussions on the NS flags and the corresponding A-MPR requirements for the new limits -35dBW/200MHz or -5dBm/200MHz in EESS frequency range of 23.6GHz-24GHz.

# 2	Discussion

## 2.1 NS flags and A-MPR

In current TS38.101-2 specification, NS_202 and NS_203 were already defined to meet the regulatory requirement. Regarding the protection on the EESS passive band frequency range 23.6-24 GHz, only the regulatory requirements of +1 dBm/200 MHz were considered for the 5G UE brought into use after 1st September 2027.

As stated in the WID objectives, to support the commercial deployment, the more stringent UE spurious requirement of −35 dBW/200 MHz or -5dBm/200MHz is needed according the studies on mmWave usage.

- The -5dBm/200MHz on 23.6GHz-24GHz UE spurious emission for commercial deployment  needs to be satisfied in China.

- EU Decision 2020-590 has decided to implement −35 dBW/200 MHz (i.e. equivalent to -5dBm/200MHz) on 23.6GHz-24GHz from Jan. 1, 2024.

It means the more stringent regulatory requirements are needed to be considered before 1st September 2027. In our understanding, there are three following regulatory limits need to be considered :

1:  -10 dBm/100 MHz (Harmonic) in 7.25 GHz ≤ f ≤ 2nd

2:  +1 dBm/200 MHz in 23.6 - 24.0 GHz

3:  -5dBm/200 MHz in 23.6 - 24.0 GHz

In table 1, we give a briefly summary for the NS values for band n257 and n258 meet the regulatory requirements.

Table 1. NS value for band n257 and n258 meet the regulatory requirements

| NR bands | NS value | A-MPR | Regulatory requirements |
| --- | --- | --- | --- |
| n257, n258 | NS_200 |  | Protected range: spurious Band applicability: all Limit: -13 dBm/MHz |
| n257, n258 | NS_202 | PC1: A-MPR=11.0dB PC2/3/4/5/6/7: A-MPR=1.0 dB | (For EU) Protected range: 7.25 GHz ≤ f ≤ 2nd harmonic Band applicability: all band Limit: -10 dBm/100 MHz Protected range: 23.6 - 24.0 GHz Band applicability: n258, n257 Limit: +1 dBm/200 MHz |
| n258 | NS_203 | PC1: 3.0 dB if Offset frequency < BWchannel, 0.0 dB otherwise PC2/3/4/5/6/7: A-MPR=0dB | Protected range: 23.6 - 24.0 GHz Band applicability: n258, n257 Limit: +1 dBm/200 MHz |
| n257, n258 | NS_20x | TBD | (For EU) Protected range: 7.25 GHz ≤ f ≤ 2nd harmonic Band applicability: all band Limit: -10 dBm/100 MHz Protected range: 23.6 - 24.0 GHz Band applicability: n258, n257 Limit: -5dBm/200 MHz |
| n257, n258 | NS_20y | TBD | Protected range: 23.6 - 24.0 GHz Band applicability: n258, n257 Limit: -5dBm/200 MHz |

For NS_202, the A-MPR is defined as 11dB for PC1 and 1.0dB for PC2/3/4/5/6/7, actually this requirements were derived based on the regulatory requirements of -10dBm/100MHz of RC REC 74-01 [2].

Later, NS_202 was updated to stay consistent with EU emissions requirement changes (+1 dBm/200MHz) [3]. However, the A-MPR requirements were not updated because the -10dBm/100MHz or -7dBm/200MHz is more stringent than +1 dBm/200MHz, which means the NS_202 A-MPR gated by -10 dBm/100 MHz requirement can also met the +1 dBm/200MHz requirements.

Similarly, when relaxation of -5dBm/200MHz (or -35 dBW/200 MHz) on 23.6GHz-24GHz introduced, it will not change the back off requirements, i.e. the NS_202 A-MPR is gated by -10 dBm/100 MHz requirement can also met the -5dBm/200MHz (or -35 dBW/200 MHz) requirements.

It is similar situation for CA_NS_202 A-MPR.

Observation 1: The existing NS_202/CA_NS_202 A-MPR requirements can met the -5dBm/200MHz (or -35 dBW/200 MHz) requirements in 23.6 - 24.0 GHz ranges.

Due to different changeover date for different EESS protection emission limits in the frequency range 23.6 - 24.0 GHz, so it is proposed to introduce NS_204 (for single CC) and CA_NS_204 (for CA) for the emissions limit of -5dBm/200MHz (or -35 dBW/200 MHz) in the frequency range 23.6 - 24.0 GHz and -10 dBm/100 MHz in the frequency range 7.25 GHz ≤ f ≤ 2nd harmonic (for EU), where the A-MPR defined in NS_202 (for single CC) and CA_NS_202 (for CA) can be applied, respectively.

Proposal 1. It is proposed to introduce NS_204 (for single CC) and CA_NS_204 (for CA) for the emissions limit of -5dBm/200MHz (or -35 dBW/200 MHz) in the frequency range 23.6 - 24.0 GHz and -10 dBm/100 MHz in the frequency range 7.25 GHz ≤ f ≤ 2nd harmonic (for EU), where the A-MPR defined in NS_202 (for single CC) and CA_NS_202 (for CA) can be applied, respectively.

For NS_203 (and also CA_NS_203), the A-MPR requirements are only defined for band n258 to met the +1 dBm/200MHz requirements on 23.6GHz-24GHz. Actually, +1 dBm/200MHz requirement on 23.6GHz-24GHz is also applied to band n257, but there are no A-MPR requirements defined for n257, instead the default NS_200 would be used. It is because the MPR requirement can absorb the emission impact for band n257 due to large frequency separation for the lower edge of n257 to 24GHz. See the Fig 1.

Fig 1. A-MPR for band n257&n258 to pretect the EESS 23.6~24GHz

For the updated limits -5dBm/200MHz (or -35 dBW/200 MHz) for EESS 23.6~24GHz range, actually it was extensively discussed in the previous meeting. For the A-MPR and NS values, we think the agreements in the approved WF [5] can be reused, and also the A-MPR value in the WF were captured in the TR38.871-01 [6], we copy and paste them in the table 2 below:

Table 1. NS value for band n257 and n258 meet the regulatory requirements (extracted from Table 7.5.4.1-2[6])

| Band | Requirement | A-MPR for PC1 | A-MPR for PC2 | A-MPR for PC3 | A-MPR for PC4 |
| --- | --- | --- | --- | --- | --- |
| n258 | (Case A) Protected range: 23.6 - 24.0 GHz Limit: +1 dBm/200 MHz | 3.0 if Offset frequency < BWchannel, 0.0 otherwise | 0 | 0 | 0 |
| n257 |  | 0 (absorb as general requirement) | 0 (absorb as general requirement) | 0 (absorb as general requirement) | 0 (absorb as general requirement) |
| n258 | (Case B) Protected range: 23.6 - 24.0 GHz Limit: -5 dBm/200 MHz | 7.0 if Offset frequency < BWchannel, 6.0 otherwise | 1.0 if Offset frequency < BWchannel, 0.0 otherwise | 1.0 if Offset frequency < BWchannel, 0.0 otherwise | 1.0 if Offset frequency < BWchannel, 0.0 otherwise |
| n257 |  |  |  |  |  |

For the sake of handling the above A-MPR requirements for different limits, new NS flags, i.e. NS_205 for single CC, and CA_NS_205 for UL CA, corresponding to an 6dB lower requirement, i.e. -5 dBm/200 MHz (or -35 dBW/200 MHz).

It is proposed to adopt the above A-MPR requirements for the Protected range: 23.6 - 24.0 GHz with -5 dBm/200 MHz  (or -35 dBW/200 MHz) emission limit.

Proposal 2. To met the -5 dBm/200 MHz (or -35 dBW/200 MHz) emission limit in 23.6 - 24.0 GHz, it is proposed to introduce NS_205 (for single CC) with the A-MPR requirements defined in TS38.817-01, which are:

- PC1: 7.0 if Offset frequency < BWchannel, 6.0 otherwise

- PC2~7: 1.0 if Offset frequency < BWchannel, 0.0 otherwise

- The Offset frequency is defined as the frequency from 24.25 GHz to the lower edge of the channel bandwidth.

With the above two proposals, the A-MPR and corresponding NS flags for n257 and n258 to met -5 dBm/200 MHz (or -35 dBW/200 MHz) emission limit in 23.6 - 24.0 GHz are shown in Table 3:

Table 3: NS values for n257 and n258 single CC

| NS value | n257/n258 bands | A-MPR | Regulatory requirements |
| --- | --- | --- | --- |
| NS_200 | n257, n258 |  | Protected range: spurious Band applicability: all Limit: -13 dBm/MHz |
| NS_202 | n257, n258 | PC1: A-MPR=11.0dB PC2/3/4/5/6/7: A-MPR=1.0 dB | (For EU) Protected range: 7.25 GHz ≤ f ≤ 2nd harmonic Band applicability: all band Limit: -10 dBm/100 MHz Protected range: 23.6 - 24.0 GHz Band applicability: n258, n257 Limit: +1 dBm/200 MHz |
| NS_203 | n258 | PC1: 3.0 dB if Offset frequency < BWchannel, 0.0 dB otherwise PC2/3/4/5/6/7: A-MPR=0dB | Protected range: 23.6 - 24.0 GHz Band applicability: n258, n257 Limit: +1 dBm/200 MHz |
| NS_204 | n257, n258 | Same A-MPR as NS_202. | (For EU) Protected range: 7.25 GHz ≤ f ≤ 2nd harmonic Band applicability: all band Limit: -10 dBm/100 MHz Protected range: 23.6 - 24.0 GHz Band applicability: n258, n257 Limit: -5dBm/200 MHz |
| NS_205 | n257, n258 | PC1: 7.0 if Offset frequency < BWchannel, 6.0 otherwise PC2~PC7: 1.0 if Offset frequency < BWchannel, 0.0 otherwise | Protected range: 23.6 - 24.0 GHz Band applicability: n258, n257 Limit: -5dBm/200 MHz |

## 2.2 changeover date for UE

Due to the different 23.6-24 GHz EESS limits in different date, and it seems the original changeover date for the requirement applying UE brought into use are:

- +1 dBm/200 MHz before 1st September 2027.

- -5 dBm/200 MHz (or -35 dBW/200 MHz) after 1st September 2027.

However, in terms of the WID, the purpose is to implement the EESS protection limits (-5dBm/200 MHz @23.6-24GHz) at an earlier stage to support the commercial deployment in terms of the justification. Since EESS protection is a globe issue and the original target data for the stringent limits is from the WRC-19 resolution, i.e. After 1 September 2027. However, as mentioned in the WID

- The -5dBm/200MHz on 23.6GHz-24GHz UE spurious emission for commercial deployment needs to be satisfied in China.

- EU Decision 2020-590 has decided to implement −35 dBW/200 MHz (i.e. equivalent to -5dBm/200MHz) on 23.6GHz-24GHz from Jan. 1, 2024.

It can be seen that EU decision decide to implement the stringent limits earlier than 1 September 2027. Although the final decision haven’t been as of today in China, it is also needed to implement the stringent limits earlier to support the commercial deployment. Therefore, the different changeover date should be applied for different regions to void delay implementation of the new emission limit. Also the new NS values are mandatory for UE brought into use at least after the changeover date.

Proposal 3. For the requirement applying UE brought into use for -5 dBm/200 MHz (or -35 dBW/200 MHz) limits, applying different changeover dates for different countries/regions.

Proposal 4. The new NS values are mandatory for UE brought into use at least after the changeover dates.

## 2.3 ModifiedMPRbehavior bits

In terms of the TR38.817-01[6], adopting the modifiedMPR bits mainly to solve the issue that BS cannot know which NS values each of UEs can support in the case of two types of UE existing in a real environment: one is UE working with an existing NS and the other is UE working with a new NS.

In the current TS38.101-2, the modifiedMPRbehavior indication for band n258 NS_203 and CA_NS_203 have already been defined. We think the same approach shall be adopted for the new introduced NS flags and CA_NS flags, which is also mentioned in TR38.817-01:  ‘Note that NS_20X and NS_20Y described in Table 7.5.4.1-3 have not yet been specified in TS 38.101-2 v15.14.0.The same approach with introduction of NS_203 should apply to these NSs according to approved WF [R4-2009141], i.e., Alt 1-2(Explicit signaling for a UE to report newly supported NS value(s) for a legacy band to the network (reuse modifiedMPR bits)) ...’

Proposal 5: The same approach with introduction of NS_203, i.e. modifiedMPRbehavior indication, should apply to the NS_204, NS_205 and CA_NS_204, CA_NS_205 for band n257 and n258, respectively.

## 2.4 Spurious emission band UE co-existence

Due to this WID include both n257 and n258, so the protection for the frequency range 23.6GHz to 24 GHz should be introduced in the spurious emission band UE co-existence table to include the -5dBm/200MHz limits. So it is proposed to introduce EESS protection of -5dBm/200MHz for n257 and n258 into general spurious emission band UE co-existence requirements. Otherwise, UE may violate regulatory requirements in some regions who need regulation requires EESS protection in 23.6GHz-24GHz.

Proposal 6: It is proposed to introduce 23.6GHz-24GHz protection of -5dBm/200MHz for n257 and n258 into general spurious emission band UE co-existence requirements.

# 3 Conclusion

In this contribution, we give some discussions on the NS flags and the corresponding A-MPR requirements for the new limits -35dBW/200MHz or -5dBm/200MHz in EESS frequency range of 23.6GHz-24GHz based on the the RAN4 previous discussion and agreements, also referring to the TR38.817-01.

The observations, proposals and conclusions are:

Observation 1: The existing NS_202/CA_NS_202 A-MPR requirements can met the -5dBm/200MHz (or -35 dBW/200 MHz) requirements in 23.6 - 24.0 GHz ranges.

Proposal 1. It is proposed to introduce NS_204 (for single CC) and CA_NS_204 (for CA) for the emissions limit of -5dBm/200MHz (or -35 dBW/200 MHz) in the frequency range 23.6 - 24.0 GHz and -10 dBm/100 MHz in the frequency range 7.25 GHz ≤ f ≤ 2nd harmonic (for EU), where the A-MPR defined in NS_202 (for single CC) and CA_NS_202 (for CA) can be applied, respectively.

Proposal 2. To met the -5 dBm/200 MHz (or -35 dBW/200 MHz) emission limit in 23.6 - 24.0 GHz, it is proposed to introduce NS_205 (for single CC) with the A-MPR requirements defined in TS38.817-01, which are:

- PC1: 7.0 if Offset frequency < BWchannel, 6.0 otherwise

- PC2~7: 1.0 if Offset frequency < BWchannel, 0.0 otherwise

- The Offset frequency is defined as the frequency from 24.25 GHz to the lower edge of the channel bandwidth.

Proposal 3. For the requirement applying UE brought into use for -5 dBm/200 MHz (or -35 dBW/200 MHz) limits, applying different changeover dates for different countries/regions.

Proposal 4. The new NS values are mandatory for UE brought into use at least after the changeover dates.

Proposal 5: The same approach with introduction of NS_203, i.e. modifiedMPRbehavior indication, should apply to the NS_204, NS_205 and CA_NS_204, CA_NS_205 for band n257 and n258, respectively.

Proposal 6: It is proposed to introduce 23.6GHz-24GHz protection of -5dBm/200MHz for n257 and n258 into general spurious emission band UE co-existence requirements.

# 4	Reference

[1] RP-242371, New WID on mmWave UE spurious emission, China Unicom, and etc.

[2] R4-1910261, dCR to 38.101-2: FR2 A-MPR updates, including ERC 74-01 changes, Qualcomm Incorporated

[3] R4-2006785, CR to 38.101-2: NS_202 update after changes to EU regulations, Qualcomm Incorporated

[5] R4-2009141, WF on WRC-19 outcome and impact on RAN4 specifications, NTT DOCOMO

[6] TR38.871-01