# 3GPP TSG-RAN WG4 Meeting # 112-bis	R4-2415909

# Hefei, China, Oct.14th – 18th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	On EESS protection for BS

# Agenda item:	5.21.3

# Document for:	Approval

# 1 Introduction

In last RAN #105 meeting, a new work item for mmWave UE spurious emission was approved in [1], the objectives  are captured below:

The objective of this WI includes the following aspects: - For UE, specify additional spurious emission requirements of -5dBm/200Mz for 23.6-24 GHz frequency range for n257 and n258, where - Introduce new corresponding Network Signalling(s) and associated A-MPR(s) if needed by taking into the consideration of the relationship with the existing NS_200, NS_202, NS_203, CA_NS_200, CA_NS_202 and CA_NS_203 - All the power classes are considered in the above work - For BS, conduct necessary modifications on protection of Earth Exploration Satellite Service requirements if any

There is an objective related to BS, which is conduct necessary modifications. In this contribution, we give some discussions on EESS(Earth Exploration Satellite Service) protection from BS perspective.

# 2	Discussion

In the past RAN4 meeting, there were several discussions (such as [2])on the protection EESS for BS study. It seems there are two following aspects for the EESS protections:

- EESS protection at 23.6 – 24 GHz

- EESS protection at 36 – 37 GHz

For the lower EESS and upper EESS frequency range, the limits are different, and also the applied band are also different in terms of the information TR38.817-01, which are shown in table 1.

Table 1. Limits and Bands for lower/upper EESS protection range.

| EESS protection range | bands | limits |
| --- | --- | --- |
| - 23.6 – 24 GHz | n257, n258 | -33 dBW/200 MHz or -3dBm/200MHz -39 dBW/200 MHz or -9dBm/200MHz |
| - 36 – 37 GHz | n259, n260 | −43 dBW/MHz or −13 dBm/MHz −23 dBW/GHz or +7 dBm/1000 MHz |

However, after several meeting cycles discussion, only the limits for EESS protection at 23.6- 24 GHz were introduced in the BS specifications. There are no consensus for EESS protection at 36 - 37 GHz due to the uncertainty in the regional regulation which could lead to conflicting requirements. Consequently, EESS protection at 36-37 GHz for band n259 and n260 was postponed.

Observation 1. For BS study, there were no consensus for EESS protection at 36 - 37 GHz in previous meetings.

For the objectives in this WID, it is unclear which EESS protections ranges are included to be discussed for BS, although the first bullet is for UE to specify limits to protect 23.6-24 GHz frequency range. In our understanding, the intention for this WID study is focus on only the 23.6-24 GHz frequency range for both UE and BS, which means the study on 36-37 GHz for BS is not in the scope of this WID. We think it should be clarified first.

Proposal 1: To clarify the scope for BS study is only for 23.6-24 GHz frequency range in this WID.

For EESS 23.6-24 GHz protection in TS38.104/38.141-1/38.141-2 specifications, additional OTA OBUE and Tx/Rx spurious emissions requirements were introduced, in which the limits and the changeover date are aligned with the above summary. For example, for the OBUE limits in TS38.104:

Table 9.7.4.3.4.1-1: OBUE limits for protection of Earth Exploration Satellite Service [TS38.104]

| Frequency range | Limit | Measurement Bandwidth |
| --- | --- | --- |
| 23.6 – 24 GHz | -3 dBm (Note 1) | 200 MHz |
| 23.6 – 24 GHz | -9 dBm (Note 2) | 200 MHz |
| NOTE 1: This limit applies to BS brought into use on or before 1 September 2027. NOTE 2: This limit applies to BS brought into use after 1 September 2027. |  |  |

Except that, there are no other BS RF requirements impacted.

Observation 2. Except additional OTA OBUE and Tx/Rx spurious emissions requirements, there are no other BS RF requirements impacted.

It can be seen from the table, there are two NOTEs to explain the date for the application of the two limits:

- +3 dBm/200 MHz before 1st September 2027.

- -9 dBm/200 MHz after 1st September 2027.

Since EESS protection is a globe issue and the original target data for the stringent limits is from the WRC-19 resolution, i.e. After 1 September 2027. However, as mentioned in the WID:

- The -5dBm/200MHz on 23.6GHz-24GHz UE spurious emission for commercial deployment needs to be satisfied in China.

- EU Decision 2020-590 has decided to implement −35 dBW/200 MHz (i.e. equivalent to -5dBm/200MHz) on 23.6GHz-24GHz from Jan. 1, 2024.

Although the above information is for UE side, and there is no explicit information for the BS EESS protection, we think the purpose is same as UE which is to implement the EESS protection limits (-9dBm/200 MHz @23.6-24GHz) for BS at an earlier stage to support the commercial deployment in terms of the justification.

It can be seen that EU decision decide to implement the stringent limits earlier than 1 September 2027. Although the final decision haven’t been as of today in China, it is also needed to implement the stringent limits earlier to support the commercial deployment. Therefore, the different changeover date should be applied for different region to void delay implementation of the new emission limit.

Proposal 2. For the requirement applying BS brought into use for -9 dBm/200 MHz limits, applying different changeover dates for different countries/regions.

# 3 Conclusion

In this contribution, we give some discussions on EESS(Earth Exploration Satellite Service) protection from BS perspective.

The observations, proposals and conclusions are:

Observation 1. For BS study, there were no consensus for EESS protection at 36 - 37 GHz in previous meetings.

Proposal 1: To clarify the scope for BS study is only for 23.6-24 GHz frequency range in this WID.

Observation 2. Except additional OTA OBUE and Tx/Rx spurious emissions requirements, there are no other BS RF requirements impacted.

Proposal 2. For the requirement applying BS brought into use for -9 dBm/200 MHz limits, applying different changeover dates for different countries/regions.

# 4	Reference

[1] RP-242371, New WID on mmWave UE spurious emission, China Unicom, and etc.

[2] R4-2008874, Email discussion summary for [95e][302] NR_maintenance_RF_BS, Moderator (Ericsson)

[3] TR38.871-01