# 33GPP TSG-RAN WG4 Meeting # 112-bis	R4-2415910

# Hefei, China, Oct.14th – 18th, 2024

# Title: 	View on methods for reducing the number of UE Rx chains

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.6.2

# Document for:	Approval

# 1 Introduction

In last meeting, some initial discussions on the fragment carrier were carried out [1] but only few of the issues were issued due to the lack of time. There are many open issues are captured in the WF[2], and meanwhile some agreements are also included.

In this contribution, we give some discussions on methods for reducing the number of UE Rx chains for fragment carriers.

# 2	Discussion

For the methods for reducing the number of UE Rx chains, there is one highlight agreements captured in the WF [2], which is:

Topic #2: Methods for reducing the number of UE Rx chains #### Issue 2-1-1: Applicability and clarification on the scope Online agreements: Agreement: - The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain - The two non-contiguous CCs is still under the current DL non-contiguous CA framework - Each fragment is treated as an individual CC - The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz. - [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands - Consider n2/n25, n3, n7, n66, n41, n39 as the example bands. - Provide the details related to gap between fragmented DL blocks]

In terms of the above agreements, it can be clearly saw that the goal of this SID is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain. Here, it uses ‘single Rx RF chain’, rather than ‘single Rx RF chain’, our understanding is that it does not include the baseband according to the term of ‘single Rx RF chain’.

- Observation: Using a single Rx RF chain method to support DL non-contiguous 2CC is the study goal

The assumption for the baseband assumptions is pended on the following issues.

Issue 2-1-2: Reference architecture Recommended WF: - The existing requirements for DL NCCA was specified based on partially-shared Rx architecture according to TR36.823. - FFS on following options for reducing number of Rx chains for DL non-contiguous CCs: - Option 1: Only the “fully shared Rx chain architecture” needs to be studied - Option 2: RAN4 also studies the possibility of supporting only 2Rx in the mandatory 4Rx band below 2700MHz - Other options are not precluded - Companies are encouraged to provide baseband assumptions i.e., shared/separated BB filter that may relate to rejection ability toward unwanted signal for the study - FFS on whether a single FFT are expected to cover the fragmented carriers or separate FFT are expected for each fragmented carrier.

For the option 2 above, it looks a bit wired since it seems not related to the SID objectives, and to our understanding, for a band mandatory support 4Rx, if it is allowed to support only 2Rx to save the Rx chain, the benefit and performance loss may need to be further discussion although there is a precedent for the non-collocated intra-band non-contiguous CA scenarios where each CC in type 2 UE is 2Rx. In this SID, one of the objectives mention ‘Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs’, which means it should be collocation scenario.

Considering the above, we propose:

Proposal 1: Only the “fully shared RF Rx chain architecture” needs to be studied.

For the basedband assumption, here we think it should divide into analogy baseband (ABB) and digital baseband(DBB).

For ABB assumption, it belongs to RF chain and since a common Rx RF chain method is used, so a single analogy baseband with wide filter to accommodate the two non-contiguous carrier/fragments within 100MHz separation at the same time.

For the DBB assumption, at least we think separate DBB is allowed for the in-gap interference/image rejection with different digital filter. At the same time, for the common DBB, it seems it is difficult to reject the in-gap interference/image using a common digital filter which may cause bad DL performance, depends on the in-gap interference. In our understanding, how much the in-gap interference between different operators seems it is highly implementation dependence. Maybe the in-gap interference level from operator A to operator B can keep in a certain low level which will not cause additional degradation. In this situation, a single DBB may still be feasible. Otherwise, separate DBB should be used for good blocker rejections.

For whether to use a single FFT size, it depends on the SCS for each CC. The use of a common FFT to handle different SCS for each carrier is generally not straightforward in CA operation, and there are several challenges such as it is difficult and impractical to accommodate all different rates due to the OFDM length/CP length in time domain, sampling rates per symbol are different for different SCS. But of course, a single FFT size is feasible when the SCS for different carriers/fragments are aligned.

Considering the above, we propose:

Proposal 2: A single analogy baseband is used to receive two non-contiguous CCs.

Proposal 3: either single digital baseband or single digital baseband is feasible, depending on how much the in-gap interference between different operators.

- A single FFT size is feasible when the SCS for different fragments are aligned. Otherwise, separate FFT are expected .

# 3 Conclusion

In this contribution, we give some initial discussion on fragment carriers by reducing the Rx chain. The conclusions are summarized in the following:

Proposal 1: Only the “fully shared RF Rx chain architecture” needs to be studied.

Proposal 2: A single analogy baseband is used to receive two non-contiguous CCs.

Proposal 3: either single digital baseband or single digital baseband is feasible, depending on how much the in-gap interference between different operators.

- A single FFT size is feasible when the SCS for different fragments are aligned. Otherwise, separate FFT are expected .

# 4	Reference

[1]  R4-2412827, Topic summary for [112][125] FS_NR_DL_Frag_Carrier, Mediatek

[2]  R4-2414457, WF on fragmented DL, Mediatek