# 33GPP TSG-RAN WG4 Meeting # 112-bis	R4-2415911

# Hefei, China, Oct.14th – 18th, 2024

# Title: 	View on Fragmented carrier general part and RF requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.6.3

# Document for:	Approval

# 1 Introduction

In last meeting, some initial discussions on the fragment carrier were carried out [1] but only few of the issues were issued due to the lack of time. There are many open issues are captured in the WF[2], and meanwhile some agreements are also included.

In this contribution, we give some discussions on the methods for reducing the number of UE Rx chains for fragment carriers.

# 2	Discussion

## 2.1 Clarification on the scope

For the methods for reducing the number of UE Rx chains, there is one highlight agreements captured in the WF [2], which is:

Topic #2: Methods for reducing the number of UE Rx chains #### Issue 2-1-1: Applicability and clarification on the scope Online agreements: Agreement: - The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain - The two non-contiguous CCs is still under the current DL non-contiguous CA framework - Each fragment is treated as an individual CC - The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz. - [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands - Consider n2/n25, n3, n7, n66, n41, n39 as the example bands. - Provide the details related to gap between fragmented DL blocks]

In terms of the above agreements, some high level clarifications for the SID are included, such as:

1. one CC in each fragment for intra-band DL NC CA (current goal)

2. More than one CC in one fragment or higher order inter/intra-band combinations (long term goal)

3. frequency span≤ 100 is for fragments frequency range, rather than band frequency range

4. Scope for the band/example bands

For the last one, square bracket is added due to companies share different views on how to select the example bands. In our understanding, at least the scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands considering the long term goal for this SID.

For the example, in terms of the last discussion, there are no criteria to decide the example bands, instead some companies verbally propose some example bands. In our understanding, the selections for the example bands mainly to study the impact RF requirements (including ΔRIB,NC). In current spec, for the Rx RF requirements for the intra-band NC CA:

1. ΔRIB,NC requirements are only defined for FDD intra-band NC CA

2. The other Rx requirements are defined for all the bands (≤2700MHz, ≥ 3300MHz)

For FDD band intra-band non-contiguous CA, the UL Tx noise caused by Tx chain/PA leakage and Tx spectrum spreading into DL Rx path shall be taken into account since the duplexer gap for some FDD bands are small, and such small duplexer gap cannot provide enough attenuation for the filters design. The UL Tx noise falls into DL Rx associated with its own Rx IIP2 will cause the REFSENS degradation.

Sharing Rx RF chain means the shared filter cannot filter out each carrier anymore. the interference appeared in the gap is located in the pass band of the filter, and the filter cannot provide the attenuation in the gap between the two DL carriers. Thus the ΔRIBNC requirements for FDD intra-band NC CA would become larger [3] which are needed to be re-evaluated.

In the TS38.101-1, there are several intra-band DL CA configurations with PC2/3 1/2UL have the ΔRIBNC requirement defined, which is summarized in Table 1.

Meanwhile, we think the existing test points are reasonable to be reused

Table 1. ΔRIBNC requirements for the Intra-band non-contiguous CA with (2A) configuration

| CA configuration (ΔRIBNC is defined) | Tables in sub-clause 7.3A.2.2 ( TS38.101-1 v18.6.0) |
| --- | --- |
| CA_n1(2A), CA_n2(2A), CA_n3(2A), CA_n5(2A), CA_n7(2A), CA_n12(2A), CA_n25(2A), CA_n26(2A), CA_n66(2A), CA_n71(2A) | Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration (PC3) |
| CA_n25(2A), CA_n66(2A), CA_n71(2A) | Table 7.3A.2.2-1a: Power class 2 intra-band non-contiguous CA reference sensitivity with one uplink carrier. |
| CA_n26(2A) | Table 7.3A.2.2-2: Intra-band non-contiguous CA with two uplink carriers configuration for reference sensitivity in FDD bands. (PC3) |

Unlike FDD, there is no duplexer gap for TDD band, so there are no ΔRIBNC requirements defined for TDD intra-band non-contiguous CA.

For the ΔRIBNC requirements study purpose, it is impossible to do the re-evaluations for all the defined intra-band non-contiguous CA scenario in the SID time phase. That’s the main reason that companies propose to select some example bands. We think the selected example bands should be in the above scope in the table 1, and both PC3 and PC2 should be considered. At this stage, it seems only consider one UL carrier is enough to reduce the workload. Meanwhile, it is reasonable to reuse the existing test points for the ΔRIBNC requirements re-evaluation to compare with the traditional way.

Considering the above, we propose:

Proposal 1: The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands.

Proposal 2: For ΔRIBNC requirements re-evaluation:

- The example bands should be in the scope of existing intra-band non-contiguous CA operating bands

- The existing test points are reused;

- Priority to one uplink carrier

- both PC3 and PC2 should be considered

  - 2.2 PSD difference between carriers of co-located adjacent channel operators

The discussion can be simply illustrated in the Fig 1.

Fig 1. Simply illustration for the PSD different between carriers

In Fig 1., there are three PSD differences, where X and Y are the received PSD difference between carriers of co-located adjacent channel operators in inter-operator deployment scenario, while Z is the received PSD difference between the two non-contiguous CCs of the same operators, which is up to 6dB according to the objectives.

The discussion point is the X and Y value, or only Y value since Y is the larger PSD difference between carriers of co-located adjacent channel operators.

Note that the terminology of ‘PSD imbalance’ is used in the objectives, rather than power difference. The power difference between the interference carrier and wanted carrier are related to the channel bandwidths, which equals to PSD difference + 10log(BWinterference/BWwanted). When BWinterference=BWwanted, then PSD difference value equal to power imbalance value. But when the BWinterference < BWwanted, then the power between the interference carrier and wanted carrier would become smaller if keep the power difference unchanged or the PSD between the interference carrier and wanted carrier would become larger if keep the power imbalance unchanged.

Observation 1. The power difference, PSD difference and the carrier channel bandwidths are dependent each other.

Regarding the value for the receive PSD different or power different between fragments, we think it might be difficult to decide how much the PSD difference or power difference between carriers of co-located adjacent channel operators even the for the same BS classes for the inter-operators since it would be highly implementation dependence, such as site space constraints, antenna type/directions, supported channel bandwidths and etc.

For co-located adjacent channel operators, our understanding is that the co-located gNBs in inter-operator deployment scenario means sharing the same site/tower for the co-location BS from the different operators, where MCL is assumed as 30dB between transmitter and receiver for the same BS class according to TS38.104. Moreover, the co-located gNBs could be operated in different bands.

Observation 2. It might be difficult to decide how much the PSD difference between carriers of co-located adjacent channel operators due to implementation consideration.

## 2.3 RF requirements

In the WF, it shows:

Issue 3-1-2: Prerequisite conditions FFS on following proposals in next meeting: - Proposal 1: Maintain the current ACS and IBB requirements to ensure UE selectivity - Proposal 2: The fourth sub-bullet in the SID should be simplified to “Determine a reasonable upper level of the power spectral density difference of the in-gap signal compared to the two non-contiguous CCs” - Proposal 3: RAN4 to confirm it is feasible to receive two non-contiguous CCs in a shared RF chains with the assumption that power spectral density imbalance between any of the CC and also the signal in the gap are within 6dB - FFS on assuming higher PSD for the signal in the gap of 2CC #### Issue 3-1-3: Requirements for evaluation Recommended WF: Companies’ views are diverged but not controversial. Further discuss following bullets see if they can be agreeable as a package - Both TDD and FDD intra-band DL contiguous CA should be included - ΔRIBNC, ACS, IBB, NBB requirements [on both PCC and SCC] as well as the minimum guard band sizes for single Rx RF chain, all need to be evaluated - Out-of-band blocking and spurious response do not need requirement adjustment - Adjusted requirements for fragmented carriers shall be separated from the existing FDD/TDD intra-band carrier non-contiguous aggregation ΔRIBNC, ACS, IBB, NBB - Discussion on if a rejection level of the image caused by the in-gap interferer is needed - Discussion on new requirement(s) if identified is not precluded

As we discussed in [3], by sharing the Rx RF chain, there is no impact on maximum input level requirements, and the mainly impacts are the requirements between the two DL carrier, a.k.a in-gap requirements.

For ACS value, it is related to co-existence study, so we think it should be caution to modify the ACS dB level when sharing the Rx RF chain to void the increasing interference potentially degrades overall network performance and user experience.

In current spec, the UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all downlink carriers are active, where the -25dBm is the same as the maximum input level requirements. For a UE supporting fragments carrier within 100MHz by sharing Rx RF chain should also met this kind of test parameter since we think the maximum interferer power condition (equals to maximum input level for the upper Rx dynamic range) would not be changed regardless separated or shared Rx RF chain is used.

Proposal 3. The existing ACS level should be maintained when reducing the Rx chain number.

Proposal 4: To adopt up to -25dBm interferer power for fragments ACS requirements.

To achieve the above goal, as mentioned in [3], the focus should be on innovative receiver designs, leaving the implementation up to UE vendor.

Moreover, the in-gap tests is applied only if the Wgap satisfy the equation conditions as specified in the spec, which is:

In our understanding, the existing conditions of the Wgap shall be also applied for the shared RF chain, which is if the interferer frequency position does not meet the Wgap condition, then no test is foreseen.

Proposal 5: For the in-band gap test, the existing conditions of the Wgap shall be also applied for the shared RF chain.

# 3 Conclusion

In this contribution, we give some discussions on the methods for reducing the number of UE Rx chains for fragment carriers. The conclusions are summarized in the following:

Observation 1. The power difference, PSD difference and the carrier channel bandwidths are dependent each other.

Observation 2. It might be difficult to decide how much the PSD difference or power difference between carriers of co-located adjacent channel operators due to implementation consideration

Proposal 1: The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands.

Proposal 2: For ΔRIBNC requirements re-evaluation:

- The example bands should be in the scope of existing intra-band non-contiguous CA operating bands

- The existing test points are reused;

- Priority to one uplink carrier

- both PC3 and PC2 should be considered

Proposal 3. The existing ACS level should be maintained when reducing the Rx chain number.

Proposal 4: To adopt up to -25dBm interferer power for fragments ACS requirements.

Proposal 5: For the in-band gap test, the existing conditions of the Wgap shall be also applied for the shared RF chain.

# 4	Reference

[1]  R4-2412827, Topic summary for [112][125] FS_NR_DL_Frag_Carrier, Mediatek

[2]  R4-2414457, WF on fragmented DL, Mediatek

[3]  R4-2411886, View on Fragmented carrier, ZTE Corporation, Sanechips

[4]  R4-2413271, Discusson on impact on UE RF requirement of fragmented carriers,Ericsson