# 3GPP TSG-RAN WG4 Meeting #112bis                               R4-2415914

# Hefei, Anhui, China, 14th – 18th October, 2024

# Title: 	Discussion on UE Capability/UE behavior and network signaling for Type 4a Type 4b EN-DC/NR-CA

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.5.2.2

# Document for:	Approval

# 1 Introduction

This contribution shares our views about UE Capability/UE behavior and network signaling type 4a and type 4b.

- Discussion

Some agreements are reached in RAN4 #112 meeting [2] in which there are some issues need further discussion, one issue is the differentiation of type 4a and 4b shown as following.

- < Issue 2-3-1: New UE Capability for Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC/NR-CA > Way forward: - Define one capability for NR CA and one UE capability for EN-DC - FFS on how to differentiate type 4a and 4b.

Table 2-1 summarize different UE types with type 3 included.

Table 2-1 UE types summary[3]

With regard to new UE Capability for Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC/NR-CA, the scheme of type 2 capability can be a reference and the description about these signaling are appended in the Annex for reference.

Table 2-2 summary of signaling for UE type 2

| UE type and transmission scenario | UE capability or NW signaling |
| --- | --- |
| UE type 2 ENDC | interBandMRDC-WithOverlapDL-Bands-r16 (UE capability) requirementTypeIndication-r18 (UE capability) nonCollocatedTypeMRDC-r18 (Network signaling) |
| UE type 2 NRCA | intraBandNR-CA-non-collocated-r18(UE capability) nonCollocatedTypeNR-CA-r18(Network signaling) |

Comparing with UE type 2, type 4 is divided into type 4a which is capable of total 6Rx where 2Rx (2 layers) in one carrier and 4Rx (4 layers) in the other carrier and type 4b which is capable of total 8Rx where 4Rx (4 layers) in each carrier. Regarding to the relationship between the UE types and ENDC/NR CA configurations, we can only find it in the following issues which was proposed by Moderator in #110-bis meeting:

< Issue 2-3-1: New UE Capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) > Way Forward: - Continue further discussion on the following two options in the next meeting. - Option 1 - Add new UE capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA) support indication. - Option 2 - Reuse intraBandNR-CA-non-collocated-r18 and maxNumberMIMO-LayersPDSCH=4 for FR1 NR intra-band CA and interBandMRDC-WithOverlapDL-Bands-r16 and maxNumberMIMO-LayersPDSCH=4 for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands.

However, there were no official agreements about the above relationships, and this relationships was argued by few of companies. Nevertheless, it seems the majority views are that the type 4a is only for ENDC since 4Rx is optional for LTE band 42, while type 4b can be either ENDC or NR CA since 4Rx is mandatory for NR band n77.

Observation 1. There are no official agreements on the relationship between the UE types and ENDC/NR CA configurations, the majority views are that the type 4a is only for ENDC, while type 4b can be either ENDC or NR CA.

Following the majority views in the above observation, we think when the NW receives a UE capability of type 4b EN-DC, the NW would assume that the UE has the capability of type 4a ENDC as well. On the other hand, if NW receives a UE capability of NRCA, the NW assumes that the UE has the capability of type 4b.

Observation 2. NW determines UE type based on UE capabilities reported by UE.

- If capability of NR CA(4 NR layers + 4 NR layers) is reported by a UE, NW assumes type 4b is supported by the UE.

- If capability of ENDC(2 LTE layers + 4 NR layers) is reported by a UE, NW assumes type 4a is supported by the UE.

- If capability of ENDC(4 LTE layers + 4 NR layers) is reported by a UE, NW assumes type 4a and type 4b are supported by the UE.

- For type 4a ENDC and type 4b ENDC, since the 4Rx is optional supported for LTE band 42, and the verification for 4Rx LTE requirement are based on 2Rx, which means when 4Rx is optionally supported via capability, 2Rx shall be supported in advanced and it is baseline, also the number of supported layers/CC can be reported using legacy signallings. Therefore, we think there is no need to define additional capability to differentiate type 4a and 4b.

- Proposal 1. There is no need to define additional UE capability to differentiate type 4a and 4b for ENDC.

Issue 2-3-4 and Issue 2-3-5 identified last meeting are related to UE transmission switching between UE type 4, type 2and UE type 1.

- < Issue 2-3-4: New BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated)> Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting. - < Issue 2-3-5: New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated) > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting. - Issue 2-3-7: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b - Option 1: (KDDI) Confirm the following understanding on maxMIMO-layers and Rel-18/19 BS signaling: - The case of maxMIMO-layers larger than 4 is Type 1. - The case of maxMIMO-layers equal to 4 and - Rel-19 New BS signaling is present with value 0 is Type 4. - Rel-19 New BS signaling is present with value 1 is Type 1. - Rel-19 New BS signaling is absent is that UE follows Rel-18 specification. - The case of maxMIMO-layers equal to 2 and - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is present is Type 1. - Rel-18 nonCollocatedTypeNR-CA/nonCollocatedTypeMRDC is absent is Type 2. - Option 2: (Samsung) Confirm the following understanding for the usage of maxMIMO-layers and the new Rel-19 NW signaling and Rel-18 NW signaling: For the initial connection, if 1）maxMIMO-layers≥8, it is collocated 2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification. 3）maxMIMO-layers=2, it could be either collocated(Type-1) or non-collocated(Type-2), pending on whether existing NW signaling (nonCollocatedTypeNR-CA or nonCollocatedTypeMRDC) is present - - Note: “If Rel-19 BS signaling IE is absent” intends for legacy NW or Rel-19 NW without Type-4 feature, “If Rel-19 BS signaling IE is present” intends for Rel-19 NW with Type-4 feature.

Figure 2-1 shows the UE types and the corresponding fallback transmission.

Figure 2-1 The UE type and the corresponding fallback scheme

If a UE only support the capability of UE type 2, it can fallback to type 1 with total 4 layers transmission when NW signaling(nonCollocatedTypeMRDC-r18 or nonCollocatedTypeNR-CA-r18) is configured to the UE. For a UE supporting both UE type 2 and UE type 4, the fallback can be type 1 with 4 layers, type 1 with 6 layers or type 1 with 8 layers.

For Rel-18 NW, only type 2 can be identified and the fallback transmission is type 1 with 4 layers even type 4 is reported by a Rel-19 UE.

For Rel-19 NW, type 2 and type 4 can be identified if both UE types are reported by a UE. A network signaling can indicate a UE to fallback to type 1 transmission. If a ENDC UE report 6Rx, NW can indicate type 1 with 6 layers as the fallback transmission which is type 1 with 6 layers. If a NR CA UE report 8Rx, NW can indicate type 1 with 8 layers as the fallback transmission. In addition, we think R19 type 4 UE should also support the R18 type 1 UE with 4 layers per CC considering the maxMIMO layer is up to 8 for R19 type 4 UE.

When the maxMIMO-layers per carrier is larger than 4, then it is collocated scenarios which means only collocated (type 1) UE is considered, and when the maxMIMO-layers per carrier is less than or equal to 2, then the traditional R18 manner can be reused. In addition, when the maxMIMO-layers per carrier equals to 4 for R19 type 4 UE, we think type 1 UE with R18/R19 4 layers/CC, 6 layers, 8 layers/CC should be considered for the New BS Signaling.

Proposal 2. The new NW signalling should consider type 4 UE ‘fallback’ to R18/R19 Type 1 UE with 4 layers/CC, 6 layers/CC, 8 layers/CC.

# 3 Conclusion

In this contribution, we give our viewpoint about requirements for type 4a/4b for NonCol_intraB_ENDC_NR_CA.

The conclusions are:

Observation 1. There are no official agreements on the relationship between the UE types and ENDC/NR CA configurations, the majority views are that the type 4a is only for ENDC, while type 4b can be either ENDC or NR CA.

Observation 2. NW determines UE type based on UE capabilities reported by UE.

- If capability of NR CA(4 NR layers + 4 NR layers) is reported by a UE, NW assumes type 4b is supported by the UE.

- If capability of ENDC(2 LTE layers + 4 NR layers) is reported by a UE, NW assumes type 4a is supported by the UE.

- If capability of ENDC(4 LTE layers + 4 NR layers) is reported by a UE, NW assumes type 4a and type 4b are supported by the UE.

- Proposal 1. There is no need to define additional UE capability to differentiate type 4a and 4b for ENDC.

Proposal 2. The new NW signalling should consider type 4 UE ‘fallback’ to R18/R19 Type 1 UE with 4 layers/CC, 6 layers/CC, 8 layers/CC.

# 4	Reference

- RP-240838, Rel-19 WID_Intra-band non-collocated EN-DC and NR-CA, Apple

- R4-2414453, WF on Rel-19 non-collocated scenario, KDDI

- RP-240101, Motivation on supporting Intra-band non-collocated EN-DC/NR-CA deployment in Release 19, KDDI, etc.

- R4-2410686, WF on NonCol_intraB_ENDC_NR_CA, KDDI

- 3GPP TS 38.331 V18.2.0

# Annex

nonCollocatedTypeMRDC-r18 (NW signaling) This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements when indicating support of interBandMRDC-WithOverlapDL-Bands-r16.

nonCollocatedTypeNR-CA-r18 (NW signaling) This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NRCA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

interBandMRDC-WithOverlapDL-Bands-r16(UE capability) -- R4 2-19 FDD-FDD or TDD-TDD inter-band MR-DC with overlapping or partially overlapping DL spectrum Indicates whether the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [5] and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 [5] and inter-band RF requirements. If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially overlapping DL bands with (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [5] and intra-band RF requirements.

requirementTypeIndication-r18(UE capability) -- R4 33-2: Support network control of requirement network applicability for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 Indicates whether the UE supports network controlled indication of the MTTD/MRTD and RF requirements by nonCollocatedTypeMRDC-r18 for TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands as specified in TS 38.331 [9]. This field is only applicable to the UE indicating interBandMRDC-WithOverlapDL-Bands-r16.

intraBandNR-CA-non-collocated-r18 (UE capability) -- R4 33-1: Support of intra-band non-collocated NR CA operation Indicates whether the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [2], and TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 [5] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [2]. If the capability is not reported, the UE only supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 [5] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [2]. A UE supporting this feature shall also support network controlled indication of the MTTD/MRTD and RF requirements by nonCollocatedTypeNR-CA-r18 for intra-band non-collocated NR-CA, as defined in TS 38.331 [9].