3GPP TSG-RAN4 Meeting #112bis	R4-2415926

Hefei, Anhui, China, Oct 14 – 18, 2024

Agenda item:	4.2.2.2

Source: 	Qualcomm Incorporated

Title: 	On performance requirements for SL positioning

Document for:	Discussion

# Introduction

RAN4 discussed RRM performance requirements for SL positioning measurements in RAN4#112 [1]. In this brief contribution we make further proposals to complete SL positioning performance requirements.

# Measurement accuracy requirements

Simulation results for SL positioning measurements have been collected and updated for several meetings, with the latest summary provided in [2]. In RAN4#112 there was an agreement to define accuracy requirements based on averaged results contained in the summary. We believe that agreement should be revised and outliers should be excluded prior to averaging. That was the approach followed when defining Rel-16 NR positioning performance requirements [3].

Issue 1-2-4: Measurement accuracy for SL PRS-RSRP(P)/Rx-Tx

Agreement

- Accuracy requirements:

  - The numbers refer to the simulation summary (R4-2410196) where the accuracies have been derived.

Plain averaging is acceptable when there is good alignment between all the simulation results submitted by interested companies. However, in the results submitted for SL Rx-Tx and SL RSTD we observe large discrepancies. Specifically, some of the accuracy results for AWGN clearly stand out as outliers, showing performance that is orders of magnitude better than the average accuracy. In our view, such results, which may have been obtained by applying advanced processing, should be discarded when deriving minimum performance requirements in RAN4. Even though there are multiple such ‘low’ outliers in the summary, we can compromise to removing the lowest and highest outliers prior to averaging to define the requirements.

Proposal 1: Exclude the lowest and highest outliers prior to averaging simulation results to derive measurement accuracy requirements for SL Rx-Tx and SL RSTD.

# Test cases

For the issue below regarding colliding PRS allocations, our view is that persistent collisions should not be configured in the test cases. The simulation assumptions did not include scenarios with SL PRS collisions. Also, in Uu positioning none of the test cases have colliding PRS configurations.

### Issue 1-2-6: SL-PRS signal level configurations

Agreements:

- Check the RAN1 specification and the Uu TCs in relation to overlapping SL-PRS from different UEs. Further discuss the options:

  - Option 1: overlapping SL-PRS can be used in TCs.

  - Option 2: overlapping SL-PRS should not be used in SL TCs.

Proposal 2; Do not use overlapping/colliding SL PRS allocations in the SL positioning test cases.

The last issue concerns the choice of SL PRS resource pool type (shared vs. dedicated) in the test cases. Note that RAN1 defined a separate UE capability for each type of resource pool and there is no baseline capability. i.e. a UE could support either or both types. Therefore, ideally, each test case would be defined so that it can be run with either shared or dedicated resource pool and a UE would be tested in one configuration (by UE choice if the UE supports both). If the effort to the define the test cases with flexible resource pool type is too high, then RAN4 should split the test cases evenly between the two types.

Issue 1-2-8: Other test case configurations

Agreements:

- FFS: Shared and dedicated resource pools are randomly used among different TCs.

- FFS: (symbol num, comb size): (4, 4) and (2, 4).

Proposal 3: On the choice between dedicated vs. shared resource pool:

- If the effort is deemed acceptable, each test case would be defined so that it can be run with either shared or dedicated resource pool, and a UE would be tested in one configuration (by UE choice if the UE supports both).

- Otherwise, RAN4 should split the test cases evenly between the two types of resource pool.

# Conclusions

Proposal 1: Exclude the lowest and highest outliers prior to averaging simulation results to derive measurement accuracy requirements for SL Rx-Tx and SL RSTD.

Proposal 2; Do not use overlapping/colliding SL PRS allocations in the SL positioning test cases.

Proposal 3: On the choice between dedicated vs. shared resource pool:

- If the effort is deemed acceptable, each test case would be defined so that it can be run with either shared or dedicated resource pool, and a UE would be tested in one configuration (by UE choice if the UE supports both).

- Otherwise, RAN4 should split the test cases evenly between the two types of resource pool.

# References

[1] R4.2414033, WF on SL positioning and carrier phase positioning, RAN4#112

[2] R4-2410196, Simulation results summary for SL positioning accuracy requirements, RAN4#112

[3] R4-2113156, Summary of link level simulation result of RSTD, PRS RSRP and UE Rx-Tx time difference, RAN4#100-e