3GPP TSG-RAN4 Meeting #112bis	R4-2415927

Hefei, Anhui, China, Oct 14 – 18, 2024

Agenda item:	4.2.2.4

Source: 	Qualcomm Incorporated

Title: 	On performance requirements for RedCap positioning

Document for:	Discussion

# Introduction

RAN4 discussed RRM performance requirements for RedCap positioning measurements in RAN4#112 [1]. In this brief contribution we make further proposals to complete the performance requirements.

# Measurement accuracy requirements

Simulation results for RedCap positioning measurements have been collected and updated for several meetings, with the latest summary provided in [2]. So far RAN4 has not reached a formal agreement on how to define accuracy requirements based on the results contained in the summary. In draft CRs submitted in previous meetings, accuracy values were left in square brackets or TBD.

When there is good alignment between all the simulation results submitted by interested companies, it is reasonable to derive the accuracy requirements by averaging all the results. However, at least in the results submitted for UE Rx-Tx and RSTD we observe large discrepancies. Specifically, some of the accuracy results for AWGN clearly stand out as outliers, showing performance that is orders of magnitude better than the average accuracy. In our view, such results, which may have been obtained by applying advanced processing, should be discarded when deriving minimum performance requirements in RAN4. Note that in Rel-16 NR positioning, outliers were discarded when defining performance requirements [3].

Proposal 1: Do not average all simulation results to derive measurement accuracy requirements for RedCap positioning, unless there is good alignment between all of them. RAN4 to discuss how to exclude outliers prior to averaging simulation results to derive measurement accuracy requirements for RedCap positioning.

# Conclusions

Proposal 1: Do not average all simulation results to derive measurement accuracy requirements for RedCap positioning, unless there is good alignment between all of them. RAN4 to discuss how to exclude outliers prior to averaging simulation results to derive measurement accuracy requirements for RedCap positioning.

# References

[1] R4-2414032, WF on RedCap positioning and PRS/SRS bandwidth aggregation, RAN4#112

[2] R4-2412688, Summary of simulation results for RedCap positioning, Ericsson, RAN4#112

[3] R4-2113156, Summary of link level simulation result of RSTD, PRS RSRP and UE Rx-Tx time difference, RAN4#100-e