3GPP TSG-RAN4 Meeting #112bis	R4-2415928

Hefei, Anhui, China, Oct 14 – 18, 2024

Agenda item:	4.2.2.5

Source: 	Qualcomm Incorporated

Title: 	On performance requirements for PRS bandwidth aggregation

Document for:	Discussion

# Introduction

In this paper we propose measurement accuracy requirements for UE Rx-Tx time difference and RSTD measurements with PRS bandwidth aggregation. The proposed accuracy values are based on simulation results submitted by multiple companies [1] and do not include any margins (e.g. for delay calibration and frequency drift).

# UE Rx-Tx time difference measurement accuracy

The results shown in the following tables were obtained by averaging results summarized in [1] and rounding up to an integer number of Tc. In some cases, we excluded results that appear as outliers. E.g. for AGWGN, results submitted by CATT at the higher side conditions (0 dB, -3 dB) were mostly excluded, except for their results for 3 PFLs in FR2. Results from Ericsson for AWGN and TDL-C (FR2) were also excluded.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -6 dB |
| FR1 | 104 | 15 | 64 | 2 | 4 | 15 | 13 |
|  | 132 | 30 | 32 | 2 | 4 | 7 | 10 |
|  | 272 |  | 16 | 2 | 4 | 4 | 4 |
|  | 64 | 60 | 32 | 2 | 4 | 7 | 10 |
|  | 132 |  | 16 | 2 | 4 | 4 | 4 |
| FR2 | 64 | 60 | 32 | 2 | 4 | 7 | 10 |
|  | 132 |  | 16 | 2 | 4 | 4 | 4 |
|  | 64 | 120 | 16 | 2 | 4 | 4 | 4 |
|  | 128 |  | 8 | 2 | 4 | 2 | 3 |

Table 1: UE Rx-Tx measurement accuracy for 2 PFLs in AWGN with reduced number of samples.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -13 dB |
| FR1 | 104 | 15 | 64 | 4 | 4 | 16 | 14 |
|  | 132 | 30 | 32 | 4 | 4 | 8 | 9 |
|  | 272 |  | 16 | 4 | 4 | 4 | 4 |
|  | 64 | 60 | 32 | 4 | 4 | 8 | 9 |
|  | 132 |  | 16 | 4 | 4 | 4 | 4 |
| FR2 | 64 | 60 | 32 | 4 | 4 | 8 | 9 |
|  | 132 |  | 16 | 4 | 4 | 4 | 4 |
|  | 64 | 120 | 16 | 4 | 4 | 4 | 4 |
|  | 128 |  | 8 | 4 | 4 | 2 | 3 |

Table 2: UE Rx-Tx measurement accuracy for 2 PFLs in AWGN with 4 samples.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = 0 dB | SINR = -6 dB |
| FR1 | 104 | 15 | 64 | 2 | 4 | 13 | 13 |
|  | 132 | 30 | 32 | 2 | 4 | 4 | 4 |
|  | 272 |  | 16 | 2 | 4 | 2 | 3 |
|  | 64 | 60 | 32 | 2 | 4 | 3 | 4 |
|  | 132 |  | 16 | 2 | 4 | 2 | 3 |
| FR2 | 64 | 60 | 32 | 2 | 4 | 3 | 4 |
|  | 132 |  | 16 | 2 | 4 | 2 | 3 |
|  | 64 | 120 | 16 | 2 | 4 | 2 | 3 |
|  | 128 |  | 8 | 2 | 4 | 1 | 1 |

Table 3: UE Rx-Tx measurement accuracy for 3 PFLs in AWGN with reduced number of samples.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -13 dB |
| FR1 | 104 | 15 | 64 | 4 | 4 | 16 | 14 |
|  | 132 | 30 | 32 | 4 | 4 | 4 | 4 |
|  | 272 |  | 16 | 4 | 4 | 3 | 3 |
|  | 64 | 60 | 32 | 4 | 4 | 4 | 4 |
|  | 132 |  | 16 | 4 | 4 | 2 | 3 |
| FR2 | 64 | 60 | 32 | 4 | 4 | 4 | 4 |
|  | 132 |  | 16 | 4 | 4 | 2 | 3 |
|  | 64 | 120 | 16 | 4 | 4 | 2 | 3 |
|  | 128 |  | 8 | 4 | 4 | 2 | 1 |

Table 4: UE Rx-Tx measurement accuracy for 3 PFLs in AWGN with 4 samples.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -13 dB |
| FR1 | 104 | 15 | 64 | 4 | 4 | 39 | 39 |
|  | 132 | 30 | 32 | 4 | 4 | 33 | 34 |
|  | 272 |  | 16 | 4 | 4 | 29 | 30 |
|  | 64 | 60 | 32 | 4 | 4 | 34 | 34 |
|  | 132 |  | 16 | 4 | 4 | 29 | 30 |
| FR2 | 64 | 60 | 32 | 4 | 4 | 71 | 71 |
|  | 132 |  | 16 | 4 | 4 | 38 | 38 |
|  | 64 | 120 | 16 | 4 | 4 | 42 | 42 |
|  | 128 |  | 8 | 4 | 4 | 56 | 56 |

Table 5: UE Rx-Tx measurement accuracy for 2 PFLs in fading conditions (TDL-A for FR1, TDL-C for FR2) with 4 samples.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [90%] (Tc) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINR = -3 dB | SINR = -13 dB |
| FR1 | 104 | 15 | 64 | 4 | 4 | 38 | 38 |
|  | 132 | 30 | 32 | 4 | 4 | 28 | 29 |
|  | 272 |  | 16 | 4 | 4 | 29 | 29 |
|  | 64 | 60 | 32 | 4 | 4 | 28 | 30 |
|  | 132 |  | 16 | 4 | 4 | 29 | 29 |
| FR2 | 64 | 60 | 32 | 4 | 4 | 39 | 39 |
|  | 132 |  | 16 | 4 | 4 | 56 | 56 |
|  | 64 | 120 | 16 | 4 | 4 | 56 | 56 |
|  | 128 |  | 8 | 4 | 4 | 64 | 64 |

Table 6: UE Rx-Tx measurement accuracy for 3 PFLs in fading conditions (TDL-A for FR1, TDL-C for FR2) with 4 samples.

Proposal 1: Define UE Rx-Tx measurement accuracy for 2 PFLs in AWGN with reduced number of samples as shown in Table 1

Proposal 2: Define UE Rx-Tx measurement accuracy for 2 PFLs in AWGN with 4 samples as shown in Table 2.

Proposal 3: Define UE Rx-Tx measurement accuracy for 3 PFLs in AWGN with reduced number of samples as shown in Table 3.

Proposal 4: Define UE Rx-Tx measurement accuracy for 3 PFLs in AWGN with 4 samples as shown in Table 4.

Proposal 5: Define UE Rx-Tx measurement accuracy for 2 PFLs in fading conditions with 4 samples as shown in Table 5.

Proposal 6: Define UE Rx-Tx measurement accuracy for 3 PFLs in fading conditions with 4 samples as shown in Table 6.

# RSTD measurement accuracy

The results shown in the following tables were obtained by averaging results summarized in [1] and rounding up to an integer number of Tc. In some cases, we excluded results that appear as outliers. E.g. results from Ericsson for AWGN were excluded.

We do not propose RSTD measurement accuracy for fading conditions due to large disparities in the simulation results submitted to date.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -3 dB, SINRi = -6 dB |
| FR1 | 104 | 15 | 64 | 2 | 4 | ±23 |
|  | 132 | 30 | 32 | 2 | 4 | ±13 |
|  | 272 |  | 16 | 2 | 4 | ±6 |
|  | 64 | 60 | 32 | 2 | 4 | ±13 |
|  | 132 |  | 16 | 2 | 4 | ±6 |
| FR2 | 64 | 60 | 32 | 2 | 4 | ±13 |
|  | 132 |  | 16 | 2 | 4 | ±6 |
|  | 64 | 120 | 16 | 2 | 4 | ±6 |
|  | 128 |  | 8 | 2 | 4 | ±4 |

Table 7: RSTD measurement accuracy for 2 PFLs in AWGN with reduced number of samples.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -6 dB, SINRi = -13 dB |
| FR1 | 104 | 15 | 64 | 4 | 4 | ±18 |
|  | 132 | 30 | 32 | 4 | 4 | ±10 |
|  | 272 |  | 16 | 4 | 4 | ±5 |
|  | 64 | 60 | 32 | 4 | 4 | ±10 |
|  | 132 |  | 16 | 4 | 4 | ±4 |
| FR2 | 64 | 60 | 32 | 4 | 4 | ±10 |
|  | 132 |  | 16 | 4 | 4 | ±4 |
|  | 64 | 120 | 16 | 4 | 4 | ±5 |
|  | 128 |  | 8 | 4 | 4 | ±3 |

Table 8: RSTD measurement accuracy for 2 PFLs in AWGN with 4 samples.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -3 dB, SINRi = -6 dB |
| FR1 | 104 | 15 | 64 | 2 | 4 | ±20 |
|  | 132 | 30 | 32 | 2 | 4 | ±6 |
|  | 272 |  | 16 | 2 | 4 | ±4 |
|  | 64 | 60 | 32 | 2 | 4 | ±6 |
|  | 132 |  | 16 | 2 | 4 | ±4 |
| FR2 | 64 | 60 | 32 | 2 | 4 | ±6 |
|  | 132 |  | 16 | 2 | 4 | ±4 |
|  | 64 | 120 | 16 | 2 | 4 | ±4 |
|  | 128 |  | 8 | 2 | 4 | ±2 |

Table 9: RSTD measurement accuracy for 3 PFLs in AWGN with reduced number of samples.

| FR | PRS BW (PRBs) per PFL | SCS (kHz) | sample rate (Tc) per PFL | Num. samples | Num symbols | Accuracy [5%, 95%] (Tc) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | SINRref = -6 dB, SINRi = -13 dB |
| FR1 | 104 | 15 | 64 | 4 | 4 | ±14 |
|  | 132 | 30 | 32 | 4 | 4 | ±5 |
|  | 272 |  | 16 | 4 | 4 | ±3 |
|  | 64 | 60 | 32 | 4 | 4 | ±5 |
|  | 132 |  | 16 | 4 | 4 | ±3 |
| FR2 | 64 | 60 | 32 | 4 | 4 | ±5 |
|  | 132 |  | 16 | 4 | 4 | ±3 |
|  | 64 | 120 | 16 | 4 | 4 | ±3 |
|  | 128 |  | 8 | 4 | 4 | ±1 |

Table 10: RSTD measurement accuracy for 3 PFLs in AWGN with 4 samples.

Proposal 7: Define RSTD measurement accuracy for 2 PFLs in AWGN with reduced number of samples as shown in Table 7.

Proposal 8: Define RSTD measurement accuracy for 2 PFLs in AWGN with 4 samples as shown in Table 8.

Proposal 9: Define RSTD measurement accuracy for 3 PFLs in AWGN with reduced number of samples as shown in Table 9.

Proposal 10: Define RSTD measurement accuracy for 3 PFLs in AWGN with 4 samples as shown in Table 10.

Proposal 11: Do not define RSTD measurement accuracy with PRS bandwidth aggregation in fading conditions based on the simulation results summarized in R4-2412692.

# Conclusions

Proposal 1: Define UE Rx-Tx measurement accuracy for 2 PFLs in AWGN with reduced number of samples as shown in Table 1.

Proposal 2: Define UE Rx-Tx measurement accuracy for 2 PFLs in AWGN with 4 samples as shown in Table 2.

Proposal 3: Define UE Rx-Tx measurement accuracy for 3 PFLs in AWGN with reduced number of samples as shown in Table 3.

Proposal 4: Define UE Rx-Tx measurement accuracy for 3 PFLs in AWGN with 4 samples as shown in Table 4.

Proposal 5: Define UE Rx-Tx measurement accuracy for 2 PFLs in fading conditions with 4 samples as shown in Table 5.

Proposal 6: Define UE Rx-Tx measurement accuracy for 3 PFLs in fading conditions with 4 samples as shown in Table 6.

Proposal 7: Define RSTD measurement accuracy for 2 PFLs in AWGN with reduced number of samples as shown in Table 7.

Proposal 8: Define RSTD measurement accuracy for 2 PFLs in AWGN with 4 samples as shown in Table 8.

Proposal 9: Define RSTD measurement accuracy for 3 PFLs in AWGN with reduced number of samples as shown in Table 9.

Proposal 10: Define RSTD measurement accuracy for 3 PFLs in AWGN with 4 samples as shown in Table 10.

Proposal 11: Do not define RSTD measurement accuracy with PRS bandwidth aggregation in fading conditions based on the simulation results summarized in R4-2412692.

# References

[1] R4-2412692, Summary of simulation results for PRS aggregation, Ericsson, RAN4#112