3GPP TSG-RAN WG4 Meeting #112bis	R4-2415933

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.22.2.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on OD-SSB requirements

Document for:	Discussion

- Introduction

During last meeting, RAN4 reached some agreements, but there are still many open items and it is summarized in WF [1]. This document focus on the OD-SSB requirements and proposes possible enhancements.

- Discussion

In the last meeting, several issues for OD-SSB based SCell activation was discussed. WF is copied below.

Issue 1-2-1: Case 1- OD-SSB based deactivated SCell measurement - Proposals - Option 1: For deactivated SCell measurement requirements, follow the periodicity of configured OD-SSB (not the measurement cycle). - Option 2: For deactivated SCell measurement requirements, follow the measCycleSCell. - Other option is not precluded Issue 2-2-10: Case #1 and Scenario #2A - unknown SCell activation - Wayforward: - Option 1: RAN4 to only define unknown SCell activation provided that the OD-SSB is the first OD-SSB triggered by NW for (Case #1, Scenario #2A). - Option 2: RAN4 only specifies requirements assuming the target SCell is unknown in (Case #1, Scenario #2A). - Option 3: RAN4 needs to further consider the scenario (Case #1, Scenario #2A) together with (Case #1, Scenario #2) for deactivated SCell OD-SSB measurement. - Other option is not precluded

According to the previous agreements, RAN4 firstly focuses on Case #1/2 of scenario #2/2A. The key issues for deactivated SCell activation based on OD-SSB are listed below:

- Known/unknown state handling

- SSB related delay definition

About measurement cycle, since measCycleSCell is not directly related to SSB periodicity, it can be used as same as current case if OD-SSB is used.

Proposal 1: For deactivated SCell measurement requirements, follow the measCycleSCell.

About known/unknown state, each case can be considered as follows:

- Case #1 of scenario #2

  - In this case, no always-on SSB on the cell and OD-SSB is triggered at the time instance of SCell configuration, or during the time period between SCell configuration and activation. In some cases, the OD-SSB has been transmitted before the UE receiving OD-SSB triggering command. However the UE cannot know whether OD-SSB has been transmitted or not before receiving the command. Therefore known/unknown should be evaluated based on the time period between OD-SSB triggering command and SCell activation command as same manner as that of current specification.

- Case #1 of scenario #2A

  - In this case, no always-on SSB on the cell and OD-SSB is triggered at the time instance of SCell activation. In some cases, the OD-SSB has been transmitted before the UE receiving OD-SSB triggering command. However the UE cannot know whether OD-SSB has been transmitted or not before receiving the command. Therefore the SCell to be activated should be always treated as unknown.

- Case #2 of scenario #2

  - In this case, there is always-on SSB and OD-SSB is triggered at the time instance of SCell configuration, or during the time period between SCell configuration and activation. Therefore the UE can evaluate the SCell condition based on always-on SSB and/or OD-SSB. According to this assumption, known/unknown should be evaluated as same manner as that of current specification by both always-on SSB and OD-SSB.

- Case #2 of scenario #2A

  - In this case, there is always-on SSB and OD-SSB is triggered at the time instance of SCell activation. In some cases, the OD-SSB has been transmitted before the UE receiving OD-SSB triggering command. However the UE cannot know whether OD-SSB has been transmitted or not before receiving the command. Therefore known/unknown should be evaluated as same manner as that of current specification.

Proposal 2: About known/unknown state, each case can be considered as follows:

- Case #1 of scenario #2

  - In this case, no always-on SSB on the cell and OD-SSB is triggered at the time instance of SCell configuration, or during the time period between SCell configuration and activation. In some cases, the OD-SSB has been transmitted before the UE receiving OD-SSB triggering command. However the UE cannot know whether OD-SSB has been transmitted or not before receiving the command. Therefore known/unknown should be evaluated based on the time period between OD-SSB triggering command and SCell activation command as same manner as that of current specification.

- Case #1 of scenario #2A

  - In this case, no always-on SSB on the cell and OD-SSB is triggered at the time instance of SCell activation. In some cases, the OD-SSB has been transmitted before the UE receiving OD-SSB triggering command. However the UE cannot know whether OD-SSB has been transmitted or not before receiving the command. Therefore the SCell to be activated should be always treated as unknown.

- Case #2 of scenario #2

  - In this case, there is always-on SSB and OD-SSB is triggered at the time instance of SCell configuration, or during the time period between SCell configuration and activation. Therefore the UE can evaluate the SCell condition based on always-on SSB and/or OD-SSB. According to this assumption, known/unknown should be evaluated as same manner as that of current specification by both always-on SSB and OD-SSB.

- Case #2 of scenario #2A

  - In this case, there is always-on SSB and OD-SSB is triggered at the time instance of SCell activation. In some cases, the OD-SSB has been transmitted before the UE receiving OD-SSB triggering command. However the UE cannot know whether OD-SSB has been transmitted or not before receiving the command. Therefore known/unknown should be evaluated as same manner as that of current specification.

- Conclusion

In this contribution, we propose possible enhancements.

Proposal 1: For deactivated SCell measurement requirements, follow the measCycleSCell.

Proposal 2: About known/unknown state, each case can be considered as follows:

- Case #1 of scenario #2

  - In this case, no always-on SSB on the cell and OD-SSB is triggered at the time instance of SCell configuration, or during the time period between SCell configuration and activation. In some cases, the OD-SSB has been transmitted before the UE receiving OD-SSB triggering command. However the UE cannot know whether OD-SSB has been transmitted or not before receiving the command. Therefore known/unknown should be evaluated based on the time period between OD-SSB triggering command and SCell activation command as same manner as that of current specification.

- Case #1 of scenario #2A

  - In this case, no always-on SSB on the cell and OD-SSB is triggered at the time instance of SCell activation. In some cases, the OD-SSB has been transmitted before the UE receiving OD-SSB triggering command. However the UE cannot know whether OD-SSB has been transmitted or not before receiving the command. Therefore the SCell to be activated should be always treated as unknown.

- Case #2 of scenario #2

  - In this case, there is always-on SSB and OD-SSB is triggered at the time instance of SCell configuration, or during the time period between SCell configuration and activation. Therefore the UE can evaluate the SCell condition based on always-on SSB and/or OD-SSB. According to this assumption, known/unknown should be evaluated as same manner as that of current specification by both always-on SSB and OD-SSB.

- Case #2 of scenario #2A

  - In this case, there is always-on SSB and OD-SSB is triggered at the time instance of SCell activation. In some cases, the OD-SSB has been transmitted before the UE receiving OD-SSB triggering command. However the UE cannot know whether OD-SSB has been transmitted or not before receiving the command. Therefore known/unknown should be evaluated as same manner as that of current specification.

References

[1] 3GPP, R4-2413885, “WF on Netw_Energy_NR_enh”, Ericsson