3GPP TSG-RAN WG4 Meeting #112bis	R4-2415935

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.25.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RRM core requirements for XR

Document for:	Discussion

- Introduction

During last meeting, RAN4 reached some agreements, but there are still many open items and it is summarized in WF [1]. This document proposes RRM core requirements for XR and LS reply for RAN1.

- Discussion

  - Types of measurement gaps to consider

During previous meeting, the MG type to be considered is listed and companies proposes priority. We propose priorities as follows.

Proposal 1: RAN4 to work based on the following table

| Cap types | First prio | Second prio | Don’t include |
| --- | --- | --- | --- |
| Type 1 (Gap(s) configured via GapConfig without suffix) | Should be first priority because most basic MG and will be the baseline for XR as well |  |  |
| Type 2 (Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17) |  |  | Can be excluded |
| NCSG |  | Should be second priority because it can be a solution for XR |  |
| MUSIM |  |  | Can be excluded |
| Positioning |  |  | Can be excluded |
| Rel 18 concurrent gap |  |  | Can be excluded |
| PreConfigured |  | Should be second priority because it can be a solution for XR |  |

- LS reply

The incoming LS from RAN1 asks whether UE assistance information related to measurement occasions is needed or not. The candidates of measurement related information are copied below.

- Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period.

- Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

Generally, these types of information are specified like minimum measurement samples or maximum duration between previous measurement as a minimum requirement in RAN4 RRM spec.

Observation 1: The candidates of measurement related information derived in LS are generally specified like minimum measurement samples or maximum duration between previous measurement as a minimum requirement in RAN4 RRM spec.

Therefore, no UE assistance information related to measurement occasion is needed.

Proposal 2: No UE assistance information related to measurement occasion is needed.

- Conclusion

In this contribution, we proposed our views on RRM core requirements for XR.

Proposal 1: RAN4 to work based on the following table

| Cap types | First prio | Second prio | Don’t include |
| --- | --- | --- | --- |
| Type 1 (Gap(s) configured via GapConfig without suffix) | Should be first priority because most basic MG and will be the baseline for XR as well |  |  |
| Type 2 (Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17) |  |  | Can be excluded |
| NCSG |  | Should be second priority because it can be a solution for XR |  |
| MUSIM |  |  | Can be excluded |
| Positioning |  |  | Can be excluded |
| Rel 18 concurrent gap |  |  | Can be excluded |
| PreConfigured |  | Should be second priority because it can be a solution for XR |  |

Proposal 2: No UE assistance information related to measurement occasion is needed.

References

[1] 3GPP, RP-2414043, “WF on RRM requirements for XR_Ph3”, Nokia

[2] 3GPP, R4-2411007, “LS on UE assistance information”, RAN1