# 3GPP TSG-RAN WG4 Meeting #112bis                            R4-2415937

# Hefei, China, 14th-19th Oct, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on BS RF requirements for LP-WUS/WUR

# Agenda Item:   6.23.3

# Document for:  Approval

- Introduction

After the online discussion in RAN4#112 meeting, there are still many controversial issues left. In this contribution, we continue to discuss the BS requirements for LP-WUS/LP-WUR.

- Discussion

## 2.1 Concept of LP-WUS Power Boosting

In RAN4#112 meeting, RAN4 discussed the definition of power boosting of LP-WUS and there are three options to be considered as follows[1]. The first two options prefer to introduce EPRE ratio to define LP-WUS power boosting, while Option 3 is proposed by us to reuse the legacy definition of dynamic range for NB-IoT.

Concept of LP-WUS dynamic range/power boosting FFS on the following options: - Introduce EPRE ratio to define LP-WUS power boosting, since it has the merit that power boosting does not vary with different CBWs. - Option 1: The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs). - Option 2: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of an LP-WUS RE (averaged power per RE based on all LP-WUS REs within a NR transmission bandwidth configuration) and the average power of a non-LP-WUS RE (averaged power from all REs from the NR carrier excluding the LP-WUS REs). - Adopt following definition rather than introduce EPRE ratio. - Option 3: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

The difference between dynamic range and EPRE ratio is shown in Fig 1. There is a consensus in the last meeting that dynamic range and EPRE ratio are mathematically equivalent and if know one we can derive the other one.

Fig. 1  The difference between dynamic range and EPRE ratio.

We understand that EPRE ratio is proposed for the purpose to declare power boosting level without considering channel bandwidth. However, there are some aspects need to be noticed as follows.

First, EPRE ratio has a premise that there are both LP-WUS RBs and NR RBs at the same time. When there is only LP-WUS transmitted, this definition does not work while dynamic range can work well no matter whether NR signal is transmitted.

Second, EPRE ratio is a relative value, which can not reflect the effective information we want directly. What we really care about are how much power LP-WUS can be boosted, and at the same time how much power NR signal has to be deboosted. Dynamic range can reflect the boosted power of LP-WUS by original LP-WUS RE power and the deboosted power of NR by original NR RE power quite directly.

Third, Option 3 comes from legacy definition of power boosting for NB-IoT, which is friendly for manufactures to reuse some design for NB-IoT in LP-WUS. When talking about power boosting, the only difference between NB-IoT and LP-WUS is that NB-IoT only occupies 1 RB while LP-WUS occupies multiple RBs. Power boosting for NB-IoT and LP-WUS is the same operation, such that to avoid ambiguity, we should not introduce another different definition for this kind of operation.

For all the reasons above, we still prefer Option 3 as the concept of LP-WUS power boosting.

Proposal 1: Use concept of LP-WUS dynamic range/power boosting in Option 3 rather than EPRE ratio: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within an NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

## 2.2 Manufacture Declaration

RAN4 has achieved a consensus that the transmit power is shared between NR OFDM signal and LP-WUS for the same carrier and the total transmit power is fixed[2]. In this case, when the transmission power of LP-WUS increases, the transmission power of NR signal decreases. As we discussed in our previous contribution [3], the permitted power boosting of LP-WUS depends on the channel bandwidth and the position of WUS RBs. Besides according to RE power control dynamic range in TS38.104 as follows, if NR signal is modulated by 64QAM, 256QAM or 1024QAM, the up and down range is 0dB.

In this case, if LP-WUS is transmitted together with PDSCH modulated by 64QAM, 256QAM or 1024QAM, no LP-WUS power boosting is permitted. Therefore, the permitted LP-WUS power boosting also depends on the NR modulation orders, but the modulation orders depend on implementation.

Considering that the power boosting of LP-WUS depends on implementation, LP-WUS power boosting should be completely manufacture declared and no minimum limitation or maximum limitation should be introduced.

Proposal 2: Not to introduce a minimum limitation or a maximum limitation for LP-WUS power boosting.

Proposal 3: Set the LP-WUS power boosting a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the supported boosting level.

## 2.3 Transmitted Signal Quality

Regarding transmit signal quality, based on the signal design, LP-WUS information is contained in time-domain for envelope detection and time-domain correlation, and is contained in frequency-domain for frequency-domain correlation. Since RAN1 has not finished the design of correlation detection for LP-WUS, here we only talk about the transmitted signal quality requirements for envelope detection.

Traditional Tx EVM is tested after FFT process based on constellations, which may not work for LP-WUS with envelope detection. Therefore, new RF envelope requirements is needed for LP-WUS to guarantee transmitted signal quality and decoding performance.

In A-IoT, companies are considering to refer to RFID RF envelope related parameters to define signal transmission quality requirement[4], such as modulation depth, RF envelope ripple, RF pulse width. The waveform generation methods of LP-WUS and A-IoT are not completely same, which results in different time-domain signal characteristics as shown in Fig. 2. The time-domain signal of A-IoT has stable ripple while the time-domain signal of LP-WUS is severe.

Fig. 2  The difference between time-domain signals of LP-WUS and A-IoT (2 OFDM symbols with CP)

However, it can also be a reference direction for LP-WUS transmitted signal quality because they are both OOK-based signal. In LP-WUS, we care about the power difference between OOK ON and OOK OFF most. Therefore, we can FFS whether the modulation depth can be used as transmitted signal quality for LP-WUS.

Proposal 4: FFS whether the modulation depth can be used as transmitted signal quality for LP-WUS.

## 2.4 Applicable BS Types

In last meeting, no agreement on the applicable BS types to support LP-WUS has been achieved yet[1].

On applicable BS type for LP-WUS - FFS whether all available BS types can be considered for LP-WUS due to the support of LP-WUS is optional and the power boosting level could be up to manufacturer declaration. - FFS whether only conductive based BS RF requirements should be defined for LP-WUS

Currently in TS38.104, only BS Type 1-C supports NR with in-band NB-IoT operation. BS Type 1-H and BS Type 1-O are excluded considering the power consumption issues. When it comes to LP-WUS, similar issues exist. LP-WUS occupies more RBs than NB-IoT. In this case, we prefer to consider BS Type 1-C as applicable type and FFS other types.

Proposal 5: It is propose to use BS Type 1-C as applicable BS type as a start point.

- Conclusion

Proposals:

Proposal 1: Use concept of LP-WUS dynamic range/power boosting in Option 3 rather than EPRE ratio: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within an NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

Proposal 2: Not to introduce a minimum limitation or a maximum limitation for LP-WUS power boosting.

Proposal 3: Set the LP-WUS power boosting a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the supported boosting level.

Proposal 4: FFS whether the modulation depth can be used as transmitted signal quality for LP-WUS.

Proposal 5: It is propose to use BS Type 1-C as applicable BS type as a start point.

- Reference

- R4-2413512 Way For [112][309] NR_LPWUS, RAN4#112, Maastricht, Netherlands, Aug. 2024.

- R4-2406140 Way Forward for [110bis][314] NR_LPWUS, RAN4#110bis, Changsha, China, Apr. 2024.

- R4-2411894 Discussion on BS RF requirements for LP-WUS/WUR, RAN4#112, ZTE, Maastricht, Netherlands, Aug. 2024.

- R4-2412834 Topic summary for [112][132] FS_Ambient_IoT_solutions_part2.