# 3GPP TSG-RAN WG4 Meeting #112bis                            R4-2415938

# Hefei, China, 14th-19th Oct, 2024

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on REFSENS, ASCS, ACS for LP-WUR

# Agenda Item:   6.23.2.2

# Document for:  Approval

- Introduction

In RAN4#112 meeting, RAN4 achieved some agreements on the SNR, ASCS and ACS simulations[1]. In this contribution, we provided some updated simulation results based on the agreements in the last meeting and discussed the issues left.

- Discussion

## 2.1 SNR Evaluation

In last meeting, RAN4 agreed to use AWGN channel to specify LP-WUS RF requirements and no repetition should be used when simulate target SNR [1]. For target SNR value, the following issues are under discussion.

Issue 2-1-3: Whether RAN4 should conclude a target SNR first Agreement: - Target SNR is the basis for many Rx requirements discussion. Group should conclude SNR definition and SNR values first. Issue 2-1-4: Target SNR value Agreements: - RAN4 further discuss target SNR value for LP-WUS - FFS single SNR for OOK-based and OFDM-based receiver

First of all, the definition of SNR was proposed in the last meeting to align the understanding. In TR38.869, SNR is calculated as average EPRE divided by power of noise. This defination is reused in RAN1 and we think it can be reused in RAN4 too.

Proposal 1: SNR could be calculated as the average EPRE of LP-WUS divided by power of noise.

Then we updated the simulation conditions based on the discussion in the last meeting. According to out previous simulation, we can find that OOK-4 waveform with M=4 has higher SNR requirement, so here we only provide the simulation results of OOK-4, M=4 to derive the target SNR for OOK-based receiver with 3rd and 5th order filter. The simulation assumptions follows the agreements in [1] and [2] unless otherwise explicitly stated. Fig. 1 shows the impacts of SNR on BLER.

Fig. 1  SNR evaluation for OOK-based receiver

According to the simulation results, at least -2dB SNR is needed for OOK-based receiver.

Proposal 2: -2dB SNR is needed for OOK-based receiver.

## 2.2 NF and IM

For REFSENS requirements, RAN4 reached some agreements in last meeting as follows[1].

Issue 2-2-2: Baseline architecture for OOK-based LP-WUS Agreement: - Use zero-IF receiver as a baseline architecture for envelop based LP-WUR Issue 2-2-3: Baseline architecture for OFDM-based LP-WUS Agreements: - Wait for RAN1’s conclusion on the OFDM-based LP-WUR. Issue 2-2-4: whether different NF for OOK-based and OFDM-based Agreement: - Differentiate NF for OOK-based and OFDM-based receiver. Issue 2-2-9: MDR value for REFSENS test case Agreements: - FFS the MDR for conformance testing should be aligned with the value for RF requirement.

Usually, REFSENS for NR UE is expressed as

REFSENS(dBm) = kT0 + 10log10(NRB*12*SCS) + NF + SNR + IM

where some other related parameters are not included for LP-WUS, such as diversity gain, because compared to the main receiver, LP-WUR architecture is a separate Rx chain without the corresponding diversity Rx chain, i.e. 1Rx architecture, so there is no need to consider diversity gain.

Except for SNR, we still need Regarding the NF values, OOK-based receiver and OFDM-based receiver have quite different architecture and we should differentiate NF for OOK-based receiver and OFDM-based receiver. In RAN4, we can follow the assumptions in RAN1, i.e., +2 dB and +5 dB on top of NF of MR (9dB) for OFDM-based receiver and OOK-based receiver, respectively.

Proposal 3: NF Gap between LR and MR for REFSENS could be +2dB and +5dB for OFDM-based receiver and OOK-based receiver, respectively.

For NR MR, IM is ~2.5dB considering implementation issues. For LR, the architecture is simpler than and power consumption is less than MR, we think 2 dB is okay for LR implementation.

Proposal 4: 2dB IM is okay for LR.

## 2.3 ASCS Evaluation

ASCS is defined as the ratio of the receive filter attenuation on the assigned LP-WUR channel frequency to the receive filter attenuation on the adjacent NR subcarrier [2], as shown in Fig. 2 (with 1 guard RB). NR PDSCH has the same PSD as LP-WUS.

Fig. 2  Receive filter attenuation for LP-WUS and NR signal under different CBW

The issues that are still under discussion are listed as follows [1].

Issue 2-3-1: ASCS requirements value Agreements: - RAN4 further discuss how to specify ASCS requirement based on simulation outcome. Issue 2-3-2: Required number of guard RB for ASCS Agreements: - RAN4 further discuss the required number of guard RBs for ASCS requirement based on simulation outcome. Issue 2-3-3: Test parameters for ASCS Agreements: - RAN4 further discuss the test parameters for ASCS requirement.

### 2.3.1 Guard RB for ASCS

According to the WF, we updated the simulation conditions and show the simulation results on the guard RB evaluation for ASCS in Fig. 3.

Fig. 3  LP-WUR performance in ASCS case for filter order = 3 and 5

We can find that with 1 guard RB, there is little difference on the required SNR to achieve 1% BLER for given waveform compared to 0 guard RB. Besides, there is no obvious improvement compared to REFSENS environment in Fig. 1, which means that WUR can endure adjacent sub-carrier interference well with 3rd and 5th order filters. Moreover, we also find that this phenomenon does not vary with channel bandwidth.

Observation 1: With 1 guard RB, there is little difference on the required SNR to achieve 1% BLER for given waveform compared to 0 guard RB for all CBW.

Proposal 5: It is proposed that no guard RB is needed for ASCS.

### 2.3.2 ASCS Requirements

It is needed to define ASCS requirements for LP-WUR since the filter capability should be guaranteed. As we discussed in our previous contribution [3], the receive filter attenuation on the adjacent NR subcarrier depends on CBW, WUS location and filter characteristic (order). Considering filter suppression characteristic, WUS at center can be seen as the worst case to determine ASCS requirements. As shown in Table 1, ASCS results vary with CBW, taking 20ppm frequency error, 0 guard RB, 30kHz SCS and waveform OOK-4, M=4 as example.

Table 1  ASCS results for different CBW (WUS at center)

| CBW(MHz) | SCS(kHz) | NRB | Filter order = 3 | Filter order = 5 |
| --- | --- | --- | --- | --- |
| 5 | 30 | 11 | - | - |
| 10 | 30 | 24 | -8.85 | -11.43 |
| 15 | 30 | 38 | -11.92 | -14.61 |
| 20 | 30 | 51 | -13.64 | -16.34 |
| 25 | 30 | 65 | -14.93 | -17.63 |
| 30 | 30 | 78 | -15.86 | -18.56 |
| 35 | 30 | 92 | -16.70 | -19.38 |
| 40 | 30 | 106 | -17.34 | -20.04 |
| 45 | 30 | 119 | -17.89 | -20.61 |
| 50 | 30 | 133 | -18.43 | -21.15 |
| 60 | 30 | 162 | -19.35 | -22.07 |
| 70 | 30 | 189 | -20.07 | -22.78 |
| 80 | 30 | 217 | -20.70 | -23.41 |
| 90 | 30 | 245 | -21.25 | -23.97 |
| 100 | 30 | 273 | -21.74 | -24.46 |

In this case, we should define ASCS requirements for all available CBW.

Observation 2: ASCS varies with CBW since receive filter attenuation on the adjacent NR subcarrier varies with the number of NR RBs.

Proposal 6: Consider ASCS requirements for all available CBW as shown in Table 2.

Table 2  Adjacent subcarrier selectivity

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |
| ASCS | dB | - | [-8] | [-7-10log10(BWChannel/5)] | [-8-10log10(BWChannel/5)] |

### 2.3.3 Test Parameters for ASCS

To perform test, test parameters are needed. Based on the analysis above, no guard RB is needed and NR PDSCH should occupy the RBs not used for LP-WUS with the same PSD as WUS signal.

Proposal 7: Define the test parameters for ASCS of LP-WUR in Table 3. For these test parameters, the missed detection rate shall be ≤1%.

Table 3  Test parameters for ASCS of LP-WUR

| RX parameter | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- |
|  | 11RB (for 30kHz SCS) |
| LP-WUS Power (dBm) | REFSENS |
| BWinterferer | NR PDSCH is mapped on RBs not used for LP-WUS |
| In-band Pinterferer | Same PSD as LP-WUS |
| NOTE 1: LP-WUS is set at center of the NR carrier. |  |

## 2.4 ACS Evaluation

ACS is defined as the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s) as shown in Fig. 4[2].

Fig. 4  Filter attenuation for LP-WUS and adjacent carrier interference

The issues that are still under discussion are listed as follows [1].

Issue 2-4-2: ACS requirements value Agreements: - RAN4 further discuss how to specify ACS requirements based on simulation outcome. - FFS same as MR, or further relaxed Issue 2-4-3: Required guard RB for ACS requirements Agreements: - RAN4 further discuss the number of guard RBs for ACS requirement based on simulation outcome. Issue 2-4-4: Test parameters for LP-WUR ACS case Agreements: - RAN4 further discuss the test parameters for ACS requirement.

### 2.4.1 Guard RB for ACS

According to WF, we updated the simulation conditions, and show the simulation results with the EPRE ratio between adjacent channel interference and LP-WUS as 31.5dB to evaluate guard RBs for ACS in Fig. 5 and Fig. 6.

Fig. 5  LP-WUR performance in ACS case for OOK-4 M=2/4, Filter order=3, EPRE ratio=31.5dB

Fig. 6  LP-WUR performance in ACS case for OOK-4 M=2/4, Filter order=5, EPRE ratio=31.5dB

From the simulation results, we can find that whether guard RBs are needed depends on filter suppression characteristics a lot under the same interference level as MR. For 3rd order Butterworth filter, up to 5 guard RBs are needed to guarantee the performance of WUR, but for 5th order Butterworth filter, no guard RB is needed if SNR condition can be 2~3dB better than RENSENS environment. Therefore, we prefer not to set guard RB limitation for ACS. As for how many guard RBs are used actually, it depends on implementation.

Observation 3: Required number of guard RBs vary with filter suppression characteristics a lot. For 3rd order Butterworth filter, up to 5 guard RBs are needed to guarantee the performance of WUR, but for 5th order Butterworth filter, no guard RB is needed.

Proposal 8: It is proposed that not to set guard RB limitation for ACS.

### 2.4.2 ACS Requirements

Similar with ASCS requirements, we need to define ACS requirements for LP-WUR since the filter capability should be guaranteed. We provide some simulation results on ACS in Table 4. As shown in Table 4, ACS results vary with CBW because of different guard bands and the number of guard RBs, taking 20ppm frequency error, 30kHz SCS and waveform OOK-4, M=4 as example (to save time, we did not fill all the grids).

Table 4  ACS results for different CBW (WUS at edge with guard RBs)

| CBW (MHz) | SCS (kHz) | NRB | Guard band (kHz) | 0 guard RB |  | 1 guard RB |  | 2 guard RB |  | 3 guard RB |  | 4 guard RB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fo = 3 | Fo = 5 | Fo = 3 | Fo = 5 | Fo = 3 | Fo = 5 | Fo = 3 | Fo = 5 | Fo = 3 | Fo = 5 |
| 5 | 30 | 11 | 505 | -18.84 | -28.55 | - | - | - | - | - | - | - | - |
| 10 | 30 | 24 | 665 | -19.97 | -30.61 | -22.16 | -34.66 | -24.31 | -38.52 | -26.29 | -42.05 | -28.12 | -45.29 |
| 15 | 30 | 38 | 645 | -19.86 | -30.40 |  |  |  |  |  |  |  |  |
| 20 | 30 | 51 | 805 | -20.75 | -32.11 | -23.02 | -36.21 | -25.10 | -39.93 | -27.02 | -43.34 | -28.80 | -46.48 |
| 25 | 30 | 65 | 785 | -20.61 | -31.87 |  |  |  |  |  |  |  |  |
| 30 | 30 | 78 | 945 | -21.65 | -33.75 |  |  |  |  |  |  |  |  |
| 35 | 30 | 92 | 925 | -21.53 | -33.52 | -23.73 | -37.48 | -25.76 | -41.09 | -27.63 | -44.41 | -29.36 | -47.48 |
| 40 | 30 | 106 | 905 | -21.39 | -33.29 |  |  |  |  |  |  |  |  |
| 45 | 30 | 119 | 1065 | -22.67 | -35.46 |  |  |  |  |  |  |  |  |
| 50 | 30 | 133 | 1045 | -22.28 | -34.89 | -24.43 | -38.73 | -26.40 | -42.23 | -28.22 | -45.46 | -29.92 | -48.45 |
| 60 | 30 | 162 | 825 | -20.88 | -32.35 |  |  |  |  |  |  |  |  |
| 70 | 30 | 189 | 965 | -21.78 | -33.98 |  |  |  |  |  |  |  |  |
| 80 | 30 | 217 | 925 | -21.53 | -33.52 | -23.73 | -37.48 | -25.76 | -41.09 | -27.63 | -44.41 | -29.36 | -47.48 |
| 90 | 30 | 245 | 885 | -21.27 | -33.06 |  |  |  |  |  |  |  |  |
| 100 | 30 | 273 | 845 | -21.01 | -32.58 |  |  |  |  |  |  |  |  |

Observation 4: ACS varies with CBW since receive filter attenuation on the adjacent channel varies with guard bands.

Observation 5: ACS varies with the number of guard RBs a lot.

Based on the simulation results, the ACS requirement for LP-WUR should be relaxed compared to MR.

Proposal 9: ACS requirement for LP-WUR should be relaxed compared to MR.

### 2.4.3 Test Parameters for ACS

To perform test, test parameters are needed.

Proposal 10: Define the test parameters for ACS of LP-WUR in Table 5 and Table 6. For these test parameters, the missed detection rate shall be ≤1%.

Table 5  Test parameters for ACS of LP-WUR, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB (for 30kHz SCS) |
| LP-WUS Power | dBm | REFSENS + 14 dB |
| Pinterferer | dBm | REFSENS + 45.5 dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + guard RB + 2.5 / -(minimum guard band3 + guard RB + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

Table 6  Test parameters for ACS of LP-WUR, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB (for 30kHz SCS) |
| LP-WUS Power | dBm | -56.5 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + guard RB + 2.5 / -(minimum guard band3 + guard RB + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

- Conclusion

Observations:

Observation 1: With 1 guard RB, there is little difference on the required SNR to achieve 1% BLER for given waveform compared to 0 guard RB for all CBW.

Observation 2: ASCS varies with CBW since receive filter attenuation on the adjacent NR subcarrier varies with the number of NR RBs.

Observation 3: Required number of guard RBs vary with filter suppression characteristics a lot. For 3rd order Butterworth filter, up to 5 guard RBs are needed to guarantee the performance of WUR, but for 5th order Butterworth filter, no guard RB is needed.

Observation 4: ACS varies with CBW since receive filter attenuation on the adjacent channel varies with guard bands.

Observation 5: ACS varies with the number of guard RBs a lot.

Proposals:

Proposal 1: SNR could be calculated as the average EPRE of LP-WUS divided by power of noise.

Proposal 2: -2dB SNR is needed for OOK-based receiver.

Proposal 3: NF Gap between LR and MR for REFSENS could be +2dB and +5dB for OFDM-based receiver and OOK-based receiver, respectively.

Proposal 4: 2dB IM is okay for LR.

Proposal 5: It is proposed that no guard RB is needed for ASCS.

Proposal 6: Consider ASCS requirements for all available CBW as shown in Table 2.

Table 2  Adjacent subcarrier selectivity

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |
| ASCS | dB | - | [-8] | [-7-10log10(BWChannel/5)] | [-8-10log10(BWChannel/5)] |

Proposal 7: Define the test parameters for ASCS of LP-WUR in Table 3. For these test parameters, the missed detection rate shall be ≤1%.

Table 3  Test parameters for ASCS of LP-WUR

| RX parameter | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- |
|  | 11RB (for 30kHz SCS) |
| LP-WUS Power (dBm) | REFSENS |
| BWinterferer | NR PDSCH is mapped on RBs not used for LP-WUS |
| In-band Pinterferer | Same PSD as LP-WUS |
| NOTE 1: LP-WUS is set at center of the NR carrier. |  |

Proposal 8: It is proposed that not to set guard RB limitation for ACS.

Proposal 9: ACS requirement for LP-WUR should be relaxed compared to MR.

Proposal 10: Define the test parameters for ACS of LP-WUR in Table 5 and Table 6. For these test parameters, the missed detection rate shall be ≤1%.

Table 5  Test parameters for ACS of LP-WUR, Case 1

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB (for 30kHz SCS) |
| LP-WUS Power | dBm | REFSENS + 14 dB |
| Pinterferer | dBm | REFSENS + 45.5 dB |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + guard RB + 2.5 / -(minimum guard band3 + guard RB + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

Table 6  Test parameters for ACS of LP-WUR, Case 2

| RX parameter | Units | LP-WUS transmission bandwidth configuration (RB)1 |
| --- | --- | --- |
|  |  | 11RB (for 30kHz SCS) |
| LP-WUS Power | dBm | -56.5 |
| Pinterferer | dBm | -25 |
| BWinterferer | MHz | 5 |
| Finterferer (offset to LP-WUS)2 | MHz | minimum guard band3 + guard RB + 2.5 / -(minimum guard band3 + guard RB + 2.5) |
| NOTE 1: LP-WUS is set at edge of the NR carrier without adjacent sub-carrier interference. NOTE 2: Finterferer (offset to LP-WUS) is the frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the LP-WUS). The interferer is an NR signal with an SCS equal to that of the LP-WUS. NOTE 3: Minimum guard band depends on the NR channel bandwidth that the LP-WUS is located in. |  |  |

- Reference

- R4-2414309 WF for LP-WUS UE RF, RAN4#112, Maastricht, Netherlands, Aug. 2024.

- R4-2410569 WF for [111][136] NR_LPWUS_UERF, RAN4#111, Fukuoka, Japan, May 2024.

- R4-2411896 Discussion on REFSENS, ASCS and ACS for LP-WUR, ZTE, RAN4#112, Maastricht, Netherlands, Aug. 2024.