3GPP TSG-RAN WG4 Meeting #112bis				                        R4-2415955

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.22.2.2

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for adaptation of common signal/channel transmission

WID/SID:        Netw_Energy_NR_enh-Core

Document for:	Discussion

# 1  Introduction

In RAN#104 meeting, the “Revised WID: Enhancements of network energy savings for NR” has been approved in [1].

The RRM requirement for adaptation of common signal/channel transmission was discussed in WF [3].

# 2 Time domain adaptation of L1/L3 SSB measurement in CONN

By following the WF [3] as below, the RRM requirements for L1/L3 SSB periodicity adaptation in CONNECTED mode will be discussed.

Issue 3-1-1: SSB adaptation in IDLE/CONNECTED mode

- Agreement

  - RAN4 to prioritize the SSB adaptation discussion in CONNECTED mode.

  - Whether to discuss SSB adaptation in IDLE mode depends on RAN1 progress

Issue 3-1-2: SSB adaptation impact in RRM requirement

- Agreement: SSB adaptation impact in RRM requirement

  - RAN4 to discuss the L1/L3 measurement requirement impact as starting point.

- Wayforward:

  - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point.

    - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress.

The concern scenario is that the transitions measurement behavior when the SSB is updated from the longer periodicity to the shorter periodicity. However, it can be analogous the existing minimum requirement at transitions, as specified in 9.1.6, where when the UE transitions when DRX cycle periodicity changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition.

Besides, for L3 measurements, which is performed based on MO and SMTCs, therefore it is probable that the MO and SMTC will be reconfigured accordingly, and it appears not necessary to define new requirements if it has been reconfigured.

Proposal 1: When the UE transitions when SSB periodicity changes, the L1/L3 measurement requirements apply based on the longer delay before or after the transition.

# Conclusion

Proposal 1: When the UE transitions when SSB periodicity changes, the L1/L3 measurement requirements apply based on the longer delay before or after the transition.

# References

- RP-241650, Revised WID: Enhancements of network energy savings for NR

- Chair notes RAN1_117 eom1

- R4-2413885, WF on Netw_Energy_NR_enh, Ericsson, Aug. 2024.