3GPP TSG-RAN WG4 Meeting #112bis				                        R4-2415956

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.26.4

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for NR NTN phase 3

WID/SID:        NR_NTN_Ph3-Core

Document for:	Discussion

# 1  Introduction

In RAN#104 meeting, the “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3” has been approved in [1].  In RAN4#112, the impact on RRM requirement for has been discussed in WF [2].

# 2  Regenerative payload and RAN

In RAN4#112, the impact on support of regenerative payload has been discussed and the following WF has been reached [2].

#### Issue 5-2-1: Timing requirements for regenerative payload

<Way Forward>

- The common understanding is that NTA,adjcommon is configured by network.

- Further discuss the impact on timing requirements in TS 38.133 for regenerative mode of NTN.

# 1. Overall Description:

In RAN2#127 meeting, the setting of common TA and Kmac for regenerative payload with full gNB on board was discussed, and a question related to the value setting was raised.

Related to this, RAN2 kindly asks RAN1 and RAN4:

Question:  Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common.

Additionally, RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0.

And one corresponding LS from RAN2 has been received [3], regarding the common TA setting.

The common Timing Advance (TA) parameter, ta-Common-r17, currently does not supports negative values. Whether to introduce negative value for regenerative mode is raised in the LS [3].

In the legacy non-regenerative NTN, in case that UE may over-compensate its TA, network can correct/reduce the UE TA, by (1) N_TA in RAR or (2) MAC-CE TAC.

In (1), because N_TA in RAR only supports >=0 in the current spec, one way to correct/reduce the over-compansated UE TA is to set a smaller TA_common than needed in advance. For instance,

- Case 1-1: If the UE has over-compensated its TA, subject to the N_TA in RAR is >=0, then a smaller TA_common can equivalently reduce the UE TA.

- Case 1-2: If the UE has under-compensated its TA, network can still correct it by configuring a larger N_TA in RAR, as a smaller TA_common is used.

In the R19 regenerative payload scenario, if the reference point is on the satellite, then the feeder link delay is zero and the TA_common cannot be larger than zero. If the network aims to correct the over-compensated TA during RACH procedure, in the same way as in the legacy non-regenerative NTN as previously described, then it requires the TA_common smaller than needed (i.e. smaller than zero).

However, it is also noted that this negative common TA approach can only help the R19 UEs, as the legacy UE cannot understand the negative common TA.

Proposal 1: For R19 UEs, the negative common TA provides more network flexibility. However, further discussion is required to accommodate legacy UEs.

# 3  (e)RedCap with NR NTN

The specified TN Rel-17 RedCap and Rel-18 eRedCap RRM requirements can be leveraged to update the requirements for FR1 NTN Redcap UE. The potential impact from (e)Redcap is analyzed below, with considerations such as 1 RX, HD-FDD, eDRX.

| Requirements | The potential impact from (e)Redcap |
| --- | --- |
| IDLE/INACTIVE, including SDT | - Update for eDRX - Update for 1 Rx, including UE measurement capability - Update for HD-FDD, for paging reception |
| CONN Mobility (Handover) & CONN Control | - Update for 1 Rx - Update for HD-FDD |
| Timing | - No major update foreseen |
| Signaling characteristics (RLM) (CONN), BFD/CBD | - Update for 1 Rx - Update for HD-FDD |
| Meas. Procedure in CONN | - Update for 1 Rx - Update for HD-FDD |

## [1 RX] Issue 6-3-2: How to relax the requirements for 1Rx (e)RedCap UEs with FR1-NTN ?

The TN 1Rx RedCap NTN UE can be leveraged, as the follows:

Proposal 2: For 1Rx RedCap NTN UE, RAN4 to adopt the same requirement relaxations and modifications as follows:

- Measurement delay

  - PSS/SSS detection delay is extended by 2-3 samples, based on whether it is intra-f/inter-f and whether it is with MG or not.

  - SSB index identification (PBCH decoding) delay is extended by 2 samples.

- Handover and CONN mobility

  - Extended handover delay

    - Unknown intra-f by 1 sample

    - Unknown inter-f by 2 samples.

  - Extended re-establishment delay by 1 sample.

  - Extended connection and release with redirection delay by 1 sample

- L1, RLM, BFD, CBD

  - Double the RLM OOS and BFD evaluation periods

  - Aggregation level of 16 for RLM/BFD hypothetical PDCCH parameters

    - CSI-RS based: the AL is twice that of legacy requirements with 2Rx.

    - SSB based: the AL and BW are twice that of legacy requirements with 2Rx.

- Measurement accuracy relaxation

  - SS-RSRP, SS-RSRQ, SS-SINR, relax relative and absolute accuracy by 1dB

  - L1-RSRP, relax relative and absolute accuracy by 3dB.

## [HD-FDD] Issue 6-4-1: The specific impact of HD-FDD for (e)Redcap UE with FR1-NTN

It has been agreed as the following in 2024.08:

<Agreement>

- RAN4 to consider the impact of HD-FDD for (e)Redcap UE with FR1-NTN.

  - The TN HD-FDD related requirements and applicable conditions defined for (e)RedCap UE can be used as a starting point, and RAN4 take into account NTN specific aspects if any.

## [eDRX] Issue 6-5-1: How to consider the impact of eDRX enhancement for (e)Redcap UE with FR1-NTN?

Similar, the TN eDRX can be used as a starting point.

Proposal 3: The TN eDRX requirement relaxations and modifications can be used as a starting point for NTN eDRX requirements in IDLE/INACTIVE mode.

For NGSO, as deliberated in R17 NR NTN, requirements involving DRX cycles >= 2.56s are deemed inapplicable due to the long delay. This rationale is also applicable to eDRX. In a similar vein, having two Paging Time Windows (PTWs) in eDRX would result in an impractically long delay for NGSO scenarios.

Proposal 4: For NGSO, eDRX requirements are not applicable if DRX >= 2.56s.

Proposal 5: For NGSO, eDRX requirements are not applicable if the measurement cannot be completed within a single PTW.

## Stationary requirements

Requirements based stationary was introduced for R17 RedCap. RAN4 may need to discuss whether the requirement can be applied to NTN. For GSO, it seems applicable, while it is impractical for NGSO.

Proposal 6: For GSO, RAN4 to discuss requirements based on stationary conditions for NR NTN Redcap.

# Conclusion

Proposal 1: For R19 UEs, the negative common TA provides more network flexibility. However, further discussion is required to accommodate legacy UEs.

Proposal 2: For 1Rx RedCap NTN UE, RAN4 to adopt the same requirement relaxations and modifications as follows:

- Measurement delay

  - PSS/SSS detection delay is extended by 2-3 samples, based on whether it is intra-f/inter-f and whether it is with MG or not.

  - SSB index identification (PBCH decoding) delay is extended by 2 samples.

- Handover and CONN mobility

  - Extended handover delay

    - Unknown intra-f by 1 sample

    - Unknown inter-f by 2 samples.

  - Extended re-establishment delay by 1 sample.

  - Extended connection and release with redirection delay by 1 sample

- L1, RLM, BFD, CBD

  - Double the RLM OOS and BFD evaluation periods

  - Aggregation level of 16 for RLM/BFD hypothetical PDCCH parameters

    - CSI-RS based: the AL is twice that of legacy requirements with 2Rx.

    - SSB based: the AL and BW are twice that of legacy requirements with 2Rx.

- Measurement accuracy relaxation

  - SS-RSRP, SS-RSRQ, SS-SINR, relax relative and absolute accuracy by 1dB

  - L1-RSRP, relax relative and absolute accuracy by 3dB.

Proposal 3: The TN eDRX requirement relaxations and modifications can be used as a starting point for NTN eDRX requirements in IDLE/INACTIVE mode.

Proposal 4: For NGSO, eDRX requirements are not applicable if DRX >= 2.56s.

Proposal 5: For NGSO, eDRX requirements are not applicable if the measurement cannot be completed within a single PTW.

Proposal 6: For GSO, RAN4 to discuss requirements based on stationary conditions for NR NTN Redcap.

# References

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

- R4-2414037, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT, Aug. 2024.

- R2-2407623, LS on common TA in a regenerative payload scenario, CMCC, Aug. 2024.