3GPP TSG-RAN WG4 Meeting # 112-bis	R4-2415960

Hefei City, CN, Oct 14 – Oct 18, 2024

Agenda Item: 5.17.2

Source: MediaTek Inc.

Title: Discussion on NTN-TN coexistence study for new NTN SBand

Document for: Discussion

Introduction

This document provides simulation results focusing on adjacent channel interference for coexistence between NTN and TN based on the NTN-TN coexistence study [1].

Simulation scenarios and assumptions

Simulation scenario

According to WF [2], RAN4 to evaluate the impact on the downlink performance of TN for new Band (n252). The aggressor is NTN UE in UL. The power class (PC) is PC3 (23 dBm). The Victim is TN DL from UE. We assume the coexistence scenarios shown in Table 1. This document presents the simulation observations relating to the coexistence of NTN with TN.

Table 1: Simulation scenarios

| Satellite | Environment | Aggressor | Victim |
| --- | --- | --- | --- |
| GEO | Rural Macro | NTN UL | TN DL |
| GEO | Urban Macro | NTN UL | TN DL |

The results presented are for urban/rural environments described in TR38.901, using both AAS and non-AAS antenna models in TR38.821.

We reused the simulation procedure and assumption refer to TR 38.863 [1] as below in details.

Agreement: Adopt n252 as the band number for the new NR NTN S-band. Agreement: Adopt 252 as the band number for the new IoT NTN S-band. TR 38.863 [1] 6.2.2 System parameters Table 6.2.2.4-1: Simulation assumptions of TN respectively based on NB-IoT and NR NB-IoT standalone; NR. Carrier frequency in GHz; 2; 2. Size of each nominal channel BW in MHz; 0.2; 20. Transmission bandwidth in MHz; 0.18; N/A. Environment; Urban macro Rural; Deployment scenario related, see Table 6.2.2.4-2.. Network layout; 19-sites [57 sectors] with wrap-around; 19-sites 57 sectors with wrap-around. Inter-site distance in meter; 500 for 2GHz band for UMA TBD For Rural; Deployment scenario related, see Table 6.2.2.4-2. System loading and activity; Full buffer 100%; See Table 6.2.1.1-1. Network location; TBD; See Table 6.2.1.1-1. DL subcarrier spacing; 15kHz; 15kHz. UL; See RP-152284[7]; OFDMA. DL power control; No; No. UL power control; TR 36.942[8] section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB; TR 36.942[8]. Frequency reuse; 1; 1. Number of scheduled UE per cell (DL); 1; 1. Number of scheduled UE per cell (UL); 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone; 3. UE antenna height in meter; 1.5; 1.5m. UE TX power in dBm; -40 to 23; -40 to 23. UE antenna gain in dBi; 0; 0. Building penetration loss; TR 45.820[9] Annex D.1; In pathloss model, TR 38.901[10]. Cell selection margin in dB; 3; 3. BS-MS min distance in meters; 35; 35. BS noise figure in dB; 5; 5. UE noise figure in dB; 9; 9. BS-UE path-loss model; TR 36.942[8] macro urban; TR 38.901[10]. Standard deviation of BS-UE log-normal shadow fading in dB; 10; Deployment scenario related, referring to TR 38.901[10]. Shadowing correlation; Inter-cell 0.5 Intra-cell 1; Inter-cell 0.5 Intra-cell 1. Link-level performance model; See Section 6.2.8 Throughtput-SINR mapping. UE distribution; Uniform. Evaluation metrics; SINR vs ACS (as victim); See Section 6.2.8 Throughtput or SNR loss criteria. Table 6.2.2.4-2: Deployment-related parameters of TN (2 GHz) Urban Macro; Rural Macro; Remarks. ISD in meters; 750; 7500; ITU-R Report M.2292[11]. BS Antenna height in meters; 25; 30. UE Outdoor/indoor; 100% Outdoor. UE height in meter; 1.5; 1.5; RP-200559 [12] 3GPP LS to ITU-R WP5D and ITU-R WP5D [IMT_Parameters] [13]. Table 6.2.2.1-2: Set-1 satellite parameters for co-existence study Satellite orbit; GEO; LEO-1200; LEO-600. Satellite altitude; 35786 km; 1200 km; 600 km. Payload characteristics for DL transmissions. Satellite EIRP density; 2GHz; 59 dBW/MHz; 40 dBW/MHz; 34 dBW/MHz. Satellite max TX power in dBm; BW (MHz); 5; 10; 15; 20; 5; 10; 15; 20; 5; 10; 15; 20. SCS 15kHz; 44.53; 47.71; 49.53; 50.81; 46.53; 49.71; 51.53; 52.81; 40.53; 43.71; 45.53; 46.81. SCS 30kHz; 43.98; 47.37; 49.36; 50.64; 45.98; 49.37; 51.36; 52.64; 39.98; 43.37; 45.36; 46.64. Satellite Tx max Gain; 51 dBi; 30 dBi; 30 dBi. Channel bandwidth; 5/10/15/20MHz; 5/10/15/20MHz; 5/10/15/20MHz. 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam; 0.4011 deg; 4.4127 deg; 4.4127 deg. ABS (Adjacent Beam Spacing) of adjacent beams from the central beam; 0.3474 deg; 3.8206 deg; 3.8206 deg. Satellite beam diameter; 250 km; 90 km; 50 km. Payload characteristics for UL transmissions. G/T; 2 GHz; 19 dB K-1; 1.1 dB K-1; 1.1 dB K-1. Satellite Rx max Gain; 51 dBi; 30 dBi; 30 dBi. - 6.2.6 Transmission power control model - 6.2.6.2 NTN UL TPC For the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted to align with UE UL power control parameters used in TR 38.821[6]. - 6.2.6.3 NTN DL TPC For downlink scenario, no power control scheme is applied. 6.3.1 Simulation procedure Adopt following simulation steps. - Step 1: Generate aggressor and victim networks. - NTN central beam is at satellite nadir, surrounded with 6 co-frequency beams. NTN FRFs higher than 1 need to be considered. Assume one NTN aggressor as default. - Deployment of TN network (19 cells with wraparound) refers to Table 6.2.1.1-1 - Step2: UE associations - TN UE are generated randomly inside the TN network, make sure enough TN UEs are associated to each TN sectors based on coupling loss. - Deployment of NTN UE refers to Table 6.2.1.1-1. - Step 3: Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS/SAN-UE. This is done for both victim and aggressor networks. - Step 4: Throughput is computed in the victim systems without considering ACI as below: ThputNO ACIbpshz=fSINRICI=fSN+IICI, where: IICI is the inter-cell interference. For TN-NTN SINR calculation, the satellite receiver off angle should be considered in the satellite receiver gain calculation when calculating SINR. Note that such angle is not considered in TR 38.821[6] section 6.1.3 equations. Thus those equations should be used for SINR calculation. - Step 5: Throughput is computed considering ACI as below: ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where: IACI is the adjacent channel interference. - Step 6: RF parameters are determined based on the degradation cause by ACI as below: LossACI=1-ThputACIThputSINGLE

Network layout model

According to [1], the system level simulation deployment is implemented as Network layout model as below.

TR 38.863 [1] 6.2.1 Network layout model Figure 6.2.1.1-1: Isolation distance for Case 1 As defined in Figure 6.2.1.1-1, isolation distance is the distance between the blue-dotted line which represents TN cell boarder and the red line. No UEs deployed in the isolation region is assumed to reduce the calculation complexity. No.; Combination; Aggressor; Victim; Which NTN cell/UE to observe?; Which TN/UE to observe?; Which TN cells in a TN to observe?. 5; TN with NTN satellite; NTN UL; TN DL; NTN cell: Nadir point NTN UE: NTN UEs dropped at the edge of TN clusters; TN clusters randomly placed in this NTN beam; All active TN clusters which has the NTN UE(s) at its edge.

In this deployment, we assumed there’re 3 TN UEs in each TN sector and total 9 NTN UEs in the system. To evaluate the impact from NTN UL to TN DL, we assume the isolation is starting from 1m to 1.5 km for scenario 5 shown in Figure 1. NTN UEs placed at the TN cluster edge; those are at the radius of isolation distance from the TN cluster edge.

Figure 1: Urban Macro for NR-NTN deployment for NTN UE and TN UE (isolation from 1m to 1500m)

Since there’s no impact observed from NTN UL to TN DL in simulation result for above deployment. We assumed another deployment to shrink the radius of NTN UEs from TN cluster edge. As shown in Figure 2, the isolation shrunk from 50m to 300m. With this kind of deployment, we could observe some NTN UEs are close to TN UEs.

Observation 1: For the deployment to shrink the radius of NTN UEs from TN cluster edge, the location of certain NTN UEs is close to TN UEs and within the TN coverage area.

Figure 2: Urban Macro for NR-NTN deployment for NTN UE and TN UE (isolation shrunk from 50m to 300m)

System parameters

We assume to reuse the agreement and parameters from NTN HPUE WF [3] except for the parameters related to higher power class than PC3. The parameters in system level simulation shown in Table 2.

NTN HPUE WF [3] Agreement - Consider 20MHz system bandwidth for NR-NTN and NR in co-existence study. Agreement - Consider 9 UEs for 15kHz, 18 and 36UEs for 3.75kHz single tone as starting point - FFS whether to down scope. Agreement - Baseline: Use NF as 9dB, and Tx power 26/29/31 dBm for PC2/1.5/1 NTN HPUE - FFS option of 7dB NF for PC2 and LEO600. (Daniel: NTN UE aggressor RX NF does not make sense, TN UE RX NF?, S5 only related) Agreement - To update the existing uplink power control parameters considering different power classes and bandwidth (Samsung, Qualcomm, Huawei) - Update Pmax to 26/29/31 for PC2/1.5/1; - Update Rmin to -66/-69/-71 assuming Pmin as -40; - Update BW to corresponding transmission BW. - Agree on the following detailed parameters where: - Pmax = 26dBm for PC2, 29 dBm for PC1.5, 31 dBm for PC1. - Rmin = -66 dB for PC2, -69 dB for PC1.5, -71 dB, assuming -40dBm minimum output power, - CLx-ile and γ are set as following: - CLx-ile = 10*log10(Pmax) – (SNRtarget + 10*log10(kTB) + NF ), - Where: - SNRtarget and BW For NR-NTN HPUE: - Option 1: 3 dB SNR target and 5MHz UL BW (One company proposed for LEO only) - Option 2: 15 dB SNR target and 2RB UL BW as baseline assumption - NF is the SAN noise figure, i.e. 4.3dB. - 10log10(kT) = -174dBm/Hz. - γ = 1 For uplink scenario.

Table 2: System level simulation parameters

| Scenarios | DL bandwidth | TN UL bandwidth | NR-NTN UL bandwidth | IoT-NTN UL bandwidth | RX NF | Power control |
| --- | --- | --- | --- | --- | --- | --- |
| Scenarios 5 | 20MHz | N/A | 0.36MHz (2RB) | 0.015MHz | 9 dB | Follow above agreement. (Option 2) |

Simulation results

The key performance metric as highlighted in TR 38.863 [1] is the throughput loss ratio, thus simulation results in this document will focus on this.

Throughput Loss Ratio vs ACIR

The throughput loss ratio of each scenario is determined across different ACIR values to determine the minimum required ACIR of the victim system to prevent impact from interference.

Table 3: ACIR requirements for NR-NTN (GEO, isolation: 1m~1500m)

|  | BS Antenna type | ACIR Requirement @ 5%-tile Throughput Loss Ratio |  |  |
| --- | --- | --- | --- | --- |
| Satellite |  | GEO |  |  |
| Aggressor |  | NR NTN UL |  |  |
| Victim |  | TN DL (TDD) |  |  |
| PC level |  | PC1 (31 dBm) | PC1.5 (29 dBm) | PC2 (26 dBm) |
| Urban | Non-AAS | 0 dB | 0 dB | 0 dB |
|  | AAS | 0 dB | 0 dB | 0 dB |

Table 4: ACIR requirements for NR-NTN (GEO, isolation shrunk: 50m~300m)

|  | BS Antenna type | ACIR Requirement @ 5%-tile Throughput Loss Ratio |  |  |
| --- | --- | --- | --- | --- |
| Satellite |  | GEO |  |  |
| Aggressor |  | NR NTN UL |  |  |
| Victim |  | TN DL (TDD) |  |  |
| PC level |  | PC1 (31 dBm) | PC1.5 (29 dBm) | PC2 (26 dBm) |
| Urban | Non-AAS | 0 dB | 0 dB | 0 dB |
|  | AAS | 0 dB | 0 dB | 0 dB |

Table 5: ACIR requirements for IoT-NTN (GEO, isolation: 1m~1500m)

|  | BS Antenna type | ACIR Requirement @ 5%-tile Throughput Loss Ratio |  |  |
| --- | --- | --- | --- | --- |
| Satellite |  | GEO |  |  |
| Aggressor |  | NR NTN UL |  |  |
| Victim |  | TN DL (TDD) |  |  |
| PC level |  | PC1 (31 dBm) | PC1.5 (29 dBm) | PC2 (26 dBm) |
| Urban | Non-AAS | 0 dB | 0 dB | 0 dB |

Table 6: ACIR requirements for IoT-NTN (GEO, isolation shrunk: 50m~300m)

|  | BS Antenna type | ACIR Requirement @ 5%-tile Throughput Loss Ratio |  |  |
| --- | --- | --- | --- | --- |
| Satellite |  | LEO 1200 |  |  |
| Aggressor |  | NR NTN UL |  |  |
| Victim |  | TN DL (TDD) |  |  |
| PC level |  | PC1 (31 dBm) | PC1.5 (29 dBm) | PC2 (26 dBm) |
| Urban | Non-AAS | 0 dB | 0 dB | 0 dB |

From the above simulation results, all the ACIR are zero. The ACIR requirement is determined based on the 5%-tile throughput loss which should be less than 5%.

Plots are given in Annex 1, with TN as the victim.

Observation 2: Based on the simulation results in Annex, for the TN DL throughput loss evaluation, the TN DL ICI level is dominant factor and is much higher than the NTN ACI level.

Based on the results in above tables, the following observations can be made:

- No tighten ACIR requirement since NTN UL interference to TN DL with a required ACIR close to 0dB.

Proposal 1: Based on the simulation results in above Tables:

- No tighten ACIR requirement since the evaluation of NTN UE UL interference to TN UE DL shows a required NTN ACIR of 0dB.

Proposal 2: Based on the system-level simulation results from table 2 to table 6, for NR-NTN and IoT-NTN, even considering NTN UEs within the TN coverage area, there is almost no NTN interference impact or tightening of NTN ACIR requirements from NTN UE UL to TN UE DL.

Annex – 1 Throughput Loss Ratio vs ACIR with TN as victim network

| Figure 3: TN DL Throughput loss ratio with nAAS @ GEO NR-NTN, isolation: 1m | Figure 4: TN DL Throughput loss ratio with nAAS @ GEO NR-NTN, isolation: 1500m |
| --- | --- |
| Figure 5: TN DL Throughput loss ratio with AAS @ GEO NR-NTN, isolation: 1m | Figure 6: TN DL Throughput loss ratio with AAS @ GEO NR-NTN, isolation: 1500m |
| Figure 7: TN DL Throughput loss ratio with nAAS @ GEO IoT-NTN, isolation: 1m | Figure 8: TN DL Throughput loss ratio with nAAS @ GEO IoT-NTN, isolation: 1500m |

| Figure 9: TN DL Throughput loss ratio with nAAS @ GEO NR-NTN, isolation shrunk: 50m. | Figure 10: TN DL Throughput loss ratio with AAS @ GEO NR-NTN, isolation shrunk: 50m. |
| --- | --- |
| Figure 11: TN DL Throughput loss ratio with nAAS @ GEO NR-NTN, isolation shrunk: 300m. Figure 12: TN DL signal/interference with nAAS @ GEO NR-NTN, isolation shrunk: 300m. | Figure 13: TN DL Throughput loss ratio with AAS @ GEO NR-NTN, isolation shrunk: 300m. Figure 14: TN DL signal/interference with AAS @ GEO NR-NTN, isolation shrunk: 300m. |

| Figure 15: TN DL signal/interference with nAAS @ GEO NR-NTN, isolation: 1m. (Urban) | Figure 16: TN DL signal/interference with nAAS @ GEO NR-NTN, isolation: 1m. (Rural) |
| --- | --- |
| Figure 17: TN DL Throughput loss ratio with nAAS @ GEO IoT-NTN, isolation: 1m. | Figure 18: TN DL Throughput loss ratio with nAAS @ GEO IoT-NTN, isolation: 1500m. |
| Figure 19: TN DL Throughput loss ratio with nAAS @ GEO IoT-NTN, isolation shrunk: 50m. | Figure 20: TN DL Throughput loss ratio with nAAS @ GEO IoT-NTN, isolation shrunk: 300m. |

Reference

[1] 3GPP TR 38.863 Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

[2] R4-2414287, “WF on Rel-19 FR1 NTN bands”, Inmarsat, RAN4#112

[3] R4_2414452, “WF on UE RF requirements for NTN HPUE”, Samsung, RAN4#112