3GPP TSG-RAN WG4 Meeting #112bis	R4-2415964

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	On General Aspects of AI/ML for NR Air Interface

Agenda item:	6.17.1

Document for:	Discussion

Introduction

One of the objectives of Rel-19 part of RAN4 study on AIML in the air interface is to formulate and finalize a general testing framework for one-sided and two-sided AIML models, considering such aspects as [1]

- Generalization

- Non-static scenarios/conditions

- Post-deployment validation

At the previous meeting the definition of the testing goal was updated. Correspondingly, in this paper, we are analyzing what could be the impacts of functionality-based testing in RAN4 on the previously discussed aspects listed above.

Discussion

Testing goal, generalization and non-stationary conditions

The TR 38.843 [2], listed the following options for the testing goals in RAN4:

For testing goals, Option 1 and/or Option 2 below will be selected depending on the test - Option 1: The testing goal is to verify whether a specific AI/ML model (if model identification is possible)/functionality can be conducted in a proper way. - FFS how to define the specific AI/ML model (e.g., a model captured in RAN4 spec as baseline) - FFS how to define that the model is properly conducted (e.g., by defining AI/ML dedicated performance/core requirements associated with model outputs) - Option 2: The testing goal is to verify whether the minimum performance gain of AI/ML model (if model identification is possible) /functionality/feature can be achieved for a static scenario/configuration. - FFS how to define a static scenario/configuration (e.g., by defining a related testing dataset based on channel models in TR 38.901) - FFS whether and how to define non-static specific scenarios/configurations

At the previous RAN#112 meeting, it was agreed to introduce the new definition of the testing goals that is defined in the feature/functionality level [3]:

Issue 1-3: Testing goals Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

Firstly, we would like to confirm our understanding that the new testing goal above is substituting the former Option 1 and Option 2 listed in Rel-18 TR 38.843:

RAN4 to confirm that the new agreement on functionality-based testing goal is substituting Option 1 and Option 2 testing goals considered in Rel-18 and listed in the TR 38.843.

Next, we are going to analyze, how this definition can be mapped to the current understanding of AI/ML functionality applicability procedure discussed in RAN2 and how to address the RAN4 FFS on the number of test configurations.

At RAN2#127, RAN2 has achieved agreements about understanding of functionality LCM signalling framework [4], also reflected in Figure 1:

Agreements on definitions: - Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) - Applicable functionalities refers to functionalities that the UE is ready to apply for inference - Activated functionalities refers to functionalities already enabled for performing inference Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig. - Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. - FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: - Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI - As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: - Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). - If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

Figure 1: A flow chart of AI/ML functionality configuration and activation from RAN2.

Additionally, at the previous RAN2 meetings it was agreed that supported functionalities (may or may not have available models) are reported in UE capability (RAN2#125bis) and that, for BM use case, as a baseline the UE determines whether a functionality is applicable (RAN2#126) [5].

Furthermore, RAN1#118 has confirmed the following working assumptions [6]

Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Based on the agreements in the other WGs, we can come up with the following possible illustration of interplay in between AI/ML-based Feature, Functionality, Associated ID and applicability of the functionality shown in Figure 2. The models are shown but just for illustration, because as agreed above we stay on the level of functionality in RAN4, even though the functionalities can be provisioned by one or several AI/ML models, and this is left up to UE implementation.

Figure 2: Diagram demonstrating relation in between AI/ML-enabled, Feature, Functionalities, Associated IDs and models.

The point of the diagram in Figure 2 is to understand how the requirement/test parameters/configurations will need to be defined in RAN4 so that the functionalities could be (adequately) tested. Following the diagram, the parameters (i.e., the configurations of the requirements than need to be specified) can be split at least into the following groups:

- Parameters that are defined by the functionality configuration (i.e., UE capability)

- Parameters that are defined by the Associated IDs (NW side additional conditions, i.e., NW-related parameters) that help the UE to determine/check the functionality applicability and enable consistency between training and inference conditions for the UE-side ML functionalities.

- Other/additional parameters that are not defined neither by the functionality nor by the Associated ID (e.g., propagation conditions/channel model, UE-side configurations/additional conditions, etc.).

Correspondingly, it can be expected that every specified AI/ML-enabled functionality (with configurations (1) specific to it) shall be tested, i.e., at least one requirement/test per functionality shall be defined. However, the total number of requirements per functionality will be dependent on whether an explicit list of Associated ID will be specified or not. If yes, then it will be necessary to define requirements per each of those because which of the Associated IDs are applicable to a UE AI/Ml-enabled functionality will be UE-specific (UE-side training specific). For example, some UEs might support best beam prediction for certain BS beam codebooks, but others - not. However, more realistic scenario could be that Associated IDs will be vendor specific. Then, the explicit list of Associated IDs will not be specified, and the definition of requirements becomes more challenging because potentially it could be UEs without any applicable Associated IDs leaving the functionality without test. This should not be the case. Therefore, at least a minimum set of representative Associated ID(s), considered as mandatory/default should, be applicable and testable for the functionality.

The way how requirements in RAN4 will be defined will depend on the way how Associated IDs will be standardized, e.g., whether any corroding parameters related to Associated ID will be specified or will they be fully vendor specific.

Each specified AI/ML-enabled functionality shall have RAN4 requirements and pass at least one test for device conformance for this functionality.

A minimum set of Associated IDs should be defined as mandatory (mandatorily applicable) for testing of each functionality.

The change of the Associated ID (Figure 1) may happen together with RRC reconfiguration because UE will need to check the applicability/compatibility of the Associated ID to tits AI/ML-enabled functionality. Thus, the UE should have sufficient time to adapt to the new conditions, e.g., to switch the underlining models if needed or to report that such configuration is not applicable at all. Therefore, we do not see a need to define requirements that will cover a possibly of change Associate ID during the test.

A considerable change of conditions that require the change of Associated ID may require RRC reconfiguration.

RAN4 does not need to define performance requirements that imply the change of Associated ID during the test.

However, we still have a group of parameters/configurations (e.g., marked as (3) in the list above) that are not explicitly defined neither by the feature/functionality nor by the Associated ID. For example, the capability of the UE to maintain the minimal level of performance in different radio propagation conditions is the core question of generalization and should be ensured by the sufficient test/requirement coverage. We envision two open issues related to such parameters:

- What should be the number of requirements per given UE AI/ML-enabled functionality and Associated ID?

  - This will depend on the configurations/parameters that will be defined by the feature and Associated IDs themselves. For example, whether the deployment, UE configurations or channel model will be specified by those or not.

  - On top of that, it will be necessary to study how the performance of the AI/ML feature is sensible to variations in those test parameters, i.e., how well the functionality generalizes to changes in those.

- Are there any potential degradations in the performance when the change of test parameters/configurations happens dynamically, i.e., in non-static way?

  - We cannot expect that RRC reconfiguration happens when additional conditions/parameters/configurations (e.g. channel conditions) change. UE should use its internal mechanisms/implementation to ensure good performance of the functionality, e.g., by changing to the proper underlying model within functionality to match the conditions.

Regarding the first issue above, we think that more clarity from other WGs and a study of generalization UE capabilities to different scenarios will be needed in RAN4. It will still take some time.

The number requirements per given UE AI/ML-enabled functionality and Associated ID will depend on their concrete definition in the other WGs and the generalization capabilities of the UEs that require further study in per use-case manner.

RAN4 will need to wait for more progress in specification of concrete AIML-enabled features and Associated IDs and study UE generalization capabilities to conclude about the number of requirements/tests per given functionality and Associated ID.

Regarding the second issue above, when additional conditions might change without the change of the functionality and Associated ID, it brings us to the question of non-static test conditions. At the previous RAN4 meetings, we have already discussed such tests and definition of non-static conditions [7]:

Agreement: - Both static and non-static scenarios/configurations could be needed for AI testing - RAN4 will further discuss how to use them case by case - FFS whether to use static scenarios/configurations as baseline. - Refine the definitions of static and non-static scenarios/configurations based on two bullets below - Static: channel model and SNR settings are fixed and do not change over the test, specific channel realizations may be dynamic - Non-static: Non-static scenarios/configuration can be further considered in application to use cases. The details of models are FFS and may include non-stationary SNR and other conditions.

Even though at RAN4#111, the Chairman recommended to continue the discussion of non-static conditions in use-case specific manner

Static and non-static conditions Chair: It is not expected that companies continue discussing this topic under the general agenda for AI/ML.

We still believe that it is necessary to emphasize the difference in the discussion of generalization aspects and non-static conditions.

Non-static radio scenarios/conditions for a given UE AI/ML-enabled functionality and Associated ID should be discussed separately from the study of AI/ML-enabled functionality generalization to different but static conditions (legacy way of testing).

Post-Deployment Aspects

Post-deployment testing also depends on the definition of the testing goal above that was updated at RAN4#112. If updates/changes to AIML-enabled functionality can be identified and/or reported by the UE then, this could trigger a need to verify that the device with such updates can still provide a minimum level of expected performance according to RAN4 requirements.

The ongoing discussion of post-deployment testing already includes several options to handle the issue of using untested functionalities in the field. Several options to treat post-deployment testing/verification were agreed at RAN#110 [8]:

Agreement: - To ensure the AI performance after device deployment, discuss the following options further - Option 1: Conduct the conformance testing for AI model/functionality before deployment - FFS on the feasibility - Option 2: Design the test to verify the performance monitoring - Depend on the other WG progress - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable - Other options are not precluded

These options were further discussed at RAN4#112, however, only tentative agreement is captured in the chairman notes:

Tentative agreement: - Option 1: Conduct the conformance testing for new/updated AI model/functionality before its deployment in already deployed UEs (considering the UE hardware that it will be used in) - FFS on the feasibility - Option 2: Design the test to verify the performance monitoring and proactive recovery from potential performance degradation - Depend on the other WG progress - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable - Option 3: Capture model input during conformance testing for later testing of new models. - Other options are not precluded

Following the tentative agreement above and the greed definition of the functionality-based testing goal, Option 1 needs to be updated:

Option 1 of post-deployment testing shall be applicable only to changes in UE AI/ML-enabled functionality following the agreed testing goal definition, i.e., Conduct the testing for changed AI/ML functionality before its deployment in already deployed UEs (considering the UE hardware that it will be used in).

Potentially, multiple models can be associated with a functionality. During pre-deployment phase, this functionality will be validated only with the models available at the time of the testing. New models may be added to the functionality in the future, i.e., after device deployment to the field. In this case, the functionality will be updated, and it may have different performance in comparison to the original functionality that was earlier validated during the conformance testing. On the other hand, if the change in the functionality or in its applicability happens due to some other reasons different from the change of the underlining models, post-deployment testing might not be needed at all. Alternatively, if the changed functionality was already tested by the vendor, there is not no need in further testing and monitoring like in Option 2 will be sufficient for verification. Therefore, any update in the functionality impacting its performance should be coordinated with the network and then network can decide about starting a post-deployment verification procedure. If the reason of the change of the functionality is known, post-deployment testing can be carried out only on a need basis.

A need for post-deployment testing will depend on the reason/type of change in the functionality or in its applicability or whether the change has been already tested or not (e.g., offline).

In Option 2, RAN4 to consider a cause/reason of the change in AIML functionality when deciding about a need for post-deployment testing.

For example, Option 1 from former discussions assumes that UE vendors could indicate that changed AI/ML-enabled functionality has passed RAN4 test cases in a similar way as the original version of the functionality. The device with changed functionality may be tested again in same testing setup as before it was allowed to be used in the field. Such approach might be feasible for any offline updates of the functionality, e.g., when there is a major update, and a new version is distributed to the devices in a centralized manner. On the other hand, if in the future 3GPP releases, more dynamic mechanism may be considered when the functionalities are updated in the device itself, then the approach with offline testing of such updates becomes practically infeasible. In our understanding, online training is not in the scope of the Rel-19 WI, hence, Option 1 is possible, but only for infrequent centralized updates.

Therefore, some of the main challenges with Option 1 are:

- Frequency of the updates to the AI/ML functionalities: if companies need rather frequent functionality updates that needs to be tested. This will increase the test times proportionally.

- High turnaround time for the conformance tests: as the tests are done per device model/type, execution of all the test for the given updated functionality might take significant amount of time.

- Pre-deployment testing of changed AI/ML-enabled functionality before it is allowed in the field (Post-deployment Option 1: conformance testing before deployment of the functionality) can cause a significant overhead, especially for frequent or online updates. Moreover, a standardized mechanism for such post-deployment testing will need some support from the other RAN WGs (e.g., with RAN2), e.g., to allow/indicate the use of only tested functionalities in the live network.

Option 1 of post-deployment discussion can be feasible only for the major implementation updates of the UE AI/ML-enabled UE functionalities.

In Option 2, updated functionality is tested/validated as a part of functionality performance monitoring mechanism (working assumption in RAN WG#1 and WG#2), which is a more flexible and dynamic solution. However, performance monitoring of active functionality is a reactive procedure: the performance issues can be identified only after the functionality is already in use in the live network. Therefore, it should also be ensured that the UE always has at least one conformance test validated AI/ML-enabled functionality as backup.

Option 2 based on monitoring does not completely address the issue when changed UE functionality is already used in the live network without testing and could demonstrate performance below minimal requirements.

If the malfunction of changed/updated UE AI/ML-enabled functionality is identified during performance monitoring (e.g., due to the UE specific update in the underlaying models, processing blocks, etc.), then the fallback/switch should be allowed only to the functionality or legacy (non-AI/ML) feature that has passed conformance testing. In particular, this would mean that at least one UE functionality (for the same AI/ML-enabled Feature) that has passed conformance testing should be always available at the device. The conformance tested UE functionality may be either an AI/ML based functionality or a non-AI/ML/legacy functionality as well. This conformance tested functionality will be required for all mandatory features.

For example, if there are multiple functionalities defined for beam prediction feature, e.g., as shown in Figure 2, it can be expected that at least one of the functionalities (e.g., single best beam prediction) will need to pass RAN4/5 conformance testing before it can be used in the device.

RAN4 to ensure that at least one functionality (for an AI/ML-enabled Feature) that has passed conformance testing shall be present in the device.

Another alternative solution to the issues identified in Option 2 is to use a proactive verification/monitoring in the field, i.e., to verify updated/new UE AI/MÆ-enabled functionality before it can be used. To achieve that, the UE device needs to support inference for several functionalities or versions of functionalities available in the device. In this case one functionality could be used for inference and the other(s) may be in an inactive or “standby” mode (i.e. not use to generate reports/feedback to the NW). These two functionalities might also require different configurations:

RAN4 to consider handling post-deployment verification/testing of updated/new UE AI/ML functionalities in inactive/standby state/mode in parallel with active functionalities.

As it can be seen from the discussion above, handling of post-deployment issue is hardly possible without involvement and clarifications from the other WGs. Therefore, we see it important to share potential issues of post-deployment conformance and testing of AI/ML enabled functionalities with RAN1/2 so they could be aware about and consider those in the discussions and designs. At least the following aspects should be highlighted:

- RAN4 has a concern that modified/updated/new AI/ML-enabled functionalities can be used in the field without passing conformance testing explicitly.

- The value of conformance testing might be compromised if the AI/ML enabled functionalities (or functionality configurations) that passed RAN4 conformance testing are not actually used or available in the devices in the real networks (i.e., after deployment).

- RAN4 is discussing the issues above (with the list of potential options) but they require consideration of the other WGs.

RAN4 to send an LS to RAN2 and RAN1 WGs on a need to consider conformance testability of AI/ML enabled functionalities when designing such LCM operations as monitoring/activation/update/fallback for the devices already deployed in the field.

Conclusion

In this contribution we are elaborating on the open issues of RAN4 conformance testing framework. Specifically, we discuss how AIML functionalities will be tested against different conditions and how many of such tests will be needed. Additionally, we share our further views on post-deployment testing.

The following Observations and Proposals were made:

On functionality-based conformance testing:

Proposal 1: RAN4 to confirm that the new agreement on functionality-based testing goal is substituting Option 1 and Option 2 testing goals considered in Rel-18 and listed in the TR 38.843.

Observation 1: The way how requirements in RAN4 will be defined will depend on the way how Associated IDs will be standardized, e.g., whether any corroding parameters related to Associated ID will be specified or will they be fully vendor specific.

Proposal 2: Each specified AI/ML-enabled functionality shall have RAN4 requirements and pass at least one test for device conformance for this functionality.

Proposal 3: A minimum set of Associated IDs should be defined as mandatory (mandatorily applicable) for testing of each functionality.

Observation 2: A considerable change of conditions that require the change of Associated ID may require RRC reconfiguration.

Proposal 4: RAN4 does not need to define performance requirements that imply the change of Associated ID during the test.

Observation 3: The number requirements per given UE AI/ML-enabled functionality and Associated ID will depend on their concrete definition in the other WGs and the generalization capabilities of the UEs that require further study in per use-case manner.

Proposal 5: RAN4 will need to wait for more progress in specification of concrete AIML-enabled features and Associated IDs and study UE generalization capabilities to conclude about the number of requirements/tests per given functionality and Associated ID.

Proposal 6: Non-static radio scenarios/conditions for a given UE AI/ML-enabled functionality and Associated ID should be discussed separately from the study of AI/ML-enabled functionality generalization to different but static conditions (legacy way of testing).

On post-deployment testing:

Proposal 7: Option 1 of post-deployment testing shall be applicable only to changes in UE AI/ML-enabled functionality following the agreed testing goal definition, i.e., Conduct the testing for changed AI/ML functionality before its deployment in already deployed UEs (considering the UE hardware that it will be used in).

Observation 4: A need for post-deployment testing will depend on the reason/type of change in the functionality or in its applicability or whether the change has been already tested or not (e.g., offline).

Proposal 8: In Option 2, RAN4 to consider a cause/reason of the change in AIML functionality when deciding about a need for post-deployment testing.

Observation 5: Option 1 of post-deployment discussion can be feasible only for the major implementation updates of the UE AI/ML-enabled UE functionalities.

Observation 6: Option 2 based on monitoring does not completely address the issue when changed UE functionality is already used in the live network without testing and could demonstrate performance below minimal requirements.

Proposal 9: RAN4 to ensure that at least one functionality (for an AI/ML-enabled Feature) that has passed conformance testing shall be present in the device.

Proposal 10: RAN4 to consider handling post-deployment verification/testing of updated/new UE AI/ML functionalities in inactive/standby state/mode in parallel with active functionalities.

Proposal 11: RAN4 to send an LS to RAN2 and RAN1 WGs on a need to consider conformance testability of AI/ML enabled functionalities when designing such LCM operations as monitoring/activation/update/fallback for the devices already deployed in the field.

- References

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#105, Melbourne, Australia, September 9-12, 2024.

- TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface, Rel-18, ver. 18.0.0.

- R4-2414323, WF on AI/ML, Qualcomm Incorporated, RAN4#112, Maastricht, Netherlands, 19th – 23rd August, 2024.

- R2-xxxxxx, RAN2#127 Meeting Report, RAN2 Chair (InterDigital), RAN2#127-bis, Hefei, China, Oct 14 – Oct 18, 2024.

- R2-2406381, Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2, Intel Corporation, RAN2#127, Maastricht, Netherlands, 19th – 23rd August, 2024.

- R1-2407305, Summary #4 for other aspects of AI/ML model and data, Moderator (OPPO), RAN1#118, Maastricht, Netherlands, 19th – 23rd August 2024.

- R4-2406617, WF on AI/ML, Qualcomm, RAN4#110-bis, China, Changsha, April 15th – 19th, 2024.

- R4-2403712, WF on AI/ML, Qualcomm, RAN4#110, Athens, Greece, February 26th – March 1st, 2024