3GPP TSG-RAN WG4 Meeting #112bis	R4-2415967

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	On AI/ML Based CSI Prediction

Agenda item:	6.17.3.1

Document for:	Discussion

Introduction

At RAN1#118 meeting the following conclusion was reached on the study of AI/ML-based CSI prediction:

Agreement From RAN1 perspective, study of CSI prediction has been completed and performance improvement is observed with increased complexity.

Correspondingly, the WID [1] has been updated to reflect the fact that CSI prediction use-case is included in Rel-19 for the normative work:

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

Even though the normative work in RAN1 is planned to start from Q1 of 2025, in this paper we share our view on testability issues and the ways to specify demodulation and LCM requirements related to AI/ML-based CSI prediction use-case.

Discussion

On eType2Doppler-r18 CSI/PMI reporting

There is clearly a certain level of similarity between the AI/ML-based CSI prediction and the eTypeII Doppler codebook introduced in Rel-18 NR_MIMO_evo_DL_UL WI [RP-233028]:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows: - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis - UE reporting of time-domain channel properties measured via CSI-RS for tracking

Correspondingly, the eType2Doppler-r18 UE capability was introduced in Rel-18, TS 38.306:

codebookParametersetype2DopplerCSI-r18 Indicates the UE support of additional codebooks and the corresponding parameters supported by the UE of Enhanced Type II Codebook (eType-II) based on doppler CSI as specified in TS 38.214 [12]. The UE shall include eType2Doppler-r18 to indicate basic features of eType-II doppler codebook. This capability signalling comprises the following parameters: - supportedCSI-RS-ResourceList-r18 indicates the list of supported CSI-RS resources in a band by referring to codebookVariantsList. The following parameters are included in codebookVariantsList: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource of a band - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs in a band, simultaneously - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs in a band, simultaneously - valueY-P-SP-CSI-RS-r18 indicates value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR - valueY-A-CSI-RS-r18 indicates value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR - scalingfactor-r18 indicates scaling factor for active resource counting Kp The UE indicating eType2Doppler-r18 shall support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI='1-1'), support eType-II regular codebook refinement for predicted PMI with PMI subband R=1 3, support parameter combinations with L=2,4, support for rank = 1,2, and support for the size of DD-basis, N4=1. The UE indicating support of eType2Doppler-r18 shall also indicate support of csi-ReportFramework and simultaneousCSI-ReportsAllCC. NOTE 1: When N4=1, OCPU =4. NOTE 2: OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR. NOTE 3: when K=12, OCPU =8 NOTE 4: A UE that supports CSI enhancement for Rel. 16 based type-II doppler must support this feature. The UE optionally includes eType2DopplerN4-r18 to indicate whether the UE supports doppler measurement with N4>1 for eType-II doppler codebook. This capability signalling comprises the following parameters: - supportedCSI-RS-ReportSettingList1-r18 indicates the list of supported combinations across all CCs simultaneously by referring to supportedCSI-RS-ReportSettingList The following parameters are included in supportedCSI-RS-ReportSettingList-r18 - maxN4-r18 indicates the max number of N4 - maxNumberTxPortsPerResource-r18 indicates the maximum number of Tx ports in a resource of a band - maxNumberResourcesPerBand-r18 indicates the maximum number of resources across all CCs in a band, simultaneously - totalNumberTxPortsPerBand-r18 indicates the total number of Tx ports across all CCs in a band, simultaneously - supportedCSI-RS-ReportSettingList2-r18 indicates the list of supported combinations for one CSI report setting by referring to supportedCSI-RS-ReportSettingList-r18. The UE indicating support of eType2DopplerN4-r18 shall also indicate support for the size of DD-basis, N4>1, and Value of d=m for the DD unit size when A-CSI-RS is configured for CMR. The UE optionally includes ddUnitSize-A-CSI-RS-CMR-r18 to indicate the support of value of d=1 for the DD unit size when A-CSI-RS is configured for CMR. A UE supporting this feature shall also indicate support of eType2DopplerN4-r18. The UE optionally includes maxNumberAperiodicCSI-RS-Resource-r18 to indicate the maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for eType-II doppler measurement. The UE optionally includes eType2DopplerR2-r18 to indicate whether the UE supports R=2 for eType-II doppler codebook. This capability signalling comprises the list of supported CSI-RS resources in a band by referring to codebookVariantsList. The UE optionally includes eType2DopplerX1-r18 to indicate whether the UE support X=1 based on first and last slot of WCSI, for eType-II doppler codebook. The UE optionally includes eType2DopplerX2-r18 to indicate whether the UE support X=2 CQI based on 2 slots for eType-II doppler codebook. The UE optionally includes eType2DopplerL-N4D1-r18 to indicate whether the UE support l = (n – nCSI,ref ) for CSI reference slot for eType-II doppler codebook. The UE optionally includes eType2DopplerL6-r18 to indicate whether the UE support L=6 for eType-II doppler codebook. The UE optionally includes eType2DopplerR3R4-r18 to indicate whether the UE support rank equals 3 and 4 for eType-II doppler codebook. For codebookVariantsList-r16 related to the eType-II: - The minimum of maxNumberTxPortsPerResource is 'p4'; - The minimum of maxNumberResourcesPerBand is 2, except for eType2DopplerR2-r18. - The minimum value of totalNumberTxPortsPerBand is 4.

Importantly, the UE optionally includes eType2DopplerN4-r18 to indicate whether the UE supports doppler measurement with N4>1 for eType-II doppler codebook. This means that optionally the UE can predict CSI for several instances in the future.

On the NW/RRC configuration side defined in TS 38.331, TypeII-DopplerPortSelection-r18 of CodebookConfig IE, the following parameters are specified:

valueOfN-Doppler Field describes the size of the window of FD basis for Rel-18 Type II Doppler PS codebook. For M=2, N=2,4 (see TS 38.214 [19], clause 5.2.2.2.9) predictionDelay Prediction delay for Doppler and Doppler port selection codebooks see TS 38.214 [19], Clause 5.2.1.4. The first value m0 means that the first slot for which the CSI corresponds to is the slot where the CSI reference resource is located at. For the other three candidate values (n0, n1, n2), then the first slot for which the CSI corresponds to is given by l= n+delta, where delta can take on values of 0, 1, 2 and n the slot in which CSI is reported.

CSI prediction feature and corresponding etypeII-doppler-r18 codebook type was introduced in Rel-18 NR_MIMO_evo_DL_UL WI, including reporting of multiple predictions (eType2DopplerN4-r18 >1).

The corresponding RAN4 Rel-18 requirements are defined in TS 38.101-4, e.g., 6.3.3.1.8 Multiple PMI with 16Tx Enhanced Type II codebook for predicted PMI (FR1, FDD, 4RX). The requirements are defined for:

- Propagation channel: TDLA30-20

- Antenna configuration: XP Medium 16 x 4 (N1,N2) = (4,2)

- CSI-RS resource Type: Aperiodic

- reportQuantity: cri-RI-PMI-CQI

- Aperiodic Report Slot Offset: 10 slots

- Codebook configuration

  - Codebook Type: typeII-doppler-r18

  - Doppler/time-domain basis vector length (N4): 1

  - Doppler-/time domain unit duration (d): 1 slot

  - Prediction delay (delta): 1 slot

- CQI/RI/PMI delay: 15 ms

- PDSCH & PDSCH DMRS Precoding configuration for random Precoding: Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity

- Note 1:	When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. The random precoder generation shall follow 'typeI-SinglePanel' codebook configuration as specified in table 6.3.2.2.3-1.

- Note 2:	If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6).

Following the definition of the requirement metric in Clause 6.3:

The minimum performance requirements of PMI reporting are defined based on the precoding gain, expressed as the relative increase in throughput when the transmitter is configured according to the UE reported PMI compared to the case when the transmitter is using random precoding, respectively. When the transmitter uses random precoding, for each PDSCH allocation a precoder is randomly generated with equal probability of each applicable i1 and i2 combination and applied to the PDSCH. A fixed transport format (FRC) is configured for all requirements. … The requirements for transmission scheme 1 with higher layer parameter codebookType set to 'typeII' or 'typeII-r16' or 'typeII-doppler-r18' or 'typeII-CJT-r18' are specified in terms of the ratio: In the definition of γ, for 16TX PMI requirements, is 90 % of the maximum throughput obtained at using the precoders configured according to the UE reports, and is the throughput measured at with random precoding.

The minimum requirements on γ are [1.8] for FDD and [2.5] for TDD.

The RAN4 Demod requirements on Multiple PMI with 16Tx Enhanced Type II codebook for predicted PMI are defined for typeII-doppler-r18 codebook type, N4=1 and for TDLA30-20 channel conditions. The requirements are specified on (γ) - the throughput relative to the random precoding from typeI-SinglePanel codebook.

On Rel-19 AIML-based CSI reporting

Since time domain prediction reporting mechanism is already introduced in Rel-18, RAN1 at 116bis meeting agreed to assume R18 eType II doppler codebook for AI/ML-based prediction:

Agreement For Rel-19 study on CSI prediction, companies are encouraged to evaluate throughput performance by comparing performance with non-AI/ML based CSI prediction. - R18 eType II doppler codebook is assumed for CSI report for both AI/ML and Non AI/ML prediction. - Companies to report the assumption for N4, which could be 1, 2, 4, 8. Note: Non-AI/ML based CSI prediction (Benchmark 2) can include statistical model based CSI prediction (e.g., based on Kalman filter, Wiener filter, Auto-regression). Conclusion For Rel-19 study on CSI prediction only, consider UE-sided model only. Agreement - For the AI/ML based CSI prediction, for CSI report, adopt following as a baseline for evaluation purpose - N4 value: 1, 4 - Others can be additionally submitted, e.g., 2, 8 - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1) - Others can be additionally submitted. - Note: The same selected parameter combination shall be applied for benchmarks. - CSI report periodicity: 5ms, 20ms (encouraged) - Others can be additionally submitted, e.g., 10ms

RAN1 considered UE-side model only and assumed R18 eType II doppler codebook for CSI report for both AI/ML and Non AI/ML predictions.

Therefore, considering Observations 1-3, it will be logical to follow the same assumptions regarding CSI reporting format in RAN4:

RAN4 to assume R18 eType II Doppler codebook for CSI report for AI/ML prediction. Correspondingly, R16 eType II codebook is supported by the UE as well.

Regarding the assumptions on the number of predictions provided by the UE, the starting point can be the same as in RAN4 as well (Observation 3), i.e., for initial evaluation N4 can be assumed equal to 1.

RAN4 can start the discussion with a single AIML-based CSI prediction (N4=1).

Furthermore, RAN1 simulation assumptions are captured in TS 38.843, e.g.,

For the performance evaluation of the AI/ML based CSI feedback enhancement, system level simulation approach is adopted as baseline. Link level simulations are optionally adopted. Table 6.2.1-1: Baseline System Level Simulation assumptions for AI/ML based CSI feedback enhancement evaluations: Parameter; Value. Duplex, Waveform; FDD (TDD is not precluded), OFDM. Multiple access; OFDMA. Scenario; Dense Urban (Macro only) is a baseline. Other scenarios (e.g., UMi@4GHz 2GHz, Urban Macro) are not precluded.. Channel model; According to TR 38.901.

For Rel-18 Doppler codebook (non-AI/ML CSI prediction), the mode assumed during the RAN1 development was that the prediction filters are calculated based on observations of the channel (i.e., through measurements). That is not the assumption in the AI/ML case.

Since the goal in RAN4 is to test the models that are as close as possible to the models that will be used in practice, it will be reasonable to assume that for the derivation of the requirements RAN4 will also need to assume that the prediction model is trained in system scenario similar to what was done in RAN1.

RAN1 baseline scenario and model training uses UMa channel model according to TR 38.901.

RAN4 to assume that the training of AI/ML CSI prediction model is done based on system-level scenario (e.g., based on UMa channel model according to TR 38.901).

The issue could be that CDL-based channel models are not usually used for demodulation testing of CSI reporting. Typically, TDL channel conditions are defined in the demodulation tests. Therefore, RAN4 will need to agree and evaluate which channel model to be used in the test for AIML CSI prediction.

The problem is that it would not make much sense to use prediction in TDL environments. If TDL with low correlation is considered, the spatial dimensions of the channel evolve “randomly”, or at least non-predictably, because only speed-based Jakes max Doppler distribution is modeled, i.e., no real velocity-based/physical model-based channel evolution is present.
Alternatively, if TDL with medium or high correlation is used, the channel has a single spatial dimension (overlayed by small scale fading) that does not change during the test, i.e., the AIML model does not have much to learn and predict either.
Such behavior already causes issues for the Doppler compression codebook testing introduced in Rel-18

Due to unrealistic spatial behavior of TDL models, CSI prediction would have only trivial outcomes: spatial characteristics of the channel either demonstrate always the same behavior for medium or high correlation, or random for - low correlation.

RAN4 to consider Spatially Correlated Channel models (SCM) for the requirements/test on AIML CSI prediction.

On metric for RAN4 requirements

In RAN1 side, the following metrics for evaluation of AIML-based CSI prediction were considered:

- SGCS metric:

  - Benchmark#1: AIML CSI prediction is compared to the nearest historical CSI

  - Benhcmark#2: AIML CSI prediction is compared to non AI/ML based CSI prediction

- User throughput (UPT) based on predicted AIML CSI is compared to the UPT based on:

  - Benchmark#1: the nearest historical CSI

  - Benhcmark#2: non AI/ML based CSI prediction

RAN1#113: Observation For the AI/ML based CSI prediction, till the RAN1#113 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS … Observation For the AI/ML based CSI prediction, till the RAN1#113 meeting, in terms of mean UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach RAN#116: Agreement For Rel-19 study on CSI prediction, companies are encouraged to evaluate throughput performance by comparing performance with non-AI/ML based CSI prediction. - R18 eType II doppler codebook is assumed for CSI report for both AI/ML and Non AI/ML prediction. - Companies to report the assumption for N4, which could be 1, 2, 4, 8. Note: Non-AI/ML based CSI prediction (Benchmark 2) can include statistical model based CSI prediction (e.g., based on Kalman filter, Wiener filter, Auto-regression).

However, the definition of the monitoring metrics is still under discussion in RAN1. Therefore, it is unclear for now whether testing of the functionality can be based on monitoring mechanism (e.g., Type 2 when the ground truth reported by the UE would be used at the TE).

RAN1 has discussed two metrics (SGCS and user TPut) and two benchmarks (the nearest historical CSI and non-AI/ML based CSI prediction) for the evaluation of the gain of AIML -based CSI prediction. Additionally, performance monitoring, including a possibility of ground-truth reporting (Type 2), is under discussion but still lacking the details.

Regarding the SGCS metric, it was agreed at RAN4#112 [2] that only throughput-based requirement metric will be considered:

Issue 4-5: CSI prediction accuracy metric Agreement: Use relative throughput as requirement metric for CSI prediction - companies are invited to bring contribution on how the requirement could be defined: - what is the baseline that throughput would be compared to

Next, following the RAN4 WF, it is necessary to discuss to what throughput we shall compare the throughput achieved with AIML CSI feedback. In general, several options can be considered:

- Like in legacy RAN4 CSI reporting tests, throughput based on random precoding (i.e., randomized typeI-SinglePanel codebook)

  - In this case, the throughput to be compared will be derived directly based on the PMI indicated by the TE, i.e., UE-reported CSI is ignored.

  - This approach is feasible since it already used in the existing tests.

- Throughput to compare with can be also based on non-AIML CSI reported by the UE, i.e., derived directly from measurements without time-domain prediction.

  - If we follow RAN1 approach, the nearest historical CSI can be used for precoding PDSCH. Then the question is: Why we need the assumption that the CSI reported by the UE has to be nearest historical to the prediction?

    - Currently in RAN4, the stages of UPT evaluation based on tested CSI reporting mechanism and reference one are split in time. Therefore, technically it will not matter how close tested and reference CSIs to each other because they cannot be utilized in the same time. TE will just need to use lates CSI reported by the UE to precode PDSCH.

  - Since we assume above that eTypeII CSI reporting is supported together with AIML CSI prediction, one valid option would be to use eTypeII CSI codebook as a reference

  - One more alternative would be to use also Type1 signle-panel codebook reported by the UE as reference (i.e., not randomized). Hower, in this case it could be more straightforward use randomized feedback following the legacy test practice.

RAN4 does not need to use the metric from RAN1 that is based on closest historical CSI for the definition of the reference user throughput because just latest reported CSI can be used.

RAN4 can consider further two possible baselines to compare AIML-enabled predicted CSI:

  - Use throughput based on randomized typeI-SinglePanel codebook like in legacy test

  - Use throughput based on eTypeII codebook measured and reported by the UE (without prediction)

Non-static scenarios

The definition of the static scenarios was clarified already at RAN4#110bis, whereas the understanding of the non-static conditions requires further clarification [3]:

Agreement: - Both static and non-static scenarios/configurations could be needed for AI testing - RAN4 will further discuss how to use them case by case - FFS whether to use static scenarios/configurations as baseline. - Refine the definitions of static and non-static scenarios/configurations based on two bullets below - Static: channel model and SNR settings are fixed and do not change over the test, specific channel realizations may be dynamic - Non-static: Non-static scenarios/configuration can be further considered in application to use cases. The details of models are FFS and may include non-stationary SNR and other conditions.

In our view, non-static scenarios are aimed at testing the case when UE keeps using the same functionality but might still switch models internally without any explicit signaling to the NW. Such transitions should not impact the performance of the functionality significantly and this what needs to be justified by the test.

In case of CSI prediction, changing radio conditions (e.g., channel model from indoor to outdoor, or Doppler related to UE speed, or delay spread, etc.) may trigger UE internal switch from one AI/ML model to another. Such switching may interrupt inference for a small duration of time that should not impact the performance (such as (relative) PDSCH throughput) above the acceptable tolerance margin.

Non-static scenarios in RAN4 need to be considered to verify that there is no performance degradation while UE is autonomously switching in between different AI/ML models due to the change of radio conditions.

RAN4 needs to define a test when some of conditions (e.g., channel type, SINR, speed/Doppler, etc.) change during the test without the change of the functionality.

Generalization and test coverage

In the RAN4#110 meeting, the agreements for Generalization are noted as below.

Issue 1-1: Generalization update Agreement: - For AI/ML generalization [tests/requirements] - RAN4 should discuss it and decide the requirements/tests for each AI feature in the case-by-case manner

In the RAN1#116, RAN1#116bis the below agreements and conclusions were made on the verification of generalization performance of an AI/ML model

RAN1 #116 Agreement - For CSI prediction evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations, to evaluate one or more of the following aspects: - Various UE speeds (e.g., 10km/h, 30km/h, 60km/h, 120km/h) - Various deployment scenarios - Various carrier frequencies (e.g., 2GHz, 3.5GHz) - Various frequency granularity assumptions - Various antenna port numbers (e.g., 32 ports, 16 ports) RAN1 #116-bis Conclusion For model generalization results table, adopt Rel-18 Table 2 and Generalization Case 1 / 2 / 3 as starting point with same additions above. For generalization aspects, adopt the following - Various UE speed - UE distribution - Various CSI-RS periodicity

In RAN4#111, the concern regarding overfitting of UE models of a functionality to the test environment, was raised by some companies and it was captured in the agreement and companies were encouraged to bring proposals to avoid overfitting issue.  In our view, testing generalization aspects of a functionality provide a rather realistic and feasible solution to the overfitting issue. Since, testing a UE functionality in different scenarios and configuration would ensure that the functionality is/are not overfitted to a certain test setup/environment. A UE functionality, if tested in different generalization scenarios, would not be overfitted to a certain test environment. However, we think that this is an important topic to be discussed. And in this section, we discuss about the different aspects of testing the generalization and scalability of the AIML enabled functionality.

Below table summarizes the parameters that can be used to verify different generalization scenarios from the list of parameters that are agreed to be used for verification in RAN1.

Table 4: Parameters for Generalization Scenarios

| Parameters | Description |
| --- | --- |
| UE Speed | Slow / Medium / Fast |
| SINR (Deployment Scenario) | Good / Bad Radio conditions |
| Carrier Frequency | 2GHz / 3.5GHz |
| Propagation Model | Change of TDL/CDL model type, e.g., CDL-A, CDL-B, etc. |

RAN4 should discuss the parameters in Table 4 to define the generalization scenarios for CSI prediction use case.

After the identification of different scenarios, RAN4 should further study the following aspects.

- Minimum performance for identified scenarios.

Minimum performance for identified scenarios should be comparable to the minimum RAN4 performance requirements. The functionality should be able to achieve this as baseline performance.

- Tolerance margin for other scenarios.

Tolerance margin is the margin by which the performance of the functionality is degraded with respect to the performance of the identified scenarios. The tolerance margin can be defined based on the simulation results and can be adapted for in field performance of the functionality.

AI/ML functionalities have their limits on the conditions where they can be used. These limitations can be imposed by the training procedure or dataset, based on the definition of the UE capabilities, and importantly based on the generalization capabilities.

Less generalized functionalities across a set of scenarios can result in frequent switching of model/functionality resulting in performance degradation.

To mitigate the problems caused by less generalized functionalities, it is important to ensure that the functionality is generalized enough. Ensuring the generalization aspects of functionality across different scenarios and conditions is not straightforward. At the outset we need the scenarios and conditions need to be identified and aligned. KPIs for generalization need to be defined. And then the tests need to be carried out on all the scenarios and based on the validation result it can be decided if the functionality is generalized enough. To carry out all these identified steps, it becomes imperative to have a test framework to test the generalization aspects of a functionality.

In order to address this issue, we propose a new metric to indicate the generalization capabilities of an AI/ML functionality, that may be referred as “generalization score”. In case of CSI feedback enhancement use case, this may be used to verify/check whether the CSI feedback enhancement functionality (which may be developed for a specific scenario) generalize well to other scenario(s).

As an example, if it is identified that there are 3 scenarios of interest. Any one scenario of these 3 scenarios can be the baseline setup (depending on functionality implementation) and the remaining 2 scenarios will then be used as other set ups to test the generalization aspects. An overall generalization score can be calculated as a function of individual scenario verdicts, e.g., as simple average or weighted average of the individual generalization verdict of each Scenario ‘j’ (i.e., Scenario ‘1’ to Scenario ‘N), where the verdict of Scenario ‘j’ = 1 if the functionality generalizes to Scenario ‘j’ else it is equal to 0.

Overall Generalization Score= (VerdictScenario1+…+VerdictScenarioN)N×100%

RAN4 needs to design a new metric, indicative of generalization capabilities of AI/ML functionality, to verify it’s the generalization performance in different scenarios.

LCM requirements

One of the factors that influence the performance of the AIML enabled functionality is the latency of the LCM actions.

If performance monitoring detects a performance degradation to a point where a decision to either switch this functionality with another functionality is taken or a fallback decision is taken, it means that the AI/ML functionality is degrading the system performance and if this functionality, with detected performance degradation, keeps running then the impact on system performance may result in catastrophic consequences.

Therefore, it is crucial to stop this functionality, either by falling back to legacy method or by switching to another functionality, within a specified time. For a use case like AIML enabled CSI feedback, it would be very urgent to stop/switch functionality because a wrong channel information would lead to wrong link adaptation and scheduling decisions impacting the throughput.

For UE-assisted or NW-based performance monitoring, if required LCM action is not taken in a timely manner, the performance of AI/ML-based CSI feedback may be degraded to undesirable levels.

Use-case specific core requirements should be considered to limit latency of LCM actions (e.g. activation, deactivation, fallback, switching etc.) for AI/ML-enabled CSI prediction functionality.

LCM for localized models

RAN1#116 has the below agreements on testing of localized models as below.

Agreement For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, consider the following options as a starting point to model the spatial correlation in the dataset for a local region: - Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901. - E.g., Dropped in a specific cell or within a specific boundary. - Option 2: By using a scenario/configuration specific to the local region. - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

Now in case of localized models that are deployed in the field they can have impact on the performance of feature based on the characteristics of the localized model being used. So, it becomes imperative to monitor the localized models also as part of model LCM. An LCM action needs to be taken based on the localization of the models in a given region.

If localized models are considered for CSI prediction, RAN4 will need to study model ID-based LCM requirements for a given functionality.

Conclusion

In this contribution we analyze which assumptions RAN4 needs to make and with requirements/test will be needed for AIML-based CSI prediction agreed for normative work in Rel-19.

The following Observations and Proposals were made:

On eType2Doppler-r18 CSI/PMI reporting:

Observation 1: CSI prediction feature and corresponding etypeII-doppler-r18 codebook type was introduced in Rel-18 NR_MIMO_evo_DL_UL WI, including reporting of multiple predictions (eType2DopplerN4-r18 >1).

Observation 2: The RAN4 Demod requirements on Multiple PMI with 16Tx Enhanced Type II codebook for predicted PMI are defined for typeII-doppler-r18 codebook type, N4=1 and for TDLA30-20 channel conditions. The requirements are specified on (γ) - the throughput relative to the random precoding from typeI-SinglePanel codebook.

On Rel-19 AIML-based CSI reporting:

Observation 3: RAN1 considered UE-side model only and assumed R18 eType II doppler codebook for CSI report for both AI/ML and Non AI/ML predictions.

Proposal 1: RAN4 to assume R18 eType II Doppler codebook for CSI report for AI/ML prediction. Correspondingly, R16 eType II codebook is supported by the UE as well.

Proposal 2: RAN4 can start the discussion with a single AIML-based CSI prediction (N4=1).

Observation 4: RAN1 baseline scenario and model training uses UMa channel model according to TR 38.901.

Proposal 3: RAN4 to assume that the training of AI/ML CSI prediction model is done based on system-level scenario (e.g., based on UMa channel model according to TR 38.901).

Observation 5: Due to unrealistic spatial behavior of TDL models, CSI prediction would have only trivial outcomes: spatial characteristics of the channel either demonstrate always the same behavior for medium or high correlation, or random for - low correlation.

Proposal 4: RAN4 to consider Spatially Correlated Channel models (SCM) for the requirements/test on AIML CSI prediction.

On metric for RAN4 requirements:

Observation 6: RAN1 has discussed two metrics (SGCS and user TPut) and two benchmarks (the nearest historical CSI and non-AI/ML based CSI prediction) for the evaluation of the gain of AIML -based CSI prediction. Additionally, performance monitoring, including a possibility of ground-truth reporting (Type 2), is under discussion but still lacking the details.

Observation 7: RAN4 does not need to use the metric from RAN1 that is based on closest historical CSI for the definition of the reference user throughput because just latest reported CSI can be used.

Proposal 5: RAN4 can consider further two possible baselines to compare AIML-enabled predicted CSI:

- Use throughput based on randomized typeI-SinglePanel codebook like in legacy test

- Use throughput based on eTypeII codebook measured and reported by the UE (without prediction)

On Non-static scenarios:

Observation 8: Non-static scenarios in RAN4 need to be considered to verify that there is no performance degradation while UE is autonomously switching in between different AI/ML models due to the change of radio conditions.

Proposal 6: RAN4 needs to define a test when some of conditions (e.g., channel type, SINR, speed/Doppler, etc.) change during the test without the change of the functionality.

On Generalization, test coverage, and LCM:

Proposal 7: RAN4 should discuss the parameters in Table 4 to define the generalization scenarios for CSI prediction use case.

Observation 9: Less generalized functionalities across a set of scenarios can result in frequent switching of model/functionality resulting in performance degradation.

Proposal 8: RAN4 needs to design a new metric, indicative of generalization capabilities of AI/ML functionality, to verify it’s the generalization performance in different scenarios.

Observation 10: For UE-assisted or NW-based performance monitoring, if required LCM action is not taken in a timely manner, the performance of AI/ML-based CSI feedback may be degraded to undesirable levels.

Proposal 9: Use-case specific core requirements should be considered to limit latency of LCM actions (e.g. activation, deactivation, fallback, switching etc.) for AI/ML-enabled CSI prediction functionality.

Proposal 10: If localized models are considered for CSI prediction, RAN4 will need to study model ID-based LCM requirements for a given functionality.

- References

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#105, Melbourne, Australia, September 9-12, 2024.

- R4-2414323, WF on AI/ML, Qualcomm, RAN4#112, Maastricht, Netherlands, Aug 19 – Aug 23, 2024.

- R4-2406617, WF on AI/ML, Qualcomm, RAN4#110-bis, China, Changsha, April 15th – 19th, 2024.