3GPP TSG-RAN WG4 Meeting #112bis	R4-2415968

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia, OPPO, MediaTek

Title:	Work Plan for RAN4 Discussions on AI/ML for Mobility in NR

Agenda item:	6.18.1

Document for:	Discussion

Introduction

During the RAN4#112bis, the discussion on the AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR (FS_NR_AIML_Mob study item) will start in RAN4. The recent RAN#105 plenary has agreed the updated version of the SID with the following RAN4 objectives [1]:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

The time budget for the concerning WGs is presented in Table 1. 0.5 TU is uninterruptedly allocated for AI/ML for mobility agenda in RAN4 until August 2025.

Table 1: Time budget allocation for FS_NR_AIML_Mob feasibility study.

| Apr. 2024 | May 2024 | Aug. 2014 | Oct. 2024 | Nov. 2024 | Feb. 2025 | Apr. 2025 | May 2025 | Aug. 2025 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RAN2#125bis | #126 | #127 | #127bis | #128 | #129 | #129bis | #130 | #131 |
| 1.5 | 1.5 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
|  |  |  | RAN4#112bis | #113 | #114 | #114bis | #115 | #116 |
|  |  |  | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |

In this discussion paper we overview the main objectives of the study and propose the work plan for RAN4.

Discussion

Main objectives and RAN2 status

The FS_NR_AIML_Mob focuses on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made at the network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively.

In addition to the RAN4 objectives already presented in the Introduction section above, the following RAN2 objectives and use-cases are listed in the SID [1]:

The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2]

In the SID [1], TSG RAN identified the following use-cases to be considered that are further clarified during the RAN2 meetings and reflected in the TP to the TR based on the achieved agreements [3]:

- RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model)

At RAN2#125bis, the following sub-use cases were agreed for RRM measurement prediction:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- RLF/HOF prediction (UE sided model)

  - Both direct and indirect approaches for RLF prediction are being discussed in RAN2.

    - Direct RLF prediction: A method that utilizes AI/ML to directly predict if an RLF is occurring or the probability of it occurring in the future time window based on current and historical radio measurements and other potential inputs. The output of direct prediction is probability of RLF within an window.

    - Indirect RLF prediction: The output for indirect prediction is predicted SINR. Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models.

  - HOF prediction is down prioritized.

- Measurement events prediction (UE sided model).

  - Measurement event A3 is taken as starting point, other events are FFS.

  - At least indirect measurement event prediction will be studied. And direct measurement event prediction is also allowed.

The study of the use cases is driven mainly by two study goals [3]:

- The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements.
The following metrics are considered:

  - MRRS	Measurement reduction rate in spatial domain

  - MRRT	Measurement reduction rate in temporal domain

- The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

The use-cases are considered in the following dimensions:

- Frequency (intra- and inter-frequency predictions):

  - FR1 to FR1 intra-frequency

  - FR2 to FR2 intra-frequency

    - Both measurements (i.e., model inputs) and predictions belong to the same frequency.

  - FR1 to FR1 inter-frequency

    - Serving frequency measurements can be taken as an input/reference but the predictions are done for another frequency.

- Cell (intra- and inter-cell predictions):

  - Inter-cell case

    - It is considered that serving cell measurements can be taken as an input/reference, but the predictions are done for another cell.

      - Intra-frequency prediction based on inter-cell measurements were discussed in RAN2 but was deprioritized.

    - Cluster-based approach is also considered when measurement results from multiple cells (e.g., including serving and/or neighbouring cells) can be used as an input to the model.

      - Cluster-based approach evaluation is optional and lower priority for now.

  - Intra-cell covers only intra-frequency case

    - Both measurements (i.e., model inputs) and predictions belong to the same cell.

- Temporal domain predictions:

  - Case A: predicting measurement result(s) in prediction window based on actual measurement result(s) in observation window of the same cell for both FR1 and FR2 scenario without reducing any measurement in observation window. This case is evaluated for the 2nd study goal.

  - Case B: predicting a sub set of measurement instances in temporal domain of the same cell for both FR1 and FR2 scenario evaluated for 1st study goal.

- Spatial domain predictions:

  - Intra-frequency intra-cell spatial domain prediction is evaluated for the 1st study goal by measuring a sub set of configured SSB as input to the model to derive L3 filtered cell-level measurements for every time instance of the same cell

Correspondingly, the following scenarios for RRM measurement prediction use case were captured and prioritized in RAN2:

Table 2: Prioritization of evaluation scenarios in RAN2 (Table 5.2.1-1 in TR 38.744 draft) for measurement prediction use-case.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

The work in RAN2 starts with the discussion of the simulation assumptions and the RRM measurements use-case, as shown in Figure 1 from RAN2#126 [2]. The updated RAN2 work plan is submitted to the RAN2#127bis meeting [4].

At the first two RAN2 meetings, the focus was mostly on RRM measurement prediction use-case and common evaluation assumptions for it. At the RAN2#127 meeting, the discussion of the RLF use-case has also started. However, no major contribution on RLF is planned at RAN2#127bis and simulation results evaluation is expected to take place only in 2025. On the other hand, at RAN2#127bis, the measurement even prediction use-case will be discussed, as it assumed by the agenda below:

8.3.3 Measurement event predictions Contributions should focus on measurement event prediction use cases/scenarios to focus during the study, simulation assumptions and relevant performance metrics/KPIs to evaluate 8.3.4 RLF/HO failure prediction No contributions expected for this meeting Contributions should focus on discussing RLF specific methodology and simulation assumptions (addressing the differences or additional aspects from RRM predicution asssumptions). Relevant metrics and assumptions not covered by email discussion No evaluations/simulation results expected for this meeting

It was also agreed in RAN2 that HOF discussion is deprioritized. Additionally, mobility-related LCM and generalization discussions in RAN2 are planned for the second half of the study item.

RAN2 has started the study with the discussion of simulation assumptions and RRM measurement prediction use-case. RLF use-case was already considered but simulation result evaluation will continue only in 2025. HOF use-case is deprioritized. Evaluation of measurement event prediction use-case will be treated at the upcoming RAN2 meeting. LCM and generalization aspects and other scenarios for mobility are expected to be discussed at a later stage.

The work plan for the study item was discussed at RAN2#126 [2]. The updated plan of RAN2 work is submitted at the RAN2#127bis meeting [4]. However, the plan of RAN4 work was pending for the clarification of the SID at RAN#105 [1].

The work plan for the FS_NR_AIML_Mob does not consider RAN4 work yet.

RAN4 work plan

In RAN4, the study can be arranged in the following directions that are described below and reflected in the RAN4 work plan Table 3 and its illustration in Figure 3:

- RAN4 should start the study with RRM measurement prediction use-case and scenarios. Then, RLF prediction and Mobility even prediction use-cases should be considered taking RAN2 progress into account.

- It will be necessary to evaluate the use-cases from RAN4 perspective:

  - Discuss RAN4 metrics and ground truth and their feasibility/availability

  - Review RAN2 simulations assumptions and analyze whether RAN4 aspects are considered or what needs to be additionally considered in RAN4.

- RAN4 will need to discuss some new aspects of testability and testing setup, e.g.,

  - Testing in FR1

  - Testing inter-carrier scenario, i.e., inter-frequency inter-cell prediction

  - Testing L3-RSRP measurements instead of L1-RSRP for beam management considered in the AIML WI.

  - Potential presence of more than one cell in the testing setup (e.g., source and target cell for the HO)

    - Note that inter-cell cluster scenario has low priority in RAN2.

  - At the same time, overlap with NR_AIML_Mob WI testing setup discussions should be avoided.

- Following the SID, RAN4 needs to identify relevant/impacted RAN4 requirements and how they can be applied and/or modified and whether new requirements might be needed

  - However, the discussion of the requirement values and concrete formulations should be left to the WI phase.  The goal is to list the options.

- In the second half of the SI consider impacts of test coverage, generalization, monitoring, LCM, and other AI/ML specific aspects in application to considered use-cases.

Table 3: RAN4 work plan.

| Time slot | Meeting# | RAN4 work plan |
| --- | --- | --- |
| October | RAN4#112bis | - Discussion/agreement of the work plan, checking RAN4 TR 38.744 section. - Start RAN4 study of RRM measurement prediction use-case: - Agree on the scenarios to be considered in RAN4 - Discuss relevant core/performance requirements - Discus feasible requirement metrics - Start discussing Testability and Interoperability aspects for RRM measurement prediction use-case. |
| November | RAN4#113 | - Continue discussion of relevant RAN4 requirements for RRM measurement prediction use-case. - Continue the discussion of RAN4 evaluation methodology, metrics, ground truth. - Continue discussion of Testing setup/Testability and Interoperability aspects. |
| Feb. 2025 | RAN4#114 | - Continue discussion of relevant RAN4 RRM requirements per RAN2 use-cases - Initiate the discussion of Measurement event prediction use-case depending on the progress in RAN2. - Continue the discussion of RAN4 evaluation methodology, metrics, ground truth. - Continue discussion of Testing setup/Testability, considering Rel-19 WI progress. |
| April | RAN4#114bis | - Continue discussion of relevant RAN4 RRM requirements per RAN2 use-case and corresponding metrics - Initiate the discussion of RLF use-case depending on the progress in RAN2. - Continue discussion of Testability aspects - Continue the discussion of RAN4 evaluation methodology, metrics, ground truth. - Initiate the discussion of monitoring, generalization, scalability, etc. – related requirements based on the progress in RAN2 and Rel-19 WI on NR_AIML_Air - If possible, start the discussion of LCM-related issues based on the progress in RAN2 and Rel-19 WI on NR_AIML_Air |
| May | RAN4#115 | - Continue the discussion and evaluation of potential requirements per use-case. - Continue the discussion of testability issues - Initial draft of the TR/Draft CR(s) to the TR 38.744 - Continue or initiate the discussion of LCM-related issues and requirements |
| June | RAN#108 | Initial TR for information |
| August | RAN4#116 | - Conclude evaluation of testability, interoperability, and specifications impacts on RRM requirements and performance - Conclude identified LCM issues and their spec impact - Submit the final CR(s) to the TR with RAN4 conclusions |
| September | RAN#109 | - TR for approval - Close the SI |

Figure 3: Illustration of the study work plan in RAN4.

RAN4 to agree the RAN4 Work plan from the Tabel 3.

Conclusion

In this paper we overview the objectives and the RAN2 progress on the FS_NR_AIML_Mob. Based on this analysis the RAN4 work plan is proposed.

The following Observations and Proposals were made:

Observation 1: RAN2 has started the study with the discussion of simulation assumptions and RRM measurement prediction use-case. RLF use-case was already considered but simulation result evaluation will continue only in 2025. HOF use-case is deprioritized. Evaluation of measurement event prediction use-case will be treated at the upcoming RAN2 meeting. LCM and generalization aspects and other scenarios for mobility are expected to be discussed at a later stage.

Observation 2: The work plan for the FS_NR_AIML_Mob does not consider RAN4 work yet.

Proposal 1: RAN4 to agree the RAN4 Work plan from the Tabel 3.

Table 3: RAN4 work plan.

| Time slot | Meeting# | RAN4 work plan |
| --- | --- | --- |
| October | RAN4#112bis | - Discussion/agreement of the work plan, checking RAN4 TR 38.744 section. - Start RAN4 study of RRM measurement prediction use-case: - Agree on the scenarios to be considered in RAN4 - Discuss relevant core/performance requirements - Discus feasible requirement metrics - Start discussing Testability and Interoperability aspects for RRM measurement prediction use-case. |
| November | RAN4#113 | - Continue discussion of relevant RAN4 requirements for RRM measurement prediction use-case. - Continue the discussion of RAN4 evaluation methodology, metrics, ground truth. - Continue discussion of Testing setup/Testability and Interoperability aspects. |
| Feb. 2025 | RAN4#114 | - Continue discussion of relevant RAN4 RRM requirements per RAN2 use-cases - Initiate the discussion of Measurement event prediction use-case depending on the progress in RAN2. - Continue the discussion of RAN4 evaluation methodology, metrics, ground truth. - Continue discussion of Testing setup/Testability, considering Rel-19 WI progress. |
| April | RAN4#114bis | - Continue discussion of relevant RAN4 RRM requirements per RAN2 use-case and corresponding metrics - Initiate the discussion of RLF use-case depending on the progress in RAN2. - Continue discussion of Testability aspects - Continue the discussion of RAN4 evaluation methodology, metrics, ground truth. - Initiate the discussion of monitoring, generalization, scalability, etc. – related requirements based on the progress in RAN2 and Rel-19 WI on NR_AIML_Air - If possible, start the discussion of LCM-related issues based on the progress in RAN2 and Rel-19 WI on NR_AIML_Air |
| May | RAN4#115 | - Continue the discussion and evaluation of potential requirements per use-case. - Continue the discussion of testability issues - Initial draft of the TR/Draft CR(s) to the TR 38.744 - Continue or initiate the discussion of LCM-related issues and requirements |
| June | RAN#108 | Initial TR for information |
| August | RAN4#116 | - Conclude evaluation of testability, interoperability, and specifications impacts on RRM requirements and performance - Conclude identified LCM issues and their spec impact - Submit the final CR(s) to the TR with RAN4 conclusions |
| September | RAN#109 | - TR for approval - Close the SI |

- References

- RP-242393, Revised SID on AIML for mobility in NR, OPPO (rapporteur), RAN#105, Melbourne, Australia, September 9-12, 2024.

- R2-2404711, Discussion on work plan of AI mobility SI, OPPO, Nokia, MediaTek, RAN2#126, Fukuoka.

- R2-2406309, Text proposal of 38.744, OPPO, RAN2#127, Maastricht, Netherlands, Aug. 19 – Aug. 23, 2024.

- R2-24xxxxx, Discussion on work plan of AI mobility, RAN2#127bis, Hefei, China, Oct 14th – 18th, 2024.