TSG-RAN Working Group 4 (Radio) meeting #112-bis	R4-2415970

Hefei, China, 14th to 18th October 2024

Source:	Ericsson

Title:	TP to TR 38.922: Addition of new subclause for MIMO modelling aspects in subclause 7.3

Agenda item:	6.2.5

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#112 in Maastricht) technical information relevant for future IMT systems was extensively discussed. At this stage in the study information have been provided to ITU-R WP 5D for the frequency range 4400 to 4800 MHz in [1] and the frequency range 7125 to 8400 MHz in [2]. A third LS to ITU-R WP 5D with information relevant for 14800 to 15350 MHz and additional information is planned to be provided at the end of RAN4#113.

In the LS received by RAN4, ITU-R WP 5D [3] is asking for information on specific questions:

- The particular type of AAS implementation and associated model for IMT base stations, i.e., AAS composed of an array-of-sub-arrays or AAS composed of single-element arrays with its technical justification.

- If providing specific technical AAS implementation parameters for the array-of-sub-arrays case, WP 5D would welcome:

- The pre-set sub array tilt is understood as a parameter (a pre-tilt within the elements combined to form a sub array) that is used to focus the main beam below the boresight where a base station is intended to serve its UEs, similarly to the mechanical down-tilt. Noticing that the mechanical down-tilt of the BS antenna is environment (urban/suburban/rural) dependent, WP 5D welcomes further confirmation whether the preset sub-array down-tilt varies with the environment or not, what would be the range of values for this parameter and the associated environments and if multiple set of configurable preset sub-array down-tilt per BS is foreseen in the future.

- Information of advancements in MIMO architectures that could be applicable for future systems.

- Whether AAS at IMT user equipment is likely or not. If AAS at IMT user equipment is considered as a possibility WP 5D welcomes information relating to the structure of the AAS at the IMT user equipment, associated frequency bands, and the associated parameters of the AAS.

- IMT base station (inclusive of its AAS-related parameters) and IMT user equipment technical parameters for analysing adjacent band compatibility of terrestrial IMT systems with other co-primary services in the ITU Radio Regulations.

WP 5D also seeks information about how the AAS performs in adjacent bands i.e., does its performance drops down to that of a single element or still some array features remain. If the latter is true how far in frequency should one assume the continuity of array features even with some degradation?

- Considering the system-level simulation work typically conducted in WP 5D for sharing and compatibility studies using 3D modelling of antenna pattern, WP 5D is seeking information on whether for implementation of IMT BS multiuser spatial beamforming techniques, such as zero-forcing (ZF) or minimum mean-square error (MMSE) based schemes, would be necessary to improve the assessment of interference and the accuracy of studies. If considered necessary, guidance on the process of deriving the necessary beamforming weights for the IMT AAS BS to compute its radiation pattern is welcomed.

Answers to questions a, b and c will be provided per frequency range. In this contribution we will provide technical input relevant for developing answers to question e.

In addition to parameter assumptions ITU-R WP 5D have also requested information related to some additional topics. In this contribution we will provide information related to modelling of AAS BS beamforming including simulation results comparing different beamforming approaches.

This contribution is based on revised information presented in an earlier contribution [4]. Technical details related to simulation methodology is captured in Annexes at the end in section 5 and section 6 of this contribution.

Technical background information relevant for adjacent channel antenna gain modelling is captured as a text proposal to TR 38.922 in Annex in section 7 in this contribution.

# Discussion

In this contribution, we address some of the points presented in [3] regarding the modelling of active antenna systems (AAS) in terrestrial International Mobile Telecommunication (IMT) networks. Accurate modelling of AAS is important for analysing how IMT can coexist with other in-band or adjacent band services. The current approach for IMT-2020 and beyond mainly relies on the IMT AAS radiation pattern model described in Recommendation ITU-R M.2101 [5] and the document 5D/716 (Annex 4.4 - WRC-23) [6] for the extension of the model to support sub-array structures.

In [3] ITU-R WP 5D requested additional information on how commercial IMT Base Stations (BSs) use digital beamforming techniques, such as Zero-Forcing (ZF) beamforming, Maximum-Ratio Transmission (MRT), and Minimum Mean-Squared Error (MMSE) transmission. These techniques adjust the transmitted beams patterns to enhance the signal reception at the intended User Equipment (UE) while reducing interference with other co-scheduled UEs. The current model used for co-existence and sharing evaluations does not account for these adjustments, which could affect the accuracy of interference calculations.

However, our simulation studies indicate that the differences introduced by ZF beamforming technique, compared to the traditional array antenna model, are minor. Despite the different beamforming techniques, the simulation results consistently show minimal impact on the overall AAS BS gain towards a number of potential interfered-with receivers and on the BS antenna gain integrated over parts of or the entire angular sphere around the antenna.

  - Introduction

In modern wireless communication systems, effective management of inter-user interference is important to maximize the performance of IMT networks. Beamforming is a critical technique to achieve this goal by focusing signal transmissions towards the intended users while reducing interference with other co-scheduled users, and therefore improving signal quality.

Among the advanced techniques employed in Multiple-Input Multiple-Output (MIMO) systems, ZF beamforming is an appealing solution as it has a moderate computational complexity while achieving spatial multiplexing. This technique decouples the multi-user channel into multiple independent subchannels by nullifying unwanted signals in the direction of other users, ensuring that each user receives their intended signal with minimal interference.

Mathematically, the ZF precoder can be derived as the pseudo-inverse of the channel matrix:

WZF=HHHHH-1		(Eq. 2.1-1)

where H represents the channel matrix with its number of rows corresponding to the number of RX antennas (i.e., UEs) and its number of columns corresponding to the number of TX antennas (BS antenna ports), and HH is the conjugate transpose of H.

In the Annex in section 5 of this contribution, the detailed methodology to compute the channel matrix H and the ZF precoder weights is provided.

In this study, Monte Carlo simulations are used to compare the IMT BS gain towards a number of potential interfered-with receivers when using ZF beamforming and the current beamforming model.

It is noted that the applicability of ZF beamforming depends mainly on the angular separation between UEs and the number of antennas of the BS. Put simply, this is because as more antenna elements are stacked, the beamwidths of the BS beams narrow, therefore improving the BS beamforming angular resolution. This allows closer UEs to be paired for ZF beamforming. Consequently, the channel matrix H only includes the UEs that fulfil a minimum angular reparation requirement depending on the size of the BS antenna, so H may have two or three rows in each Monte Carlo snapshot depending in the geometry of the BS and the UEs. For the UEs that cannot be paired, the current beamforming model will be used.

For simplicity, to determine a suitable minimum angular separation for pairing UEs in the horizontal domain, we use a uniform linear array (ULA) with 8 antenna elements spaced 0.5 wavelengths and the single element pattern model with the parameters listed in Table 2.1-1.

Table 2.1-1: Antenna characteristics

| Parameter | Macro Urban |
| --- | --- |
| Element gain (dBi) | 5.5 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |

The previous parameters are taken from the antenna characteristics for IMT in 6 425-10 500 MHz for macro urban deployments in the document 5D/716 (Annex 4.4 - WRC23) [6].

For this assessment, as shown in Figure 2.1-1, we assume only two co-channel UEs served by a BS (UE #1 always at the BS boresight and UE #2 located at a certain angular separation from UE #1).

# Figure 2.1-1: Scenario to determine a suitable minimum angular separation for pairing UEs in the horizontal domain

Figure 2.1-2 shows the radiation patterns towards UE #1 of the ULA. As example, Figure 2.1-2a shows the radiation pattern with no ZF beamforming, while Figures 2.1-2b, 2.1-2c, and 2.1-2d show the radiation patterns when a ZF precoder is applied to generate nulls towards the UE #2 at an angular separation of 20, 10, and 5 degrees from UE #1, respectively. As can be seen, the gain reduction towards UE #1 is 0.22, 0.68, and 4.72 dB, respectively. This reduction is due to the limited resolution provided by the ULA with 8 elements. Therefore, in the preceding example and for the rest of this study, a UE minimum angular separation of 10 degrees is deemed appropriate as the antenna gain reduction is under 0.7 dB (i.e., less than 15% reduction in the linear domain).

# Figure 2.1-2: Radiation patterns of a ULA (8 antenna elements and a spacing of 0.5λ) with and without ZF beamforming

  - Simulation parameters

The IMT terrestrial component relevant parameters are based on the agreed technical and operational characteristics in the document 5D/716 (Annex 4.4 - WRC23). Table 2.2-1 contains the deployment related parameters of BSs and UEs, and Table 2.2-2 contains the beamforming antenna characteristics.

Table 2.2-1: IMT deployment-related parameters

| Parameter | Macro Urban |
| --- | --- |
| Base station |  |
| Carrier frequency | 6 GHz |
| Channel bandwidth | 100 MHz |
| BS antenna height | 18 m |
| Cell size | 300 m |
| Sectorization | 1 sector 1 |
| Frequency reuse | 1 |
| User Equipment |  |
| UE height | 1.5 m |
| UE density for terminal that are transmitting simultaneously | 3 UEs per sector |
| NOTE 1: This study considers only a single-entry scenario. |  |

Table 2.2-2: IMT base station beamforming antenna characteristics for IMT

| Parameter | Macro Urban |
| --- | --- |
| Antenna pattern | Refer to Recommendation ITU-R M.2101 Annex 1 |
| Element gain (incl. Ohmic loss) (dBi) | 5.5 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-120 1 |
| Mechanical down-tilt (degrees) | 10 |
| NOTE 1: The vertical coverage range includes the mechanical down-tilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

For the sake of completeness, additional results are provided with an AAS with a subarray configuration. The parameters for this AAS are taken from the WRC-23 studies 5D/1235 and 5D/1461 and are presented in the Table 2.2-3.

Table 2.2-3: IMT base station beamforming antenna characteristics for IMT with subarray configuration

| Parameter | Macro Urban |
| --- | --- |
| Antenna pattern | Refer to the extended AAS model in 5D/716 (Annex 4.4 - WRC23) |
| Element gain (incl. Ohmic loss) (dBi) | 6.4 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 16 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 1.4 of wavelength for V |
| Number of element rows in sub-array | 2 |
| Vertical radiating element spacing in sub-array | 0.7 of wavelength |
| Pre-set sub-array down-tilt (degrees) | 3 |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-100 1 |
| Mechanical down-tilt (degrees) | 6 |
| NOTE 1: The vertical coverage range includes the mechanical down-tilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

  - Deployment

The IMT network consists of a single site with a single sector. In each snapshot of the Monte Carlo simulation, 3 UEs are uniformly distributed within the sector, and the BS serves each UE with a dedicated beam. These beams can be generated by both the traditional array antenna model and as described in Section 2.1, assuming a UE minimum angular separation of 10 degrees.

To assess the interference to possible interfered-with receivers, two scenarios are considered, where these receivers are represented by sample points:

- Equidistant sample points towards the horizon around the IMT BS (Figure 2.3-1a)

- Equidistant sample points on a hemisphere above the IMT BS excluding the sample points towards the horizon (Figure 2.3-1b)

It is noted that the angular separation between sample points is approximately 15.5 degrees, with scenario 1 having 23 sample points and scenario 2 having 62 sample points.

# Figure 2.3-1: Equidistant sample points (a) towards the horizon around the BS, and (b) on a hemisphere above the BS

To validate the effectiveness of the ZF beamforming technique, for a BS without subarrays, Figure 2.3-2 shows the BS gain towards the co-scheduled UEs when serving a specific UE. As expected, ZF reduces the BS gain towards the co-scheduled UEs. Note that the gain reduction provided by ZF is not as large as one might expect from a beam null because the resolution of the beam patterns is 1 degree, so the null gain is usually rounded to the nearest integer angular direction. Note as well that all CDFs in Figure 2.3-2 are nearly identical because all UEs are deployed randomly, following the exact same distribution.

Furthermore, it can be seen that the traditional array antenna model and ZF model gains are the same in about 16% of cases, corresponding to situations where no UE can be paired for ZF beamforming because the angular separation between UEs is less than 10 degrees. The rest of the cases correspond to situations where either two UEs or all three UEs can be paired, depending on whether only two or all UEs fulfil the minimum angular separation requirement.

# Figure 2.3-2: BS gain towards the co-scheduled UEs when serving a specific UE with a BS without subarrays

Additionally, Figure 2.3-3 shows the BS gain towards the intended UEs. The antenna gain difference for all cases is less than 0.7 dB, as expected, with a minimum angular separation of 10 degrees. Again, all CDFs are nearly identical because all UEs are deployed randomly, following the exact same distribution.

# Figure 2.3-3: BS gain towards the intended UEs with a BS without subarrays

  - Simulation results

In this section the Monte Carlo simulations results are presented. For each sample point in the scenarios in Figure 2.1-3, 10.000 snapshots are simulated. The cumulative distribution functions (CDFs) curves of the IMT BS gain towards the sample points of the different scenarios are shown in Figures 2.4-1 to 2.4-4. The CDF results for scenario 1 using a BS without and with subarrays are shown in Figures 2.4-1 and 2.4-2 respectively. The CDF results for scenario 2 using a BS without and with subarrays are shown in Figures 2.4-3 and 2.4-4 respectively. For instance, since scenario 1 has 23 sample points, Figures 2.4-1a and 2.4-2a show 23 pairs of CDFs.

# Figure 2.4-1: Scenario 1 (without subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

# Figure 2.4-2: Scenario 1 (with subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

# Figure 2.4-3: Scenario 2 (without subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

# Figure 2.4-4: Scenario 2 (with subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

To assess the impact on long- and short-term statistics, the 80th percentile and 99.9th percentile are considered respectively. For Figures 2.4-1a, 2.4-2a, 2.4-3a, and 2.4-4a, Table 2.4-1 contains the summary of the average BS antenna gain difference between traditional beamforming model and ZF beamforming model.

Table 2.4-1: Average BS antenna gain difference between traditional beamforming model and ZF beamforming

| Scenario | Long-term (80th percentile) | Short-term (99.9th percentile) |
| --- | --- | --- |
| Scenario 1 (without subarrays) | 0.39 dB | 0.14 dB |
| Scenario 1 (with subarrays) | 0.05 dB | ~0 dB |
| Scenario 2 (without subarrays) | 0.48 dB | 0.06 dB |
| Scenario 2 (with subarrays) | 0.08 dB | ~0 dB |

Furthermore, the BS antenna gain integrated over parts of or the entire angular sphere around the antenna, i.e., the Total Integrated Gain (TIG), is also evaluated to provide a set of more consistent results by considering all angular directions. The TIG is calculated as follows:

TIG≈π2NθNφφ=0Nφ-1θ=0Nθ-1Gθ,φsinθ		(Eq. 2.4-1)

where θ and φ represent the elevation and azimuth angles, respectively, Nθ and Nφ are the number of samples on elevation and azimuth, respectively, and G is the antenna gain in linear units at the spherical coordinates (θ, φ).

For this assessment, the BS antenna gain is integrated over the whole sphere and over only the upper hemisphere by limiting the range of θ. The integrated gain is calculated for each individual beam for 10000 snapshots, resulting in three values per snapshot. Note that the integrated gain of the upper hemisphere of the AAS, where θ = 90 degrees is the AAS boresight direction including down-tilt, is not exactly the integrated above the horizon but includes most of it. This is depicted in Figure 2.4-5.

# Figure 2.4-5: Upper hemisphere: (a) upper hemisphere above horizon, and (b) upper hemisphere considered in this study

Figures 2.4-6 and 2.4-7 show the BS antenna gain integrated over the whole sphere and over only the upper hemisphere (Figure 2.4-5b), including the horizon, for the BS without and with subarrays, respectively. For simplicity, it is noted that for all the presented results, the integrated gain is always normalized by π⁄(2Nθ Nφ), as indicated in (Eq. 2.4-1), regardless of the number of samples considered.

# Figure 2.4-6: CDFs of the BS antenna gain without subarrays, integrated over (a) the upper hemisphere, including the horizon, and (b) the entire angular sphere around the antenna

# Figure 2.4-7: CDFs of the BS antenna gain with subarrays, integrated over (a) the upper hemisphere, including the horizon, and (b) the entire angular sphere around the antenna

As can be seen, in all cases considered, the TIG difference between the beamforming schemes is negligible.

# Conclusion

Based on our updated simulation results, we have corroborated that there is no need to change the AAS BS beamforming assumptions in 3GPP RAN4 and ITU-R WP 5D. The differences introduced by ZF beamforming technique, compared to the traditional array antenna model, are minor. The simulation results show minimal impact on the overall AAS BS gain towards a number of potential interfered-with receivers and on the BS antenna gain integrated over parts of or the entire angular sphere around the antenna. Therefore, the current modelling approach remains effective for its intended purposes. Modifying the existing beamforming model is not necessary, as the variations observed are insignificant and do not justify the complexity of updating the model.

Based on the simulation results we observe following:

Observation 1: Based on our simulation results, there is no need to change the beamforming assumptions in RAN4 TRs and Recommendation ITU-R M.2101. The differences introduced by ZF beamforming technique, compared to the model currently used, are minor. The simulation results show minimal impact on the overall AAS BS gain towards a number of potential interfered-with receivers and on the BS antenna gain integrated over parts of or the entire sphere around the antenna.

To progress the work, propose following:

Proposal 1: Based on outcome of beamforming modelling evaluation, the current modelling approach remains effective for its intended purposes. Modifying the existing beamforming model is not necessary, as the variations observed are insignificant and do not justify the complexity of updating the model.

Proposal 2: Capture simulation assumption, simulation results and corresponding conclusion in TR 38.922.

A text proposal for TR 38.922 with technical background information is captured in section 7 of this contribution.

# References

[1]	R4-2410576, “LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, RAN4

[2]	R4-2414449, “LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27”, RAN4

[3]	R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz”, ITU-R WP 5D

[4]	R4-2411873, “On the topic of additional information requested by ITU-R WP 5D”, Ericsson

[5]	Recommendation ITU-R M.2101, “Modelling and simulation of IMT networks and systems for use in sharing and compatibility studies”, ITU-R

[6]	ITU-R WP 5D document 5D/716 (Annex 4.4 - WRC-23), ITU-R

[7]	C. A. Balanis, Antenna Theory: Analysis and Design, 3rd ed. Hoboken, NJ, USA: Wiley, 2005.

# Annex: Methodology

The simulation methodology can be summarized as:

- For the k-th UE, generate the uniform planar array (UPA) coefficients, Ukn,m, using the weighting formulation in Rec. ITU-R M.2101 – Table 4, to reflect the UE angles from the BS antenna array in both azimuth and elevation:

Ukn,m=expj2πn-1dvλcosθk+m-1dhλsinθksinφk

where, for the k-th UE, θk and φk are the angles of elevation and azimuth, respectively. The ranges of θk and φk are 0,π and [-π,π] radians, respectively. The size of the matrix Ukn,m is N×M, where N is the number of rows and M is the number of columns of the antenna array.

- Assess the angular separation between the K UEs in the cell (ITU-R assumption, K = 3) (NOTE 1).

- For each UE (NOTE 2), if there is at least another UE which with the minimum angular separation requirement is fulfilled, create a concatenated P×MN channel matrix,  Hk (NOTE 3):

Hk=hkT …hpTT

where T is the transpose operator and hk corresponds to the vectorization of Ukn,m resulting in a vector of size MN×1 (NOTE 4). Note that, for simplicity, hkT is always the first row of Hk, and hpT represent the UE(s) that can be paired with the k-th UE.

- For each UE, the zero-forcing beamforming (precoding) matrix WkZF (MN x P) is:

WkZF=HkHHkHkH-1

where Hk represents the channel matrix with its number of columns corresponding to the number of Rx antennas (i.e., UEs) and its number of rows corresponding to the number of Tx antennas (BS antenna ports), and HkH is the conjugate transpose of Hk.

- The entire WkZF matrix can be normalised by diving it by its Frobenius norm, or, doing a column wise normalisation by diving each column with its norm.

- The actual precoding weights for each UE (wk) are obtained by selecting the first column of WkZF. The size of wk is MN×1.

- Obtain the individual Zero Forcing beam pattern: The precoding weights for each beam wk of size MNx1 may be used to obtain the k-th zero forcing beam pattern by reordering the column vector wk of size MNx1 into a matrix Wk of size NxM which can be used as element weight as:

Akθ,ϕ=AEθ,ϕ+10log10(|m=1NHn=1NVwkn,m⋅νn,m​2)

where wkn,m refers to the entry in the n-th row and m-th column of weighting matrix Wk and νn,m is the superposition vector. 

Note that for AAS with sub-arrays, AEθ,ϕ is updated by Asubθ,ϕ: 

Akθ,ϕ=Asubθ,ϕ+10log10(|m=1NHn=1NVwkn,m⋅νn,m​2)
where Asubθ,ϕ calculation follows the method defined in the extended AAS modelling in Annex 4.4 to Working Party 5D Chairman’s Report ANNEX 3.

- The resulting composite ZF beam pattern may be obtained by summing up the individual Zero Forcing beam patterns in linear scale (and transforming them back to dB):

AZFθ,ϕ=10log10p=1P10Akθ, ϕ10 

where AZFθ,ϕ refers to the composite ZF beam pattern and Akθ,ϕ refers to the k-th individual zero forcing beam pattern.

NOTE 1: The total 3D angular separation should be considered to determine the fulfillment of the minimum angular separation requirement. Additionally, only the azimuth angular separation may be considered in cases where the elevation angle does not significantly contribute to the total 3D angular separation.

NOTE 2: Hk must be constructed for each UE separately, since in some instances, one UE can be paired with all other UEs while another UE could be paired with only a single other UE. In Figure 5-1, the figure depicts this situation: UE1 can be paired with UE2 and UE3 since the min. angular separation is fulfilled (as shown in Figure 5-1a), UE2 can only be paired with UE1 (as shown in Figure 5.1b), and UE3 can only be paired with UE1 (as shown in Figure 5-1c).

# Figure 5-1: UE distribution

NOTE 3: Hk only includes the UEs that fulfill the minimum angular separation requirement depending on the size of the BS antenna, so Hk may have two or three columns (P = 2 or 3) in each Monte Carlo snapshot depending on the geometry of the BS and the UEs. For the UEs that cannot be paired with any other UEs (P = 1), Hk is not constructed, and the current beamforming model is used.

NOTE 4: The vectorization must ensure that the vector of size 1×MN correspond to the concatenation of the rows of Ukn,m:

u11⋯u1m⋮⋱⋮un1⋯unm→concatenation→u11…u1m u21…u2m… un1…unm

# Annex: Considerations for Pseudo-Inverse Calculations in Zero-Forcing Beamforming

There have been some inquiries about the correctness of the methodology used in this study. Based on the methodology in Annex in section 5, the main aspect being questioned is the assumption that the matrices Ukn,m alone can be vectorized to form the entries of the concatenated channel matrix Hk for performing the pseudo-inverse operation to calculate the ZF precoder. Instead, it is suggested that the concatenated channel matrix should be formed using the product of the vectorization of conj(Ukn,m) and the super position vector νn,m. For simplicity, we will refer to this product as wn,mνn,m from now on. Next, we show that the matrices Ukn,malone provide very similar results as when considering the product wn,mνn,m:

# Figure 6-1: Scenario 1 (without subarrays) CDFs: BS gain towards each sample point

# Figure 6-2: Scenario 1 (with subarrays) CDFs: BS gain towards each sample point

# Figure 6-3: Scenario 2 (without subarrays) CDFs: BS gain towards each sample point

# Figure 6-4: Scenario 2 (with subarrays) CDFs: BS gain towards each sample point

To assess the impact on long- and short-term statistics, the 80th percentile and 99.9th percentile are considered respectively. For Figures 6-1, 6-2, 6-3, and 6-4, Table 6-1 contains the summary of the average BS antenna gain difference between traditional antenna beamforming model and ZF beamforming model when considering the product wn,mνn,m.

Table 6-1: Average BS antenna gain difference between traditional beamforming model and ZF beamforming (considering the product wn,mνn,m)

| Scenario | Long-term (80th percentile) | Short-term (99.9th percentile) |
| --- | --- | --- |
| Scenario 1 (without subarrays) | 0.37 dB | 0.13 dB |
| Scenario 1 (with subarrays) | 0.06 dB | 0.03 dB |
| Scenario 2 (without subarrays) | 0.35 dB | 0.02 dB |
| Scenario 2 (with subarrays) | 0.28 dB | 0.66 dB |

The results in Table 6-1 are very similar to those in Table 2.4-1 corroborating that the matrices Ukn,malone provide very similar results as when considering the product wn,mνn,m. Thus, both approaches can be used to assess the difference between the traditional antenna model and the ZF precoder scheme.

# Annex: Text Proposal to TR 38.922

Overview of text proposal:

- A new section in subclause 7.3 is created to captured technical background information relevant for evaluating different AAS beamforming assumptions. The heading is “MIMO modelling aspects”.

- In subclause 7.3.1 “Overview”, the background for comparing different AAS beamforming assumptions is captured.

- In subclause 7.3.2 “Assumptions”, all the assumptions considered in the simulation is captured.

- In subclause 7.3.3 “Simulation results”, all the results are captured.

- In subclause 7.3.4, “Conclusion”, the conclusion is captured.

TEXT PROPOSAL:

## 7.3	MIMO modelling aspects

### 7.3.1	Overview

In modern wireless communication systems, effective management of inter-user interference is important to maximize the performance of IMT networks. Beamforming is a critical technique to achieve this goal by focusing signal transmissions towards the intended users while reducing interference with other co-scheduled users, and therefore improving signal quality.

Among the advanced techniques employed in Multiple-Input Multiple-Output (MIMO) systems, Zero-Forcing (ZF) beamforming is an appealing solution as it has a moderate computational complexity while achieving spatial multiplexing. This technique decouples the multi-user channel into multiple independent subchannels by nullifying unwanted signals in the direction of other users, ensuring that each user receives their intended signal with minimal interference.

Mathematically, the ZF precoder can be derived as the pseudo-inverse of the channel matrix:

WZF=HHHHH-1		(Eq. 7.3.1-1)

where H represents the channel matrix with its number of rows corresponding to the number of RX antennas (i.e., UEs) and its number of columns corresponding to the number of TX antennas (BS antenna ports), and HH is the conjugate transpose of H.

In this study, Monte Carlo simulations are used to compare the IMT BS gain towards a number of potential interfered-with receivers when using ZF beamforming and the current beamforming model.

It is noted that the applicability of ZF beamforming depends mainly on the angular separation between UEs and the number of antennas of the BS. Put simply, this is because as more antenna elements are stacked, the beamwidths of the BS beams narrow, therefore improving the BS beamforming angular resolution. This allows closer UEs to be paired for ZF beamforming. Consequently, the channel matrix H only includes the UEs that fulfil a minimum angular reparation requirement depending on the size of the BS antenna, so H may have two or three rows in each Monte Carlo snapshot depending in the geometry of the BS and the UEs. For the UEs that cannot be paired, the traditional beamforming model in is used.

For simplicity, to determine a suitable minimum angular separation for pairing UEs in the horizontal domain, we use a uniform linear array (ULA) with 8 antenna elements spaced 0.5 wavelengths and the single element pattern model with the parameters listed in Table 7.3.1-1.

Table 7.3.1-1: Antenna characteristics

| Parameter | Macro Urban |
| --- | --- |
| Element gain (dBi) | 5.5 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |

For this assessment, as shown in Figure 7.3.1-1, we assume only two co-channel UEs served by a BS (UE #1 always at the BS boresight and UE #2 located at a certain angular separation from UE #1).

# Figure 7.3.1-1: Scenario to determine a suitable minimum angular separation for pairing UEs in the horizontal domain

Figure 7.3.1-2 shows the radiation patterns towards UE #1 of the ULA. As example, Figure 7.3.1-2a shows the radiation pattern with no ZF beamforming, while Figures 7.3.1-2b, 7.3.1-2c, and 7.3.1-2d show the radiation patterns when a ZF precoder is applied to generate nulls towards the UE #2 at an angular separation of 20, 10, and 5 degrees from UE #1, respectively. As can be seen, the gain reduction towards UE #1 is 0.22, 0.68, and 4.72 dB, respectively. This reduction is due to the limited resolution provided by the ULA with 8 elements. Therefore, in the preceding example and for the rest of this study, a UE minimum angular separation of 10 degrees is deemed appropriate as the antenna gain reduction is under 0.7 dB (i.e., less than 15% reduction in the linear domain).

# Figure 7.3.1-2: Radiation patterns of a ULA (8 antenna elements and a spacing of 0.5λ) with and without ZF beamforming

The simulation methodology can be summarized as:

- For the k-th UE, generate the uniform planar array (UPA) coefficients, Ukn,m, using the weighting formulation in subclause 7.1, Table 7.1.3-1, to reflect the UE angles from the BS antenna array in both azimuth and elevation:

Ukn,m=expj2πn-1dvλcosθk+m-1dhλsinθksinφk

where, for the k-th UE, θk and φk are the angles of elevation and azimuth, respectively. The ranges of θk and φk are 0,π and [-π,π] radians, respectively. The size of the matrix Ukn,m is N×M as defined in subclause 7.1, Table 7.1.3-1, where N is the number of rows and M is the number of columns of the antenna array.

- Assess the angular separation between the K UEs in the cell (ITU-R assumption, K = 3) (NOTE 1).

- For each UE (NOTE 2), if there is at least another UE which with the minimum angular separation requirement is fulfilled, create a concatenated P×MN channel matrix,  Hk (NOTE 3):

Hk=hkT …hpTT

where T is the transpose operator and hk corresponds to the vectorization of Ukn,m resulting in a vector of size MN×1 (NOTE 4). Note that, for simplicity, hkT is always the first row of Hk, and hpT represent the UE(s) that can be paired with the k-th UE.

- For each UE, the zero-forcing beamforming (precoding) matrix WkZF (MN x P) is:

WkZF=HkHHkHkH-1

where Hk represents the channel matrix with its number of columns corresponding to the number of Rx antennas (i.e., UEs) and its number of rows corresponding to the number of Tx antennas (BS antenna ports), and HkH is the conjugate transpose of Hk.

- The entire WkZF matrix can be normalised by diving it by its Frobenius norm, or, doing a column wise normalisation by diving each column with its norm.

- The actual precoding weights for each UE (wk) are obtained by selecting the first column of WkZF. The size of wk is MN×1.

- Obtain the individual Zero Forcing beam pattern: The precoding weights for each beam wk of size MNx1 may be used to obtain the k-th zero forcing beam pattern by reordering the column vector wk of size MNx1 into a matrix Wk of size NxM which can be used as element weight in the formulation in subclause 7.1, Table 7.1.3-1:

Akθ,ϕ=AEθ,ϕ+10log10(|m=1NHn=1NVwkn,m⋅νn,m​2)

where wkn,m refers to the entry in the n-th row and m-th column of weighting matrix Wk and νn,m is the superposition vector as given in subclause 7.1, Table 7.1.3-1. 

Note that for AAS with sub-arrays, AEθ,ϕ is updated by Asubθ,ϕ: 

Akθ,ϕ=Asubθ,ϕ+10log10(|m=1NHn=1NVwkn,m⋅νn,m​2)
where Asubθ,ϕ calculation follows the method defined in the extended AAS modelling in subclause 7.1, Table 7.1.3-1.

- The resulting composite ZF beam pattern may be obtained by summing up the individual Zero Forcing beam patterns in linear scale (and transforming them back to dB):

AZFθ,ϕ=10log10p=1P10Akθ, ϕ10 

where AZFθ,ϕ refers to the composite ZF beam pattern and Akθ,ϕ refers to the k-th individual zero forcing beam pattern.

NOTE 1: The total 3D angular separation should be considered to determine the fulfillment of the minimum angular separation requirement. Additionally, only the azimuth angular separation may be considered in cases where the elevation angle does not significantly contribute to the total 3D angular separation.

NOTE 2: Hk must be constructed for each UE separately, since in some instances, one UE can be paired with all other UEs while another UE could be paired with only a single other UE. In Figure 7.3.1-3, the figure depicts this situation: UE1 can be paired with UE2 and UE3 since the min. angular separation is fulfilled (as shown in Figure 7.3.1-3a), UE2 can only be paired with UE1 (as shown in Figure 7.3.1-3b), and UE3 can only be paired with UE1 (as shown in Figure 7.3.1-3c).

# Figure 7.3.1-3: UE distribution

NOTE 3: Hk only includes the UEs that fulfill the minimum angular separation requirement depending on the size of the BS antenna, so Hk may have two or three columns (P = 2 or 3) in each Monte Carlo snapshot depending on the geometry of the BS and the UEs. For the UEs that cannot be paired with any other UEs (P = 1), Hk is not constructed, and the current beamforming model in subclause 7.1, Table 7.1.3-1is used.

NOTE 4: The vectorization must ensure that the vector of size 1×MN correspond to the concatenation of the rows of Ukn,m:

u11⋯u1m⋮⋱⋮un1⋯unm→concatenation→u11…u1m u21…u2m… un1…unm

### 7.3.2	Assumptions

The IMT terrestrial component relevant parameters are based on the agreed technical and operational characteristics in the document 5D/716 (Annex 4.4 - WRC23). Table 7.3.2-1 contains the deployment related parameters of BSs and UEs, and Table 7.3.2-2 contains the beamforming antenna characteristics.

Table 7.3.2-1: IMT deployment-related parameters

| Parameter | Macro Urban |
| --- | --- |
| Base station |  |
| Carrier frequency | 6 GHz |
| Channel bandwidth | 100 MHz |
| BS antenna height | 18 m |
| Cell size | 300 m |
| Sectorization | 1 sector 1 |
| Frequency reuse | 1 |
| User Equipment |  |
| UE height | 1.5 m |
| UE density for terminal that are transmitting simultaneously | 3 UEs per sector |
| NOTE 1: This study considers only a single-entry scenario. |  |

Table 7.3.2-2: IMT base station beamforming antenna characteristics for IMT

| Parameter | Macro Urban |
| --- | --- |
| Antenna pattern | Refer to Recommendation ITU-R M.2101 Annex 1 |
| Element gain (incl. Ohmic loss) (dBi) | 5.5 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 0.5 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-120 1 |
| Mechanical down-tilt (degrees) | 10 |
| NOTE 1: The vertical coverage range includes the mechanical down-tilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

For the sake of completeness, additional results are provided with an AAS with a subarray configuration. The parameters for this AAS are taken from the WRC-23 studies 5D/1235 and 5D/1461 and are presented in the Table 7.3.2-3.

Table 7.3.2-3: IMT base station beamforming antenna characteristics for IMT with subarray configuration

| Parameter | Macro Urban |
| --- | --- |
| Antenna pattern | Refer to the extended AAS model in 5D/716 (Annex 4.4 - WRC23) |
| Element gain (incl. Ohmic loss) (dBi) | 6.4 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 65º for V |
| Horizontal/vertical front to back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 16 |
| Horizontal/Vertical radiating element spacing | 0.5 of wavelength for H 1.4 of wavelength for V |
| Number of element rows in sub-array | 2 |
| Vertical radiating element spacing in sub-array | 0.7 of wavelength |
| Pre-set sub-array down-tilt (degrees) | 3 |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-100 1 |
| Mechanical down-tilt (degrees) | 6 |
| NOTE 1: The vertical coverage range includes the mechanical down-tilt. A minimum BS-UE distance along the ground of 35 m should be used for urban/suburban and rural macro environments. |  |

The IMT network consists of a single site with a single sector. In each snapshot of the Monte Carlo simulation, 3 UEs are uniformly distributed within the sector, and the BS serves each UE with a dedicated beam. These beams can be generated by both the traditional array antenna model and as described in subclause 7.3.1, assuming a UE minimum angular separation of 10 degrees.

To assess the interference to possible interfered-with receivers, two scenarios are considered, where these receivers are represented by sample points:

- Equidistant sample points towards the horizon around the IMT BS (Figure 7.3.2-1a)

- Equidistant sample points on a hemisphere above the IMT BS excluding the sample points towards the horizon (Figure 7.3.2-1b)

It is noted that the angular separation between sample points is approximately 15.5 degrees, with scenario 1 having 23 sample points and scenario 2 having 62 sample points.

# Figure 7.3.2-1: Equidistant sample points (a) towards the horizon around the BS, and (b) on a hemisphere above the BS

### 7.3.3	Simulation results

#### 7.3.3.1	Company A

In this section the Monte Carlo simulations results are presented. For each sample point in the scenarios in Figure 7.3.2.1-1, 10.000 snapshots are simulated. The cumulative distribution functions (CDFs) curves of the IMT BS gain towards the sample points of the different scenarios are shown in Figures 7.3.3.1-1 to 7.3.3.1-4. The CDF results for scenario 1 using a BS without and with subarrays are shown in Figures 7.3.3.1-1 and 7.3.3.1-2 respectively. The CDF results for scenario 2 using a BS without and with subarrays are shown in Figures 7.3.3.1-3 and 7.3.3.1-4 respectively. For instance, since scenario 1 has 23 sample points, Figures 7.3.3.1-1a and 7.3.3.1-2a show 23 pairs of CDFs.

# Figure 7.3.3.1-1: Scenario 1 (without subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

# Figure 7.3.3.1-2: Scenario 1 (with subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

# Figure 7.3.3.1-3: Scenario 2 (without subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

# Figure 7.3.3.1-4: Scenario 2 (with subarrays) CDFs: BS gain towards (a) each sample point, and (b) all sample points (composite)

To assess the impact on long- and short-term statistics, the 80th percentile and 99.9th percentile are considered respectively. For Figures 7.3.3.1-1a, 7.3.3.1-2a, 7.3.3.1-3a, and 7.3.3.1-4a, Table 7.3.3.1-1 contains the summary of the average BS antenna gain difference between traditional beamforming model and ZF beamforming model.

Table 7.3.3.1-1: Average BS antenna gain difference between traditional beamforming model and ZF beamforming

| Scenario | Long-term (80th percentile) | Short-term (99.9th percentile) |
| --- | --- | --- |
| Scenario 1 (without subarrays) | 0.39 dB | 0.14 dB |
| Scenario 1 (with subarrays) | 0.05 dB | 0.00 dB |
| Scenario 2 (without subarrays) | 0.48 dB | 0.06 dB |
| Scenario 2 (with subarrays) | 0.08 dB | 0.00 dB |

#### 7.3.3.2	Company B

{Editorial note: Simulation results from other companies to be added}

### 7.3.4	Conclusion

Based on our updated simulation results, we have corroborated that there is no need to change the beamforming assumptions for the antenna model used in RAN4 and ITU-R WP 5D. The differences introduced by ZF beamforming technique, compared to the traditional array antenna model, are minor. The simulation results show minimal impact on the overall AAS BS gain towards a number of potential interfered-with receivers and on the BS antenna gain integrated over parts of or the entire angular sphere around the antenna. Therefore, the current modelling approach remains effective for its intended purposes. Modifying the existing beamforming model is not necessary, as the variations observed are insignificant and do not justify the complexity of updating the model.

Based on our simulation results, there is no need to change the beamforming assumptions. The differences introduced by ZF beamforming technique, compared to the traditional array antenna model, are minor. The simulation results show minimal impact on the overall AAS BS gain towards a number of potential interfered-with receivers and on the BS antenna gain integrated over parts of or the entire sphere around the antenna.

Based on outcome of beamforming modelling evaluation, the current modelling approach remains effective for its intended purposes. Modifying the existing beamforming model is not necessary, as the variations observed are insignificant and do not justify the complexity of updating the model.