TSG-RAN Working Group 4 (Radio) meeting #112-bis	R4-2415971

Hefei, China, 14th to 18th October 2024

Source:	Ericsson

Title:	On the topic of aspects related to BS-to-BS co-location

Agenda item:	6.11.3

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#112 in Maastricht) aspects related to BS-to-BS colocation was discussed as part of the Rel-19 work (with WI code NR_BS_RF_req_evo) related to evolving BS RF requirements and test methods. In addition to the last RAN4 meeting, an extra ad-hoc session was scheduled with the intention to find a way-forward. At this time, it can be concluded that currently involved companies have different views on how to evolve co-location requirements and corresponding test methods to be able to support frequencies in the upper region of FR1 (3500 to 7125 MHz) and potential future spectrum in the cm-wave range with large AAS base station antennas (4x8, sub-array 3x1 or larger).

There are different reasons to investigate the possibilities to evolve and enhance the description of BS-to-BS colocation aspects:

- Current FR1 bands: For FR1 BS RF co-location requirements the current assumption of BS-to-BS isolation is set to 30 dB regardless of frequency offset between victim and aggressor. This approach has been used since the introduction of co-location requirements in RAN4 BS specifications. For bands defined below 3500 MHz the assumption of 30 dB is still seen as relevant. However, with the introduction of NR bands in the range 3500 to 7125 MHz the relevance of the 30 dB isolation assumption requires further analysis to confirm relevance.

- Large AAS BS antennas: With large AAS antennas (typically larger than 4x8 array with 3x1 sub-array configuration) operating at higher frequencies above 3500 MHz work related to establish a relevant assumption for BS-to-BS isolation is essential. Without proper technical background, BS RF requirements could be non-relevant with the risk to not achieve the purpose or set to stringent putting constraints on the base station implementation which would drive cost without benefiting network performance. RAN4 need to define relevant BS-to-BS isolation assumption to be considered for requirement derivation within the frequency range 3500 to 7125 MHz and above when it is needed.

- Practical test issues: For BS type 1-O the BS-to-BS isolation assumptions is used together with an assumption on isolation between the test object and co-location reference antenna. Experience from AAS development and testing gives that the test concept created around the Co-Location Test Antenna (CLTA) is difficult to scale to frequencies above 3500 MHz. More details related to technical challenges is listed in a previous contribution [1, 2, 3].

- Protection of other BS within the same network: Currently, the TDD OFF power requirement use the 30 dB BS-to-BS isolation assumption for requirement level derivation. For non-AAS BS 30 dB isolation was deemed relevant as isolation between two BS within the same network. However, for AAS BS the isolation between two BS within the same network have significantly larger isolation due to the fact that the BS focus the antenna gain towards UEs on the ground. Therefore, RAN4 should study the BS-to-BS isolation between base station within the same network to be able to decide on the relevance for this requirement for AAS BS operating above 3500 MHz.

- Transmitter intermodulation requirement: The transmitter intermodulation requirement is defined to secure the robustness of the test object transmitter to maintain specified unwanted emission levels while the test object is exposed for an interferer signal. Currently, the co-location BS-to-BS isolation is used for requirement derivation for all BS types. For AAS base stations the strongest interferer is coming from neighbouring elements within the AAS array. The interferer from an external source will be significantly lower. If the AAS base station is meeting the regular unwanted emission requirements with full power, it would be safe to say that it will also meet unwanted emission when an external interferer is injected. RAN4 should study the need to specify transmitter intermodulation requirement for AAS base stations with large antennas operating at frequencies above 7125 MHz.

In addition:

- Co-existence evaluation modelling: In the RAN4 co-existence evaluation the BS-to-BS interference due to grid shift between aggressor network and victim network is normally not of interest when requirement such ACLR and ACS are determined. It can be noticed that for the specific situation when grid shift 0% is analysed assumptions similar to the one used for BS-to-BS co-location would be relevant to consider. Especially in the case where BS-to-BS interference is considered. For grid shift 0%, meaning that the victim and aggressor base station is located at the same site, the BS-to-BS isolation should resemble the assumption for BS-to-BS isolation used for co-location requirement derivation. Currently, no such models exist in RAN4.

- Other situations: In some cases (e.g. the Rel-19 study on fragmented carriers) the network grid shift has impact on how the UE experience the RF environment. In the case of 0% grid shift site specific details (such as assumptions related to mechanical down-tilt, sector alignment, difference in BS heights, etc.) will affect how much power difference the UE will see from two co-located base stations. The scenario with 0% grid shift is of most importance for the study, hence more details relevant for BS co-location should be established by RAN4.

To be able to evolve RAN4 BS RF specifications and develop relevant requirements supporting new features and use-cases from 3500 to 7125 MHz and above it is obvious that the assumptions used for BS-to-BS isolation for co-location situations needs to be evaluated, understood and described properly.

In several previous contributions different technical aspects, challenges and possibilities related to BS-to-BS co-location technical background information and requirement derivation information have been presented to RAN4. In Table 1-1 all contributions have been collected as an overview.

Table 1-1: Overview of previous contributions

| Title | Reference | Abstract |
| --- | --- | --- |
| Overview of BS OTA co-location requirements | R4-2404984 [1] | This contribution presents an overview of technical issues related to requirement derivation and corresponding test methods for the requirement concept introduced for AAS BS in Rel-15. |
| Evolution of BS co-location requirements | R4-2408086 [2] | This contribution focus on the co-location scenario and aspects related to the requirement level derivation. It is proposed to evaluate BS-to-BS isolation for different co-location scenario using simulations. |
| On the topic of BS RF evolution related to co-location requirements | R4-2412912 [3] | This contribution focus on details relevant for transmitter spurious emission and receiver out-of-band blocking. A workplan is presented on how to evaluate isolation based on simulation. Measured values of BS-to-BS isolation is presented as an indication of technical issues presented in previous contributions. It is suggested to evaluate a requirement concept based on field strength instead of a co-location reference antenna. |
| On general aspects related to BS RF evolution WI | R4-2411872 [4] | This contribution presents an overview of the workplan for BS RF evolution. For co-location requirements a brief summary is captured with technical challenges with current requirement approach. |

In this contribution we continue the discussion related to aspects related to BS-to-BS colocation situations. Now we have expanded the scope beyond BS RF co-location requirements to also capture other aspects relevant for RAN4 specification evolution.

# Discussion

The intention with a standard is to define a common set of requirements and assumptions that can be used and understood by multiple vendors, operators, laboratories, and administrations. Standardized requirements would guarantee operation of co-located base stations for a defined situation with guaranteed performance. A standard would guarantee interoperability between different base station manufacturers equipment and provide guidance to site configurations handled by operators.

In RAN4 aspects related to BS-to-BS colocation comes up in discussions related to many topics. For non-AAS base stations and AAS base stations operating below 3500 MHz, the current assumption of 30 dB is still valid. But for non-AAS base stations and AAS base stations operating above 3500 MHz some careful considerations related to how requirements are derived and tested are needed.

In the following sub-sections, we provide some more technical details on some aspects related to BS-to-BS co-location.

  - Implementation aspects

The co-location spurious emission requirement specifies a maximum emission level within another 3GPP band. The co-location spurious emission requirement is an optional requirement. This means that the base station manufacturer declares for what bands the emission requirement is fulfilled. For a non-AAS BS the emission level at the Transceiver Array Boundary (TAB) connector shall not exceed -96 dBm/100kHz. This level is set such that the impact on the victim UL is low (0.5 dB degradation of receiver sensitivity). This requirement level is also applicable for hybrid AAS BS type 1-H and used as baseline for OTA AAS BS type 1-O requirements.

The co-location spurious emission requirement is derived based on allowing a very low degradation of the victim receiver sensitivity. Traditionally, the additional maximum allowed noise from the aggressor is recalculated to a maximum allowed emission level at the aggressor TAB connector, as showed in the link budget visualized in Figure 2.1-1. As an example, values for aggressor output power and ACLR is typical values. Depending on considered frequency and base station power capability the values in the link budget can vary. The BS-to-BS isolation equal to 30 dB have been fixed since it was derived for GSM operating at 900 MHz. Now with bands defined to support frequencies up to 7125 MHz it would be appropriate to study technical background and relevance of current assumption.

# Figure 2.1-1: Co-location spurious emission link budget

The co-location spurious emission level is the most stringent emission requirement level defined for base stations in 3GPP specifications, hence it will have significant impact on the band specific RF filter in the radio front-end. Using the current approach for requirement derivation the required RF filter suppression (marked in red in Figure 2.1-1) will be directly related to BS-to-BS isolation (marked in blue in Figure 2.1-1), the aggressor BS output power and the aggressor ACLR. The link budget is representative for a scenario where two bands are allocated close to each other with worst-case the aggressor carrier configured at the top RF channel in the aggressor band and the victim carrier configured at the bottom RF channel in the victim band. This would correspond to a worst-case situation, often the bands to be considered for co-location requirements are not located edge-to-edge. The intention is to visualise the impact from co-location spurious emission on radio implementation.

It can be noticed that the BS-to-BS isolation have direct impact on the required RF filter suppression. For non-AAS BS (BS type 1-C) and AAS-BS (BS type 1-H and BS type 1-O) operating below 3500 MHz the RF filter suppression required for co-location requirements can be achieved. However, for AAS BS and operation above 3500 MHz, the space for the RF filters is typically constrained by additional physical magnitudes like the antenna aperture, the volume of the enclosure and weight. Therefore, it is essential to study BS-to-BS isolation properties and characteristics for AAS base stations with the intention to find relevant of BS-to-BS isolation assumptions values that can be used for requirements derivation still meeting the goal with the requirement to achieve minimal impact on victim network UL.

Observation 1: The assumed BS-to-BS isolation of 30 dB have direct impact on required RF filter suppression for any base station implementation. For AAS BS operating above 3500 MHz the filter suppression has direct impact on the physical size of the RF filter and corresponding routing losses, volume, weight and cost. Since the current assumption was derived for GSM operating at 900 MHz it would be reasonable to further study the relevance for AAS base stations operating above 3500 MHz.

It is essential to evaluate BS-to-BS isolation for relevant co-location scenarios considering implications such as beamforming (including statistical aspects related to beamforming) and frequency offset between victim and aggressor.

Observation 2: Using a fixed value for BS-to-BS isolation would risk to over specify emission requirements for the situation where the frequency offset is large between the aggressor base station and victim base station. For AAS base stations it would be reasonable to further study statistical properties due beamforming and how they affect BS-to-BS isolation. Also, aspects related to victim/aggressor frequency offset would be relevant to study. It could be transmission mismatch, different physical size of victim and aggressor antenna aperture that have impact on BS-to-BS isolation.

In Rel-15 a concept based on a colocation reference antenna was defined for AAS BS (BS type 1-O) for all bands. To resolve identified technical issues attached to this concept RAN4 need to take step back to the original background and find a new approach on how the requirement can be formulated as an OTA requirement. Eventually, the Rel-15 can be used for bands up to 3500 MHz with some modifications and a new concept based on OTA parameters adopted for bands defined above 3500 MHz.

  - AAS dynamic behaviour aspects

With the introduction of AAS base station capable of MIMO transmission schemes the radiation properties changed from beam spatially static to be very dynamic.

When the BS-to-BS isolation of 30 dB was originally derived a static operation based on passive antenna assumption was considered. The isolation was derived from a simple link budget calculation. However, for AAS base stations the inherent dynamic behaviour will have impact on the BS-to-BS isolation. This means that the isolation needs to be evaluated for a situation where the base station beamforming to achieve coverage to all UEs within the whole considered coverage area is captured. This will produce a statistical outcome of BS-to-BS isolation. Typically, RAN4 decide on a point in a Cumulative Distribution Function (CDF) curve to derive a requirement level. For BS-to-BS isolation, RAN4 need to evaluate the statistical spread on BS-to-BS isolation and consider what point the CDF would be proper to use to establish a relevant assumption for BS-to-BS isolation. For this purpose, a simulation considering several UEs spread within the coverage area needs to be considered for both the aggressor network and the victim network. Based on the array excitations, array geometry assumptions and proper propagation model (near-field or far-field) the BS-to-BS isolation can be evaluated. Since the outcome will be produce different results based on array antenna excitation a statistical approach is needed to decide on what isolation value should be picked for requirement derivation. An example of simulation results is visualised in Figure 2.2-1.

Figure 2.2-1: An example of BS-to-BS isolation statistical outcome

It can be noticed that the isolation heavily depends on mast height difference. This indicates careful consideration in relation to simulation parameters assumptions used during the evaluation.

Based on the empirical CDF plot, RAN4 needs to establish what probability point (e.g. 50%, 70%, 90% or 99.999%) to pick for requirement derivation.

Observation 3: Based on statistical simulation results RAN4 need to establish criteria for requirements derivation.

With a thorough evaluation of BS-to-BS statistical properties for near-field scenarios (side-by-side, on top of each other) and far-field scenarios (mounted on different poles on a rooftop, or different masts) RAN4 would build up better understanding on relevant deployment scenarios. This information can be used if additional co-location scenarios need to be established. E.g., 0.5 m edge-to-edge separation instead of 0.1 m separation.

  - Isolation model aspects

In RAN4 discussions related to BS-to-BS co-location can be divided into two different main categories:

- Uncoordinated co-location: With this scenario two networks are arranged so that the network grid shift is 100%. Practically, this scenario would not be expected in real situations. Its only purpose is to evaluate impact on coexistence due to different network deployment situations.

- Coordinated co-location: With this scenario two network are arranged so that the network shift is 0%. This would correspond to that both operators are using the same network grid with common location for base stations.

The definition of network grid shift used in RAN4 is visualised in Figure 2.3-1.

Figure 2.3-1: Network grid shift

For uncoordinated deployments the isolation between base stations can be derived based on established array antenna models and propagation models using traditional link budget calculations. While for coordinated deployments models capturing near-field effects would be required. Sweeping grid shift from 100% to 0% would require careful considerations with respect to the condition for where far-field models are appropriate, see aspects mentioned in section 2.4. Since new features requires more advanced coexistence evaluation including aspects related to BS-to-BS isolation (e.g., interference from other BS from adjacent network or the same network) would need a new model to be adopted for proper handling of the transition between far-field to near-field condition.

Since both categories are evaluated in RAN4 models and both situations are required. Near-field propagation models for evaluating BS-to-BS isolation is currently not established by RAN4. Based on antenna and circuit theory accurate models tend to be very complex and processing heavy. Therefore, RAN4 should evaluate the possibility to define near-field models tailored to evaluate BS-to-BS isolation between two AAS base stations.

Observation 4: RAN4 need to study more relevant and accurate models to evaluate BS-to-BS isolation for both coordinated, uncoordinated and intermediate co-location scenarios.

With proper models for coordinated network deployment the BS-to-BS isolation between AAS base stations in the edge-to-edge arrangement defined in Rel-15 (in TS 38.104, subclause 4.9) and scenarios where aggressor and victim are located at different sites in close vicinity can be evaluated.

  - Definition of isolation

It can be noticed that measurement results submitted to RAN4 typically refer to Array (passive antenna) to single element or sub-array (on AAS side) or in few cases Array on aggressor side to Array on victim side. Depending on isolation considered the results will significantly differ. It can also be noticed that depending on considered requirement background a specific definition of BS-to-BS isolation may need to be considered.

In previous discussion some confusion about the fundamental core of the definition was identified. When evaluating BS-to-BS isolation for AAS base station different kind of isolation can be considered depending on what aspect or requirement that is considered.

Below some common type of BS-to-BS isolation definitions are listed:

- Sub-Array to Sub-Array isolation (SA-to-SA)

- Array to Sub-Array (A-to-SA)

- Array to Array Isolation (A-to-A)

From a radio systemization perspective, it is only relevant to consider the isolation seen at the Transceiver Array Boundary (TAB) interface. This means that the isolation between induvial elements is not relevant since is many cases elements are combined into sub-arrays in the Radio Distribution Network (RDN).

As indicated in section 2.1, A-to-A isolation is relevant for co-location spurious emission requirement level derivation. While for example co-location receiver blocking A-to-SA may be more relevant.

Evaluating wrong type of isolation from measurements and then applying the results for requirement derivation requiring a different type of isolation would risk to not capture the full consequences for radio electronics. This would risk deriving incorrect requirement levels.

Observation 5: When evaluating BS-to-BS isolation from simulation results and measurement results, it is essential to be clear on isolation definition and the purpose of the result. The requirement derivation for different requirements may depend on different isolation definitions.

With proper model simulations can produce results for all types of isolation. For measurement results it may sometime be difficult to measure A-to-A isolation since the array response is achieved in the digital domain in base band.

# Conclusion

Define co-location requirements based on a fixed BS-to-BS isolation value of 30 dB have worked for base stations operating below 3500 MHz. Considering AAS base station with a large antenna (more electrically steered elements/sub-arrays) and extending the frequency support to include frequencies up to 7125 MHz it is essential to study BS-to-BS isolation including statistical properties and methodology to derive relevant requirement levels.

Experiences from previous work in the area gives that RF core and conformance should be handled together.

In this contribution we observe following:

Observation 1: The assumed BS-to-BS isolation of 30 dB have direct impact on required RF filter suppression for any base station implementation. For AAS BS operating above 3500 MHz the filter suppression has direct impact on the physical size of the RF filter and corresponding routing losses, volume, weight and cost. Since the current assumption was derived for GSM operating at 900 MHz it would be reasonable to further study the relevance for AAS base stations operating above 3500 MHz.

Observation 2: Using a fixed value for BS-to-BS isolation would risk to over specify spurious emission requirements for the situation where the frequency offset is large between the aggressor base station and victim base station. For AAS base stations it would be reasonable to further study statistical properties due beamforming and how they affect BS-to-BS isolation. Also, aspects related to victim/aggressor frequency offset would be relevant to study. It could be transmission mismatch, different physical size of victim and aggressor antenna aperture that have impact on BS-to-BS isolation.

Observation 3: Based on statistical simulation results RAN4 need to establish criteria for requirements derivation.

Observation 4: RAN4 need to study more relevant and accurate models to evaluate BS-to-BS isolation for both coordinated, uncoordinated and intermediate co-location scenarios.

Observation 5: When evaluating BS-to-BS isolation from simulation results and measurement results, it is essential to be clear on isolation definition and the purpose of the result. The requirement derivation for different requirement may depend on different isolation definitions.

From the observations we propose:

Proposal 1: Since the technical background for 30 dB BS-to-BS isolation is obsolete for large array AAS base station operating above 3500 MHz RAN4 should create a study with the intention to better describe relevant co-location scenarios and evaluate expected BS-to-BS isolation based on simulations.

Since the time allocated in current Rel-19 BS RF evolution WI is very limited, it would be appropriate for RAN4 to discuss and agree to justification and objectives for work related to improve co-location aspects in Rel-19, rather than solving technical issues or initiating simulation and measurement activities. That would put RAN4 in a good position before work start in Rel-20 to study co-location aspects.

# References

[1]	R4- 2404984, “Overview of BS OTA co-location requirements”, Ericsson, RAN4#110-bis

[2]	R4-2408086, “Evolution of BS co-location requirements”, Ericsson, RAN4#111

[3]	R4-2412912, “On the topic of BS RF evolution related to co-location requirements”, Ericsson, RAN4#112

[4]	R4-2411872, “On general aspects related to BS RF evolution WI”, Ericsson