3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415977

Hefei, China, Oct 14 –18, 2024

Source:	Nokia

Title:	Discussion on NR NTN UE RF requirement for Ku band

Agenda item:	6.9.4

Document for:	Approval

Introduction

WI on the introduction of Ku Band for NR NTN has been approved in [1]. RAN4 has discussed UE and SAN RF aspects since RAN4#112 meeting [3][4] and the following WF has been agreed for NTN Ku band UE RF.

Issue 1-1-1: Supported VSAT Types in the NR NTN Ku band work - The current NTN VSAT types in 38.101-5 are considered in this Ku band work item - Both mechanical and electrically steered antennas can be considered - The discussion on down selecting between FR1/FR2 approaches for the NTN Ku band will not be impacted by this WF. Issue 1-1-2: Open issues to be discussed - Companies are encourage to provide the following information for the NTN Ku band in the next meeting: - Equivalent antenna aperture for the Ku band VSAT - Aspects on whether the common Tx/Rx antenna or separate Tx/Rx antenna can be assumed for VSAT in Ku band - Parameterized array antenna model for electronic steering antenna (the table 8.1.1-1 in TR 38.921 can be referred as an example format) - The range of TRP/EIRP for the Ku band - The UL/DL budget - Regulatory requirements

In this contribution, we discuss issues on UE RF requirement.

Discussion

Supported VSAT Types

NTN VSAT UE types are defined in TS 38.101-5 Table 9.2.1.0-1 for FR2-NTN and are currently applicable to Ka bands n512, n511 and n510. OTA-based UE RF requirements have been introduced to these VSAT UE types.

| UE class | UE type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 2 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 5 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| Note 1: The UE types are assuming UE has only one antenna beam towards one satellite at a given time in this release. Note 2: The Mobile VSAT communicating with non-GSO is not considered in this release. |  |  |

The scope of the WI is to specify the requirement for NTN VSAT types in 38.101-5. All UE types 1-5 can be introduced to Ku band.

All UE types 1-5 in TS 38.101-5 can be introduced to Ku band.

A few satellite operators have indicated that Mobile VSAT with NGSO is also necessary and the WID is considered updated in December [4]. As the coexistence requirement may not differ much from Fixed VSAT to Mobile VSAT, this may be incorporated easily in the coexistence analysis if the WID scope is updated with a new VSAT type.

A new mobile VSAT type with NGSO may be later incorporated if the WID scope is updated.

The existing UE capability signaling, ntn-VSAT-AntennaType-r18, ntn-VSAT-MobilityType-r18 and ntn-ScenarioSupport-r17, can be reused for the new UE type.

During the online discussion RAN4#112, one satellite operator indicated both OTA and conducted requirements were of interest for Ku band. The conducted requirement in TS 38.101-5 has been introduced to L/S-bands, however, high directional antenna has not been considered in the earlier coexistence analysis as well as in RF requirement derivation [2]. It is for further study whether the conducted requirement for L/S-bands can simply be reused for Ku band. The UE type is not specified in the current specifications for FR1-NTN bands as only the isotropic antenna is assumed. Instead of the UE type, only the power class is defined for FR1-NTN. No antenna requirement has been specified in the conducted requirement so far in RAN4. It is for further study if such approach is fine for Ku band VSAT.

It is for further discussed if both conducted and OTA requirements are specified for Ku band.

Currently, conducted requirements for FR1-NTN are specified in clauses 6, 7 and 8 in TS 38.101-5. OTA requirements for FR2-NTN are specified in clauses 9 and 10 in TS 38.101-5. In our view, FR1 numerology is preferred for Ku band [6], therefore, the clauses 9 and 10 should cover also FR1-NTN. For example, FR1 based channel bandwidths need to be included in these clauses.

FR1 numerology should be captured in the OTA requirement in TS 38.101-5.

Equivalent antenna aperture for VSAT

The equivalent aperture diameter is agreed as 60 cm for coexistence analysis [5][7]. It is for further study if another UE characteristics is introduced.

Common or separate antenna for Tx/Rx

Common antenna is typical implementation for Ku band VSAT. Tx/Rx isolation is expected large enough for VSAT and the receiver desensitization due to Tx leakage is minimized. For low-cost terminals without enough isolation, half duplex is an option to avoid the isolation issue. For the sake of simplification, we can first focus on the Tx and Rx requirement separately assuming separate antenna as done for Ka band. The duplexer insertion loss and isolation can be considered if the typical duplexer data for Ku band VSAT is provided in RAN4.

Common antenna is the basic assumption for VSAT. The impact to UE RF requirement can be considered if typical duplexer data in Ku band VSAT is provided.

Array antenna model

VSAT parameters for Ku band are provided in [7] in the following table and proposed for the coexistence analysis [5].

| Characteristics | VSAT (Note 1) |
| --- | --- |
| Frequency band | Ku-band (i.e. 14 GHz UL and 12 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 36.7 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 35.1 dBi |
| NOTE 1: VSAT terminal characteristics could be implemented with phased array antenna |  |

The antenna model in 6.4.1 of TR 38.811 is based on the reflector antenna. The flat panel phased array antenna is widely used for Ku band terminals. For the coexistence study, the similar characteristics as the reflector antenna should be assumed also for the array antenna. UE RF requirement can be separately discussed for the array antenna, as we have done for Ka band.

The coexistence analysis is based on the reflector antenna characteristics, which may be reused for the phased array antenna.

Range of TRP/EIRP and UL/DL budget

VSAT parameters are provided in [7] and agreed for the coexistence analysis. TRP and Tx antenna gain (in the table above) are in the following.

- TRP = 33 dBm

- Tx Antenna gain = 35.1 dBi

Therefore, the simple addition of TRP and gain leads to the following EIRP.

- EIRP = 68.1 dBm.

In Ka band work, the maximum EIRP was agreed from the coexistence parameters, thus 68.1 dBm could be considered for Ku band.

The assumption for TRP is 33 dBm in Ku band (in the above table), which is the same as Ka band VSAT type 1 (mechanical steering). We have added 2 dB margin to allow some implementation flexibility in the maximum TRP requirement for Ka band. Thus, 35 dBm maximum TRP can be considered for Ku band, too. It is FFS for electronic steering as array antenna parameter needs to be agreed first.

After we analyze the UL link budget, we can further discuss the minimum peak EIRP requirement for Ku band.

It is for further study if the agreed parameters for the coexistence analysis is used to derive TRP/EIRP requirement.

Regulatory requirements

Requirement for VSAT communicating with NGSO for Region 1 can be found in EN 303 980 and EN 303 981, which should be introduced to TS 38.101-5.

On-axis spurious requirement can be specified in the following according to the EN standards.

Table 9.5.3.2.2-1: On-axis spurious limits in “Emissions disabled” and “Carrier-off” states

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 27.5 – 30.0 | 19 | 1 |
| 14.0 – 14.5 | 19 | 1 |

Table 9.5.5.2.2.3-1: On-axis spurious limits in “Carrier-on” state - outside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| 27.5 – 30.0 | 4, 5 | 44 - K (NOTE) | 1 |
|  | 1, 2, 3 | 4 - K (NOTE) | 1 |
| 14.0-14.5 | TBD | 14 – K (NOTE) | 1 |
| NOTE: K=10log(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [17] for Mobile VSAT or sub-clause 4.2.4.2 in [18] for Fixed VSAT. The manufacturer shall declare the value of N. |  |  |  |

Off-axis spurious requirement for Ku band can specified in the following according to the EN standards.

Table 9.5.3.3.2-1: Radiated field strength limits at 10m from the NTN VSAT

| Frequency range (MHz) | EIRP Limit (dBμV/m) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 30 – 230 | 30 | 120 |
| 230 – 1000 | 37 | 120 |

Table 9.5.3.3.3-1: Off-axis spurious limits in “Emissions disabled” state

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 1.0 – 2.0 | -38 | 1 |
| 2.0 – 10.7 | -32 | 1 |
| 10.7 – 21.2 | -26 | 1 |
| 21.2 – 60.0 | -20 | 1 |

For VSAT communicating with NGSO for Region 1, on-axis and off-axis spurious emission requirements in EN 303 980 and EN 303 981 shall be specified in TS 38.101-5.

Conclusion

The following Observations and Proposals were made:

Observation 1: All UE types 1-5 in TS 38.101-5 can be introduced to Ku band.

Observation 2: A new mobile VSAT type with NGSO may be later incorporated if the WID scope is updated.

Proposal 1: It is for further discussed if both conducted and OTA requirements are specified for Ku band.

Observation 3: FR1 numerology should be captured in the OTA requirement in TS 38.101-5.

Observation 4: Common antenna is the basic assumption for VSAT. The impact to UE RF requirement can be considered if typical duplexer data in Ku band VSAT is provided.

Observation 5: The coexistence analysis is based on the reflector antenna characteristics, which may be reused for the phased array antenna.

Observation 6: It is for further study if the agreed parameters for the coexistence analysis is used to derive TRP/EIRP requirement.

Proposal 2: For VSAT communicating with NGSO for Region 1, on-axis and off-axis spurious emission requirements in EN 303 980 and EN 303 981 shall be specified in TS 38.101-5.

- References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, RAN#104, Shanghai, China, 17 – 20 June 2024.

- TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

- R4-2413415 Topic summary for [112][315] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#112

- DRAFT Meeting Report for TSG RAN WG4 meeting: 112

- R4-2413519 Way Forward for [112][314] NR_NTN_Ku_Band_General, RAN4#112

- R4-2415976 Discussion on NR NTN system parameters for Ku band, Nokia, RAN4#112-bis

- R4-2411507 Ku Band Co-existence, Intelsat, RAN4#112