3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415978

Hefei, China, Oct 14 –18, 2024

Source:	Nokia

Title:	Discussion on NR NTN SAN requirement for Ku band

Agenda item:	6.9.5

Document for:	Approval

Introduction

WI on the introduction of Ku Band for NR NTN has been approved in [1] and one objective is to specify RF requirement for SAN in the following.

- Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band.

RAN4 discussed SAN RF aspects in RAN4#112 meeting [3][4][5][6][7]. The following WF was agreed for NTN Ku band SAN [8].

Issue 2-1-1: Supported SAN classes for NR NTN Ku band - The supported SAN classes for NR NTN Ku band include GEO, LEO-600, and LEO-1200.

Furthermore, coexistence scenarios were agreed in [9] according to [10], in which SAN parameters are proposed in the following.

Ku Band Satellite Parameters

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 14 GHz for UL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | FFS dBW/MHz | FFS dBW/MHz | FFS dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.295 deg | 2.948 deg | 2.948deg |
| Satellite beam diameter (Note 2) |  | 184 km | 62 km | 31 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 12 GHz for DL ) | 5 m | 0.5 m | 0.5 m |
| G/T |  | 28.3 dB K-1 | 8.3 dB K-1 | 8.3 dB K-1 |
| Satellite RX max Gain |  | 55.4 dBi | 35.4 dBi | 35.4 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite. NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

In this contribution, we discuss issues on SAN requirement.

Discussion

RF requirement for SAN is specified in TS 38.108, where three requirement types 1-H, 1-O and 2-O are currently used. As our preference is to use FR1-NTN numerology for Ku band [11], it is proposed to consider 1-O and 1-H requirement for Ku band SAN.

It is proposed to consider 1-O and 1-H requirement as baseline for Ku band SAN.

The requirement for 1-O and 1-H type SANs are specified for SCS 15 kHz, 30 kHz and 60 kHz. As the WID [1] mentions the priority 1 option for Ku band FR1-NTN numerology (though not limited to) to support 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing. If we agree FR1-NTN with only 30 kHz SCS, other SCS (e.g., 15 kHz and 60 kHz) do not apply to Ku band.

It is for further discussion if only FR1-NTN 30 kHz requirement in TS 38.108 is applicable to Ku band SAN.

The impact of Ku band introduction to SAN specifications was discussed in RAN4#112 [5][6][7]. The main open issue for SAN transmitter and receiver requirement is to derive ACLR and ACS requirement through the coexistence analysis in Ku band. Other than ACLR and ACS, the specifications impact is relatively straightforward by scaling some requirements according to the additional channel bandwidths, since most SAN requirements are already specified in band agnostic way.

Except for ACLR and ACS (and other requirements scaling needed for additional channel bandwidth), the exiting SAN transmitter and receiver specifications in TS 38.108 for FR1-NTN are reused for Ku band.

Conclusion

The following Observations and Proposals were made:

Proposal 1: It is proposed to consider 1-O and 1-H requirement as baseline for Ku band SAN.

Proposal 2: It is for further discussion if only FR1-NTN 30 kHz requirement in TS 38.108 is applicable to Ku band SAN.

Proposal 3: Except for ACLR and ACS (and other requirements scaling needed for additional channel bandwidth), the exiting SAN transmitter and receiver specifications in TS 38.108 for FR1-NTN are reused for Ku band.

- References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, RAN#104, Shanghai, China, 17 – 20 June 2024.

- TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

- R4-2413415 Topic summary for [112][315] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#112

- DRAFT Meeting Report for TSG RAN WG4 meeting: 112

- R4-2411122 Discussion on SAN RF core requirments for Ku-band for NR NTN, CATT, RAN4#112

- R4-2411191 NTN Ku-band – SAN RF requirements overview, Ericsson, RAN4#112

- R4-2411861 Discussion on SAN RF requirements for NR NTN Ku band, ZTE, Sanechips, RAN4#112

- R4-2413525 Way Forward for [112][315] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#112

- R4-2413519	Way Forward for [112][314] NR_NTN_Ku_Band_General, RAN4#112

- R4-2411507 Ku Band Co-existence, Intelsat, RAN4#112

- R4-2415976 Discussion on NR NTN system parameters for Ku band, Nokia, RAN4#112-bis