3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415979

Hefei, China, Oct 14 –18, 2024

Source:	Nokia

Title:	UE RF requirement for HAPS

Agenda item:	5.21.2

Document for:	Approval

Introduction

RAN#105 has approved the introduction of additional HAPS operating bands for Rel-19 [1] according to WRC-23 decision to include more frequency ranges for HAPS, i.e., 694-960 MHz and 2500-2690 MHz. We discuss the UE RF requirement and UE parameters for coexistence study in the additional HAPS operating bands.

Discussion

As the concept of HAPS is summarized in [2][3][4], HAPS provides the IMT network coverage of remote areas that may not be covered by the conventional IMT network on the ground. The connectivity between UE and HAPS base station is provided by the same air interface as the one between UE and IMT base station on the ground. Therefore, UE in HAPS network is not different from UE in the ground network. The same UE can be connected to either the ground network or HAPS network without any additional feature for HAPS.

Band n1 has been already specified in TS 38.104 clause 5.2 as a HAPS operating band in Rel-17. The assumption has been that no new requirement for UE is necessary since the existing UE requirement for band n1 also is applicable to HAPS. Indeed, no change specific to HAPS has been introduced to TS 38.101-1 for band n1. Further, no UE spec is included in the affected documents of this WI [1]. Therefore, for the additional HAPS operating bands, no change or addition to the UE RF requirement is expected.

No change or addition to the UE RF requirement in TS 38.101-1 is necessary for the additional HAPS bands.

Furthermore, UE parameters for HAPS coexistence study for the 2 GHz band are found in [3], which are the same as UE parameters of IMT for the ground network. The ones for the new additional HAPS frequency ranges are found in [2], which are in principle the same as UE parameters for 2 GHz, i.e., UE parameters for coexistence study of additional HAPS bands are the same as ones for TN.

UE parameters for coexistence study of additional HAPS bands are the same as ones for TN.

As no discussion on UE RF aspects is expected, RAN4 shall not further discuss UE requirements under this WI and remove this agenda at next meeting as it seems not needed.

RAN4 shall not further discuss UE requirements under this WI.

- References

- RP-242373 New WID Proposal: Introduction of HAPS additional operating bands, SoftBank Corp., Deutsche Telekom, NTT DOCOMO, KDDI, RAN#105

- ITU-R WP5D#43 [HIBS-CHARACTERISTICS] (Annex 4.10 in Doc. 5D/1668)

- TR 38.863 Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects (Release 18)

- TR 38.811 Study on New Radio (NR) to support non-terrestrial networks (Release 15)