3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415980

Hefei, China, Oct 14 –18, 2024

Source:	Nokia

Title:	Discussion on additional operating bands for HAPS

Agenda item:	5.21.3

Document for:	Approval

Introduction

RAN#105 has approved the introduction of additional HAPS operating bands for Rel-19 [1] according to WRC-23 decision to include more frequency ranges for HAPS, i.e., 694-960 MHz and 2500-2690 MHz. The objectives of the core part of the work item are in the following.

The core part of this work item includes: - Assess whether existing ACLR and ACS requirements can be reused for the newly proposed HAPS bands. - The frequency ranges other than the 694-960 MHz should be prioritized. - Specify the following NR bands for HAPS operation in TS 38.104. - n2, n3, n5, n7, n8, n20, n25, n26, n28, n34, n38, n39, n41, n67, n85, n90 Note 1: HAPS can be supported using TN functionality available in Rel-17. Note 2: The frequency range of the downlink in bands n2 and n25 is limited to 1930-1980 MHz. The frequency in band n41 is limited to 2500-2690 MHz.

Discussion

Band n1 has been already specified in TS 38.104 clause 5.2 as a HAPS operating band in Rel-17. The coexistence analysis has been made in TR 38.863 [2]. The required ACLR and ACS for HAPS to coexist with terrestrial networks and other HAPS have been concluded the same as the requirement for TN wide are base stations as specified in clause 4.4 of TS 38.104. Therefore, HAPS RF requirements for band n1 are simply reused from TN base station requirement.

The frequency ranges for HAPS are extended to include 694-960 MHz, 1710-2170 MHz and 2500-2690 MHz in WRC-23. Coexistence analysis for additional frequency ranges is required. HAPS parameters including new frequency ranges can be found in [4], where most parameters are the same as 2 GHz band used in TR 38.863 [2]. However, a few HAPS deployment related parameters are slightly different such as area radius for 2500-2690 MHz and the platform antenna tilt for 694-960 MHz. Furthermore, the platform EIRP is common to all frequency ranges however, it is slightly different from what has been used in the previous coexistence study. They are highlighted in yellow in the following table.

Table 1 Deployment related parameters of HIBS [4]

| No. | Parameter | Band 1 (694-960 MHz) | Band 2 (1 710-1 980 MHz 2 010-2 025 MHz 2 110-2 170 MHz) | Band 3 (2 500-2 690 MHz) |
| --- | --- | --- | --- | --- |
| 1 | Network topology and characteristics |  |  |  |
| 1.1 | BS density or ISD | 1 BS/HIBS area |  |  |
| 1.2 | HIBS area radius | 100 km | 100 km | 90 km |
| 1.3 | HIBS Network Configuration (Duplex Mode)(1) | FDD/TDD(1a) | FDD/TDD(1b) | FDD/TDD(1c) |
| 2 | Base station characteristics/Cell Structure |  |  |  |
| 2.1 | HIBS Platform Altitude | 18-25 km(3) |  |  |
| 2.2 | Number of cells/HIBS | 7 |  |  |
| 2.3 | Frequency reuse | 1 |  |  |
| 2.5 | HIBS Platform Antenna pattern | Recommendation ITU-R M.2101(2) |  |  |
|  | Element gain | 8 dBi |  |  |
|  | Horizontal/vertical 3 dB beamwidth of single element | 65º for both H/V |  |  |
|  | Horizontal/vertical front-to-back ratio | 30 dB for both H/V |  |  |
|  | Antenna polarization | Linear/±45 degrees |  |  |
|  | Antenna array configuration (Row × Column) | 2 × 2 elements (1st layer cell), 4 × 2 elements per cell (2nd layer cell) |  |  |
|  | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for both H/V |  |  |
|  | Ohmic losses | 2 dB |  |  |
| 2.6 | HIBS Platform Antenna tilt | 90º (1st layer cell), 33º (2nd layer cell) | 90º (1st layer cell), 23º (2nd layer cell) | 90º (1st layer cell), 23º (2nd layer cell) |
| 2.7 | HIBS Conducted power per antenna element | 37 dBm (1st layer cell), 34 dBm (2nd layer cell) |  |  |
| 2.8 | HIBS Platform e.i.r.p./cell | 55 dBm (1st layer cell), 58 dBm (2nd layer cell) |  |  |
| 2.9 | HIBS Platform e.i.r.p. Spectral Density/cell | 42 dBm/MHz (1st layer cell), 45 dBm/MHz (2nd layer cell) |  |  |

HAPS parameters for 694-960 MHz, 1710-2170 MHz and 2500-2690 MHz are mostly common for the three frequency ranges however a few parameters such as the area radius for 2500-2690 MHz and the platform antenna tilt for 694-960 MHz.

HAPS EIRP in the recent LS in ITU-R WP5D is 1.8 dB lower than the one used in coexistence study in TR 38.863.

It is for further evaluation, how much impact is expected to the existing coexistence analysis in TR 38.864 [2] by these changes. The area radius is smaller for 2500-2690 MHz. A larger path loss than 2 GHz is in general expected. They may compensate each other so the coexistence requirement may not be that different from 2 GHz.

The antenna tilt is larger for 649-960 MHz, which may be for reducing the interference to other HAPS, as the path loss for 694-960 MHz is in general smaller than 2 GHz. They may help HAPS-to-HAPS coexistence, however, the impact to the coexistence with TNs may not be straightforward.

EIRP in Table 1 is 1.8 dB lower than the one in TR 38.863 [2]. This may reduce the interference to TNs and other HAPS.

It is for further discussion if the HAPS system and deployment parameters different from 2 GHz would affect conclusion to existing co-existence analysis.

Other than these HAPS parameters, the general methodology for the coexistence analysis should be reused from TR 38.863 [2]. The propagation model is dependent on the frequency and thus it should be considered for the analysis of the new frequency ranges 694-960 MHz and 2500-2690 MHz. The propagation model used in the analysis for 2 GHz is found in TS 38.811 [3]. ITU-R has developed a recommendation P.1409 for the propagation model applicable to HAPS over 0.7 GHz. For the frequency range 1710-2170 MHz, the previous study for 2 GHz should be applicable. It is for further discussion whether and if so how RAN4 should update the propagation model in TS 38.811 [3] for 694-960 MHz and 2500-2690 MHz.

It is for further discussion if RAN4 shall revise the propagation model for HAPS in 694-960 MHz and 2500-2690 MHz.

Conclusion

The following Observations and Proposals were made:

Observation 1: HAPS parameters for 694-960 MHz, 1710-2170 MHz and 2500-2690 MHz are mostly common for the three frequency ranges however a few parameters such as the area radius for 2500-2690 MHz and the platform antenna tilt for 694-960 MHz.

Observation 2: HAPS EIRP in the recent LS in ITU-R WP5D is 1.8 dB lower than the one used in coexistence study in TR 38.863.

Proposal 1: It is for further discussion how the HAPS system and deployment parameters different from 2 GHz are affected.

Proposal 2: It is for further discussion if RAN4 shall revise the propagation model for HAPS in 694-960 MHz and 2500-2690 MHz.

- References

- RP-242373 New WID Proposal: Introduction of HAPS additional operating bands, SoftBank Corp., Deutsche Telekom, NTT DOCOMO, KDDI, RAN#105

- TR 38.863 Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects (Release 18)

- TR 38.811 Study on New Radio (NR) to support non-terrestrial networks (Release 15)

- Document 5D/TEMP/64R3, 4D/103-E, 7B/64-E REPLY LIAISON STATEMENT TO WORKING PARTIES 4C AND 7B FOR WRC-27 AGENDA ITEMS 1.12, 1.13, 1.14, 1.15, Technical and operational characteristics of HIBS for sharing and compatibility studies in preparation for WRC-27, Working Party 5D

- Rec. ITU-R P.1409-3 Propagation data and prediction methods for systems using high altitude platform stations and other elevated stations in the stratosphere at frequencies greater than about 0.7 GHz

- R4-2110690 HAPS uplink simulation results and simulation alignment data, Nokia, RAN4#99-e

- R4-2118615 HAPS simulation assumptions and results for coexistence study, Nokia, RAN4#101-e

- R4-2201077 HAPS coexistence simulation assumptions, Nokia, RAN4#101-bis-e

- R4-2202992 Simulation assumptions for HAPS co-existence, Nokia, RAN4#101-bis-e

- R4-2205556 HAPS coexistence simulation results, Nokia, RAN4#101-bis-e

- R4-2205558 HAPS BS ACLR and ACS requirements, Nokia, RAN4#101-bis-e