3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415982

Hefei, China, Oct 14 –18, 2024

Source:	Nokia

Title:	IMT parameters: General system and UE aspects

Agenda item:	6.2.1

Document for:	Approval

Introduction

ITU-R WP 5D has requested that external organizations provide IMT parameters for 4400-4800 MHz, 7125-8400 MHz and 14800-15350 MHz for the sharing studies towards WRC-27 [1]. RAN has agreed SI FS_4400_15350MHz_NR and tasked RAN4 to work on these parameters [2].

RAN4 started the work in RAN4#110-bis and LS replies for 4400-4800 MHz and 7125-8400 MHz have been already agreed [11][12]. The open issues regarding the UE parameters for 14800-15350 MHz was agreed in the way-forward [13].

Discussion

Typical channel bandwidth

Issue 1-2: Channel Bandwidth

Agreement:

- For LS about 15GHz, choose the single value out of 100MHz, 200MHz or 400MHz as baseline with understanding that the other channel bandwidths are not precluded.

The agreement about the typical channel bandwidth in RAN4#112 [14] is in the following for 14800-15350 MHz.

The required channel bandwidth for IMT-2030 depends on the required data rate and achievable spectrum efficiency. The values of 50, 100 or 200 Gbps are mentioned in IMT-2030 Framework [3] as possible examples for the peak data rates. This is 2.5 to 10 times greater than IMT-2020 peak data rate (10 Gbps for Uplink and 20 Gbps in Downlink) [4]. IMT-2030 Framework targets 1.5 times higher averages spectrum efficiency [3]. As it is not easy to significantly improve the peak spectrum efficiency for IMT-2030, much wider total bandwidth than IMT-2020 would be needed to achieve the target peak data rate.

Therefore, it is important to assume a wider channel bandwidth than IMT-2020 to achieve higher peak data rate for IMT-2030. Therefore, we propose to consider typical maximum channel bandwidth 200-400 MHz for the frequency range for 14800 – 15350 MHz.

200-400 MHz is considered as typical maximum channel bandwidth for 14800 – 15350 MHz.

Typical signal bandwidth

Issue 3-3: Typical signal bandwidth

Agreement:

- Quote formula of RBs * SCS without number of RBs

The following agreement is made for typical signal bandwidth for 7125-8400 MHz [14] and included in the LS [12]. For 14800 – 15350 MHz, a similar approach can be made with the reference to TS 38.922.

Typical signal bandwidth is given by a formula of RBs * SCS without number of RBs with the reference to the TR 38.922.

UE Noise Figure

RAN4 earlier informed [9-13] dB as the IMT parameters of the frequency range 6.425-10.5 GHz for the sharing study toward WRC-23 [8]. We understand that the conclusion without square bracket was later summarized in TR 38.921 [6], where Typical UE NF value for 10 GHz band is 9 dB.

TR 38.921 Table 6.2.1-1: Typical noise figure for 7 – 24 GHz example frequencies

| Example frequency (GHz) | Typical NF values for NR BS (dB) | Typical NF values for NR UE (dB) |
| --- | --- | --- |
| 10 | 7 | 9 |
| 15 | 8 | 10 |
| 20 | 9 | 10 |

Issue 3-3: UE noise factor for both simulation and LS reply

- FFS the below options:

  - Option 1: 11 dB i.e. follow simulation assumption

  - Option 2: 8 dB

  - Option 3: More than 11 dB

In the WF [13] in RAN4#112, the following options were discussed but without conclusion. 11 dB has been already agreed for the coexistence simulation parameter in RAN4#111.

These options are worse than the typical NF value agreed earlier in TR 38.921. We think this additional margin is not really needed and there may be some confusions about typical NF and the worst NF including all industrial margins (such as in the extreme condition with all production variations). However, for the sake of progress, we propose to use it for 14800-15350 MHz in the reply LS to WP5D.

Proposal 4: It is proposed that NF=11 dB for 14800 – 15350 MHz in the reply LS to WP5D.

Maximum UE output power

In the WF [9] in RAN4#111, two architecture assumptions are agreed in the following for 14800-15350 MHz, where the maximum output power assumptions are also clarified.

Agreement:

- For simulations, consider both options.

  - FR1 like

    - RX diversity gain: [0 or 5] dB assuming 4RX

    - TX: 0dBi omnidirectional

    - Max Power: 23 dBm, 26dBm

  - FR2 like

    - Two panels (one in each direction) 2x2 antenna

      - 5dBi element gain. Array gain comes on top

    - Power: 23 dBm as max TRP

For FR1 option, we propose to follow the same as 7125-8400 MHz, i.e., only 23dBm is indicated in LS as typical value of maximum output power and a generic statement about referring to the TR mentioning other powers.

For FR2 option, we propose to indicate the same information as the coexistence simulation assumption. mentioning the TR if other powers are included.

For the maximum UE output power of FR1 option, 23 dBm is indicated in the LS as a typical value, mentioning the TR for other power classes for FR1 option.

For the maximum UE output power of FR2 option, the agreed coexistence simulation assumptions are informed in the LS, mentioning the TR for other power classes if they are present in the TR.

UE architecture FR1 vs FR2 for 14800 – 15350 MHz

## Sub-topic 1-1: UE type

Issue 3-1: UE Type assumption for comparison of FR1 Vs FR2 like UE performance

- FFS the below options for UE types:

  - Revisit RAN4#111 UE type assumption with the following options

  - Companies are encouraged to clarify their assumption when they provide simulation results.

  - FR1-like UE:

    - Option 1: 1TX assumption with omnidirectional antenna

    - Option 2: Two discrete elements following Table 1-1 below with maximum-gain across elements selection criterion.

    - Option 3: For [N] Tx/Rx, [G] dBi gain for link of interest regardless the AoA, and 0dB for signals from all non-serving co-channel and adjacent-channel links regardless the AoA.

      - Companies to clearly state what value of N and G assumed.

  - FR2-like UE:

    - FR2-likUE: Two panels with 1x2 array at each panel following Table 1-2 below with maximum gain across panels critertion.

## Sub-topic 1-4: UE RF and Antenna parameter update to TR 38.922

Since the discussion on the UE beamforming feasibility is still ongoing, it is recommended to note R4-2411143 for this meeting and continue the discussion in the next two meetings.

For the coexistence simulation, both FR1 and FR2 options were considered in RAN4#111 [9]. More discussion has been suggested by some companies [10] and the following WF was agreed in RAN4#112 [13].

FR2 band such as band n257/n258 is already commercially implemented in UE. Such architecture would be also feasible for 14800-15300 MHz unless the size of antenna array is not a big concern such as in CPE type or vehicular mounted devices. Since the handheld device has a limited space for antenna array, it may be still preferred to consider FR1-like architecture for lower than the existing FR2 bands.

More investigation would be needed to decide which architecture is assumed in the final specifications for IMT-2030. If FR1 is of only interest, we may only specify FR1 requirement and include it in the LS reply to WP5D. If both FR1 and FR2 are of interest, both requirements may need to be specified like in the BS specifications where conducted, hybrid and OTA requirement are all specified as 1-C, 1-H, and 1-O requirements. We may take such approach also for UE specifications if both architectures are of interest by the UE vendors. In that case both architectures may better be informed in the LS reply to WP5D and captured in the TR 38.922.

FR1-like architecture may be more feasible for handheld devices for 14800-15350 MHz. FR2 may not be excluded for other UE types. It may be premature to single out the UE architecture for 14800-15350 MHz.

Unless there is no clear consensus among UE vendors, both FR1 and FR2 architecture assumptions are informed to WP5D in the LS reply and captured in the TR 38.922.

Conclusion

The following Observations and Proposals were made:

Proposal 1: 200-400 MHz is considered as typical maximum channel bandwidth for 14800 – 15350 MHz.

Proposal 2: Typical signal bandwidth is given by a formula of RBs * SCS without number of RBs with the reference to the TR 38.922.

Proposal 3: For the maximum UE output power of FR1 option, 23 dBm is indicated in the LS as a typical value, mentioning the TR for other power classes for FR1 option.

Proposal 4: For the maximum UE output power of FR2 option, the agreed coexistence simulation assumptions are informed in the LS, mentioning the TR for other power classes if they are present in the TR.

Observation 1: FR1-like architecture may be more feasible for handheld devices for 14800-15350 MHz. FR2 may not be excluded for other UE types. It may be premature to single out the UE architecture for 14800-15350 MHz.

Proposal 5: Unless there is no clear consensus among UE vendors, both FR1 and FR2 architecture assumptions are informed to WP5D in the LS reply and captured in the TR 38.922.

- References

- R4-2400333 Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz, ITU-R Working Party 5D

- RP-240787 New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz

- M.2160 Framework and overall objectives of the future development of IMT for 2030 and beyond

- M.2410 Minimum requirements related to technical performance for IMT-2020 radio interface(s)

- R4-1700305 LS on Characteristics of terrestrial IMT systems for frequency sharing/interference analysis in the frequency range between 24.25 GHz and 86 GHz

- TR 38.921 Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz

- R4-2406615 WF on IMT parameters for other frequency ranges

- R4-2103104 LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)

- R4-2410741 WF on IMT parameter study, Ericsson, RAN4#111

- R4-2410738 Ad hoc minutes #2 for [111][127] FS_NR_IMT, Ericsson, RAN4#111

- R4-2410576, Draft LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, RAN4#111

- R4-2414449	LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-27, RAN4#112

- R4-2414284 WF for 15GHz UE parameters, RAN4#112

- DRAFT Meeting Report for TSG RAN WG4 meeting: 112