3GPP TSG-RAN WG4 Meeting #112bis									                     R4- 2415986

Hefei, China, Oct. 14 – 18, 2024

Agenda item:			6.10.2.2

Source:	Ericsson

Title:	Discussion on RF requirement of ATG UE inter-band CA

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, some agreements addressed the general concerns [2]. Part of the agreements regarding the inter-band are shown below:

  - inter-band CA

Issue 3-1-1: clarify the antenna type for DL CA_n3-n39

Agreement:

- Omni-antenna type can be assumed for both band n3 and n39.

Issue 3-1-2: whether new capability is needed for ATG CA

Agreement:

- no need for new capability for UE antenna type for ATG CA

- Antenna type per band could be applied to the same band in the band combination.

Issue 3-2-1: whether to allow n39 UL for CA_n3A-n39A

Agreement:

- Whether to allow n39 UL for CA_n3A-n39A depend on companies’ input.

  - The filter with [X] MHz frequency gap can be considered.

Issue 3-2-1: whether to allow n39 UL for CA_n3A-n39A

Agreement:

- Whether to allow n39 UL for CA_n3A-n39A depend on companies’ input.

  - The filter with [X] MHz frequency gap can be considered.

Issue 3-2-2: BCS for DL CA_n3_n39

Agreement:

- Table 1: BCS for CA_n3A-n39A

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39A | - | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |

  - Rx requirement for inter-band CA_n3A-n39A

Issue 3-4-1: delta RIB for NR CA_n3A-n39A could be used as the baseline

Agreement:

Table 3: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n3-n39 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

Issue 3-4-2: MSD for CA_n3A-n39A

Agreement:

- There is no MSD issue for n3 impact n39.

Issue 3-4-3: OOB exception for CA_n3A-n39A

Agreement:

- No need to specify OOB blocking exception for CA_n3-n39 since the second order intermodulation product of the n3 UL carrier and the CW interfering signal will not overlaps with the n39 DL carrier.

There remain numerous unresolved matters requiring attention, and the upcoming session will delve into the RF requirement for ATG UE inter-band CA.

  - Inter-band CA

### Whether to allow n39 UL for CA_n3A-n39A

Issue 3-2-1: whether to allow n39 UL for CA_n3A-n39A

Agreement:

- Whether to allow n39 UL for CA_n3A-n39A depend on companies’ input.

  - The filter with [X] MHz frequency gap can be considered.

The bands n3 and n39 are defined in the table below.

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |

The CA_n3-n39 is an FDD and TDD inter-band combination, specifically, the upper bound of the n3 DL is next to the lower bound of the n39 without any gap in between. Therefore, the actual deployment scenarios/configuration would impact the potential interference. There was a discussion during the #110 meeting and a WF was reached as shown below [4]:

<Way forward 1>:

Single UL only in band n3 for DL CA_n3A-n39A is assumed.

Essentially, for the inter-band CA, PCell can operate either in FDD band or TDD band. To mitigate the potential interference, if only one UL transmission is assumed in band n3 for DL CA_n3-n39, then FDD band n3 could serve as the PCell and TDD band n39 serve as SCell while n39 is configured as DL to avoid interference with the n3 DL. It would be easier to adopt the following proposal to mitigate the potential interference:

- FDD band n3 serves as PCell and TDD band n39 could serve as SCell while n39 is configured as DL to avoid interference with the n3 DL.

However, it is worth noting that the above discussion was based on the handheld UE architecture. ATG UE is a different type of UE with a larger form factor. Therefore, the isolation is larger than that of handheld UE, which could make the n39 UL feasible for CA_n3A-n39A. This approach needs to be further verified by thoroughly studying the ATG UE architecture.

- The n39 UL may be feasible for CA_n3A-n39A due to its larger form factor and isolation than handheld UE.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	The n39 UL may be feasible for CA_n3A-n39A due to its larger form factor and isolation than handheld UE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	FDD band n3 serves as PCell and TDD band n39 could serve as SCell while n39 is configured as DL to avoid interference with the n3 DL.

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2414322, WF on Rel-19 ATG UE RF requirements

- TS 38.101-1, V18.5.0

- R4-2403719, WF on CA_n3A-n39A MSD and architecture.