3GPP TSG-RAN WG4 Meeting #112bis									                     R4- 2415988

Hefei, China, Oct. 14 – 18, 2024

Agenda item:			6.10.3

Source:	Ericsson

Title:	Discussion on remaining issues of ATG BS supporting CA

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, several agreements were made, including TAE requirements and other RF requirements [2].

#### Issue 1-1:  Whether to preclude DL MIMO for TAE requirements for ATG BS supports CA?

Way Forward：

- DL MIMO is not included for ATG BS supports CA based on the current WID.

  - RAN4 can re-visit it if there are revised WID to include DL MIMO.

#### Issue 1-2: How to handle CA TAE requirements for ATG BS in TS38.104 based on the current WID?

Way Forward：

- FFS on the wordings on the TAE requirements for ATG BS CA in the TS38.104.

  - Only focus on TS38.104 in RAN4 #112-bis meeting.

The following sessions will discuss the remaining issues, i.e. how to handle CA TAE requirements for ATG BS in TS 38.104.

In Rel-18, the ATG BS does not have a designated core requirement. Instead, it follows the existing NR BS requirement; the only exception is the TAE requirement.

In the RAN4 #110bis meeting, it was agreed to reuse the existing NR BS TAE requirements for ATG BS, which supports intra-band contiguous and inter-band CA. In the current WID, the objectives solely address the “RF requirements for support of UL MIMO with 2TX for single CC for UE,” with no additional clarification provided in the latest RAN plenary meeting [3].

Therefore, it is recommended to replace “There is no TAE requirement for ATG BS” with “The TAE requirements for NR BS are applicable for ATG BS, except for MIMO which is not supported for ATG BS.”

- Replace “There is no TAE requirement for ATG BS” with “The TAE requirements for NR BS are applicable for ATG BS, except for MIMO which is not supported for ATG BS.”

# Conclusion

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Replace “There is no TAE requirement for ATG BS” with “The TAE requirements for NR BS are applicable for ATG BS, except for MIMO.”

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2413510, Way Forward for [112][306] NR_ATG_enh

- RP-240839, New WID on Enhancements for Air-to-ground network for NR.