3GPP TSG-RAN WG4 Meeting #112bis									                     R4- 2415991

Hefei, China, Oct. 14 – 18, 2024

Agenda item:			6.1.1.3.2

Source:	Ericsson

Title:	Discussion on the MIMO layers for 6Rx UE

Document for:	Approval

# Introduction

With advancements in hardware and design, modern smartphones increasingly integrate multiple Rx antennas to enhance DL performance. For instance, certain foldable mobile phones have adopted 6Rx antennas. Incorporating 6Rx facilitates a higher peak data rate by supporting more DL MIMO layers compared to 4Rx configurations. On the other hand, due to the higher diversity, it can also help increase the downlink coverage by improving the receiver sensitivity.

In 3GPP Release 18, RAN4 finalized the standardization efforts for enabling 8Rx for CPE/FWA/vehicle/industrial type of UEs. Although 6Rx was proposed and deliberated upon, it was given lower priority during the Rel-18 discussion. To enhance DL coverage and throughput for handheld UEs with larger sizes, RAN4 has approved a new WI to specify requirements for both handheld and FWA UEs [1].

6Rx for handheld and FWA UE

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

The following sections will discuss the objectives specified in the WI, focusing on the 6Rx MIMO layers.

# Support of MIMO layers

The last meeting reached a WF on the MIMO layers for 6Rx, and it mainly clarified the objectives that are not in the scope of the WI [2]:

## Sub-topic 3-1:	General considerations for MIMO layer evaluation for 6Rx UE

Issue 3-1-1: Tightening BS EVM requirement

Agreement:

-	Do not tighten the minimum requirements of BS Tx EVM

-	Further discuss the assumption of Tx EVM for 6-layer performance evaluation.

Issue 3-1-2: 6-Layer Performance Evaluation Assumptions

Way Forward: RAN4 to further discuss and align the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios.

Issue 3-1-4: Performance requirements for 6Rx

Agreement: RAN4 to defer decision after evaluation of 6 MIMO layer performance for handheld and FWA devices.

- Sub-topic 3-2:	6-layer Support

Issue 3-2-1: 6-layer Support

Way Forward: RAN4 to further discuss the following options.

-	Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios.

-	Option 2: 6-layer support is feasible for FWA devices.

-	Option 3: Companies to align on the feasibility criteria and simulation assumptions to evaluate 6-Layer feasibility using the following as a starting point.

-	Feasibility criteria

-	Consider 4-layer vs 6-layer throughput performance.

-	Simulation assumptions

-	Only consider link-level simulation for performance evaluations.

-	Simulation assumptions provided in Annex A as a starting point. Companies to clarify the settings used in the simulation assumptions for BS correlation matrix and Tx EVM.

-	Option 4: 6-layer support is not feasible for handheld UEs.

Issue 3-2-2: 6-layer Support as optional feature

Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature.

As we have pointed out in the introduction, modern smartphones increasingly integrate multiple Rx antennas to enhance DL performance. Numerous simulation results show that supporting a higher number of DL MIMO layers can improve the DL peak throughput.

The number of layers that can be supported is impacted by the radio channel and several other factors, which should be thoroughly studied and analyzed.

One factor is the receive antenna correlation. When antenna elements are closely spaced, the correlation between them increases.  In the previous 8Rx discussions, the objective is to define the requirement for FWA and CPE UEs. Given their larger form factor compared to the handheld UE, the demodulation performance study assumed a low antenna correlation [3]. In the 6Rx WI, the objective is to specify FWA and handheld UE requirements. Although the example bands specified in WI are mid-bands and the antenna size can be relatively compact, we understand the difficulty of integrating 6 antenna elements into the handheld deceive without any correlation.

During the last RAN4 meeting in August, companies discussed the parameter assumption, aiming to do some evaluations to better understand the gain from the 6 layers. We have accomplished some simulations based on the parameter assumptions listed in the WF. Below, we shared our simulation assumption in Table 1, PDSCH performance with fixed rank in Figure 1, and PDSCH performance with adaptive rank in Figure 2.

Table 1 Simulation parameter assumption

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHz |
| SCS |  | 30 |
| Antenna configuration |  | Option 1: 16Tx, 6Rx Option 2: 8Tx, 6Rx |
| Propagation channel |  | Option 2: TDLA30-10 |
| Rank |  | Fixed rank [4,6] |
| MCS |  | Option 1: Adaptive MCS (target BLER 10%) Option 2: MCS13, MCS17 from Table 1 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Rel-15 Type I |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DMRS Configurations | Option 1: [1+1(4L), 2+0(6L)] Option 3: [1+1 (4L), 2+2 (6L)] |
|  | Precoding | Random |
| UE Correlation matrix |  | Option 1: |

Figure 1 PDSCH performance with fixed rank of 4 and 6.

From Figure 1, we can observe that with adaptive MCS but fixed rank configuration, the performance of PDSCH with rank 4 is very close to that of rank 6.

- The performance of PDSCH with fixed rank 4 is close to that of fixed rank 6

Figure 2 PDSCH performance with adaptive rank and MCS.

Figure 2 shows that with adaptive MCS and rank, the performance of 6 layers outperformed that of 4 layers in the higher SNR range.

- The performance of PDSCH with adaptive rank up to 6 is better than that with adaptive rank up to 4 at high SNR.

We have also evaluated the PDSCH throughput performance with fixed MCS/rank conditions and the correlation captured in the 3GPP spec., as shown in 2, where we reuse the simulation assumption discussed in PDSCH demodulation requirements as much as possible. Figure 3 (a) and Figure 4 (a) showed the absolute PDSCH throughput with low and medium antenna correlation conditions, respectively. We have also shown the relative throughputs in Figure 1 (b) and Figure 2 (b) to evaluate the achieved peak throughput. We have also selected TDLA30-10, usually configured for PDSCH demodulation requirements with higher layers and/or MCS.

Table 2	Simulation parameters for evaluation of MIMO layers in 6Rx UE.

| Parameters | Values |
| --- | --- |
| Propagation channel | TDLA30-10 (TDL-A, delay spread 30ns, Doppler frequency 10Hz) |
| Antenna configuration | 8Tx, 6Rx |
| Antenna correlation | low, medium (See TS 38.101-4 Table B.2.3.2.2-1) |
| MCS | QPSK 1/3 (MCS4) 16QAM 1/2 (MCS13) 64QAM 1/2 (MCS17) |
| MIMO layers | 4 and 6 |
| Precoding | Follow PMI with Rel-15 Type-I codebook |
| PDSCH DMRS configuration | Type 1, 1+1 Single symbol for 4 layers Double symbol for 6 layers |
| Receiver type | MMSE-IRC |

| (a) Absolute throughput | (b) Relative throughput |
| --- | --- |

Figure 3	PDSCH fixed MCS/rank link-simulation results with low correlation.

| (a) Absolute throughput | (b) Relative throughput |
| --- | --- |

Figure 4	PDSCH fixed MCS/rank link-simulation results with medium correlation.

With the low antenna correlation condition, as illustrated in Figure 1, the performance with 6 layers achieves higher throughput compared to 4 layers for QPSK, 16QAM, and 64QAM MCS. This demonstrates a performance gain observed in the given scenario.

- With the TDLA30-10 low antenna correlation condition, the performance gain with 6 layers over 4 layers for QPSK, 16QAM and 64QAM is observed.

Under medium antenna correlation conditions, the performance with 6 layers using QPSK can achieve higher throughput, demonstrating a performance gain. For MCS13 and MCS17, no obvious performance gain has been observed.

- With TDLA30-10 medium correlation, the performance gain with 6 layers over 4 layers for low and medium MCS is observed.

The link simulation results demonstrate a performance gain with 6 layers for QPSK, 16QAM, and 64QAM under lower antenna correlation conditions, making it applicable for FWA/CPE type devices.

In the meantime, a performance improvement with 6 layers over 4 layers was also observed under medium antenna correlation for low and medium MCS levels, which could be beneficial for handheld devices.

Another factor is the likelihood of getting a rank 6 channel in a practical deployment scenario. Rank distributions depend on SINR distribution and propagation conditions. For example, the likelihood of getting a higher rank is low in a deployment scenario with a larger ISD and rural or suburban areas. With a relatively small ISD and dense urban area, the probability of getting a higher rank is probably high.

This WI considers both the handheld and FWA/CPE devices, and considering their differences in the form factor, the correlation concerning the real antenna arrangement in the device varies. From the analysis provided above, we could observe that, with low antenna correlation conditions, MIMO 6 layers could yield performance gain compared to 4 layers. Furthermore, support of 6 layers also depends on the likelihood of getting a rank 6 channel in the practical deployment scenarios with different ISDs. Therefore, we made the following proposal.

- Support of MIMO 6 layers should be evaluated considering the impact of antenna correlation and the feasibility and probability of achieving a rank 6 channel in the deployment scenario.

- Support of MIMO 6 layers could be considered as an optional feature.

According to the current RRC signalling, the number of MIMO layers can be configured as 1, 2, 3, 4, 5, 6, 7, 8. However, the existing UE capability signalling only allows a UE to report to 2, 4, and 8-layer DL MIMO [4]. If agreed to be introduced, one potential solution would be to introduce/add a new candidate value, i.e. 6-layer.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	The performance of PDSCH with fixed rank 4 is close to that of fixed rank 6

Observation 2	The performance of PDSCH with adaptive rank up to 6 is better than that with adaptive rank up to 4 at high SNR.

Observation 3	With the TDLA30-10 low antenna correlation condition, the performance gain with 6 layers over 4 layers for QPSK, 16QAM and 64QAM is observed.

Observation 4	With TDLA30-10 medium correlation, the performance gain with 6 layers over 4 layers for low and medium MCS is observed.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Support of MIMO 6 layers should be evaluated considering the impact of antenna correlation and the feasibility and probability of achieving a rank 6 channel in the deployment scenario.

Proposal 2	Support of MIMO 6 layers could be considered as an optional feature.

# References

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4.

- R4-2410751, WF on requirements for 6Rx, AT&T

- R4-2405154, Draft BigCR on 38.101-4 for introduction of 8Rx performance requirements

- TS 38.331, V18.1.0