3GPP TSG-RAN WG4 Meeting #112bis									                     R4- 2415992

Hefei, China, Oct. 14 – 18, 2024

Agenda item:			6.6.2

Source:	Ericsson

Title:	Discussion on UE Rx chains of Fragmented Carriers

Document for:	Discussion

# Introduction

Fragmented carriers in the same band requiring multiple Rx chains may limit the UE CA capabilities if part of complex band combinations with many DL CCs combined with DL MIMO in fragmented operator spectrum holdings. A new RAN4 study item was approved in the RAN #104 meeting with the aim of investigating solutions to address this.

The objectives of the SI are as follows [1]:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

In the last meeting, several agreements were captured in the following WF [2].

#### Issue 2-1-1: Applicability and clarification on the scope

Online agreements:

Agreement:

- The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain

    - The two non-contiguous CCs is still under the current DL non-contiguous CA framework

      - Each fragment is treated as an individual CC

    - The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz.

    - [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands

      - Consider n2/n25, n3, n7, n66, n41, n39 as the example bands.

        - Provide the details related to gap between fragmented DL blocks]

Topic #3: Impacts on UE RF requirements and DL performance

#### Issue 3-1-4: Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements

Recommended WF

- Proposal 1: put off the signalling related discussions until the UE performance aspects are clear.

The following sections will discuss some issues of the number of UE Rx chains of fragmented carriers.

# Discussions

## Reference UE Rx architecture and FFT

The architecture for the colocated non-contiguous CA was assumed to be a separate or partially shared Rx chain, as shown in Figure 1, from which the requirement was derived. Multiple instances of this architecture support DL MIMO and Rx diversity.

In the “fully shared Rx architecture” or single Rx chain with shared LNA, Mixer and LPF, as shown in Figure 1, the single analog LPF needs to cover a wider frequency range encompassing 2 non-contiguous CCs. Using a single LPF to filter out the combined signal of the two non-contiguous CCs may result in degraded in-gap and out-of-gap ACS performance compared to using two separate filters. However, this may depend on the separation between the two CCs and their bandwidth relative to the modulated interferer bandwidth, as well as where the LO frequency is placed. The out-of-gap selectivity and blocking performance would be similar to the non-CA requirements with a wanted signal bandwidth equal to the frequency span of the two non-contiguous carriers (we note that for the ACS minimum requirement, the margins are usually large for the non-CA case). BB selectivity is also affected and the performance depends on whether two separate digital BBs or a shared digital BB will be used after the ADC.

Figure 1. Partially shared Rx chain with two separate LPFs

Figure 2. Single Rx chain with shared LNA, Mixer and LPF.

The blocking performance is a trade-off between the level of analogue filter suppression and the ADC’s dynamic range. If the suppression level is lower, a higher ADC dynamic range will be necessary.

- The blocking performance is a trade-off between the filter suppression level and the dynamic range of the ADC.

It is important to note that for an implementation supporting non-contiguous CA, there will indeed be a selectivity and blocking performance difference between single and separate Rx chains. The key question is whether this necessitates modifications of the minimum requirements, given that the UE has tolerance and implementation margin.

- There will indeed be a selectivity and blocking performance difference between single and separate Rx chains. The key question is whether this necessitates modifications of the minimum requirements, given that the UE has tolerance and implementation margin.

Whether a single FFT is expected to cover the fragmented carriers or a separate FFT for each fragmented carrier is related to the BW and the separation between the two CCs. In the FDD case, if the separation between 2 CCs is not a multiple of 300 kHz, then a single FFT and different FFTs and digital BBs would be used anyway. In this case, the performance would not be degraded. Clarification is needed on the maximum separation between two CCs. A single FFT would cover a maximum 50 MHz frequency separation between the carriers.

- Regarding single or separate FFT, clarification is needed on the maximum separation between two CCs and the separation of the fragmented carriers.

The required number of Rx chains is not only dependent on the number of (possibly fragmented) carriers but also the number of MIMO layers supported per DL CC (optional for CA).

In practice, the problem in fragmented spectrum is the UE capability information and ensuring the total size of the message does not exceed the maximum size that the UE is allowed to use.

## Scenarios for fragmented spectrum

When studying fragmented spectrum scenarios, it is crucial to consider various deployment situation and their impact on RF requirements. In the inter-operator co-located scenario of the SID, co-location between wanted carriers is assumed, and the collaborating operator/s has full control over power levels and scheduling across all carrier components. The operators may or may not operate using a shared RAN and accept UE Rx degradation due to a non-collocated operator in the gap.

However, a relaxation to the requirements may not be acceptable for all non-contiguous intra-band carrier aggregation (CA) deployments when a non-collocated operator is present in the gap. Hence, supported band combinations with a reduced number of Rx chains and reduced selectivity performance would have to be advertised with modified UE band-combination (BC) capability. This will bring additional complexity to the UE capabilities. Unless this is restricted to complex BC, there would be nothing preventing a UE vendor from advertising BCs with reduced interference rejection capability even for band combinations where the number of Rx chains is not a problem, which implies degraded selectivity in general as support of any BC is always optional.

Reduced performance is normally not advertised as a “capability.” The UE can still implement collocated non-contiguous carriers with a single Rx chain as long as the existing requirements are met.

We observe that

- Supported band combinations with a reduced number of Rx chains and consequently reduced selectivity performance would have to be advertised with modified UE band-combination capabilities. This would increase the signalling size and add complexity to the UE capability in addition to the normally supported BC.

## Co-located conditions and PSD

The co-location definition refers to a scenario where two or more gNB are physically located at the same site or close to each other. The gNB is a logical node typically implemented within a BS deployed at a site [2]. This collocation deployment scenario should meet the conditions outlined in MRTD as specified in TS38.133 clause 7.6.4 and the power spectra density imbalance specified in TS38.101 [3][4].

Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation

| Frequency Range | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 31 |
| FR2-1 | 0.26 |
| Note 1: In the case of different SCS on different CCs, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. |  |

The current power imbalance requirement for the intra-band CA is shown below:

NOTE 6: For a UE not indicating intraBandNR-CA-non-collocated-r18, the minimum requirements for intra-band non-contiguous CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided and UE is configured with maxMIMO-Layers with value less than or equal to 2, the power imbalance requirement defined in subclause 7.10A apply. For a UE indicating intraBandNR-CA-non-collocated-r18 and when nonCollocatedTypeNR-CA-r18 is provided, the minimum requirements for intra-band non-contiguous CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable intra-band non-contiguous NR CA configuration is a subset of a higher order NR CA configuration.

It is important to note that the 6 dB PSD imbalance for intra-band collocated CA is only the minimum core requirement accessed during the conformance testing. However, in real-world deployment, a 6 dB power imbalance between two non-contiguous CCs cannot always be guaranteed in real deployment, it is not possible for the network(s) to maintain a 6 dB PSD imbalance in the field at all times.

- The collocation deployment scenario should meet the conditions outlined in MRTD as specified in TS38.133 clause 7.6.4 and the power spectra density imbalance specified in TS38.101.

There was some discussion regarding the PSD level between CCs and the blocker, where the blocker was assumed to be from a uncoordinated operator and non-collocated. The performance for a non-contiguous CA configuration should also be ensured with a non-collated blocker in the gap regardless of the receiver implementation.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	The blocking performance is a trade-off between the filter suppression level and the dynamic range of the ADC.

Observation 2	There will indeed be a selectivity and blocking performance difference between single and separate Rx chains. The key question is whether this necessitates modifications of the minimum requirements, given that the UE has tolerance and implementation margin.

Observation 3	Regarding single or separate FFT, clarification is needed on the maximum separation between two CCs and the separation of the fragmented carriers.

Observation 4	Supported band combinations with a reduced number of Rx chains and consequently reduced selectivity performance would have to be advertised with modified UE band-combination capabilities. This would increase the signalling size and add complexity to the UE capability in addition to the normally supported BC.

Observation 5	The collocation deployment scenario should meet the conditions outlined in MRTD as specified in TS38.133 clause 7.6.4 and the power spectra density imbalance specified in TS38.101.

# References

- RP-241669, Study on NR FR1 DL Fragmented Carriers

- R4-2414457, WF on fragmented DL carriers study

- RP-180557, LS response on work to support IMT-2020/5G in the Transport Network

- TS. 38.133 v18.5.0

- TS. 38.101-1 v18.5.0

- RP-240503, Fragmented carriers in the DL